THE SOCIALIST REPUBLIC OF VIETNAM

MINISTRY OF TRANSPORT

VIETNAM EXPRESSWAY CORPORATION

PROJECT MANAGEMENT UNIT 85

Consulting Service for Detailed Design for Danang Quang Ngai Expressway Development Project

IDA Credit No. 3843-VN

TECHNICAL REPORT

(Revision-1)

Alignment Alternative Study for Section between KM 70 and KM 80

September 2011

Joint Venture of

Nippon Koei Co., Ltd.

Nippon Engineering Consultants Co., Ltd.

Chodai Co., Ltd.

Thai Engineering Consultants Company Limited

Table of Contents

Page

- Answer to Comments from PMU85					1

- Comparison of Alignment Alternatives					2

- Alignment Alternatives Plan						6

- Profile Plan of Alignment Alternatives					7

- Conclusion								8

- Answer to Comments from PMU85

| Item No | Item Reference | Observations and Comments | Answers |
| --- | --- | --- | --- |
| 1 |  | Cost estimation for alternative 2&3 must be compared to identify the economic efficiency. | Rough construction cost estimate of Alternative 2, Alternative 3, and Alternative 4 are 19 million USD, 22.4 million USD, and 17.5 million USD respectively. |
| 2 |  | The difference of “high embankment and deep cut” between alternative 2 and 3. (Depth for cutting and elevation for embankment?) | Length of high embankment and deep cut of Alternative 2, Alternative 3, and Alternative 4 are as follows:. Alternative 2: Fill 5m<h<10m: 2.9km Cut 5m<h<10m: 1.05km, 10m<h<20m: 0.25km Alternative 3: Fill 5m<h<10m: 1.5km, 10m<h<20m: 0.8km Cut 5m<h<10m: 0.85km, 10m<h<20m: 0.45km, 20m<h<30m: 0.65km Alternative 4: Fill 5m<h<10m: 1.25km Cut 5m<h<10m: 1.05km, 10m<h<20m: 0.2km) |
| 3 |  | Reason of need of consensus of resident at km2+800 (as our understanding this section is belonging to realignment section). | Land acquisition is most crucial issue on smooth project implementation. Residential area at KM 2+800 on Alternative-2 is about 300m and to be seriously considered in the comparison. |
| 4 |  | “Water pollution during construction”- you are kindly requested to make this term clearly. As our understanding it is impossible to prevent pollution during construction for all alignments. Then, as for your study, how serious the pollution during construction is ? (in comparison between Alternative 2 &3) | In terms of water pollution magnitude during construction, Alternative 3 is higher than Alternative 2 and Alternative 4 due to longer passing length of hilly area which requires slope works. However, water pollution during construction on both Alternatives would be protected properly level by environmental mitigation measures. |
| 5 |  | Your proposal is Alternative 2. But according to us, Alternative 3 has more basic advantages than Alternative 2 such as: least impact to residents 3%/6%, farm land (very important with residents) 28%/58%, provide the development land for Chu Lai IZ 650/450ha and better landscape. | In terms of contribution for economic development, Alternative 3 is higher than Alternative 2. Our alternative ranking was given based on multi-criteria of economic development, technical advantages. We would like to finalize alternative selection after discussion with concerned agencies. |

2. Comparison of Alignment Alternatives

KM 71+200 - KM81+600

| Items |  | Alternative-1 | Alternative-2 | Alternative-3 | Alternative-4 |
| --- | --- | --- | --- | --- | --- |
|  |  | F/S Alignment | New Alignment |  |  |
| Length |  | Fair 10.302 km (1.00) | Good 9.992 km (0.969) | Good 9.977 km (0.968) | Good 9.948 km (0.965) |
| Land Use |  | Residential: 1.3km (13%) Farmland: 5.7km (55%) Hilly area: 3.3km (32%) | Residential: 0.6km (6%) Farmland: 5.8km (58%) Hilly area: 3.6km (36%) | Residential: 0.3km (3%) Farmland: 2.8km (28%) Hilly area: 6.9km (69%) | Residential: 0.3km (3%) Farmland: 5.5km (55%) Hilly area: 4.1km (42%) |
| Residential Area (Passing length) |  | Fair (1.3km) | Fair (0.6km) | Good (0.3km) | Good (0.3km) |
| Additional Industrial Zone |  | Fair (Restrict development area of Chu Lai development plan) | Good (Provide about 400ha potential development area for Chu Lai development plan) | Good (Provide about 650ha potential development area for Chu Lai development plan) | Good (Provide about 550ha potential development area for Chu Lai development plan) |
| Horizontal Alignment |  | Good (R≧4,900m) | Good (R≧5,000m) | Good (R≧5,000m) | Good (R≧5,000m) |
| Constructability |  | Fair | Good | Fair | Good |
| Required Structures | Bridge | Fair (Middle 1 bridge, Small 2 bridges) | Fair (Middle 1 bridge, Small 2 bridges) | Fair (Middle 1 bridge, Small 3 bridges) | Fair (Middle 1 bridge, Small 2 bridges) |
|  | Slope Protection | Good (Fill 5m<h<10m: 2.5km Cut 5m<h<10m: 0.5km 10m<h<20m: 0.2km) | Good (Fill 5m<h<10m: 2.9km Cut 5m<h<10m: 1.05km 10m<h<20m: 0.25km) | Fair (Fill 5m<h<10m: 1.5km 10m<h<20m: 0.8km Cut 5m<h<10m: 0.85km 10m<h<20m: 0.45km 20m<h<30m: 0.65km) | Good (Fill 5m<h<10m: 1.25km Cut 5m<h<10m: 1.05km 10m<h<20m: 0.2km) |
| Landscape |  | Fair | Fair | Good (High elevation) | Fair |
| Environment Impact |  | Fair (Cut: 175,000m3 Fill: 750,000m3 Hilly area: 3.3km ) | Fair (Cut: 700,000m3 Fill: 1,050,000m3 Hilly area: 3.6km) | Poor (Cut: 1,920,000m3 Fill: 1,160,000m3 Hilly area: 6.9km) | Fair (Cut: 630,000m3 Fill: 700,000m3 Hilly area: 4.1km) |
| Project Cost (Construction Cost Ratio, Land Acquisition length) |  | Fair (1.00, Residential: 1.3km) | Good (0.86, Residential: 0.6km) | Good (1.02, Residential: 0.3km) | Good (0.79, Residential: 0.3km) |
| Ranking |  | 4 | 2 | 3 | 1 |
| Remarks |  |  | Need consensus of residents at KM 2+800 |  |  |

  - Basis of Comparison Items

a) Length

CAD design data shown in “Alternative Alignment Plan”

b) Land Use

Scaled up on satellite photo of “Alternative Alignment Plan”

c) Residential Area

Scaled up on satellite photo of “Alternative Alignment Plan”

d) Additional Industrial Zone

Measured on “Alternative Alignment Plan”

e) Horizontal Alignment

Alternative-1: F/S Plan & Profile Drawing

Alternative-2, 3, 4: CAD design data shown in “Alternative Alignment Plan”

f) Constructability

Earth work balance:	 Alternative-1(575,000m3 Fill)

Alternative-2 (350,000m3 Fill)

Alternative-3 (760,000m3 Cut)

Alternative-4 (70,000m3 Fill)

g) Required Structure (Bridge)

Alternative-1: F/S Drawing

Alternative-2, 3, 4: Planned based on river information on 1:25,000 topographic map

h) Required Structure (Slope protection)

Alternative-1: F/S Plan & Profile Drawing

Alternative-2, 3, 4: CAD design shown in “Profile Plan of Alternative Alignment”

i) Landscape

Alternative-1: F/S Plan & Profile Drawing

Alternative-2, 3, 4: CAD design shown in “Profile Plan of Alternative Alignment”

j) Environmental Impact

Alternative-1: F/S Cross Section Drawing

Alternative-2, 3, 4: CAD design with 3-D terrain model obtained from Google Earth

k) Project Cost

Preliminary construction cost was estimated with referring to cost estimate by F/S review (WB):

(Unit: USD)

| No | Items | ALT-1 | ALT-2 | ALT-3 | ALT-4 |
| --- | --- | --- | --- | --- | --- |
| 1 | Earthwork | 2,570,500 | 4,490,500 | 7,208,400 | 3,313,800 |
| 2 | Soft-ground Treatment | 643,630 |  |  |  |
| 3 | Pavement | 10,878,388 | 10,551,044 | 10,535,205 | 10,504,582 |
| 4 | Drainage | 479,146 | 511,413 | 603,283 | 469,825 |
| 5 | Slope Protection | 549,608 | 586,620 | 692,001 | 538,917 |
| 6 | Miscellaneous Work | 2,226,618 | 2,376,564 | 2,803,490 | 2,183,304 |
| 7 | Culvert | 4,664,624 | 497,875 | 587,313 | 457,388 |
| Total |  | 22,012,514 | 19,014,016 | 22,429,691 | 17,467,817 |
| Cost Ratio |  | 1.00 | 0.86 | 1.02 | 0.79 |

3. Alignment Alternatives Plan

4. Profile Plan of Alignment Alternatives

Alternative-2                  Alternative-3		Alterantive-4
5. Conclusion

Alternative-4 is recommended based on alignment comparison and advantages of the alternative-4 are as follows:

- Wide area for additional industrial zone is available

- Less impact to residential area

- Slope protection works is not required

- Project cost is lowest