THE SOCIALIST REPUBLIC OF VIETNAM

MINISTRY OF TRANSPORT

VIETNAM EXPRESSWAY CORPORATION

PROJECT MANAGEMENT UNIT 85

Consulting Service for Detailed Design for Danang Quang Ngai Expressway Development Project

IDA Credit No. 3843-VN

TECHNICAL REPORT (1st Draft)

Finalization of Expressway Alignment

(Some Modifications from F/S Alignment)

October 2011

Joint Venture of

Nippon Koei Co., Ltd.

Nippon Engineering Consultants Co., Ltd.

Chodai Co., Ltd.

Thai Engineering Consultants Company Limited

Table of Contents

Page

1. Background	1

2. Overview of Major Alignment Modifications	1

2.1 Relation of Stationing between F/S and D/D	1

2.2 Summary of Alignment Modifications	1

3. Alignment Alternative Studies for Major Modifications	3

3.1 F/S: KM20+424 – KM30+681 (D/D: KM20+484 – KM30+796)	3

3.2 F/S: KM33+531 – KM36+630 (D/D: KM33+647 – KM36+756)	6

3.3 F/S: KM42+295 – KM47+246 (D/D: KM42+422 – KM47+369)	9

3.4 F/S: KM47+246 – KM52+126 (D/D: KM47+369 – KM52+252)	12

3.5 F/S: KM71+313 – KM81+569 (D/D: KM71+443 – KM81+384)	15

3.6 F/S: KM84+466 – KM91+701 (D/D: KM84+281 – KM91+537)	16

3.7 F/S: KM91+701 – KM95+949 (D/D: KM91+537 – KM95+753)	20

3.8 F/S: KM95+949 – KM106+827 (D/D: KM95+746 – KM106+509)	24

4. Summary of the Alignment Modifications	31

Appendix-1: Summary of Additional Topographic Survey (Draft)	32

1. Background

The topographic survey was commenced on 22nd August 2011 and planimetric survey for thruway has been carried out on the basis of the F/S alignment. Following to the planimetric survey, appropriateness of the F/S alignment has been verified with reference to design controls identified by site investigation.

As a result of verification of the F/S alignment, some sections are needed to modify its alignments to avoid the design control points.

This technical report is prepared to verify necessity of the alignment modification through alternative studies. Additional topographic survey arose from the alignment modification is summarized in Appendix-1.

2. Overview of Major Alignment Modifications

2.1 Relation of Stationing between F/S and D/D

Subjected major alignment modifications to alternative study are titled using stations at start and end points of the modified sections on the F/S center line and updated design center line, namely D/D centerline.

The F/S center line is stationing with four brake points as shown in table 2.1.1.

Table 2.1.1 Brake Points on F/S Center Line

| No. | station |  | Adjusted Length | Accumulated Adjusted Length |
| --- | --- | --- | --- | --- |
| 1 | back | KM019+260 | -60m | -60m |
|  | ahead | KM019+200 |  |  |
| 2 | back | KM073+057 | -57m | -117m |
|  | ahead | KM073+000 |  |  |
| 3 | back | KM096+700 | 300m | 183m |
|  | ahead | KM097+000 |  |  |
| 4 | back | KM102+295 | -173m | 10m |
|  | ahead | KM102+122 |  |  |

Therefore, apparent section length calculated from the F/S station is possibly not same with actual section length.

2.2 Summary of Alignment Modifications

As of 20 October 2011, alignments of following sections were modified from the F/S as shown in table 2.2.1:

Table 2.2.1 Summary of Alignment Modifications

| No. | Station |  |  |  | 1)Length in FS alignment (m) | Length in modified alignment (m) | Contents of Modification | Reason for Modification |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | F/S alignment |  | Modified alignment (D/D alignment) |  |  |  |  |  |
|  | BP | EP | BP | EP |  |  |  |  |
| 1 | KM020+424 | KM030+681 | KM020+484 | KM030+796 | 10,257 | 10,313 | Substitute R1200 for R1000 and shift align southward | Remediation of tunnel plan Avoid school and temple |
| 2 | KM033+531 | KM036+630 | KM033+647 | KM036+756 | 3,098 | 3,109 | Shift align eastward by R5000 curves | Avoid rocky mountain |
| 3 | KM042+295 | KM047+246 | KM042+422 | KM047+369 | 4,951 | 4,947 | Shift align westward by R80000 | Avoid bending river |
| 4 | KM047+246 | KM052+126 | KM047+369 | KM052+252 | 4,880 | 4,883 | Insert R5000 in tangent | Avoid shrine |
| 5 | KM071+313 | KM081+569 | KM071+443 | KM081+384 | 10,313 | 9,942 | Substitute spirals and curves for R5000 | Passing shooting area |
| 6 | KM084+466 | KM091+701 | KM084+281 | KM091+537 | 7,235 | 7,256 | Shift align westward by R34000 | Avoid war memorial |
| 7 | KM091+701 | KM095+949 | KM091+537 | KM095+753 | 4,248 | 4,216 | Change a series of curves | Avoid dam and HVL tower |
| 8 | KM095+949 | KM106+827 | KM095+746 | KM106+509 | 10,751 | 10,763 | Change a series of curves | Avoid HVL tower |
| Total |  |  |  |  | 55,733 | 55,428 |  |  |
| Note: 1) Length is adjusted to actual length with brake length |  |  |  |  |  |  |  |  |

3. Alignment Alternative Studies for Major Modifications

3.1 F/S: KM20+424 – KM30+681 (D/D: KM20+484 – KM30+796)

(1) General Conditions

This section starts from the end of the Chiem Son Bridge and bending to the left with radius of 1,000m. After the curve section, alignment is running through the mountain from km22+200 to km23+200 and passing through flat paddy field from km23+200 to km27+200. Then, mountain skirt section starts from km27+200 to km30+796 except flat section from km28+800 to km30+200.

Figure 3.1.1 Location Map of KM20+424 – KM30+681 (F/S)

(2) Design Controls and Alignment Modification Policy

Design controls were identified by the site investigation with topographic map of 1:1000 scale produced by the planimetric survey. Table 3.1.1 shows design controls and alignment modification policy.

Table 3.1.1 Design Controls and Alignment Modification Policy

| No | Section | Design Control |  | Alignment Modification Policy |
| --- | --- | --- | --- | --- |
| 1 | km20+600 – km21+800 | Small Radius (R=1,000) |  | Radius shall not be smaller than the F/S. Application of larger radius is desirable. |
| 2 | km22+600 – km23+200 | Portals of the Danang Tunnel | Terrain condition at portals of the Danang Tunnel was carefully investigated on the topographic map and at site. As a result of the investigation, the consultant concluded that location of the tunnel portal by the F/S is not appropriate due to possible imbalanced earth pressure. | Location of the portals shall be relocated to appropriate terrain. |
| 3 | km24+180 | Temple |  | Alignment needs to avoid the temple because that the temple is important for local residents. |
| 4 | km24+450 | School |  | Alignment needs to avoid the school. |

(3) Alignment Alternatives and Design Control Points

Since no space to examine additional alternative alignment between the F/S and a proposed alignment, one alternative alignment was proposed.

Figure 3.1.2 Alignment Alternative and Design Controls at KM20+424 – KM30+681 (F/S)

(4) Comparison of Alignment Alternatives

Table 3.1.2 Comparison of Alignment Alternatives of KM20+424 – KM30+681 (F/S)

| Items |  |  | Alternative-1 | Alternative-2 |
| --- | --- | --- | --- | --- |
|  |  |  | F/S Alignment | D/D Alignment |
| Length |  | Value | 10.257 km (1.00) | 10.313 km (1.005) |
|  |  | Evaluation | Good | Fair |
| Land Use (Passing length) |  | Value | Residential: 1.0km Farmland : 5.9km Hilly area: 3.3km | Residential: 0.8km Farmland : 6.2km Hilly area: 3.3km |
|  |  | Evaluation | Fair | Good |
| Horizontal Alignment (Minimum) |  | Value | R=1,000m | R=1,200m |
|  |  | Evaluation | Fair | Good |
| Constructability |  | Value | Skew at tunnel portal | Rectangle at tunnel portal |
|  |  | Evaluation | Poor | Good |
| Required Structures | Bridge | Value | Middle 3 bridges | Middle 3 bridges |
|  |  | Evaluation | Fair | Fair |
|  | Tunnel Length | Value | 540m | 2)540m |
|  |  | Evaluation | Fair | Fair |
| Natural Environment Impact |  | Value | Hilly area: 3.3km | Hilly area: 3.3km |
|  |  | Evaluation | Fair | Fair |
| 1)Construction Cost Ratio |  | Value | 1.00, Residential: 1.0km | 1.007, Residential: 0.8km |
|  |  | Evaluation | Fair | Fair |
| Ranking |  |  | 2 | 1 |

Note: 1) Fraction of cost per length (Residential / Farmland: 1.0, Hilly area: 1.5, Tunnel / Bridge: 5.0)

2) Tunnel length will be determined after profile survey.

(5) Conclusion and Remarks

Alternative-2 is recommended because:

- Large horizontal curve is applied to ensure driving safety (Design control No. 1)

- Skew at tunnel portals is avoided (Design control No. 2)

- Temple at km24+180 and school at 24+450 are not relocated (Design control No. 3 & No. 4)

Remarks:

- Maximum offset distance is 350m from the F/S alignment. Revision of EIA for the section more than 200m offset from the F/S which is 3,100m between km 22+802 and km 25+902 is required.

3.2 F/S: KM33+531 – KM36+630 (D/D: KM33+647 – KM36+756)

(1) General Conditions

This section is running parallel with the railway. The F/S alignment is passing through steep mountain section between km34+200 and km35+000. After crossing provincial road 611, terrain changes to flat paddy field.

Figure 3.2.1 Location Map of KM33+531 – KM36+630 (F/S)

(2) Design Controls and Alignment Modification Policy

Design controls were identified by the site investigation with topographic map of 1:1000 scale produced by the planimetric survey. Table 3.2.1 shows design controls and alignment modification policy.

Table 3.2.1 Design Controls and Alignment Modification Policy

| No | Section | Design Control |  | Alignment Modification Policy |
| --- | --- | --- | --- | --- |
| 1 | Km33+800 – km35+000 | Railway | According to “Laws of Railway, National Assembly, 27 June 2005 (No. 9/2005/L/CTN)”, necessary clearance from the foot of the embanked roadbed is 5m and 3m from the outer edge of the longitudinal drainage ditches respectively. | Alignment shall maintain at least 5m clearance from the foot of the embanked roadbed and 3m from the outer edge of the longitudinal drainage ditches respectively. |
| 2 | Km34+200 – km35+000 | Rocky mountain |  | Alignment shall be set to reduce rock excavation volume as much as possible with consideration of maintaining safety clearance of the railway and construction safety. |

(3) Alignment Alternatives and Design Control Points

Figure 3.2.2 Alignment Alternative and Design Controls at KM33+531 – KM36+630 (F/S)

(4) Comparison of Alignment Alternatives

Table 3.2.2 Comparison of Alignment Alternatives of KM33+531 – KM36+630 (F/S)

| Items |  |  | Alternative-1 | Alternative-2 |
| --- | --- | --- | --- | --- |
|  |  |  | F/S Alignment | D/D Alignment |
| Length |  | Value | 3.098 km (1.00) | 3.109 km (1.004) |
|  |  | Evaluation | Good | Fair |
| Land Use (Passing length) |  | Value | Residential: 0.0km Farmland : 1.7km Hilly area: 1.4km | Residential: 0.0km Farmland : 1.9km Hilly area: 1.2km |
|  |  | Evaluation | Fair | Fair |
| Horizontal Alignment (Minimum) |  | Value | R≧5,000 | R≧5,000 |
|  |  | Evaluation | Good | Good |
| Constructability |  | Value | Large Rock Excavation | Smaller Rock Excavation than Alt-1 |
|  |  | Evaluation | Poor | Fair |
| Required Structures | Bridge | Value | Middle 1 bridge | Middle 1 bridge |
|  |  | Evaluation | Fair | Fair |
|  | Slope Protection | Value | 860m (rocky section) | 630m (rocky section) |
|  |  | Evaluation | Poor | Fair |
| Natural Environment Impact |  | Value | Hilly area: 1.4km | Hilly area: 1.2km |
|  |  | Evaluation | Poor | Fair |
| 1)Construction Cost |  | Value | 1.00 | 0.97 |
|  |  | Evaluation | Fair | Good |
| Ranking |  |  | 2 | 1 |

Note: 1) Fraction of cost per length (Residential / Farmland: 1.0, Hilly area: 1.5, Tunnel / Bridge: 5.0)

(5) Conclusion and Remarks

Alternative-2 is recommended because:

- Less rock excavation volume (Design control No. 2)

Remarks:

- Alternative-2 alignment secures more than 5m clearance from the foot of the embanked roadbed and 3m from the outer edge of the longitudinal drainage ditches (Design control No. 1)

3.3 F/S: KM42+295 – KM47+246 (D/D: KM42+422 – KM47+369)

(1) General Conditions

This section is running on flat paddy field. Ngoc Kho River and Binh Chanh River are flowing on the section at km44+500 and km46+000 respectively.

Figure 3.3.1 Location Map of KM42+295 – KM47+246 (F/S)

(2) Design Controls and Alignment Modification Policy

Design controls were identified by the site investigation with topographic map of 1:1000 scale produced by the planimetric survey. Table 3.3.1 shows design controls and alignment modification policy.

Table 3.3.1 Design Controls and Alignment Modification Policy

| No | Section | Design Control |  | Alignment Modification Policy |
| --- | --- | --- | --- | --- |
| 1 | Km44+500 | Ngoc Kho River |  | Alignment should maintain adequate safety distance from the edge of the river. |
| 2 | Km46+000 | Binh Chanh River |  | Alignment should maintain adequate safety distance from the edge of the river. |

(3) Alignment Alternatives and Design Control Points

Figure 3.3.2 Alignment Alternative and Design Controls at KM42+295 – KM47+246 (F/S)

(4) Comparison of Alignment Alternatives

Table 3.3.2 Comparison of Alignment Alternatives of KM42+295 – KM47+246 (F/S)

| Items |  |  | Alternative-1 | Alternative-2 |
| --- | --- | --- | --- | --- |
|  |  |  | F/S Alignment | D/D Alignment |
| Length |  | Value | 4.951 km (1.00) | 4.947 km (0.999) |
|  |  | Evaluation | Good | Good |
| Land Use (Passing length) |  | Value | Residential: 0.5km Farmland : 4.4km Hilly area: 0.0km | Residential: 0.5km Farmland : 4.4km Hilly area: 0.0km |
|  |  | Evaluation | Fair | Fair |
| Horizontal Alignment (Minimum) |  | Value | R≧5,000m | R≧5,000m |
|  |  | Evaluation | Good | Good |
| Constructability |  | Value | River protection required | Not required |
|  |  | Evaluation | Fair | Good |
| Required Structures | Bridge | Value | Middle 3 bridges | Middle 2 bridges |
|  |  | Evaluation | Fair | Excellent |
|  | River Protection | Value | River protection required | Not required |
|  |  | Evaluation | Poor | Good |
| Natural Environment Impact |  | Value | Hilly area: 0.0km | Hilly area: 0.0km |
|  |  | Evaluation | Good | Good |
| 1)Construction Cost |  | Value | 1.00, Residential: 0.5km | 0.972, Residential: 0.5km |
|  |  | Evaluation | Fair | Excellent |
| Ranking |  |  | 2 | 1 |

Note: 1) Fraction of cost per length (Residential / Farmland: 1.0, Hilly area: 1.5, Tunnel / Bridge: 5.0)

(5) Conclusion and Remarks

Alternative-2 is recommended because:

- Reduce cost of one river protection at Ngoc Kho River and one bridge at Binh Chanh River (Design control No. 1 & No. 2)

Remarks:

- Nil

3.4 F/S: KM47+246 – KM52+126 (D/D: KM47+369 – KM52+252)

(1) General Conditions

This section is running on flat paddy field except hilly section between km49+600 and km51+000.

Figure 3.4.1 Location Map of KM47+246 – KM52+126 (F/S)

(2) Design Control and Alignment Modification Policy

Design control was identified by the site investigation with topographic map of 1:1000 scale produced by the planimetric survey. Table 3.4.1 shows design control and alignment modification policy.

Table 3.4.1 Design Control and Alignment Modification Policy

| No | Section | Design Control |  | Alignment Modification Policy |
| --- | --- | --- | --- | --- |
| 1 | Km51+100 | Shrine |  | Alignment needs to avoid the shrine because that the shrine is important for local residents. |

(3) Alignment Alternatives and Design Control Points

Figure 3.4.2 Alignment Alternative and Design Control at KM42+295 – KM47+246 (F/S)

(4) Comparison of Alignment Alternatives

After modifications, there is no change in the alignment.

(5) Conclusion and Remarks

Alternative-2 is recommended because:

- Shrine at km51+100 is not relocated (Design control No. 1)

Remarks:

- Nil.

3.5 F/S: KM71+313 – KM81+569 (D/D: KM71+443 – KM81+384)

Alternative alignment study for this section was carried out and the study report was submitted to PMU85 by our letter No. DQEDD-PMU85-15-11 dated 6th September 2011 enclosed the TECHNICAL REPORT (Revision-1), Alignment Alternative Study for Section between KM 70 and KM 80. The proposed modified alignment in technical report was approved by PMU85 at the meeting held on Quang Nam People’s Committee on 07 September 2011.

3.6 F/S: KM84+466 – KM91+701 (D/D: KM84+281 – KM91+537)

(1) General Conditions

This is section with its many uphills and downhills. The alignment starts from continuous mountain skirt of Bung Bang Dinh Mountain by Chau Lake at km85+400. The alignment is running hilly and flat terrain alternately from the Chau Lake to km91+537. The alignment is intersecting with 500kv electric line at km91+340.

Figure 3.6.1 Location Map of KM84+281 – KM91+537 (F/S)

(2) Design Controls and Alignment Modification Policy

Design controls were identified by the site investigation with topographic map of 1:1000 scale produced by the planimetric survey. Table 3.6.1 shows design controls and alignment modification policy.

Table 3.6.1 Design Controls and Alignment Modification Policy

| No | Section | Design Control |  | Alignment Modification Policy |
| --- | --- | --- | --- | --- |
| 1 | Km87+280 | War memorial |  | Alignment needs to avoid the war memorial because that the war memorial is very important monument. |
| 2 | Km91+340 | HVL 500kv |  | Alignment shall maintain 1)18m clearance from the lowest point of HVL in accordance with Decree No. 81/2009/ND-CP. |
| 3 | Km91+400 | HVL 500kv tower | Tower is located at 25m east of the alignment. | Alignment shall maintain adequate clearance from tower of HVL to protect foundation of the tower. |

Note: 1) Since vertical clearance for 500kv is not stipulated in Decree No. 81/2009/ND-CP, value for 220kv is temporarily applied. Further consideration for the vertical clearance is necessary because that the value is excessive in comparison with the value in Japan.

(3) Comparison of Relative Position with High Voltage Line on Alignment Alternatives

The alignment should be secured necessary clearance from tower and cable of high voltage lines (HVL). Table 3.6.2 shows relative position between HVL and alternative alignments and necessary measures.

Table 3.6.2 Comparison of Relative Position with HVL on Alignment Alternatives

| No | Classification | Alternative-1 (F/S) |  |  | Alternative-2 (D/D Alignment) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Station | Relative Position | Counter- measures | Station | Relative Position | Counter- measures |
| 2 | HVL 500kv | Km91+340 | - | - | Km91+69 | Clearance height of the lowest point from ground is 18.48m- | Maintain more than 18 m clearance |
| 3 | HVL 500kv tower | Km91+400 | Locating at 25m east of the alignment. | Relocation (Realign) | Km91+230 | Locating at 50m east of the alignment. | Not required |
| 1)Quantity of Required Relocation |  | - |  |  | - |  |  |
| 2)Cost of required Relocation (USD) |  | - |  |  | - |  |  |

Note: 1) Tower of 500kv line is not included because that interruption of power transmission by 500kv is impossible.

2) Installation of 110kv tower ((Construction & Material: 150,000USD + Design: 3,500USD) per a tower), Removal of 110kv tower (50,000USD per a tower)

(4) Alignment Alternatives and Design Control Points

Figure 3.6.2 Alignment Alternative and Design Controls at KM84+281 – KM91+537 (F/S)

(5) Comparison of Alignment Alternatives

Table 3.6.3 Comparison of Alignment Alternatives of KM84+466 – KM91+701 (F/S)

| Items |  | Alternative-1 | Alternative-2 |
| --- | --- | --- | --- |
|  |  | F/S Alignment | D/D Alignment |
| Length | Value | 7.235 km (1.00) | 7.256 km (1.003) |
|  | Evaluation | Good | Good |
| Land Use (Passing length) | Value | Residential: 1.8km Farmland : 3.5km Hilly area: 1.9km | Residential: 1.4km Farmland : 3.6km Hilly area: 2.2km |
|  | Evaluation | Fair | Good |
| Horizontal Alignment (Minimum) | Value | R≧5,000m | R≧5,000m |
|  | Evaluation | Good | Good |
| Constructability | Value | Difficult construction work is not required | Difficult construction work is not required |
|  | Evaluation | Good | Good |
| Natural Environment Impact | Value | Hilly area: 1.9km | Hilly area: 2.2km |
|  | Evaluation | Good | Fair |
| 1)Construction Cost | Value | 1.00, Residential: 1.8km | 1.018, Residential: 1.4km |
|  | Evaluation | Good | Fair |
| Ranking |  | 2 | 1 |

Note: 1) Fraction of cost per length (Residential / Farmland: 1.0, Hilly area: 1.5, Tunnel / Bridge: 5.0)

(6) Conclusion and Remarks

Alternative-2 is recommended because:

- War memorial at km87+280 is not affected (Design control No. 1)

- Alternative-2 alignment secures adequate safety clearance from tower of 500kv HVL (Design control No. 3)

Remarks:

- Nil

3.7 F/S: KM91+701 – KM95+949 (D/D: KM91+537 – KM95+753)

(1) General Conditions

This section is running on mountain foot of Hang Ba Mountain entirely. The alignment was set to run over base of Ho May dam at km94+300. The alignment is intersecting with high voltage lines at km92+740 (500kv), km94+240 (220kv), and 94+300 (110kv), respectively.

Figure 3.7.1 Location Map of KM91+701 – KM95+949 (F/S)

(2) Design Controls and Alignment Modification Policy

Design controls were identified by the site investigation with topographic map of 1:1000 scale produced by the planimetric survey. Table 3.7.1 shows design controls and alignment modification policy.

Table 3.7.1 Design Controls and Alignment Modification Policy

| No | Section | Design Control |  | Alignment Modification Policy |
| --- | --- | --- | --- | --- |
| 1 | Km94+000 – Km94+600 | Ho May Dam | Ho May dam is classified as Grade III in TCXDVN 285-2002 according to DARD, Quang Nam province. | In accordance with Decree 27-2007 ND-CP, necessary clearance for protection of the Grade III dam is minimum 100m from the base of dam. |
| 2 | Km92+650 | HVL 500kv tower | Tower is located at 35m east of the alignment. | Alignment shall maintain adequate clearance from tower of HVL to protect foundation of the tower. |
| 3 | Km92+740 | HVL 500kv |  | Alignment shall maintain 1)18m clearance from the lowest point of HVL in accordance with Decree No. 81/2009/ND-CP. |
| 4 | Km94+240 | HVL 220kv |  | Alignment shall maintain 18m clearance from the lowest point of HVL in accordance with Decree No. 81/2009/ND-CP. |
| 5 | Km94+300 | HVL 110kv |  | Alignment shall maintain adequate clearance from tower of HVL to protect foundation of the tower. |

Note: 1) Since vertical clearance for 500kv is not stipulated in Decree No. 81/2009/ND-CP, value for 220kv is temporarily applied. Further consideration for the vertical clearance is necessary because that the value is excessive in comparison with the value in Japan.

(3) Comparison of Relative Position with High Voltage Line on Alignment Alternatives

The alignment should be secured necessary clearance from tower and cable of high voltage lines (HVL). Table 3.7.2 shows relative position between HVL and alternative alignments and necessary measures.

Table 3.7.2 Comparison of Relative Position with HVL on Alignment Alternatives

| No | Classification | Alternative-1 (F/S) |  |  | Alternative-2 (D/D Alignment) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Station | Relative Position | Counter- measures | Station | Relative Position | Counter- measures |
| 2 | HVL 500kv tower | Km92+650 | Tower is located at 35m east of the alignment. | Not required | Km92+480 | Locating at 90m west of the alignment. | Not required |
| 3 | HVL 500kv | Km92+740 | - | - | Km91+314 | Clearance height of the lowest point from ground is 17.19m- | Maintain more than 18 m clearance |
| 4 | HVL 220kv | Km94+240 | - | - | Km93+868 | Clearance height of the lowest point from ground is 29.62m- | Maintain more than 18 m clearance |
| 5 | HVL 110kv | Km94+300 | - | - | Km93+929 | Clearance height of the lowest point from ground is 35.30m- | Maintain more than 15 m clearance |
| 1)Quantity of Required Relocation |  | 110kv: 0 220kv: 0 |  |  | 110kv: 0 220kv: 0 |  |  |
| 2)Cost of required Relocation (USD) |  | 110kv: 0 220kv: 0 |  |  | 110kv: 0 220kv: 0 |  |  |

Note: 1) Tower of 500kv line is not included because that interruption of power transmission by 500kv is impossible.

2) Installation of 110kv tower ((Construction & Material: 150,000USD + Design: 3,500USD) per a tower), Removal of 110kv tower (50,000USD per a tower)

(4) Alignment Alternatives and Design Control Points

Figure 3.7.2 Alignment Alternative and Design Controls at KM91+701 – KM95+949 (F/S)

(5) Comparison of Alignment Alternatives

Table 3.7.3 Comparison of Alignment Alternatives of KM91+701 – KM95+949 (F/S)

| Items |  | Alternative-1 | Alternative-2 |
| --- | --- | --- | --- |
|  |  | F/S Alignment | D/D Alignment |
| Length | Value | 4.248 km (1.00) | 4.216 km (0.992) |
|  | Evaluation | Fair | Good |
| Land Use (Passing length) | Value | Residential: 0.2km Farmland :1.8km Hilly area: 2.2km | Residential: 0.0km Farmland : 1.4km Hilly area: 2.8km |
|  | Evaluation | Fair | Good |
| Horizontal Alignment (Minimum) | Value | R≧5,000m | R≧5,000m |
|  | Evaluation | Good | Good |
| Constructability | Value | Difficult construction work is not required | Longer hilly section than alternative-1 |
|  | Evaluation | Good | Fair |
| Natural Environment Impact | Value | Hilly area: 2.2km | Hilly area: 2.8km |
|  | Evaluation | Good | Fair |
| 1)Construction Cost | Value | 1.00, Residential: 0.2km | 1.057, Residential: 0.0km |
|  | Evaluation | Good | Fair |
| Ranking |  | 2 | 1 |

Note: 1) Fraction of cost per length (Residential / Farmland: 1.0, Hilly area: 1.5, Tunnel / Bridge: 5.0)

(6) Conclusion and Remarks

Alternative-2 is recommended because:

- Alternative-2 alignment secures more than 100m clearance from the base of Ho May dam. (Design control No. 1)

- Alternative-2 alignment secures adequate clearance from tower of 500kv HVL (Design control No. 2)

Remarks:

- Maximum offset length is 230m from the F/S alignment. Revision of EIA for the section more than 200m offset from the F/S which is 810 m between km 93+030 and km 93+840 is required.

3.8 F/S: KM95+949 – KM106+827 (D/D: KM95+746 – KM106+509)

(1) General Conditions

This section is running mountain skirt of Ga Cao and Son Tra Mountains from km96+400 to km101+000 and passing beside of Ham Rong dam at km100+200. Railway is running parallel with the alignment from km98+000 to km103+000. Flat terrain is start from km101+000 to end point except hilly section from km103+200 to km104+400. Several high voltage lines are intersecting with the alignment.

Figure 3.8.1 Location Map of KM95+949 – KM106+827 (F/S)

(2) Design Controls and Alignment Modification Policy

Design controls were identified by the site investigation with topographic map of 1:1000 scale produced by the planimetric survey. Table 3.8.1 shows design controls and alignment modification policy.

Table 3.8.1 Design Controls and Alignment Modification Policy

| No | Section | Design Control |  | Alignment Modification Policy |
| --- | --- | --- | --- | --- |
| 1 | Km100+000 – Km100+400 | Ham Rong Dam | Ham Rong dam is classified as Grade IV in TCXDVN 285-2002 according to DARD, Quang Ngai province. | In accordance with Decree 27-2007 ND-CP, necessary clearance for protection of the Grade IV dam is minimum 50m from the base of dam. |
| 2 | Km99+230 | HVL 110kv tower | Towers are located at 20m east of the alignment respectively. | Alignment shall maintain adequate clearance from tower of HVL to protect foundation of the tower. |
| 3 | Km99+340 | HVL 110kv |  | Alignment shall maintain 15m clearance from the lowest point of HVL in accordance with Decree No. 81/2009/ND-CP. |
| 4 | Km99+350 | HVL 110kv tower | Towers are located at 3m west of the alignment. | Alignment shall maintain adequate clearance from tower of HVL to protect foundation of the tower. |
| 5 | Km99+520 | HVL 110kv tower | Towers are located at 25m west of the alignment. | Alignment shall maintain adequate clearance from tower of HVL to protect foundation of the tower. |
| 6 | Km100+370 | HVL 110kv tower | Towers are located at 27m west of the alignment. | Alignment shall maintain adequate clearance from tower of HVL to protect foundation of the tower. |
| 7 | Km100+565 | HVL 110kv tower | Towers are located at 1m west of the alignment. | Alignment shall maintain adequate clearance from tower of HVL to protect foundation of the tower. |
| 8 | Km100+580 | HVL 110kv |  | Alignment shall maintain 15m clearance from the lowest point of HVL in accordance with Decree No. 81/2009/ND-CP. |
| 9 | Km100+720 | HVL 110kv tower | Towers are located at 25m east of the alignment. | Alignment shall maintain adequate clearance from tower of HVL to protect foundation of the tower. |
| 10 | Km101+380 | HVL 110kv tower | Tower is located at 20m east of the alignment. | Alignment shall maintain adequate clearance from tower of HVL to protect foundation of the tower. |
| 11 | Km101+520 | HVL 110kv tower | Tower is located at 15m west of the alignment. | Alignment shall maintain adequate clearance from tower of HVL to protect foundation of the tower. |
| 12 | Km101+530 | HVL 110kv |  | Alignment shall maintain 15m clearance from the lowest point of HVL in accordance with Decree No. 81/2009/ND-CP. |
| 13 | Km103+440 | HVL 110kv |  | Alignment shall maintain 15m clearance from the lowest point of HVL in accordance with Decree No. 81/2009/ND-CP. |
| 14 | Km103+595 | HVL 500kv tower | Tower is located at 30m west of the alignment. | Alignment shall maintain adequate clearance from tower of HVL to protect foundation of the tower. |
| 15 | Km103+600 | HVL 500kv |  | Alignment shall maintain 1)18m clearance from the lowest point of HVL in accordance with Decree No. 81/2009/ND-CP. |
| 16 | Km103+635 | HVL 500kv tower | Tower is located at 5m east of the alignment. | Alignment shall maintain adequate clearance from tower of HVL to protect foundation of the tower. |
| 17 | Km103+640 | HVL 500kv |  | Alignment shall maintain 1)18m clearance from the lowest point of HVL in accordance with Decree No. 81/2009/ND-CP. |
| 18 | Km104+815 | HVL 110kv |  | Alignment shall maintain 15m clearance from the lowest point of HVL in accordance with Decree No. 81/2009/ND-CP. |

Note: 1) Since vertical clearance for 500kv is not stipulated in Decree No. 81/2009/ND-CP, value for 220kv is temporarily applied. Further consideration for the vertical clearance is necessary because that the value is excessive in comparison with the value in Japan.

(3) Comparison of Relative Position with High Voltage Line on Alignment Alternatives

The alignment should be secured necessary clearance from tower and cable of high voltage lines (HVL). Table 3.8.2 shows relative position between HVL and alternative alignments and necessary measures.

Table 3.8.2 Comparison of Relative Position with HVL on Alignment Alternatives

| No | Classification | Alternative-1 (F/S) |  |  | Alternative-2 (D/D Alignment) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Station | Relative Position | Counter- measures | Station | Relative Position | Counter- measures |
| 2 | HVL 110kv tower | Km99+230 | Towers are located at 20m east of the alignment respectively. | Relocation | Km98+730 | Towers are located at 20m east of the alignment respectively. | Relocation |
| 3 | HVL 110kv | Km99+340 | - | - | Km98+835 | 1)Clearance height of the lowest point from ground is **m- | Maintain more than 15 m clearance |
| 4 | HVL 110kv tower | Km99+350 | Towers are located at 3m west of the alignment. | Relocation | Km98+845 | Towers are located at 3m west of the alignment. | Relocation |
| 5 | HVL 110kv tower | Km99+520 | Towers are located at 25m west of the alignment. | Relocation | Km99+010 | Towers are located at 25m west of the alignment. | Relocation |
| 6 | HVL 110kv tower | Km100+370 | Towers are located at 27m west of the alignment. | Relocation | Km99+865 | Towers are located at 27m west of the alignment. | Relocation |
| 7 | HVL 110kv tower | Km100+565 | Towers are located at 1m west of the alignment. | Relocation | Km100+060 | Towers are located at 1m west of the alignment. | Relocation |
| 8 | HVL 110kv | Km100+580 | - | - | Km100+062 | 1)Clearance height of the lowest point from ground is **m- | Maintain more than 15 m clearance |
| 9 | HVL 110kv tower | Km100+720 | Towers are located at 25m east of the alignment. | Relocation | Km100+220 | Towers are located at 30m east of the alignment. | Not required |
| 10 | HVL 110kv tower | Km101+380 | Tower is located at 20m east of the alignment. | Relocation | Km100+880 | Tower is located at 60m east of the alignment. | Not required |
| 11 | HVL 110kv tower | Km101+520 | Tower is located at 15m west of the alignment. | Relocation | Km101+025 | Tower is located at 35m east of the alignment. | Not required |
| 12 | HVL 110kv | Km101+530 | - | - | Km101+005 | 1)Clearance height of the lowest point from ground is **m- | Maintain more than 15 m clearance |
| 13 | HVL 110kv | Km103+440 | - | - | Km103+095 | Clearance height of the lowest point from ground is 19.15m- | Maintain more than 15 m clearance |
| 14 | HVL 500kv tower | Km103+595 | Tower is located at 30m west of the alignment. | Relocation (Realign) | Km103+245 | Tower is located at 120m east of the alignment. | Not required |
| 15 | HVL 500kv | Km103+600 | - | - | Km103+250 | Clearance height of the lowest point from ground is 27.5m- | Maintain more than 18 m clearance |
| 16 | HVL 500kv tower | Km103+635 | Tower is located at 5m east of the alignment. | Relocation (Realign) | Km103+295 | Tower is located at 170m east of the alignment. | Not required |
| 17 | HVL 500kv | Km103+640 | - | - | Km103+300 | Clearance height of the lowest point from ground is 29.75m- | Maintain more than 18 m clearance |
| 18 | HVL 110kv | Km104+815 | - | - | Km104+720 | Clearance height of the lowest point from ground is 15.33m- | Maintain more than 15 m clearance |
| 2)Quantity of Required Relocation |  | 110kv: 8 |  |  | 110kv: 5 |  |  |
| 3)Cost of required Relocation (USD) |  | 110kv: 1,628,000USD |  |  | 110kv: 1,017,500USD |  |  |

Note: 1) Clearance height is under measuring.

2) Tower of 500kv line is not included because that interruption of power transmission by 500kv is impossible.

3) Installation of 110kv tower ((Construction & Material: 150,000USD + Design: 3,500USD) per a tower), Removal of 110kv tower (50,000USD per a tower)

(4) Alignment Alternatives and Design Control Points

Figure 3.8.2 Alignment Alternative and Design Controls at KM95+949 – KM106+827 (F/S)

Figure 3.8.3 Design Controls at KM95+949 – KM106+827 (F/S)

(5) Comparison of Alignment Alternatives

Table 3.8.3 Comparison of Alignment Alternatives of KM95+949 – KM106+827 (F/S)

| Items |  |  | Alternative-1 | Alternative-2 |
| --- | --- | --- | --- | --- |
|  |  |  | F/S Alignment | D/D Alignment |
| Length |  | Value | 10.751 km (1.00) | 10.763 km (1.001) |
|  |  | Evaluation | Good | Good |
| Land Use (Passing length) |  | Value | Residential: 1.1km Farmland : 4.1km Hilly area: 5.5km | Residential: 1.1km Farmland : 4.3km Hilly area: 5.3km |
|  |  | Evaluation | Fair | Fair |
| Horizontal Alignment (Minimum) |  | Value | R≧5,000m | R≧5,000m |
|  |  | Evaluation | Good | Good |
| Constructability |  | Value | Longer hilly section than alternative-2 | Difficult construction work is not required |
|  |  | Evaluation | Fair | Good |
| Required Structures | Relocation of HVL pole | Value | 110kv×8 nos. | 110kv×5 nos. |
|  |  | Evaluation | Fair | Good |
| Natural Environment Impact |  | Value | Hilly area: 5.5km | Hilly area: 5.3km |
|  |  | Evaluation | Fair | Fair |
| 1)Construction Cost |  | Value | 1.00, Residential: 1.1km | 0.98, Residential: 1.1km |
|  |  | Evaluation | Fair | Good |
| Ranking |  |  | 2 | 1 |

Note: 1) Fraction of cost per length (Residential / Farmland: 1.0, Hilly area: 1.5, Tunnel / Bridge: 5.0)

(6) Conclusion and Remarks

Alternative-2 is recommended because:

- Alternative-2 alignment secures more than 50m clearance from the base of Ham Rong dam. (Design control No. 1)

- Alternative-2 alignment reduces relocation of tower of 110kv HVL at km100+720, km101+380 and km101+520, respectively. (Design control No. 9 & No.10 & No.11)

- Alternative-2 alignment secures adequate clearance from tower of 550kv HVL at km103+595 and km103+635, respectively. (Design control No.14 & No.16)

Remarks:

- Nil

4. Summary of the Alignment Modifications

Eight major alignment modifications were verified through alternative alignment studies in foregoing chapter and conclusion of the alignment modifications are summarized in Table 4.1.

Table 4.1 Conclusion of Alignment Modifications

| No. | Station |  |  |  | 1)Length in FS alignment (m) | Length in modified alignment (m) | Conclusion of Alternative Study |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | F/S alignment |  | Modified alignment (D/D alignment) |  |  |  |  |
|  | BP | EP | BP | EP |  |  |  |
| 1 | KM020+424 | KM030+681 | KM020+484 | KM030+796 | 10,257 | 10,313 | - Alternative-2 (D/D alignment) is recommended - Revision of EIA is necessary |
| 2 | KM033+531 | KM036+630 | KM033+647 | KM036+756 | 3,098 | 3,109 | - Alternative-2 (D/D alignment) is recommended |
| 3 | KM042+295 | KM047+246 | KM042+422 | KM047+369 | 4,951 | 4,947 | - Alternative-2 (D/D alignment) is recommended |
| 4 | KM047+246 | KM052+126 | KM047+369 | KM052+252 | 4,880 | 4,883 | - Alternative-2 (D/D alignment) is recommended |
| 5 | KM071+313 | KM081+569 | KM071+443 | KM081+384 | 10,313 | 9,942 | - Alternative-2 (D/D alignment) is recommended |
| 6 | KM084+466 | KM091+701 | KM084+281 | KM091+537 | 7,235 | 7,256 | - Alternative-2 (D/D alignment) is recommended |
| 7 | KM091+701 | KM095+949 | KM091+537 | KM095+753 | 4,248 | 4,216 | - Alternative-2 (D/D alignment) is recommended - Revision of EIA is necessary |
| 8 | KM095+949 | KM106+827 | KM095+746 | KM106+509 | 10,751 | 10,763 | - Alternative-2 (D/D alignment) is recommended |
| Total |  |  |  |  | 55,733 | 55,428 |  |
| Balance (F/S – D/D) |  |  |  |  | 305 |  |  |
| Note: 1) Length is adjusted to actual length with brake length |  |  |  |  |  |  |  |

The Consultant will proceed the basic design, as well as the detailed design on the basis of the modified alignment (D/D alignment).

Appendix-1: Summary of Additional Topographic Survey (Draft)

As a result of the alignment modification, additional topographic survey was carried out and area of the additional topographic survey is summarized in Table A.1.1.

Table A.1.1 Summary of Additional Topographic Survey (Draft)

| Date | Section | Additional Area |
| --- | --- | --- |
| 2011/10/11 | KM22 | 4.86ha |
| 2011/10/6 | KM23-27 | 63.72ha |
| 2011/8/15 | KM34-35 | 8.00ha |
| 2011/9/12 | KM35-36 | 6.96ha |
| 2011/9/29 | KM40-47 | 13.50ha |
| 2011/10/4 | KM47-53 | 5.35ha |
| 2011/9/8 | KM71-82 | -5.20ha |
| 2011/9/26 | KM84-95 | 73.48ha |
| 2011/10/12 | KM95-107 | 57.17ha |
| Total Additional Area |  | 227.84ha |