THE SOCIALIST REPUBLIC OF VIETNAM

MINISTRY OF TRANSPORT

VIETNAM EXPRESSWAY CORPORATION

PROJECT MANAGEMENT UNIT 85

Consulting Service for Detailed Design for Danang Quang Ngai Expressway Development Project

IDA Credit No. 3843-VN

TECHNICAL REPORT

Alternative Interchanges Study at Provincial Road 609 and Provincial Road 610

October 2011

Joint Venture of

Nippon Koei Co., Ltd.

Nippon Engineering Consultants Co., Ltd.

Chodai Co., Ltd.

Thai Engineering Consultants Company Limited

Table of Contents

Page

- Background								1

- Basic Planning Conditions of Interchange on Expressway			1

- Current Conditions of Alternative Interchanges				2

- Comparison of Alternative Interchanges					3

- Conclusion								5

1. Background

Based on the Circular no 346/TB0UBND date of 13 September 2011 of Quang Nam province, letter No. 1855/VEC-KTCNMT dated 30 September 2011 of VEC, and letter No. 50/BQL-DNQN dated 4 October 2011 of PMU85, the consultants was requested to study additional interchange connecting to provincial road 609 and 620.

In this technical report, the interchange connecting to provincial road 609 is studied as alternative interchange of the My Son interchange located at km 20+200 crossing to the provincial road 610.

Figure 1-1 Location of Interchanges

2. Basic Planning Conditions of Interchange on Expressway

The interchange location shall be planned to conform following practical conditions:

- Crossing to National Highway or near it.

- Near the City/Town (Population over 30,000)

- An intersection position or a proximity point with the major Highway which leads to the important area, the important Port, the Airfield, the Track Terminal and the International Tourism area.

- The passing number of the motor vehicles are over 30,000/day.

- The interval of the interchanges, minimum is 4 km and maximum is 30 km.

- Typical condition of the intervals for the Interchanges as follow.

  - Near Capital, Big City or Major Industrial area : 5Km ~ 10Km

  - The plain area dotted with a small city         :15Km ~ 25Km

  - The district villages and the mountain area     :20Km ~ 30Km

3. Current Conditions of Alternative Interchanges

Figure 3-1 shows commune populations, planned industrial parks, tourist places, major highways, and impacts area of interchanges (10km radius). Almost one third of the impact area of the My Son interchange is hilly and water reservoir area.

Figure 3-1 Current Conditions of Alternative Interchanges

4. Comparison of Alternative Interchanges

Table 4-1 Comparison of Alternative Interchanges

| Items |  | Provincial Road 609 |  | Provincial Road 610 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Alternative Plan |  | F/S Plan |  |
| Location |  | KM13+600 |  | KM20+200 |  |
| Intersecting Road |  | PR609 |  | PR610 |  |
| Regional Population (Main District) |  | Good Dien Ban District 198,822 person (2009) 926 person/km2 (2009) |  | Fair Duy Xuyen District 120,948 person (2009) 404 person/km2 (2009) |  |
| Population Growth (Main District) |  | Good 104.27 % (2009/2005) |  | Fair 99.11 % (2009/2005) |  |
| Major Access (Existing & Plan) | District Center | Good Dien Ban District (9km) Dai Loc District (6km) |  | Fair Duy Xuyen Distric (7.5km) Dien Ban District (20km) |  |
|  | Transport Facilities | Good NH1 (9 km) NH14B (7.5km) |  | Fair NH1 (11 km) |  |
|  | Industrial | Good Hoa Khuong IZ (400ha, 5km) Hoa Minh IZ (200ha, 5 km) |  | Fair Dien Nam-Dien Ngoc IZ (418ha, 15 km) |  |
|  | Tourist Center | Fair Hoi An (22km) My Son (42km) |  | Fair My Son (15km) Hoi An (30km) |  |
| IC Distance |  | Fair |  | Good |  |
|  |  | 20.2 km |  |  | 20.9 km |
|  |  | 13.6km | 27.5 km |  |  |
| Forecasted Traffic Volume around ICs |  | Approx. 12,000 pcu/day (2015) – 40,000 pcu/day (2025) |  |  |  |
| Land Use / Land Use Plan |  | Good Residential / Farmland: 95% Hilly area / River: 5% |  | Poor Residential / Farmland: 65% Hilly area / River / Dam: 35% |  |
| IC Type |  | Trumpet |  | Trumpet |  |
| Thruway Horizontal Alignment |  | Good R=4,000m |  | Poor R=1,200m (Operation speed shall be less than 80km/h) |  |
| Traffic Regulation (Possible Operation Speed) |  | Good 120 km/h |  | Poor 80 km/h (Speed limit regulation is necessary) |  |
| Constructability |  | Good No deep cut and high embankment |  | Good No deep cut and high embankment |  |
| Social Environment Impact |  | Fair Little resettlement |  | Good Little or nothing resettlement |  |
| Construction Cost |  | Almost same project cost (3.5mil USD) |  |  |  |
| Ranking |  | 1 |  | 2 |  |

5. Conclusion and Remarks

5.1 Conclusion

Interchange at provincial road 609 is recommended based on alternative comparison and advantages of the interchange at provincial road 609 are as follows:

- Locating at potential area of population growth

- Good access to district center, major highways, industrial parks

- Wider potential development area than the My Son IC

- Smooth and safety thruway geometry

5.2 Remarks

- Revision of the ROW is necessary