Appendix 1: Major Features and Facilities of Expressway (by Package)

Package 1: KM000+000 - KM008+000, 8.000km

| No. | Items | Main Features |
| --- | --- | --- |
| 01 | Road Length | Danang city: 8.0km Beginning Point: Km0+000, Hoa Nhon Commune, Hoa Vang District, Da Nang City. Ending Point: Km8+000, Dien Tien Commune, Dien Ban District, Quang Nam Province. |
| 02 | Road Classification | Expressway: Type A, Class 120 |
| 03 | Design Speed | Expressway: 120 km/hr |
| 04 | Nos. of Lane | 4 lanes (Ultimate stage: Widened to 6 lanes) |
| 05 | Road Width | Expressway: Road 25.5m, Large River Bridge: 26.0m, Other Bridges: 25.5m, Tunnel section: 2@13.05m=26.1m, Linking Road: 12.0m |
| 06 | Design Hydrological Frequency | Expressway: 1 %, Linking Road: 4 % Pavement Drainage: 4% Bridge Deck Drainage: 4% |
| 07 | Other Bridge | 8 bridges (L=803.0m) Large River Bridge (L>100m): 2 bridges (L=445.6m) Other River Bridge (L<100m): 2 bridges (L=120.9m) Overpass: 3 bridges (L=165.9m) Cross Road Flyover: 1 bridge (L=70.6m) |
| 08 | Major River Bridge | None |
| 09 | Tunnel | None |
| 10 | Underpass | 8 Cross roads (Thruway: 8 Rampway: 0) |
| 11 | Culvert | Culvert Box: 15 (Thruway: 9, Rampway: 6) Culvert Pipe: 24 (Thruway: 20, Rampway: 4) |
| 12 | Softground | Approximately 5.6km |
| 13 | Interchange | KM0+000, Tuy Loan IC, NH14B |
| 14 | Frontage Road | (Under design) |
| 15 | Earthworks | Soil excavation: 182,295 m3 Rock excavation: 45m3 Embankment: 2,288,281m3 |
| 16 | Rough Cost | F/S (Sep 2010): 71.43 Million USD Updated (Mar 2013): 84.09 Million USD |

Package 2: KM008+000 - KM016+880, 8.880km

| No. | Items | Main Features |
| --- | --- | --- |
| 01 | Road Length | Quang Nam province: 8.880km Beginning Point: Km8+000, Dien Tien Commune, Dien Ban District, Quang Nam Province. Ending Point: Km16+880, Dien Tho Commune, Dien Ban District, Quang Nam Province. |
| 02 | Road Classification | Expressway: Type A, Class 120 |
| 03 | Design Speed | Expressway: 120 km/hr |
| 04 | Nos. of Lane | 4 lanes (Ultimate stage: Widened to 6 lanes) |
| 05 | Road width | Expressway: Road 25.5m, Large River Bridge: 26.0m, Other Bridges: 25.5m, Tunnel section: 2@13.05m=26.1m, Linking Road: 12.0m |
| 06 | Design Hydrological Frequency | Expressway: 1 %, Linking Road: 4 % Pavement Drainage: 4% Bridge Deck Drainage: 4% |
| 07 | Other Bridge | 15 bridges (L=1,518.8m) Large River Bridge (L>100m): 5 bridges (L=608.9m) Canal Bridge: 2 bridges (L=78.2m) Viaduct: 5 bridges (L=588.7m) Overpass: 1 bridge (L=40.7m) Interchange Rampway Bridge: 1 bridge (L=142.0m) Cross Road Flyover: 1 bridge (L=60.3m) |
| 08 | Major River Bridge | None |
| 09 | Tunnel | None |
| 10 | Underpass | 5 cross roads (Thruway: 5 Rampway: 0) |
| 11 | Culvert | Culvert Box: 6 (Thruway: 5, Rampway: 1) Culvert Pipe: 16 (Thruway: 10, Rampway: 6) |
| 12 | Softground | Approximately 5.6km |
| 13 | Interchange | KM13+260, My Son IC, PR609 |
| 14 | Frontage Road | (Under design) |
| 15 | Earthworks | Soil excavation: 16,221m3 Rock excavation: 15,084m3 Embankment: 1,947,854m3 |
| 16 | Rough Cost | F/S (Sep 2010): 72.69 Million USD Updated (Mar 2013): 86.10 Million USD |

Package 3A: KM016+880 - KM018+100, 1.220km

| No. | Items | Main Features |
| --- | --- | --- |
| 01 | Road Length | Quang Nam province: 1.220km Beginning Point: Km16+880, Dien Tho Commune, Dien Ban District, Quang Nam Province. Ending Point: Km18+100, Dien Quang Commune, Dien Ban District, Quang Nam Province. |
| 02 | Road Classification | Expressway: Type A, Class 120 |
| 03 | Design Speed | Expressway: 120 km/hr |
| 04 | Nos. of Lane | 4 lanes (Ultimate stage: Widened to 6 lanes) |
| 05 | Road width | Expressway: Road 25.5m, Large River Bridge: 26.0m, Other Bridges: 25.5m, Tunnel section: 2@13.05m=26.1m, Linking Road: 12.0m |
| 06 | Design Hydrological Frequency | Expressway: 1 %, Linking Road: 4 % Pavement Drainage: 4% Bridge Deck Drainage: 4% |
| 07 | Other Bridge | None |
| 08 | Major River Bridge | KM017+503, Ky Lam Bridge, L=1,044.80m, Thu Bon River |
| 09 | Tunnel | None |
| 10 | Underpass | None |
| 11 | Culvert | None |
| 12 | Softground | None |
| 13 | Interchange | None |
| 14 | Frontage Road | 110m |
| 15 | Earthworks | Soil excavation: 5,579m3 Rock excavation: 0m3 Embankment: 65,128m3 |
| 16 | Rough Cost | F/S (Sep 2010): 42.68 Million USD Updated (Mar 2013): 70.18 Million USD |

Package 3B: KM018+100 - KM021+500, 3.400km

| No. | Items | Main Features |
| --- | --- | --- |
| 01 | Road Length | Quang Nam province: 3.400km Beginning Point: Km18+100, Dien Quang Commune, Dien Ban District, Quang Nam Province. Ending Point: Km21+500, Duy Trinh Commune, Duy Xuyen District, Quang Nam Province. |
| 02 | Road Classification | Expressway: Type A, Class 120 |
| 03 | Design Speed | Expressway: 120 km/hr |
| 04 | Nos. of Lane | 4 lanes (Ultimate stage: Widened to 6 lanes) |
| 05 | Road width | Expressway: Road 25.5m, Large River Bridge: 26.0m, Other Bridges: 25.5m, Tunnel section: 2@13.05m=26.1m, Linking Road: 12.0m |
| 06 | Design Hydrological Frequency | Expressway: 1 %, Linking Road: 4 % Pavement Drainage: 4% Bridge Deck Drainage: 4% |
| 07 | Other Bridge | 4 bridges (L=1,051.1m) Viaduct: 3 bridges (L=995.9m) Cross Road Flyover: 1 bridges (L=55.2m) |
| 08 | Major River Bridge | KM20+209, Chiem Son Bridge, L=451.10m, Ba Ren River |
| 09 | Tunnel | None |
| 10 | Underpass | None |
| 11 | Culvert | Culvert Box: 2 (Thruway: 2) Culvert Pipe: 1 (Thruway:1) |
| 12 | Softground | None |
| 13 | Interchange | None |
| 14 | Frontage Road | 1,788 m |
| 15 | Earthworks | Soil excavation: 101,857m3 Rock excavation: 101,547m3 Embankment: 568,191m3 |
| 16 | Rough Cost | F/S (Sep 2010): 38.65 Million USD Updated (Mar 2013): 59.26 Million USD |

Package 4: KM021+500 - KM032+600, 11.100km

| No. | Items | Main Features |
| --- | --- | --- |
| 01 | Road Length | Quang Nam province: 11.100km Beginning Point: Km21+500, Duy Trinh Commune, Duy Xuyen Commune, Dien Ban District, Quang Nam Province. Ending Point: Km32+600, Que Xuan Commune, Que Son District, Quang Nam Province. |
| 02 | Road Classification | Expressway: Type A, Class 100 - 120, Linking Road: Class III, Delta |
| 03 | Design Speed | Expressway: 100 - 120 km/hr, Linking Road: 80 km/hr |
| 04 | Nos. of Lane | 4 lanes (Ultimate stage: Widened to 6 lanes) |
| 05 | Road Width | Expressway: Road 25.5m, Large River Bridge: 26.0m, Other Bridges: 25.5m, Tunnel section: 2@13.05m=26.1m, Linking Road: 12.0m |
| 06 | Design Hydrological Frequency | Expressway: 1 %, Linking Road: 4 % Pavement Drainage: 4% Bridge Deck Drainage: 4% |
| 07 | Other Bridge | 5 bridges (L=302.9m) Other River Bridge (L<100m): 3 bridges (L=204.9m) Overpass: 1 bridge (L=35.7m) Cross Road Flyover: 1 bridge (L=62.3m) |
| 08 | Major River Bridge | None |
| 09 | Tunnel | 1 tunnel (KM022+900, North bound : 556m; South bound: 515m ) |
| 10 | Underpass | 12 cross roads (Thruway: 12 Rampway: 0) |
| 11 | Culvert | Culvert Box: 19 (Thruway: 19, Rampway: 0) Culvert Pipe: 21 (Thruway: 21, Rampway: 0) |
| 12 | Softground | None |
| 13 | Interchange | None |
| 14 | Frontage Road | (Under design) |
| 15 | Earthworks | Soil excavation: 301,039m3 Rock excavation: 290,659m3 Embankment: 1,638,682m3 |
| 16 | Rough Cost | F/S (Sep 2010): 70.36 Million USD Updated (Mar 2013): 122.59 Million USD |

Package 5: KM032+600 - KM042+000, 9.400km

| No. | Items | Main Features |
| --- | --- | --- |
| 01 | Road Length | Quang Nam province: 9.400km Beginning Point: Km32+600, Que Xuan Commune, Que Son District, Quang Nam Province. Ending Point: Km42+000, Binh Quy Commune, Thang Binh District, Quang Nam Province. |
| 02 | Road Classification | Expressway: Type A, Class 100 - 120, Linking Road: Class III, Delta |
| 03 | Design Speed | Expressway: 120 km/hr |
| 04 | Nos. of Lane | 4 lanes (Ultimate stage: Widened to 6 lanes) |
| 05 | Road width | Expressway: Road 25.5m, Large River Bridge: 26.0m, Other Bridges: 25.5m, Tunnel section: 2@13.05m=26.1m, Linking Road: 12.0m |
| 06 | Design Hydrological Frequency | Expressway: 1 %, Linking Road: 4 % Pavement Drainage: 4% Bridge Deck Drainage: 4% |
| 07 | Other Bridge | 8 bridges (L=581.3m) Large River Bridge (L>100m): 1 bridge (L=222.1m) Other River Bridge (L<100m): 3 bridges (L=184.3m) Canal Bridge: 1 bridge (L=51.0m) Overpass: 3 bridges (L=123.9m) |
| 08 | Major River Bridge | None |
| 09 | Tunnel | None |
| 10 | Underpass | 4 cross roads (Thruway: 3 Rampway: 1) |
| 11 | Culvert | Culvert Box: 7 (Thruway: 6, Rampway: 1) Culvert Pipe: 21 (Thruway: 16, Rampway: 5) |
| 12 | Softground | None |
| 13 | Interchange | KM40+880, Ha Lam IC, NH14E |
| 14 | Frontage Road | 374m |
| 15 | Earthworks | Soil excavation: 435,079m3 Rock excavation: 214,556m3 Embankment: 2,880,820m3 |
| 16 | Rough Cost | F/S (Sep 2010): 61.28 Million USD Updated (Mar 2013): 84.75 Million USD |

Package 6: KM042+000 - KM052+000, 10.000km

| No. | Items | Main Features |
| --- | --- | --- |
| 01 | Road Length | Quang Nam province: 10.000km Beginning Point: Km42+000, Binh Quy Commune, Thang Binh District, Quang Nam Province. Ending Point: Km52+000, Binh An Commune, Thang Binh District, Quang Nam Province. |
| 02 | Road Classification | Expressway: Type A, Class 120 |
| 03 | Design Speed | Expressway: 120 km/hr |
| 04 | Nos. of Lane | 4 lanes (Ultimate stage: Widened to 6 lanes) |
| 05 | Road width | Expressway: Road 25.5m, Large River Bridge: 26.0m, Other Bridges: 25.5m, Tunnel section: 2@13.05m=26.1m, Linking Road: 12.0m |
| 06 | Design Hydrological Frequency | Expressway: 1 %, Linking Road: 4 % Pavement Drainage: 4% Bridge Deck Drainage: 4% |
| 07 | Other Bridge | 9 bridges (L=480.0m) Large River Bridge (L>100m): 1 bridge (L=112.2m) Other River Bridge (L<100m): 3 bridges (L=176.7m) Canal Bridge: 3 bridges (L=123.1m) Overpass: 2 bridges (L=68.0m) |
| 08 | Major River Bridge | None |
| 09 | Tunnel | None |
| 10 | Underpass | 11 cross roads (Thruway: 11) |
| 11 | Culvert | Culvert Box: 10 (Thruway: 10) Culvert Pipe: 18 (Thruway: 18) |
| 12 | Softground | None |
| 13 | Interchange | None |
| 14 | Frontage Road | 7,767m |
| 15 | Earthworks | Soil excavation: 66,564m3 Rock excavation: 66,564m3 Embankment: 1,718,654m3 |
| 16 | Rough Cost | F/S (Sep 2010): 63.86 Million USD Updated (Mar 2013): 63.92 Million USD |

Package 7: KM052+000 - KM065+000, 13.000km

| No. | Items | Main Features |
| --- | --- | --- |
| 01 | Road Length | Quang Nam province: 13.000km Beginning Point: Km52+000, Binh An Commune, Thang Binh District, Quang Nam Province. Ending Point: Km65+000, Tam Thai Commune, Phu Ninh District, Quang Nam Province. |
| 02 | Road Classification | Expressway: Type A, Class 120 |
| 03 | Design Speed | Expressway: 120 km/hr |
| 04 | Nos. of Lane | 4 lanes (Ultimate stage: Widened to 6 lanes) |
| 05 | Road width | Expressway: Road 25.5m, Large River Bridge: 26.0m, Other Bridges: 25.5m, Tunnel section: 2@13.05m=26.1m, Linking Road: 12.0m |
| 06 | Design Hydrological Frequency | Expressway: 1 %, Linking Road: 4 % Pavement Drainage: 4% Bridge Deck Drainage: 4% |
| 07 | Other Bridge | 10 bridges (L=668.6m) Large River Bridge (L>100m): 2 bridges (L=294.8m) Other River Bridge (L<100m): 3 bridges (L=156.0m) Overpass: 4 bridges (L=149.8m) Interchange Rampway Bridge: 1 bridges (L=68.0m) |
| 08 | Major River Bridge | None |
| 09 | Tunnel | None |
| 10 | Underpass | 17 cross roads (Thruway: 15, Rampway: 2) |
| 11 | Culvert | Culvert Box: 31 (Thruway: 25, Rampway: 6) Culvert Pipe: 21 (Thruway: 15, Rampway: 6) |
| 12 | Softground | None |
| 13 | Interchange | KM64+510, Tam Ky IC, PR616 |
| 14 | Frontage Road | 3,619m |
| 15 | Earthworks | Soil excavation: 52,431m3 Rock excavation: 46,927m3 Embankment: 2,781,573m3 |
| 16 | Rough Cost | F/S (Sep 2010): 84.61 Million USD Updated (Mar 2013): 79.84 Million USD |

Package A1: KM065+000 - KM081+150, 16.150km

| No. | Items | Main Features |
| --- | --- | --- |
| 01 | Road Length | Quang Nam province: 16.150km Beginning Point: Km65+000, Tam Thai Commune, Phu Ninh District, Quang Nam Province. Ending Point: Km81+150, Tam Anh Nam Commune, Nui Thanh District, Quang Nam Province. |
| 02 | Road Classification | Expressway: Type A, Class 120 |
| 03 | Design Speed | Expressway: 120 km/hr |
| 04 | Nos. of Lane | 4 lanes (Ultimate stage: Widened to 6 lanes) |
| 05 | Road width | Expressway: Road 25.5m, Large River Bridge: 26.0m, Other Bridges: 25.5m, Tunnel section: 2@13.05m=26.1m, Linking Road: 12.0m |
| 06 | Design Hydrological Frequency | Expressway: 1 %, Linking Road: 4 % Pavement Drainage: 4% Bridge Deck Drainage: 4% |
| 07 | Other Bridge | 5 bridges (L=585.5m) Large River Bridge (L>100m): 1 bridge (L=306.3m) Other River Bridge (L<100m): 2 bridges (L=118.3m) Cross Road Flyover: 2 bridges (L=160.9m) |
| 08 | Major River Bridge | None |
| 09 | Tunnel | None |
| 10 | Underpass | 18 cross roads (Thruway: 18, Rampway: 0) |
| 11 | Culvert | Culvert Box: 27 (Thruway: 27, Rampway: 0) Culvert Pipe: 24 (Thruway: 24, Rampway: 0) |
| 12 | Softground | None |
| 13 | Interchange | None |
| 14 | Frontage Road | (Under design) |
| 15 | Earthworks | Soil excavation: 560,325 m3 Rock excavation: 549,437m3 Embankment: 2,451,666m3 |
| 16 | Rough Cost | F/S (Sep 2010): 84.54 Million USD Updated (Mar 2013): 98.04 Million USD |

Package A2: KM081+150 - KM099+500, 18.350km

| No. | Items | Main Features |
| --- | --- | --- |
| 01 | Road Length | Quang Nam province: 18.150km, Quang Ngai province: 0.200km Beginning Point: Km81+150, Tam Anh Nam Commune, Nui Thanh District, Quang Nam Province. Ending Point: Km99+500, Binh Chanh Commune, Binh Son District, Quang Ngai Province. |
| 02 | Road Classification | Expressway: Type A, Class 120 |
| 03 | Design Speed | Expressway: 120 km/hr |
| 04 | Nos. of Lane | 4 lanes (Ultimate stage: Widened to 6 lanes) |
| 05 | Road width | Expressway: Road 25.5m, Large River Bridge: 26.0m, Other Bridges: 25.5m, Tunnel section: 2@13.05m=26.1m, Linking Road: 12.0m |
| 06 | Design Hydrological Frequency | Expressway: 1 %, Linking Road: 4 % Pavement Drainage: 4% Bridge Deck Drainage: 4% |
| 07 | Other Bridge | 9 bridges (L=539.1m) Large River Bridge (L>100m): 1 bridge (L=147.1m) Other River Bridge (L<100m): 3 bridges (L=152.5m) Canal Bridge (CB): 2 bridges (L=98.8m) Overpass: 2 bridges (L=84.2m) Cross Road Flyover: 1 bridge (L=56.5m) |
| 08 | Major River Bridge | None |
| 09 | Tunnel | None |
| 10 | Underpass | 23 cross roads (Thruway: 22, Rampway: 1) |
| 11 | Culvert | Culvert Box: 24 (Thruway: 20, Rampway: 4) Culvert Pipe: 35 (Thruway: 29, Rampway: 6) |
| 12 | Softground | Approximately 3.36km |
| 13 | Interchange | KM82+990, Chu Lai IC, Planned Road |
| 14 | Frontage Road | 3,229m |
| 15 | Earthworks | Soil excavation: 1,653,116m3 Rock excavation: 1,646,860m3 Embankment: 2,323,333m3 |
| 16 | Rough Cost | F/S (Sep 2010): 89.37 Million USD Updated (Mar 2013): 114.19 Million USD |

Package A3: KM099+500 - KM110+100, 10.600km

| No. | Items | Main Features |
| --- | --- | --- |
| 01 | Road Length | Quang Ngai province: 10.600km Beginning Point: Km99+500, Binh Chanh Commune, Binh Son District, Quang Ngai Province. Ending Point: Km101+100, Binh Nguyen Commune, Binh Soin District, Quang Nam Province. |
| 02 | Road Classification | Expressway: Type A, Class 120 |
| 03 | Design Speed | Expressway: 120 km/hr |
| 04 | Nos. of Lane | 4 lanes (Ultimate stage: Widened to 6 lanes) |
| 05 | Road width | Expressway: Road 25.5m, Large River Bridge: 26.0m, Other Bridges: 25.5m, Tunnel section: 2@13.05m=26.1m, Linking Road: 12.0m |
| 06 | Design Hydrological Frequency | Expressway: 1 %, Linking Road: 4 % Pavement Drainage: 4% Bridge Deck Drainage: 4% |
| 07 | Other Bridge | 12 bridges (L=579.0m) Other River Bridge (L<100m): 4 bridges (L=158.6m) Canal Bridge: 1 bridge (L=31.5m) Viaduct: 2 bridges (L=92.2m) Overpass: 1 bridges (L=24.8m) Interchange Rampway Bridges: 3 bridges (L=217.6m) Cross Road Flyover: 1 bridge (L=54.3m) |
| 08 | Major River Bridge | KM109+001, Tra Bong Bridge, L=454.5m, Tra Bong River |
| 09 | Tunnel | None |
| 10 | Underpass | 15 cross roads (Thruway: 14, Rampway: 1) |
| 11 | Culvert | Culvert Box: 11 (Thruway: 11, Rampway: 0) Culvert Pipe: 12 (Thruway: 10, Rampway: 2) |
| 12 | Softground | Approximately 2.46km |
| 13 | Interchange | KM101+740, Dung Quat IC, Planed Road |
| 14 | Frontage Road | (Under design) |
| 15 | Earthworks | Soil excavation: 722,598m3 Rock excavation: 722,598m3 Embankment: 992,250m3 |
| 16 | Rough Cost | F/S (Sep 2010): 76.14 Million USD Updated (Mar 2013): 74.58 Million USD |

Package A4: KM110+100 - KM124+700, 14.600km

| No. | Items | Main Features |
| --- | --- | --- |
| 01 | Road Length | Quang Ngai province: 14.600km Beginning Point: Km110+100, Binh Long Commune, Binh Son District, Quang Ngai Province. Ending Point: Km124+700, Tinh Ha Commune, Son Tinh District, Quang Ngai Province. |
| 02 | Road Classification | Expressway: Type A, Class 100 - 120 |
| 03 | Design Speed | Expressway: 100-120 km/hr |
| 04 | Nos. of Lane | 4 lanes (Ultimate stage: Widened to 6 lanes) |
| 05 | Road width | Expressway: Road 25.5 (33.0)m, Large River Bridge: 26.0m, Other Bridges: 25.5m, Tunnel section: 2@13.05m=26.1m, Linking Road: 12.0m |
| 06 | Design Hydrological Frequency | Expressway: 1 %, Linking Road: 4 % Pavement Drainage: 4% Bridge Deck Drainage: 4% |
| 07 | Other Bridge | 8 bridges (L=500.8m) Other River Bridge (L<100m): 1 bridge (L=61.9m) Canal Bridge: 1 bridges (L=46.1m) Viaduct: 1 bridge (L=116.4m) Overpass: 2 bridges (L=91.8m) Cross Road Flyover: 3 bridges (L=184.6m) |
| 08 | Major River Bridge | None |
| 09 | Tunnel | None |
| 10 | Underpass | 7 cross roads (Thruway: 7) |
| 11 | Culvert | Culvert Box: 24 (Thruway: 23, Rampway: 1) Culvert Pipe: 15 (Thruway: 15) |
| 12 | Softground | None |
| 13 | Interchange | KM123+700, Quang Ngai North IC, NH24B |
| 14 | Frontage Road | 2,753m |
| 15 | Earthworks | Soil excavation: 1,064,725m3 Rock excavation: 1,001,090m3 Embankment: 1,559,038m3 |
| 16 | Rough Cost | F/S (Sep 2010): 90.7 Million USD Updated (Mar 2013): 93.63 Million USD |

Package A5 (1/2): KM124+700 - KM131+500, 6.800km (Expressway)

| No. | Items | Main Features |
| --- | --- | --- |
| 01 | Road Length | Quang Ngai province: 6.800km Beginning Point: Km124+700, Nghia Ky Commune, Tu Nghia District, Quang Ngai Province. Ending Point: Km131+500, Nghia Dien Commune, Tu Nghia District, Quang Ngai Province. |
| 02 | Road Classification | Expressway: Type A, Class 120 |
| 03 | Design Speed | Expressway: 120 km/hr |
| 04 | Nos. of Lane | 4 lanes (Ultimate stage: Widened to 6 lanes) |
| 05 | Road width | Expressway: Road 25.5m, Large River Bridge: 26.0m, Other Bridges: 25.5m, Tunnel section: 2@13.05m=26.1m, Linking Road: 12.0m |
| 06 | Design Hydrological Frequency | Expressway: 1 %, Linking Road: 4 % Pavement Drainage: 4% Bridge Deck Drainage: 4% |
| 07 | Other Bridge | 8 bridges (L=1,369.8m) Other River Bridge (L<100m): 2 bridge (L=99.2m) Canal Bridge: 1 bridge (L=55.9m) Viaduct: 2 bridges (L=986.8m) Overpass: 3 bridges (L=227.9m) |
| 08 | Major River Bridge | KM125+367, Tra Khuc Bridge, L=774.50m, Tra Khuc River |
| 09 | Tunnel | None |
| 10 | Underpass | 4 cross roads (Thruway: 4) |
| 11 | Culvert | Culvert Box: 5 (Thruway: 5) Culvert Pipe:9 (Thruway: 9) |
| 12 | Softground | None |
| 13 | Interchange | KM130+502, Quang Ngai IC, Planned Road |
| 14 | Frontage Road | (Under design) |
| 15 | Earthworks | Soil excavation: 0m3 Rock excavation: 0m3 Embankment: 1,214,650m3 |
| 16 | Rough Cost (incl/ Linking Road) | F/S (Sep 2010): 91.65 Million USD Updated (Mar 2013): 74.01 Million USD |

Package A5 (2/2): KM131+500 - KM139+204, 7.704km (Linking Road)

| No. | Items | Main Features |
| --- | --- | --- |
| 01 | Road Length | Quang Ngai province: 7.704km Beginning Point: Km131+500, Nghia Dien Commune, Tu Nghia District, Quang Ngai Province. Ending Point: Km139+204, Nghia Thuong Commune, Tu Nghia District, Quang Ngai Province. |
| 02 | Road Classification | Linking Road: Class III, Delta |
| 03 | Design Speed | Linking Road: 80 km/hr |
| 04 | Nos. of Lane | 2 lanes |
| 05 | Road width | Linking Road: 12.0m |
| 06 | Design Hydrological Frequency | Linking Road: 4 % Pavement Drainage: 4% Bridge Deck Drainage: 4% |
| 07 | Other Bridge | 3 bridges (L=246.906m) Other River Bridge (L<100m): 1 bridge (L=48.7m) Canal Bridge: 1 bridges (L=54.5m) Overpass: 1 bridge (L=143.71m) |
| 08 | Major River Bridge | None |
| 09 | Tunnel | None |
| 10 | Underpass | 1 cross road |
| 11 | Culvert | Culvert Box: 14 Culvert Pipe: 24 |
| 12 | Softground | None |
| 13 | Interchange | None |
| 14 | Frontage Road | (Under design) |
| 15 | Earthworks | Soil excavation: 0m3 Rock excavation: 0m3 Embankment: 124,659m3 |

Package 13: Electrical/O&M Building/ITS Works and Equipment Provision

| No. | Items | Main Features |
| --- | --- | --- |
| 01 | Electrical Facilities | Power supply, road lighting, tunnel ventilation and safety facilities |
| 02 | ITS | Traffic management, toll collection and communication systems |
| 03 | O&M Building | 26 locations KM4+100, Traffic Management Center KM4+100, KM123+900, ManagementOffices KM66+800, Service Area KM36+000, KM96+000, Parking Area KM4+100, KM 129+500, Toll Barriers, Interchange Toll Gate and Office: 6locations |
| 04 | O&M Equipment | O&M vehicles, spare parts, maintenance equipment and consumables |

Package 14A/14B: Traffic Safety/Lighting

| No. | Items | Main Features |
| --- | --- | --- |
| 1 | Electrical facilities | Road lighting and tunnel ventilation |
| 2 | Traffic Safety Facilities | Guard rail (except crossing & frontage roads), Fence, Traffic signs (except crossing & frontage roads), Road Marking (except crossing & frontage roads), Anti-glare screen |