THE SOCIALIST REPUBLIC OF VIETNAM

MINISTRY OF TRANSPORT

VIETNAM EXPRESSWAY CORPORATION

PROJECT MANAGEMENT UNIT 85

Consulting Service for Detailed Design for Danang Quang Ngai Expressway Development Project

IDA Credit No. 3843-VN

TECHNICAL REPORT

Alternative Study on Location of My Son Interchange

October 2011

Joint Venture of

Nippon Koei Co., Ltd.

Nippon Engineering Consultants Co., Ltd.

Chodai Co., Ltd.

Thai Engineering Consultants Company Limited

Table of Contents

Page

1. Background	1

2. Status of Technical Study for Additional Interchange at Provincial Road 620	1

3. Evaluation Criteria to Select Interchange Location	1

3.1 Location of Interchange	1

3.2 Traffic Network	2

3.3 Accessibility to Populated Area, Industrial Area, and Tourist Area	2

3.4 Interval of Interchanges	2

3.5 Geometrical Requirement	2

3.6 Construction Cost	3

3.7 Environmental Impact	3

3.8 Traffic Safety	3

4. Evaluation Results	4

4.1 Location of Interchange	4

4.2 Traffic Network	4

4.3 Accessibility to Populated Area, Industrial Area, and Tourist Area	4

4.4 Interval of Interchanges	4

4.5 Geometrical Requirement	4

4.6 Construction Cost	5

4.7 Environmental Impact	5

4.8 Traffic Safety	5

5. Summary and Conclusion	6

5.1 Summary	6

5.2 Conclusion	6

5.3 Remarks	7

APPENDIX-1 : Map of Impact Area of Interchange	8

APPENDIX-2 : Selection of Type of Interchange	9

1. Background

Based on the Notice No.346/TB0UBND dated 13 September 2011 by Quang Nam province, letter No. 1855/VEC-KTCNMT dated 30 September 2011 by VEC, and letter No. 50/BQL-DNQN dated 4 October 2011 by PMU85, the Consultant carried out alternative study of My Son interchange. Subjected connecting roads to the study are provincial road 609 (new plan) and 610 (original plan).

Figure 1-1 Location of Interchanges in F/S

2. Status of Technical Study for Additional Interchange at Provincial Road 620

In addition to the interchange at provincial road 609, Quang Nam Peoples Committee requested to VEC to carry out technical study of additional interchange at provincial road 620 by the Notice No.346/TB0UBND dated 13 September 2011.

However, the Consultant does not receive necessary information about the provincial road 620 to the technical study yet.

3. Evaluation Criteria to Select Interchange Location

In the selection of interchange location, traffic conditions, social conditions, and natural conditions shall be taken into account. The evaluation criteria to select interchange are prepared based on practice in Japan.

3.1 Location of Interchange

Since interchange itself needs large space of 30,000m2 to 100,000m2 and the neighboring space would be developed quickly, it is desirable that an interchange should be located on spacious area without serious obstacles for development.

Following evaluation criteria is applied to select interchange location:

- Spacious land area

- Less obstacles as design control

3.2 Traffic Network

Interchange should be connected to major arterial roads with consideration of followings:

- Connecting road shall be able to accommodate access traffic to/from interchange

- Connecting road shall provide efficient access to major traffic generation points such as city center, industrial zone, tourist place

- Connecting road shall be network to arterial road network formulating appropriate road density to disperse access traffic to/from interchange efficiently

Following evaluation criteria is applied to select interchange location:

- Distance to major arterial roads

3.3 Accessibility to Populated Area, Industrial Area, and Tourist Area

Interchange should be located nearby cities of 30,000 populations or more. Alternatively, coverage area of interchange (within 10km radius) shall cover about 50,000 to 100,000 total population in the coverage area. Furthermore, interchange shall be connected to access roads which provide efficient access to industrial area and tourist area.

Following evaluation criteria is applied to select interchange location:

- Population number and population growth

- Distance to major industrial area and tourist area

3.4 Interval of Interchanges

The interval of the interchanges is practically from 4 km to 30 km. Typical condition of the intervals for the Interchanges is as follows:

- Near Capital, Big City or Major Industrial area : 5Km ~ 10Km

- The plain area dotted with a small city         :15Km ~ 25Km

- The district villages and the mountain area     :20Km ~ 30Km

Following evaluation criteria is applied to select interchange location:

- Interval of neighboring interchange

3.5 Geometrical Requirement

Thruway geometrical condition nearby the interchange shall be designed with consideration of visibility and safety. Table 3-1 shows thruway geometrical standard at IC section.

Table 3-1 Thruway Geometrical Standard at Interchange Section (TCVN5729-2007)

| The grades of freeway/expressway (km/h) |  |  | 120 | 100 | 80 | 60 |
| --- | --- | --- | --- | --- | --- | --- |
| The minimum radius of the horizontal curve (m) |  | Normal | 2,000 | 1,500 | 1,100 | 500 |
|  |  | Limited | 1,500 | 1,000 | 700 | 350 |
| The minimum radius of the vertical curve (m) | Convex | Normal | 45,000 | 25,000 | 12,000 | 6,000 |
|  |  | Limited | 23,000 | 15,000 | 6,000 | 3,000 |
|  | Concave | Normal | 16,000 | 12,000 | 8,000 | 4,000 |
|  |  | Limited | 12,000 | 8,000 | 4,000 | 2,000 |
| The biggest longitudinal gradient (%) |  | Normal | 2 | 2 | 3 | 4.5 |
|  |  | Limited | 2 | 2 | 4 | 5.5 |

Following evaluation criteria is applied to select interchange location:

- Thruway horizontal curve at interchange section

- Applicable regulation speed

3.6 Construction Cost

Since the greatest impact on construction cost of interchange is earth work, location of interchange shall not be selected at high mountain area and soft ground.

Following evaluation criteria is applied to select interchange location:

- Terrain on interchange planning area

3.7 Environmental Impact

Land use condition should be taken into account on selection of interchange location to protect living environment.

Following evaluation criteria is applied to select interchange location:

- Residential area on interchange planning area

3.8 Traffic Safety

Appropriate distance between interchange and expressway facilities shall be secured to accommodate weaving section and pre-caution traffic sign for ensuring traffic safety. Table 3-2 shows minimum distance between interchange and expressway facilities.

Table 3-2 Minimum Distance between Interchange and Expressway Facilities in Japan

| Name of Facilities | 1) Minimum Distance |  |
| --- | --- | --- |
|  | Normal | Limited |
| Interchange – Interchange Interchange – Service Area Interchange – Parking Area | 5km | 3km |
| Interchange – Bus Stop Interchange –Tunnel | 2) 4km | 2km |

Note: 1) Distance between facility center and facility center

2) Distance between facility center and portal

Following evaluation criteria is applied to select interchange location:

- Distance between Interchange and expressway facilities

4. Evaluation Results

4.1 Location of Interchange

- Spacious land area

- TL609: Located wide flat area

- TL610: Located between mountain and river

- Less obstacles as design control

- TL609: No outstanding design control point

- TL610: Near to tunnel portal

4.2 Traffic Network

- Distance to major arterial roads

- TL609: Connecting to NH 1 and NH 14B and networking with main arterial roads (See Appendix-1)

- TL610: Connecting only NH 1 (See Appendix-1)

4.3 Accessibility to Populated Area, Industrial Area, and Tourist Area

- Number of population and population growth

- TL609: Main district in the coverage area is Dien Ban District (198,822 person in 2009, growth rate 104.27% (2009/2005))

- TL610: Main district in the coverage area is Duy Xuyen District (120,948 person in 2009, growth rate 99.11% (2009/2005))

- Distance to major industrial area and tourist area

- TL609: Hoa Khuong IZ (400ha, 5km), Hoa Minh IZ (200ha, 5 km), Hoi An (22km), My Son (42km) (See Appendix-1)

- TL610: Dien Nam-Dien Ngoc IZ (418ha, 15 km), My Son (15km), Hoi An (30km) (See Appendix-1)

4.4 Interval of Interchanges

- Interval of neighboring interchange

- TL609: Tuy Loan Interchange 13.6km, Ha Lam Interchange 27.5km

- TL610: Tuy Loan Interchange 20.2km, Ha Lam Interchange 20.9km

4.5 Geometrical Requirement

- Thruway horizontal curve at interchange section

- TL609: Horizontal curve radius is 4,000m

- TL610: Horizontal curve radius is 1,200m. Therefore, operation speed shall be less than 80km/h in accordance with TCVN5729-2007

- Applicable regulation speed

- TL609: Speed limit regulation is not necessary. 120km/h regulation is available

- TL610: Speed limit regulation is necessary. 120km/h regulation is not available

4.6 Construction Cost

- Terrain on interchange planning area

- TL609: Terrain is flat

- TL610: Terrain is partially hilly

4.7 Environmental Impact

- Residential area on interchange planning area

- TL609: Little resettlement is required for residential area along TL609

- TL610: Smaller number of resettlement than TL609

4.8 Traffic Safety

- Distance between Interchange and expressway facilities

- TL609: There is no neighboring interchange and expressway facilities

- TL610: Distance between interchange center and portal of tunnel is about 1.6km

5. Summary and Conclusion

5.1 Summary

Evaluation result is summarized in Table 5-1.

Table 5-1 Summary of Alternative Interchanges Evaluation

| Items |  |  | Provincial Road 609 | Provincial Road 610 |
| --- | --- | --- | --- | --- |
|  |  |  | Alternative Plan | F/S Plan |
| Location of Interchange | Spacious land area | Value | Wide and flat (at IC area) Residential / Farmland: 95% Hilly area / River: 5% (within 10km area) | Limited flat (at IC area) Residential / Farmland: 65% Hilly area / River / Dam: 35% (within 10km area) |
|  |  | Evaluation | Excellent | Fair |
|  | Less obstacles as design control | Value | No outstanding design control | Near to tunnel portal |
|  |  | Evaluation | Good | Fair |
| Traffic Network | Distance to major arterial roads | Value | NH1 (9 km) NH14B (7.5km) | NH1 (11 km) |
|  |  | Evaluation | Excellent | Fair |
| Accessibility | Number of population and population growth (Main District) | Value | Dien Ban District 198,822 person (2009) 926 person/km2 (2009) 104.27 % (2009/2005) | Duy Xuyen District 120,948 person (2009) 404 person/km2 (2009) 99.11 % (2009/2005) |
|  |  | Evaluation | Excellent | Fair |
|  | Distance to major industrial and tourist area | Value | Hoa Khuong IZ (400ha, 5km) Hoa Minh IZ (200ha, 5 km) Hoi An (22km) My Son (42km) | Dien Nam-Dien Ngoc IZ (418ha, 15 km) My Son (15km) Hoi An (30km) |
|  |  | Evaluation | Excellent | Fair |
| Interval of Interchange | Interval of neighboring interchange | Value | 13.6km 27.5 km | 20.2 km 20.9 km |
|  |  | Evaluation | Excellent | Excellent |
| Geometrical Requirement | Thruway horizontal curve at interchange section | Value | R=4,000m | R=1,200m (Operation speed shall be less than 80km/h) |
|  |  | Evaluation | Excellent | Poor |
|  | Applicable regulation speed | Value | 120 km/h | 80 km/h (Speed limit regulation is necessary) |
|  |  | Evaluation | Excellent | Poor |
| Construction Cost | Terrain on interchange planning area | Value | Flat | Partially hilly |
|  |  | Evaluation | Fair | Good |
| Environmental Impact | Residential area on interchange planning area | Value | Little | Little or nothing |
|  |  | Evaluation | Fair | Good |
| Traffic Safety | Distance between Interchange and expressway facilities | Value | Nothing within 5km | 1.6km (Tunnel) |
|  |  | Evaluation | Excellent | Poor |

5.2 Conclusion

Interchange at provincial road 609 is recommended based on alternative comparison and advantages as follows:

- Spacious area for future development of neighboring area

- Locating at potential area of population growth

- Good access to district center, major highways, industrial parks

- Wider potential development area than the interchange TL610

- Smooth and safety thruway geometry

5.3 Remarks

Following additional work and cost are necessary in case the interchange TL609 is selected.

- Revision of the ROW is required

- Additional survey and design cost

APPENDIX-1 : Map of Impact Area of Interchange

Following figure shows commune populations, planned industrial parks, tourist places, major highways, and impacts area of interchanges (10km radius). Almost one third of the impact area of the interchange TL610 is hilly and water reservoir area.

APPENDIX-2 : Selection of Type of Interchange

A-2.1 Type of Interchange

Interchange is classified by number of legs on intersecting road and treatment of traffic stream lines as follows:

- Classification by the number of legs on intersecting road

  - Three leg interchange

  - Four-leg interchange

  - Multi-leg interchange

- Classification by treatment of traffic stream lines

- Partial grade separation type (ex. Diamond)

- Full grade separation type (ex. Trumpet, Partial Cloverleaf, Cloverleaf, Directional)

- Weaving type (ex. Rotary)

Table A-2-1 General Compatibility of Interchanges

| Items / Type |  | Partial Grade Separation | Full Grade Separation | Weaving Type |
| --- | --- | --- | --- | --- |
| Traffic Management / Operation | Intersecting Road |  | High compatibility on connection between expressway and ordinary roads | High compatibility on connection between expressway and expressway, as well as expressway and ordinary roads | Compatible for connection between expressway and ordinary roads |  |
|  | Toll Gate |  | High compatible except diamond type | Low compatible except three-leg interchange | Low compatible except three-leg interchange |  |
|  | Ramp Traffic Volume |  | Low compatibility on high interchange traffic volume due to level crossing at ramp exit | High compatibility on high interchange traffic volume | Low compatibility on high interchange traffic volume due to weaving section |  |
|  | Traffic Safety |  | Safety consideration is necessary for suspending long vehicle queue on thruway and ramp way by traffic jam at ramp exit | Safety level is high | Safety consideration is necessary because that weaving sections causes bottleneck |  |
| Economic Efficiency | Construction Cost |  | Lowest | Highest | Intermediate |  |
|  | Land Acquisition Cost |  | Lowest | High | High |  |
|  | Operation & Maintenance Cost |  | Low in case that toll gates can be consolidated one location | Low in case that toll gates can be consolidated one location | Low in case that toll gates can be consolidated one location |  |

Since high grade traffic function shall be given to Danang Quang Ngai Expressway as a part of North-South Expressway, traffic management and operation shall be prioritized on the selection. Moreover, economic efficiency shall be evaluated with consideration of initial cost and operation & maintenance cost.

According to above principle, the weaving type interchange is excluded from the selection due to less compatibility on traffic management and operation.

Table A-2-2 Result of Interchange Type Selection

| Items / Type |  |  | Partial Grade Separation | Full Grade Separation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Diamond | Trumpet |  | Cloverleaf |  |
| Traffic Management / Operation | Intersecting Road | Value | Connect between expressway and ordinary road (High compatibility on connection between expressway and ordinary roads) | Connect between expressway and ordinary road (High compatibility on connection between expressway and expressway, as well as expressway and ordinary roads) |  | Connect between expressway and ordinary road (High compatibility on connection between expressway and expressway, as well as expressway and ordinary roads) |  |
|  |  | Evaluation | Good | Good |  | Good |  |
|  | Toll Gate | Value | 4 locations (High compatible except diamond type) | 1 location (Low compatible except three-leg interchange) |  | 4 locations (Low compatible except three-leg interchange) |  |
|  |  | Evaluation | Poor | Good |  | Poor |  |
|  | Ramp Traffic Volume | Value | (Low compatibility on high interchange traffic volume due to level crossing at ramp exit) | (High compatibility on high interchange traffic volume) | (High compatibility on high interchange traffic volume) |
|  |  | Evaluation | Poor | Good | Good |
|  | Traffic Safety | Value | (Safety consideration is necessary for suspending long vehicle queue on thruway and ramp way by traffic jam at ramp exit) | (Safety level is high) | (Safety consideration is necessary because that weaving sections causes bottleneck) |
|  |  | Evaluation | Poor | Good | Fair |
| Economic Efficiency | Construction Cost | Value | (Lowest) | (Highest) |  | (Highest) |  |
|  |  | Evaluation | Good | Poor |  | Poor |  |
|  | Land Acquisition Cost | Value | (Lowest) | (High) |  | (High) |  |
|  |  | Evaluation | Good | Poor |  | Poor |  |
|  | Operation & Maintenance Cost | Value | 4 locations (Low in case that toll gates can be consolidated one location) | 1 location (Low in case that toll gates can be consolidated one location) |  | 4 locations) (Low in case that toll gates can be consolidated one location) |  |
|  |  | Evaluation | Poor | Good |  | Poor |  |
| Ranking |  |  | 2 | 1 | 3 |

A-2.2 Conclusion

Trumpet type interchange is recommended based on alternative comparison and advantages as follows:

- Only one toll gate is necessary

- High compatibility on high interchange traffic volume

- Traffic safety level is high

- Low operation & maintenance cost contributes reduction of total cost