Ministry of Physical Infrastructure and Transport, Department of Roads

Government of Nepal

Nagdhunga Tunnel Construction Project

Loan Agreement No. (NE-P10)

Contract No. DOR - NTCP -W-ICB-01

Bidding Documents

Part 2: Works Requirements

(SECTION VI)

VI-C: Special Provisions to Standard Specifications (SPSS)

VOLUME-I: ROAD AND BRIDGE WORKS

Section VI-C: Special Provisions to the Standard Specifications (SPSS)

TABLE OF CONTENTS

VOLUME-I: ROAD AND BRIDGE WORKS

SECTION 100: GENERAL

101	SITE OFFICES FOR THE EMPLOYER	1-1

102	ACCOMMODATION OF TRAFFIC	1-6

103	PUBLICLY AND PRIVATELY OWNED SERVICES	1-8

104	SURVEY AND SETTING OUT	1-9

106	DIVERSION BRIDGES	1-10

107	MAINTENANCE OF ROAD	1-10

108	NOTICE BOARD	1-11

109	ENVIRONMENTAL PROTECTION WORKS	1-12

110	PROJECT SAFETY	1-25

111	SUPPLY OF PROJECT RECORD	1-36

112	NOTES ABOUT MEASUREMENT AND PAYMENT	1-38

113	NATIONAL SPECIFICATIONS	1-39

115	UNITS OF MEASUREMENT, ABBREVIATION, TERMINOLOGY AND DEFINITIONS	1-39

116	PROGRAMME	1-41

117	CONTRACTOR’S DRAWINGS	1-44

SECTION 200: SITE CLEARANCE

201	CLEARING AND GRUBBING	2-1

202	DISMANTLING CULVERTS, BRIDGES, OTHER STRUCTURES AND PAVEMENT	2-2

SECTION 300: SOIL IMPROVEMENT

301	REINFORCED SOIL	3-1

SECTION 400: FENCING

406	MEASUREMENT	4-1

407	PAYMENT	4-1

SECTION 500: QUALITY CONTROL

501	SCOPE	5-1

502	CONTRACTOR RESPONSIBLE FOR THE QUALITY OF THE WORKS	5-1

509	ACCEPTANCE TESTS FOR COMPLETED WORKS OR PARTS OF THE WORKS	5-1

511	LABORATORY	5-2

SECTION 600: MATERIALS AND TESTING OF MATERIALS

607	TRADE NAMES AND ALTERNATIVES	6-1

612	SOIL AND GRAVELS	6-1

613	STONE, AGGREGATE, SAND AND FILLERS	6-1

627	GEOTEXTILES AND GEOGRIDS	6-1

SECTION 700: PIPE DRAINS, PIPE CULVERTS AND CONCRETE CHANNELS

701	PIPE DRAINS AND PIPE CULVERTS	7-1

702	CONCRETE CHANNELING	7-2

SECTION 800: QUARRIES, BORROW PITS, STOCKPILE AND SPOIL AREAS

802	DEFINITIONS	8-1

803	LOCATING MATERIAL SOURCES	8-2

SECTION 900: EARTHWORKS

902	DEFINITIONS AND GENERAL REQUIREMENTS	9-1

904	EXPLOSIVES AND BLASTING	9-2

905	EXCAVATION IN CUTTING	9-3

907	EXCAVATION FOR FOUNDATION	9-4

909	FORMING OF EMBANKMENT AND OTHERS AREAS OF FILL	9-5

910	COMPACTION OF EMBANKMENT AND OTHER AREAS OF FILL/BACKFILL	9-5

917	MEASUREMENT	9-6

918	PAYMENT	9-7

SECTION 1000: SUBGRADES

1002	DEFINITIONS	10-1

1007	MEASUREMENT	10-2

1008	PAYMENT	10-2

SECTION 1100: ROADWORKS OVERALL REQUIREMENTS

1104	CARE OF WORKS	11-1

SECTION 1200: SUBBASE, BASE, HARD SHOULDEr AND GRAVEL WEARING COURSE

1204	SHOULDERS	12-1

1208	CRUSHED AGGREGATE SUBBASE AND BASE	12-1

SECTION 1300: BITUMINOUS SURFACE AND BASE COURSE

1302	PRIME COAT AND TACK COAT	13-1

1308	ASPHALT CONCRETE	13-1

SECTION 1400: KERBS AND FOOTPATHS

1401	CONCRETE KERBS	14-1

1402	CONCRETE/STONE FOOTPATHS	14-2

SECTION 1500: TRAFFIC SIGNS, ROAD MARKING, ROAD MARKER STONES 
AND DELINEATORS

1501	PERMANENT TRAFFIC SIGNS	15-1

1503	ROAD MARKER STONE	15-1

1505	OVERHEAD SIGN	15-1

1506	THERMOPLASTIC ROAD MARKING	15-4

1507	REFLECTIVE PAVEMENT MARKERS (ROAD STUDS) AND SOLAR POWERED ROAD MARKERS (SOLAR STUDS)	15-9

1508	STEEL CRASH BARRIER	15-12

1509	POLYETHYLENE TANK CRASH BARRIER	15-17

SECTION 1600: PILING FOR STRUCTURES

1618	SHINSO PILES (CAST-IN-SITU PILE BY MANUAL EXCAVATION)	16-1

SECTION 1900: BEARINGS AND EXPANSION JOINTS

1901	EXPANSION JOINTS	19-1

1902	BEARINGS	19-1

SECTION 2000: CONCRETE FOR STRUCTURES

2003	MATERIAL FOR CONCRETE	20-1

2004	DESIGN OF CONCRETE MIXES	20-1

2005	MIXING CONCRETE	20-3

2007	PLACING OF CONCRETE	20-4

2017	TEST AND STANDARD OF ACCEPTANCE	20-4

2019	PAYMENT	20-7

2050	CONCRETE PAVEMENTS	20-7

2051	CONTINUOUSLY REINFORCED CONCRETE PAVEMENT (CRCP) AND JOINTED REINFORCED CONCRETE PAVEMENT (JRCP)	20-7

2052	MATERIALS	20-8

2052	EQUIPMENT	20-9

2053	CONSTRUCTION	20-11

2054	MEASUREMENT	20-23

2055	PAYMENT	20-24

2060	CEMENT TREATED BASE COURSE (CTBC)	20-25

SECTION 2400: MISCELLANEOUS STRUCTURES

2401	GABIONS	24-1

2402	CAST-IN-PLACE CRIB WORKS	24-2

2404	SUB-SURFACE DRAINS	24-4

2407	GEOSYNTHETICS FOR ROAD AND BRIDGES WORKS	24-5

2408	GEOTEXTILE FOR FILTRATION, DRAINAGE AND SEPARATION	24-8

2409	GEOGRID	24-12

2410	GEO-COMPOSITE DRAINS	24-14

2411	RIVERBED PROTECTION	24-16

2412	SHOTCRETE FOR SLOPE PROTECTION	24-17

2413	DRY STONE PACKING AND SOLING	24-21

SECTION 2600: MASONRY FOR STRUCTURES

2611	MEASUREMENT	26-1

2612	PAYMENT	26-1

SECTION 2800: BIO ENGINEERING WORKS

2801	GENERAL	28-1

2806	FINAL SLOPE PREPARATION FOR BIo-ENGINEERING	28-1

2807	SITE PLANTING AND SOWING	28-2

2811	SITE AFTERCARE AND MAINTENANCE	28-4

2812	TURFING WITH SODS	28-4

2813	TOPSOIL	28-6

SECTION 3100: MISCELLANEOUS WORKS

3105	RAILINGS	31-1

3111	RC BRIDGE DRAINAGE	31-1

3112	PVC WATER-STOPS	31-3

3120	STREET LIGHTING SYSTEM (SLS)	31-7

3130	TRAFFIC INFORMATION BOARDS (TIB)	31-15

SECTION 6000: SUPPLYING OF O&M EQUIPMENT

6001	DESCRIPTION	60-1

6002	MEASUREMENT	60-3

6003	PAYMENT	60-3

SECTION 6100: O&M FACILITIES AND BUILDINGS

6101	DESCRIPTION	61-1

6102	Standards	61-1

6103	Construction Method, Design Review and Updating	61-1

6104	MEASUREMENT	61-2

6105	PAYMENT	61-2

SECTION 6300: TOLL GATE AND FACILITIES

6301	DESCRIPTION	63-1

6302	MATERIAL	63-2

6303	CONSTRUCTION	63-3

6304	WORKS’ REQUIREMENTS FOR TOLL FACILITIES	63-4

6305	MEASUREMENT	63-5

6306	PAYMENT	63-6

SECTION 6500: TRANSMISSION LINE

6501	DESCRIPTION	65-1

6502	STANDARDS	65-1

6503	CONSTRUCTION METHOD AND DESIGN REVIEW AND UPDATING REPORT	65-2

6504	GENERAL OPERATING CONDITIONS	65-2

6505	MATERIALS AND CONSTRUCTION REQUIREMENTS	65-2

6506	MEASUREMENT	65-24

6507	PAYMENT	65-24

SECTION 7000: Roadside Service Station

7001	DESCRIPTION	70-1

7002	STANDARDS	70-1

7003	CONSTRUCTION METHOD AND DESIGN REVIEW AND UPDATING REPORT	70-1

7004	MEASUREMENT	70-1

7005	PAYMENT	70-2

SECTION 7500: STEEL BRIDGE FOR FLYOVER

7501	DESCRIPTION	75-1

7502	MATERIALS	75-1

7503	CONSTRUCTION METHOD AND DESIGN REVIEW AND UPDATING REPORT	75-2

7504	MEASUREMENT	75-2

7505	PAYMENT	75-3

VOLUME-II: TUNNELLING WORKS AND FACILITIES

Section 4000:	TunnelLing Works

Section 5000:	Tunnel Facilities

Section 5100:	Tunnel Power Supply System

Section 5200:	Tunnel Lighting System

Section 5300:	Tunnel Emergency Facilities

Section 5400:	Remote Monitoring and Control System

Section 5500:	Tunnel Ventilation System

Section 5600:	Tunnel Fire Fighting System

SecTION 5700:	MEASUREMENT AND PAYMENT

Section VI-B:

Standard Specifications (SS)

The Standard Specifications for this contract shall be the “Standard Specifications for Road and Bridge Works – ASAD 2058”, edited by the Department of Roads, Ministry of Physical Structure and Transport, Government of Nepal.

The Standard Specifications may be purchased from the Department of Roads, Central Road Laboratory, Kupondole, Lalipur.

The Standard Specifications are linked with the Special Provisions to Standard Specifications (SPSS) referred to as Section VI-C by the corresponding numbering of Clauses so that Sections VI-B and VI-C together comprise the specifications that shall be respected for performing the works of the Project.

Section VI-C:

Special Provisions to the Standard Specifications (SPSS)

The Special Provisions to Standard Specifications are set out below (the Clauses of the standard specifications that were not modified by these provisions shall remain and shall be respected).

The Special Provisions (Volume-I and Volume-II) contained herein shall be read in conjunction with the Standard Specifications and shall supplement, replace or supersede the Standard Specifications as appropriate. Where there is any ambiguity or discrepancy between the Standard Specifications and the Special Provisions; the Special Provisions shall have preference, prevail and shall govern.

SECTION 100: GENERAL

101	OFFICES FOR ENGINEER

(1)	Delete the full Sub-Section 101 and replace with the following:

101	SITE OFFICES FOR THE EMPLOYER

(1)	Description

This work consists of the provision of the Site office for the Employer to be used by the Employer and the Engineer during the construction supervision stage.

(2)	Requirements

Simultaneously with the preparation of the Contractor’s facilities on Site, the Contractor shall provide with Site Offices (one for Western Portal Site and one for Eastern Portal Site) for the use by the Employer’s Site Representatives, and for the Engineer, in accordance with the following characteristics:

| Site Office | Tentative Location | Period (months) | Capacity (No. of Staff) |
| --- | --- | --- | --- |
| Western Portal | Nearby the Contractor’s office and within 500m from the Site Laboratory | 42 | - Employer Representatives - Engineer’s Team Leader (T/L), Foreign and Local Experts, and Supporting Staff |
| Eastern Portal |  | 42 |  |

Note: The layout shall be prepared by the Contractor and submitted for review and approval by the Engineer. The locations are tentative and shall be discussed and confirmed with the Engineer.

The basic layout of each site office shall be prepared by the Contractor and submitted to the Engineer for review and respective approval.

The building shall comply with Nepal’s applicable specifications for architectural and structural works for buildings.

Basic Layout:

The layout shall be prepared in accordance with the number of staff indicated in the table shown above to accommodate properly the following (note: the plan area shown below are the minimum requirement to be considered):

- Minimum Area: (total building area excepting parking) 	260 sqm

- One office room for the Employers’ representatives	30 sqm

- One office room for Project Manager of the Engineer	20 sqm

- One office room for Foreign and Local Experts	50 sqm

- One weekly meeting Room capacity for 10 people (Western Site)	50 sqm

- One Working Room capacity for 10 people (Eastern Site)	50 sqm

- One reception/administration office room	20 sqm

- A kitchen (Pantry) plan area well equipped with sink, 
draining board, cupboards, shelving, etc.	20 sqm

- Male (2) and Female (1) toilets with shower and wash-hand
 basin facilities for the sole use of the Employer’s representative, 
Foreign Experts, Local Experts, and administration staff 
(office manager and secretaries) ( 1unit=7sqm)	3 units

- Male (1) rest room with 3 toilets and one common 
wash-hand basin facilities for Supporting staff  (1unit=8sqm)	2 units

- Corridor for connecting all rooms (1.5m width)	40 sqm

- Parking area 	10 veh.

Basic Requirements:

The Contractor shall, not later than 7 days after the Commencement Date, submit full details of the Design Drawings to the Engineer, including floor plans, elevations, construction principles and materials, before commencing the erection of the facilities.

The Contractor shall be responsible for raising the ground (if necessary), grading and drainage in the vicinity of the building(s), with suitable access and walkways.

The Contractor shall construct a covered hard-standing parking area, for the exclusive use of the Employer, the Engineer and his visitors and respective access road to the parking area. The access road shall be paved and hard enough in order to be transited even during heavy rains. Outside lighting shall be installed around the buildings and the parking area, and appropriate signs shall be erected to indicate the purpose of the facilities.

All facilities shall conform to current fabrication standards for the required types. The facilities described above shall represent the minimum requirements. The Contractor shall provide all incidentals and necessary items, so that the facilities will be completely adequate and satisfactory in every respect for their intended use. Painting both the exterior and the interior shall be as agreed with the Engineer.

The building shall be complete with all services connected with clean water supply by water bowser (including elevated water tank and the tower for capacity of 3,000 liters), electricity and sewerage. Each room shall be provided with at least four electrical outlet sockets. All power shall be 220V-240V, 50Hz except where otherwise agreed by the Engineer. All rooms shall be illuminated by fluorescent lighting. Each toilet shall be provided with flushing toilets and warm water hand washing and shower facilities and extractor fans.

Offices and meeting room shall be air-conditioned. The air-conditioning may be either individual units or a central ducted system and shall be adequate to maintain temperature around 24°C (in summer or winter ) at a relative humidity of 50% during the hottest season of the year. The noise level of the air-conditioning while working should be sufficiently low to allow normal voice level discussions to take place.

Office rooms shall be capable of providing at all times environmental conditions suitable for the operation of specified electronic office equipment.

The building shall be weather proof, fire protected, heat-insulated and secured. Windows shall give adequate light and ventilation and be protected with metal mosquito-proof gauze and have security bars and Venetian, or other approved sun blinds. Ceiling height above the floor level shall be at least 2.75 m. All internal walls shall be sound insulated. Floors shall be PVC tile covered. In toilets and other washing areas the floors shall have drains to assist cleaning.

Maintaining and Servicing of the Offices:

The construction of the new offices shall be completed within 60 days since the date of commencement of the Contract and shall be equipped and maintained by the Contractor to the satisfaction of the Engineer until 3 months after the issue of a Completion Certificate for the Works or such earlier time as instructed by the Engineer whereupon the furniture shall be removed and any internal partition walls modified as required by the Engineer.

The Contractor shall provide all laborers, materials and equipment for maintaining and cleaning offices, furniture and fittings. The Contractor shall replace and/or restore, as directed, any facilities or parts thereof that become damaged, worn out, lost or stolen. The Contractor shall provide an adequate stock of all expendable and consumable items including refreshments, clean water supply and drinking water, paper towels, toilet rolls, soaps, washing up liquid, brooms/mops and shall ensure proper and continuing functioning of all components and parts of the facilities during the contract period.

The Offices shall be provided with waste disposal material and these shall be emptied and disposed of daily by the Contractor.

Equipment and Expenses for Running-Off the Offices for Engineer

The Contractor shall provide the following furniture and equipment (one set per each office):

| Item | Unit | Quantity | Remarks |
| --- | --- | --- | --- |
| Infocus + Screen | set | 1 | Large Size for meetings |
| Binding Machine A4 Size | unit | 1 | Type to be discussed |
| Stationary, Spears, Tonners, Consumables, etc. | month | 42 | To be confirmed in monthly basis |
| Desk + Chair + Side Table for experts | set | 10 | Type and number to be discussed |
| Desk + Chair + Side Table for Administrative Staff | set | 2 | Type to be discussed |
| Desk + Chair + Side Table for Supporting Staff | set | 2 | Type of working table shall be confirmed |
| Furniture for Conference Room (West) | set | 1 | Type to be discussed |
| Furniture for Meeting Room (East) | set | 1 |  |
| Medium Size Wall White Board | set | 1 |  |
| Refrigerator (deodorizer and no-frost system) | unit | 1 | 300liters’ minimum capacity |
| Water Dispenser (19 liters capacity) | set | 2 | including bottles and 6 spares bottles / gallon |
| Electric Kitchen Stove | unit | 1 | Size and model shall be discussed |
| Electric Kettle | unit | 1 |  |
| Diesel Generator Set (125KVA) | unit | 2 | Covering more than 10hr/day and backup |

All of these equipment and furniture provided by the Contractor for the Engineer and operation of the Engineer’s Office shall remain as the property of the Contractor during and after the Project completion, and shall be removed from the Site after termination of contract period. The Contractor shall accept any conditions of these equipment and furniture returned to him after the completion of the Project and shall not be entitled to claim the Engineer to remedy any defect of the equipment.

(3)	Measurement and Payment

Excepting the Items shown here below, no separate measurement nor payment shall be made for the works described in this Section.

The works for provision of the “Site Offices for the Employer” shall be measured and paid in “lump sum” in split instalment shown here below, which shall be the full and the final compensation to the Contractor as per Clause 112 including all costs for providing and installation of the site offices as herein described, and for the cost of all operations, materials, equipment, and any incidentals involved for completion of this item.

Payment shall be made on a “lump sum” according to the following instalments for each Portal Site:

(i)	Twenty percent (20%) upon completion of the Site offices and all installations herein described and approved by the Engineer.

(ii)	Fifty percent (50%) during the execution of the Works split in partial payments in proportion to the work progress subject to certification of achievement issued by the Engineer.

(iii)	Thirty percent (30%) upon issuance of the Taking-Over Certificate.

| Item No. | Description | Unit |
| --- | --- | --- |
| 101 | Site Offices for the Employer |  |
| 101-1 | Site Office for the Employer at Western Portal Site | lump sum |
| 101-2 | Site Office for the Employer at Eastern Portal Site | lump sum |

102	ACCOMMODATION OF TRAFFIC

(3)	Temporary Diversions

(2)	Insert the following as per the 2nd and 3rd paragraphs of Sub-Clause 102 (3) (a) “Provision of Diversions”:

The works for diversions should include the construction and maintenance of temporary road and temporary access roads to bridge construction sites, temporary safety barriers and fencings for traffic safety and stopping spilling of excavated material as per the specifications and instruction of the Engineer.

The Contractor shall secure the compliance with traffic management and safety provisions.

(3)	Delete the full text of Sub-Clause 102 (3) (j) “Removal of Diversions” and replace with the following:

(j)	Removal of Diversions

When traffic is routed permanently onto the new road following the completion of construction, the diversions which are no longer required shall be removed and the ground reinstated to its original condition. This work shall include the establishment and maintenance for at least one year of suitable replacement vegetation to ensure full rehabilitation and protection of the disturbed area.

(4)	Accommodation of Traffic Where Road is Constructed in Half Widths

(4)	Delete the third and fourth paragraph of Clause 102(4) “Accommodation of Traffic where Road is Constructed in Half Widths” and replace with the following:

Contractor shall ensure that the road is opened for the traffic full time. Should the road not be in a safe trafficable condition for two-way traffic over the whole width, at the end of each day’s work the Contractor shall adequate flagmen, signs, barriers, and the necessary staff at his own cost to ensure a reasonable free flow of traffic alternately in each direction.

The Contractor shall provide and maintain for the period of construction, traffic control and safety devices including Traffic Signs, Barricade, Boards, Traffic Cones, Lighting Devices etc., at all locations where work is in progress in accordance with “Safety at Road Works: A Code of Practice for Signing at Road Works” from the Department of Roads Traffic Signs Manual.

Material and equipment temporarily stored on, or adjacent to, the existing roadway shall be so placed, and the work at all times, shall be so conducted as to cause minimum disruption to the travelling public. Warning signs and barriers will be required to separate the Contractor’s material and equipment from the public.

The Contractor will not be permitted to have excavations open on both sides of the road at a particular chainage such that there is a step adjacent to public traffic which may create a danger to traffic, i.e. the excavation and backfilling with sub-base or base shall be flush with the existing road level on one side before excavation can proceed on the other side of the road.

During construction for operations which require one way traffic the Contractor shall be responsible for the removal of broken down vehicles including vehicles damaged in accidents and shall maintain one way uninterrupted traffic flow at all times (except at those times as agreed with the Engineer where it is necessary to temporarily close the road).

The Contractor shall program his work so as to reduce disruption to all road users during construction of the Works and maintain traffic control measures for the safety of traffic at all times.

The Contractor shall propose the working procedures which will be subject to the Engineer’s approval before implementation.

The approval of the Engineer shall be required before traffic is allowed to pass over newly constructed road. Any defects caused by traffic passing over the construction layers shall be remedied in accordance with these Specifications.

The Contractor shall be fully responsible for traffic control.

(5)	Add the following new Clause 102(5) “Installation of Provisional Guard Fence” and renumber the remaining Clauses as follows:

(5)	Installation of Provisional Guard Fence

This work shall consist of the installation and on-completion of the work removal of provisional guard fence as described in Section 900: “Earthworks”, Clause 905(2): “Installation of Provisional Guard Fence”.

The works for Traffic Management at KM -0+240 to 0+240 along Tribhuvan Highway shall be performed in accordance with the requirements of Clause 905(2): “Installation of Provisional Guard Fence” as shown in the Drawings.

Renumbering:

(5) Traffic Safety and Control  → 	(6) Traffic Safety and Control

(6) Measurement and Payment  → 	(7) Measurement and Payment

(7)	Measurement and Payment

(6)	Delete the full text of the renumbered Clause(7) “Measurement and Payment” and replace with the following:

Excepting for the items shown here below (and shown in the Drawings) and unless stated otherwise in the Contract, no separate measurement and /or payment shall be made for all materials and works required under this Clause. All costs in connection with the works specified in this Section shall be considered to be deemed included in the rates of the relevant main items of the Bill of Quantities.

The works comprised by the Item No.102-1 to No.102-5 shall be measured per unit, and shall be paid in “lump sum” which shall be the full and the final compensation to the Contractor and for the cost of all operations, materials and incidentals involved for completion of this item.

The unite rate established for payment of Item No.102-1 shall consider the rescue value of the temporary fence provided and constructed as shown in the Drawings, which shall be removed after work’s completion, and will belong to the Contractor.

| Item No. | Description | Unit |
| --- | --- | --- |
| 102 | Accommodation of Traffic |  |
| 102-1 | Traffic Management at KM -0+240 to KM 0+240 along Tribhuvan Highway | lump sum |
| 102-2 | Traffic Management at KM 3+160 | lump sum |
| 102-3 | Traffic Management at KM 4+390 | lump sum |
| 102-4 | Traffic Management at KM 5+000 | lump sum |
| 102-5 | Traffic Management at KM 5+340 Tribhuvan Highway | lump sum |

103	PUBLICLY AND PRIVATELY OWNED SERVICES

(7)	Delete the full text of Clause 103 (7) and replace with the following:

- The Contractor shall ascertain the location of all access paths, watercourses, irrigation channels, sewers, drains, water pipes, electricity and telecommunication cables and other services and structures which may be encountered during the execution of the works. He shall temporarily support or divert and subsequently reinstate all such services and structures as necessary and to the satisfaction or the Engineer. As soon as any such service or structure is encountered on, over, under, in or through the Site during the performance of the Contract, the contractor shall make a record of the location and description of such service or structure and shall send the same forthwith to the Engineer.

Where permanent diversion or support of such service or structure is rendered necessary as an unavoidable result of the execution and maintenance of the Works in accordance with the Contract, the Engineer after consultation with the Employer will instruct the Contractor as to the diversion or support to be provided. All operations necessary for the execution of the Works shall be carried out so as not to interfere unnecessarily or improperly with the convenience of the public, or the access to, use and occupation of public or private roads and footpaths or of properties whether in the possession of the Government or any other person.

- Unless stated otherwise, no separate measurement nor payment shall be made for the work of temporarily supporting and protecting the publicly and privately owned services and/or for construction and maintenance of new services in lieu of them.

104	SURVEY AND SETTING OUT

(8)	Insert the following as per the 2nd paragraph of Clause 104 (6):

If at any time the Contractor believes that there exists a discrepancy between the location of the works as defined by the setting out and the apparent location of the works as shown in the Drawings he shall immediately inform the Engineer and request clarification.

(9)	Delete full text of Clause 104 (7) and replace with the following:

The Contractor shall check the existing profile and cross sections against the survey data and, should any changes be found necessary in the Survey Traverse Points and Bench Marks, after the procedures required in Clauses (1) and (2) above, the existing data set shall be amended accordingly. Should the Engineer consider that design modifications of the centreline and/or gradient is relevant or if the Contractor find that the existing survey data set is inadequate or inaccurate in any area of the works, the Engineer shall issue detailed instructions to the Contractor and the Contractor shall perform the necessary modifications in the field, as required, and modify the cross sections accordingly.

The final survey data set so obtained in combination with the final horizontal and vertical alignment of the road shall form the basis for the measurements and payments of earthworks. Measurement shall be either based on a Digital Terrain Model (DTM) generated from the agreed survey data set with the final designed road horizontal and vertical alignment and road cross section, applied or any other method approved by the Engineer. In case of DTM, the model will be used to generate cross-sections complete with cut and fill data at intervals not exceeding 2.50 meters in the hill areas. Measurement of volumes and areas of earthworks items shall be derived from these cross sections.

The software currently used for the preparation of survey and design data is SW-Roads developed by Softwel (P) Ltd. of Kathmandu Nepal. Should the Contractor wish to use alternative software he may propose this to the Engineer. He will be required to demonstrate that his proposed alternative is at least as accurate as and is functionally equivalent in all respects to SW-Roads. Should the Engineer agree to the use of the alternative software, the Contractor shall be entirely responsible for the adaptation. Under the supervision of the Engineer. Of all input data to the alternative software requirements and for the training of the Engineer’s personnel in the use of the alternative software. In the event that alternative software is approved the Contractor shall supply the Engineer with two, fully licensed, copies of the alternative for the exclusive use of the Engineer.

106	DIVERSION BRIDGES

(10)	Delete the first sentence of the Clause 106 (1) and replace with the following:

- Temporary diversion bridges shall be provided by the Contractor, upon previous approval by the Engineer, wherever required for the execution of the Works to maintain traffic.

(11)	Delete the full text of Clause 106 (3) and replace with the following:

- On completion of the works, the Contractor shall obtain the approval of the Engineer before removing the temporary bridges. Unless otherwise required by the Employer, the materials arising from the removal shall become the property of the Employer and they shall be removed to the Employer’s premises and neatly stacked at no cost to the Employer. If it is not so required, the Contractor shall remove the materials to an approved disposal area at no cost to the Employer.

(4)	Measurement and Payment

(12)	Delete the full text of Clause 106 (4): “Measurement and Payment” and replace with the following:

No separate measurement nor payment shall be made for the work required under this Sub-Section. All costs in connection with the works specified in this Sub-Section shall be considered to be deemed included in the rates of the relevant main items of the Bill of Quantities.

107	MAINTENANCE OF ROAD

(13)	Delete the fourth (4th) paragraph of the Clause 107 (1) and replace with the following:

The Contractor shall perform the maintenance works as often as required to keep the carriageway, shoulders, adjoining structures and roadside slopes stable and in proper working order as required by the Engineer. The Contractor shall strictly adhere to the stipulations of the latest publication of the document “Road Maintenance Manual” published by the Department of Roads.

The basic criteria and requirements for materials and construction specified in Section 2900: “Maintenance Works” shall be applied in general for these works, as instructed by the Engineer. Particularly, the Sub-Section 2905: “Bituminous Patch Works”, Clause (2) Bituminous Patching shall be applied for those road sections or surfaces identified by the Engineer.

“Crusher-run base” shall conform the requirements for a base course specified in Sub-Section 1202: “Graded Crushed Stone Base and Sub-Base”.

(2)	Measurement and Payment

(14)	Delete the full text of Clause 107(2) “Measurement and Payment” and replace with the following:

“Maintenance of road during construction period” as well as “Maintenance of road in Defects Notification Period” both quantified in kilometer-months shall be paid as per contract price on lump sum basis per month.

Bituminous Patching (Shallow and Deep) shall be measured in square metres of the areas actually repaired in accordance with these specifications, and as instructed by the Engineer.

“Crusher-run base” shall be measured in cubic metres of the sections actually repaired in accordance to the specifications and as instructed by the Engineer.

The contract price and/or unit rate shall be the full and the final compensation to the Contractor as per Clause 112.

| Item No. | Description | Unit |
| --- | --- | --- |
| 107 | Maintenance of Roads |  |
| 107-1 | Maintenance of existing roads from KM-0+220 to Sisne Khola clossing and Tribuvan Rajpath in Balambu interchange during construction period | month |
| 107-2 | Maintenance of existing roads from KM-0+220 to Sisne Khola clossing and Tribuvan Rajpath in Balambu interchange during Defects Notification Period | month |
| 107-3 | Shallow Patching | sq.m. |
| 107-4 | Deep Patching | sq.m. |
| 107-5 | Crusher-run base | cu.m. |

108	NOTICE BOARD

(15)	Delete the full text of Sub-Section 108: “Notice Board” and replace with the following:

- The Contractor shall erect, where directed by the Engineer or as shown in the Drawings, information boards (1.8m x 1.2m) at each end of the site giving details of the contract in the format and wording as directed by the Engineer. These boards shall be erected within 14 days after the Contractor has been given the Possession of Site.

The Contractor shall not erect any advertisement on or along the Project Site without the written approval by the Employer.

All information boards shall be removed by the Contractor by the end of the Defects Notification Period.

(2)	Measurement and Payment

Project signboards shall be measured as the number of signboards, satisfactorily provided, installed, maintained throughout the Contract period and subsequently removed. Payment shall be made at the stated unit rate per signboard. The contract price shall be the full and final compensation as per Clause 112 for all materials and labour required to perform the work described.

| Item No. | Description | Unit |
| --- | --- | --- |
| 108 | Notice Board |  |
| 108-1 | Provision and installation of Project Signboards | No. |

109	ENVIRONMENTAL PROTECTION WORKS

(16)	Delete the third (3rd) and fourth (4th) paragraphs of the Section 109: “Environmental Protection Works” and replace with the following:

The Contractor shall take full responsibility for performing the work minimizing environmental pollution and damage that can occur pursuant to his construction operations.

The Contractor shall develop and implement a site-specific Environment Management Action Plan (EMAP) in complying with all applicable government/ local laws and regulations in the Country as well as the Environmental Impact Assessment (EIA) and Initial Environmental Examination (IEE) reports approved by Ministry of Population and Environment (MoPE), of this Project, which describe the requirements for environmental protection during the entire duration of this contract. The Contractor shall be primarily responsible and accountable for the actions and activities of his subcontractors and suppliers, and for their compliance with the EMAP.

The Contractor shall assume the responsibility of securing all the necessary licenses, permits, clearances and their attendant costs and fees prior to start of any construction activities. The Contractor may request the assistance of the Employer/ Engineer thereon on a best endeavor basis. The Contractor shall prohibit employees from any use of explosives, poaching of wildlife and cutting of trees or branches. The Contractor shall be responsible for the action of his employees.

The Engineer will notify the Contractor in writing of any observed noncompliance with relevant laws, regulations, permits and other elements of the Contractor’s EMAP. The Contractor shall, after receipt of such notice, inform the Engineer of proposed corrective actions and take such actions, at his own cost, immediately when approved. If the Contractor fails to comply promptly, the Engineer may issue an order stopping all or part of the work until satisfactory corrective action has been taken. No time extension therefore shall be granted to the Contractor for any such suspensions.

The Contractor shall provide and maintain at his own cost the personnel, equipment and facilities necessary for a fulfilment of the requirements of the Environment Protection.

Submittals:

The Contractor shall submit the own EMAP to the Engineer for his approval within twenty-eight (28) calendar days after the date of the Letter of Acceptance. Any construction operation shall not begin until the EMAP has been approved by the Engineer. The EMAP shall include a series of management plans including (but not limited to) the following main items:

•	Sediment and erosion control plan for each Site;

•	Camps management plan for each camp;

•	Waste management plan;

•	Pollution control (water, air, noise) plan for each Site;

•	Traffic management plan;

•	Standard Operating Procedures for pollution spills, and management of fuels and hazardous substances.

The above referred plans shall be compiled by using the following information:

a)	Organization and lines of responsibilities including subcontractors and suppliers

b)	Methods of protection of features to be preserved such as trees/vegetation, soil, landscape features, air and surface/ ground water, fish and wildlife, historical, archaeological and cultural resources.

c)	Procedures to be implemented to provide the required environmental management, to comply with the applicable laws and regulations, and to correct pollution due to accident, natural causes, or failure to follow the procedures of the EMAP

d)	Drawings showing locations of any proposed temporary excavations or embankments for haul roads, stream crossings, material storage areas, stockpiles of excess or spoil materials and sanitary facilities and solid waste disposal areas。

e)	Training for Contractor’s personnel during the construction period.

The approval by the Engineer shall not relieve the Contractor of any of his responsibilities for the Environment Protection pursuant to his construction operations.  Furthermore, the Engineer shall have the right to require the Contractor to improve the approved EMAP whenever it is deemed necessary in the opinion of the Engineer.

Environment Monitoring Report

The Contractor, within 7 days after the last day of the period to which it relates, shall submit the Environmental Monitoring Report in monthly basis to the Engineer for his review.

Reporting shall continue until the Contractor has completed all work which is known to be outstanding at the completion date stated in the Taking-Over Certificate for the Works.

The Report shall include the items of environmental monitoring requested in this Section.

Relevant Statutes:

The Contractor shall at all times comply with all existing statutes in the Country concerning environmental protection, pollution control and abatement that are applicable to his construction operations and the JICA Guidelines for Confirmation of Environmental and Social Considerations (2010)

Assignment of Environment Protection Manager (EPM):

The Contractor shall designate an environment protection manager (EPM) with his staff.  The name and qualifications including education and work experiences of the EPM shall be submitted for approval of the Engineer prior to the appointment.

Environmental Monitoring

The Contractor shall perform environmental monitoring for the duration of this contract and submit results to the Engineer. The minimum requirement for items, frequency and number of locations of sampling or measurement shall be as indicated here below, and the detail shall be confirmed in the latest approved EIA report.

Frequency and Locations of Environmental Monitoring

| Item | Monitoring Indicator | Place of Monitoring | Frequency(Period) |
| --- | --- | --- | --- |
| Air Pollution | NO2, PM10 | (1) West Portal), (2) East Portal, (3) East end point of the east approach road | 1 measurement in every month or urgent request / complain from local residents |
| Water pollution | BOD, TSS | (1) Sisne Khola , downstream of Soil Disposal Area, (2) Balkhu Khola, culvert bridge near medical college , downstream of east approach road work area | 1 measurement in every month or urgent request / complain from local residents |
| Noise pollution | Noise | Just outside of the following work area: (1) Tunnel: West Portal, (2) Tunnel: East Portal, (3) Surface: West Approach Road / Soil Disposal Area, (4) Surface: East Approach Road | 1 measurement in every month or urgent request / complain from local residents |
| Waste | - Soil Waste | Construction work areas Soil disposal site | Once in every month |
|  | - Solid Waste | Contraction work areas | Once in every month |
|  | - Liquid Waste | (1) West Plant discharge point (2) East Plant discharge point |  |
| Water regime | - Surface water - Ground water - Springs | Construction work areas Soil disposal site Adjacent springs | 1 measurement in every month or urgent request / complain from local residents |
| Geology | Slope stability | Construction work areas Soil disposal site | Every day (morning and evening) |
| Land use | - Land use, Local resource use, - Communal/Common resource use right | Construction work areas Soil disposal site | As occurrence of issues |
| Water Rights / Water use | - Surface water resource - Ground water resource - Spring water resource | Construction work areas Soil disposal site Adjacent springs | 1 measurement in every month or urgent request / complain from local residents |
| Work Related Issues | - Sanitation, - Public Health condition, - Infectious disease such as HIV/AIDS - Industrial Safety and Health - Traffic accidents - Crime | Construction work areas Soil disposal site | As occurrence of issues |

Note: the number and frequency only shows the minimum requirements.

The environmental protection works, among others, shall also include the following:

(1)	Borrow / Quarry Sites

(17)	Delete the fifth (5th) paragraph of Clause 109(1): “Borrow/Quarry Sites” and replace with the following:

The Contractor shall obtain approval from the Engineer of all material sources before extraction, and before opening up any borrows pits or quarries. All sources of materials used under the Contract shall require such approval regardless of whether the material is obtained directly by the Contractor or supplied indirectly by others. The Contractor is not allowed to use the right of way for the borrow pit.

Such borrow pits and quarries may be prohibited or restricted in dimensions and depth by the Engineer where:

(18)	Delete the paragraph of Sub-Clause 109(1) (iii): “they may be environmentally unsuitable” and replace with the following:

(iii)	they may lead to general environmental or instability problems or may be environmentally unsuitable in any other way.

(19)	Insert the following as per the last paragraph of Clause 109 (1): “Borrow/Quarry Sites”:

The Contractor shall not deposit excavated material on land in Government or private ownership except where approved by the Engineer in writing or by leave in writing of the authority responsible for such land in Government ownership or of the owner or responsible representative of the owner of such land in private ownership and only then in those places and under such conditions as the authority, owner or responsible representative may prescribe.

Any quarry operated as part of this Contract shall be maintained and at the end of the Contract left in a stable condition without steep slopes and be either refilled or drained and be landscaped by appropriate planting.  Rock or gravel won from a river shall be removed over some distance so as to limit the depth of material removed at any one location and not to disrupt the river flow or damage or undermine river banks.

(2)	Disposal of Spoil and Construction Waste

(20)	Delete the second (2nd) paragraph of Clause 109(2) “Disposal of Spoil and Construction Waste” and replace with the following:

Under no circumstances whatsoever shall the Contractor allow spoil from any excavation or cut to fall or be pushed down the slope below. In any situation where this occurs by accident or by purpose, the Contractor shall, at his own cost, remove the debris to a location approved by the Engineer and make good the affected area through appropriate structures and re-vegetation as specified by the Engineer and following guidelines published in the Department of Roads’ manual “Roadside Bio-Engineering”. Failure in compliance with the Engineer’s instruction in respect of spoil disposal will lead to a reduction or with-holding of payment.

The Contractor shall plan his works in such a way that there is no spillage of POL products to the surface or sub-surface water.

Any construction waste that includes bitumen products, unused cement, fuel, oil or plastics shall be disposed of in a Government-approved landfill site. If this is not practical then it may be disposed of by burial in a specially excavated pit at a site approved by the Engineer. Land compensation shall be paid by the Contractor for the area occupied by the pit as if for permanent construction. If the land is owned by the Government, then the Contractor must obtain written permission from the appropriate authority for its use in this manner, and must pay any royalties due. The pit shall be located at least 50 metres from any seasonal or permanent water course or spring. A hole shall be excavated below ground level so that, when the original surface profile is regained through backfilling, the disposed waste is covered by at least two metres of soil. Following the closure of the waste disposal site, the Contractor shall provide any structures necessary to prevent erosion and shall re-vegetate the topsoil with appropriate plants.

The Contractor shall treat the vegetative wastes and construction wastes separately at the dumps approved by the relevant local authorities and the Engineer.  The Contractor shall be responsible for making necessary arrangements with private parties and with the relevant local authorities for the location of such dumps.

(3)	Provision and Maintenance of Camps, Offices, Stores, Equipment Yards and Workshops

(21)	Insert the following after the first (1st) paragraph of Clause 109(3) “Provision and Maintenance of Camps,… etc.”:

The Contractor shall provide and maintain such accommodation and amenities as necessary for all his staff and labour, employed for the purposes of or in connection with the Contract, including all fencing, water supply (both for drinking and other purposes), electricity supply, sanitation, cookhouses and other requirements in connection with such accommodation or amenities. On completion of the Contract, unless otherwise agreed with the Employer, all the temporary camps/housing provided by the Contractor shall be removed and the site(s) reinstated to its (their) original condition, all to the approval of the Engineer.

All accommodation facilities of whatsoever nature provided by the Contractor shall be provided with adequate sewage and wastewater collection and treatment facilities to a design and of a capacity approved by the Engineer. Under no circumstances shall untreated sewage or wastewater be discharged to the natural environment.

(22)	Insert the following the end of Clause 109(3) “Provision and Maintenance of Camps,… etc.”:

Construction Equipment and Plants

            - All construction plant and equipment provided by the Contractor shall, when brought to the Site, be deemed to be exclusively intended for the construction and completion of the Works, and the Contractor shall not remove the same or any part thereof without the consent of the Engineer. The equipment and plants shall not be measured nor paid separately but shall be deemed included in the rates for unit prices of the respective main items.

Contractor’s Site Office, Sheds, Stores, and Living Quarters

            - Unless otherwise provided by the Employer within the Site, the Contractor shall obtain, at his risk and cost, any additional rights of way or facilities outside the Site which he may require for the purpose of the Works.

Contractor’s Workshop and Warehouse

            - The Contractor shall have a suitable workshop in the Site, adequately equipped and provided with utilities, to allow for repairs of the equipment employed to carry out the Works.  He shall also provide a warehouse for the equipment spare parts, mainly for the parts that frequently fail or are difficult to procure.  A chief foreman qualified for mechanical repairs, with an adequate labour force shall be assigned for management and operation of the workshop.

Land for Construction Purposes, Detours, Plant, and Other Uses:

            - The Contractor shall acquire all additional working areas or any additional areas required for construction purposes and access or other uses, as stipulated in the Specifications.

            - Before entering the work site, the Contractor shall give a written notice to the Engineer. The Contractor shall give separate notices to each owner and occupier or authority having jurisdiction over the work sites.

            - Before entering any additional work areas, the Contractor shall obtain, and forward to the Engineer, a copy of the written consent of the owner and occupier or authority having jurisdiction over the land, and shall state the purposes for which such land is to be used. The Contractor shall define the extent and period of occupation for which such consent is granted.

            - The Contractor shall select, arrange for and, if necessary, pay for the use of sites for construction purposes, detours, plant, and other uses necessary for the execution of the Works.

            - Before using any land belonging to the government or to a private landowner for any purpose in connection with the execution of the Works, the Contractor shall obtain the Engineer’s prior consent for such use.

            - If any utility passing through the temporary site is affected by the Works, the Contractor, at his own expense, shall provide an appropriate plan for re-alignment or relocation of such utility in full working order to the satisfaction of the owner of the utility and the Engineer, before cutting or removing or relocating the existing utility.

            - Upon completion of the Contract, or earlier if so directed by the Engineer, all plant, temporary facilities and any other encumbrances shall be removed, the site and land use areas shall be properly cleaned, all damages shall be repaired, and, if necessary, the landowner shall be paid for the use of land.

(6)	Provision of First Aid/Medical Facilities

(23)	Modify the tittle of Clause 109(6) “Provision of First Aid/Medical Facilities” as follows:

(6)	Provision of First Aid/Medical Facilities and Prevention of HIV/AIDS

(24)	Insert the following sub-tittle before the first paragraph of the renamed Clause 109(6):

(6-1)	Provision of First Aid/Medical Facilities

(25)	Insert the following after the renamed Sub-Clause 109(6)(6-1):

(6-2)	Prevention of HIV/AIDS

This Sub-Clause deals with the prevention of HIV/AIDS at workplace and surrounding local communities during the execution of the project.

This Clause described to herein shall be interpreted without detriment to the stipulations of Clause 6.7: “Health and Safety” of the Conditions of Contract.

Regarding the HIV/AIDS as the workplace issue (like any other serious illness/condition in the workplaces), the Contractor shall conduct an HIV/AIDS awareness program via an approved and elite service provider, and shall undertake to reduce the risk of the transfer of the HIV virus between and among the Contractor’s Personnel and the surrounding local community, to promote early diagnosis and to assist the affected individuals.

The requirements established in this section shall be minimum requirements, without detriment to, the responsibilities of the Contractor. However, it is not a requirement of this Clause for the Contractor to undertake or pay for treatment or medication for personnel found to be suffering from HIV/AIDS. Such personnel, however, shall not be discriminated in any situations but shall be referred to National AIDS Program of the respective township or to other International NGOs (INGOs) which provide further testing and treatment.

Scope of Work:

The Contractor shall prepare a detailed HIV/AIDS Prevention Plan (the Plan) indicating how he shall comply with the Contract requirements as well as requirements in the IEE report, for the Engineer’s Approval. The Plan shall contain the contents, methods, frequency and cost of the activities for HIV/AIDS prevention targeting all staff related to construction works as well as communities surrounding the Site.

The Contractor shall undertake to implement activities indicating in the Plan, including all necessary facilitation. The approved Plan shall be properly implemented by the Contractor throughout the contract period including the Defects Notification Period, and the activity record with attendee lists shall be submitted to the Engineer within ten (10) working days.

The Contractor shall assign a manager and appropriate number of staff who will work for preparation and implementation of the Plan.

The Contractor shall, within fifty six (56) calendar days, after the date of the letter of Acceptance, submit the Plan to the Engineer for his approval. In case the Contractor sublets any of his obligation under the Contract, it is required that the sub-contractor shall comply with the Plan as mandatory.

The Plan shall cover the following aspects but not limited to the following:

i.  HIV/AIDS Awareness and Education Program

(a)	HIV/AIDS awareness and education program indicates the risks, danger and impact, and appropriate avoidance behavior with respect to Sexually Transmitted Diseases (STD) – or Sexually Transmitted Infections (STI) in general and HIV/AIDS in particular. The Contractor shall provide the program addressed to all the Site staffs and labours (including all the Contractor’s employees, all Sub-Contractors and Consultants’ employees and all truck drivers and crews to Site for construction activities). The HIV/AIDS awareness and education program shall be provided at least one time for all workers before commencement of their works.

(b)	HIV/AIDS awareness and education program to the surrounding local communities, at least every three months, concerning the risks, danger and impact, and appropriate avoidance behaviour with respect to Sexually Transmitted Diseases (STD) – or Sexually Transmitted Infections (STI) in general and HIV/AIDS in particular.

(c)	Arrangement for the providing of condoms for all Site staffs and labours on Site as appropriate place or other approved method of distribution throughout the contract period.

(d)	Arrangement for the hanging of the HIV/AIDS awareness posters in (A0 and A3 size) vinyl at the appropriate place of offices, work place and labour camps.

(e)	Arrangement of place, keeping and submission of record for the implementation of HIV/AIDS awareness and education program to the Engineer.

ii.  Peer Educators Program

(a)	Selection sufficient number of the Peer Educators from all Site staffs and labours to be more effective in HIV/AIDS prevention program.

(b)	Implementation schedule of the peer educator program.

(c)	Arrangement for providing the necessary support to the peer educators.

iii.  HIV Testing

(a)	Arrangement of STI and HIV/AIDS screening, diagnosis, voluntarily counselling/testing referral to a dedicated national STI and HIV/AIDS program, (Unless otherwise agreed) of all Site staffs and labours.

(b)	Confidentially keeping system of the testing results not to discriminate in working area.

(c)	Referring the staffs and labours found HIV positive in testing results to the National AIDS Program (NAP) of the relevant township or to other INGOs which provide further testing and treatment.

Relevant Statutes

The Contractor shall at all the time comply with the existing statutes in Nepal concerning Public Health Law for the implementation of the Works.

If the requirements stated in this document are less stringent than or in conflict with the country’s applicable legislation, the latter shall apply.

Code of Practice

The Code of Practice shall be in accordance with the Contract, based on a recognized standard and not less standards than those prescribed in the following code and standards:

-	The International Labor Organization (ILO) Codes of Practice on HIV/AIDS and the world of work (Guidelines for the Construction Sectors)

Notice of Contravention:

The Engineer will notify the Contractor in writing as Notice of Contravention of any observed noncompliance with the responsibilities of Contractors’ HIV/AIDS prevention plan. The Contractor shall, after receipt of such notice, inform the Engineer of proposed corrective actions and take such actions immediately.

(9)	Hazardous Materials

(26)	Delete the first (1st) paragraph of the Clause 109(9) “Hazardous Material” and replaced with the following:

Hazardous Materials comprise such material as cement, lime, bitumen and emulsion, paint, fuels, oils, battery acids, pesticides and herbicides, and any type of toxic material.

Solid wastes containing hazardous material shall be placed in containers which can be emptied on a regular schedule.  Handling and disposal shall be conducted to prevent contamination.  Segregation measures shall be employed so that no hazardous material will become co-mingled with other solid waste.

Chemicals shall be dispensed ensuring no spillage to ground or seasonal / permanent water courses or springs.  Periodic inspections of dispensing areas to identify leakage and initiate corrective action shall be performed and documented by the Contractor.  Chemical waste shall be collected in corrosion resistant, compatible containers.

The Contractor shall take sufficient measures to prevent spillage of hazardous materials during dispensing and shall collect waste in suitable containers observing compatibility.  Spills of hazardous materials shall be immediately reported to the Engineer.  Clean-up and the costs arising there from due to spills shall be the Contractor’s responsibility.

Provision of Safety Clothing and Equipment:

Besides the requirements for Hazardous Material, the Contractor shall provide all laborers, supervisors and site staff with suitable basic safety helmets, boots, clothing, etc.

All workers and supervisory staff when on any site shall wear the basic safety clothing specified. Where additional safety equipment is required by nature of the site, then all workers and supervisory staff when on any part of that site shall use the additional equipment in addition to the basic safety clothing specified.

In hot weather the Contractor shall provide appropriate lightweight safety equipment, such as cotton gloves in place of rubber gloves and canvas boots in place of rubber boots.

The Contractor shall be responsible for ensuring that all his employees and the employees of any Subcontractor and any visitor to the site complies with the Engineer’s requirement to use safety clothing and equipment at all times. Failure in compliance with the Engineer’s instruction in respect of safety clothing and equipment may lead to a suspension of works and a reduction or with-holding of payment until the Contractor complies.

(10)	Reinstatement of Environment

(27)	Insert the following as the second paragraph of the Clause 109(10):

The Contractor shall reinstate the environment of areas polluted by the use or spills of hazardous materials as defined in Clause 109(9). All such materials and polluted soil shall be removed to a safe disposal site as described in these Specifications. The site shall then be restored and revegetated as approved by the Engineer.

The Contractor shall reinstate the quarry sites at the completion of the works as mentioned in Clause 109(1) and/or as instructed by the Engineer.

(11)	Measurement and Payment

(28)	Delete the full text of Clause 109(11): “Measurement and Payment” and replace with the following:

(11)	Water Resources

The Contractor shall keep construction activities under surveillance, management, and control to avoid pollution of surface and ground waters.  Toxic or hazardous chemicals shall not be applied to soil or vegetation when such application may cause contamination of the fresh water reserve.

Proper treatment of water pollutants generated from construction works. Adequate preventive following measures shall be taken as necessary:

-	Install cover sheet on bare lands, cut and fill areas and waste soil disposal sites if necessary.

-	Surface runoff from the Site shall be directed to silt traps or sedimentation basin with the help of channels before discharge.

-	To shelter scattering of soil mud from excavation work by applying preventive measure such as submerged fence in order to avoid increase in turbidity.

-	Monitoring water quality during construction.

(12)	Air, Noise and Vibration

The Contractor shall keep construction activities under surveillance and control to minimize environment damage by air contamination, noise, vibration, odors and/or other disturbances.

Dust particles pursuant to production and preparation of materials shall be controlled at all times. The Contractor shall maintain excavations, stockpiles, haul roads, permanent and temporary access roads, spoil areas, and other work areas within or outside the Site free from particulate which would cause a hazard or a nuisance to persons and/or damage crops, orchards, cultivated field and dwellings. A discharge of dust into the atmosphere shall be controlled during processing, handling and storing cement and cementitious materials.

Night-time operations producing a high level of noise and/or vibration shall be performed only at time and places approved by the Engineer. Temporary noise barriers, or acoustic screens or enclosures shall be used at the Site close to the residential houses to shield residences from the noise as necessary. The Contractor shall indemnify the Engineer from any liability for damages due to noise, vibration and/or other disturbances caused by his construction operations and also from all claims relating to such liability.

Equipment and vehicles shall be maintained and operated at all times in such a condition as not discharge excessive exhaust gases due to poor engine adjustments or other inefficient operating conditions. The Contractor shall likewise ensure that all equipment and machinery are in proper working condition so as to minimize the amount of noise and vibration generated.  The Engineer may require, at his discretion, the Contractor to replace any equipment, machinery or vehicles emitting excessive exhaust gases, noise and/or vibration.

(13)	Social Considerations

The Contractor shall pay attention to the public morality of his staff and laborers including his subcontractors both on and off the Site.

In the interest of good community relations, the Contractor and his subcontractors shall be circumspect in dealings with people residing in the neighborhood of the Site and take adequate efforts to avoid any disturbance to them.  The Contractor shall, in his responsibility, arbitrate any dispute arising between his personnel and persons in the neighborhood of the Site. Unskilled labour shall preferably be employed from the towns/villages affected by the Works. Also, the Contractor shall distribute prior information of construction works as following but not limited through installation of signboards informing the residents who live in and around the construction area.

-	Starting and completion date of construction works

-	Location of construction work (dredging and disposal work)

-	Notices of the construction work etc.

-	Method of grievance (contact person and contact number, etc.)

(14)	Measurement and Payment

Excepting the Items shown here below, no separate measurement nor payment shall be made for the works described in this Section but it shall be deemed included in the rates and unit prices of all related main items.

The works for provision of the Items No.109-1 and No.109-2 shall be measured and paid in “lump sum” in split instalment shown here below, which shall be the full and the final compensation to the Contractor as per Clause 112 including all costs for providing and installation of the site offices as herein described, and for the cost of all operations, materials, equipment, and any incidentals involved for completion of this item.

Payment of Item No.109-1 shall be made according to the following instalments:

(i)	Twenty percent (20%) upon completion of the site offices, camp, and all installations herein described and approved by the Engineer.

(ii)	Fifty percent (50%) during the execution of the Works split in partial payments in proportion to the work progress subject to certification of achievement issued by the Engineer.

(iii)	Thirty percent (30%) upon issuance of the Taking-Over Certificate.

Payment of Item No.109-2 shall be made according to the following instalments:

(i)	Fifty percent (50%) upon completion of the works, demobilizing of all plants herein described and issuance of TOR certificate and approved by the Engineer.

(ii)	Fifty percent (50%) upon completion of the Defects Notification Period and duly reinstatement of the areas used for the plants and stockpiling sites, and approved by the Engineer.

Noncompliance by the Contractor on any of the requirements specified in this Section will cause suspension of the payments of Items No.109-1 and No.109-2.

The works for provision of the Item No.109-3 for provision and installation of “Environmental Protection Works”, as herein described, shall be measured and paid in “lump sum” prorated in monthly basis in accordance with the approved Monthly Progress of the Works. The contract price and/or unit rate shall be the full and the final compensation to the Contractor as per Clause 112 including all costs for providing and installation of the Environmental Protection Works herein described, and for the cost of all operations, materials, equipment, and any incidentals involved for completion of this item.

| Item No. | Description | Unit |
| --- | --- | --- |
| 109 | Environmental Protection Works |  |
| 109-1 | Providing and establishment of camps, adequate drinking water, firewood and toilets for Contractor's labors and staffs and demolish camp site and re-instate the ground in original condition as per specifications and Engineers instruction all complete | lump sum |
| 109-2 | Reinstatement of crusher plant, concrete mixing plant, asphalt concrete mixing plant and stock pile sites to the original stage at the completion of works as instructed by the Engineer | lump sum |
| 109-3 | All the works for Environmental Protection Works | month |

(15)	Provision for Particular Environmental Mitigation Measures

This sub-section describes the works to be implemented as per particular measures for mitigation of impacts to the environmental. The basic requirements described in these specifications shall apply for these works too; however, the following have been identified and classified for this provision as particular and are not included in other sections for measurement and payment:

Tree planting

Relocation and repair of services

Emergency water supply to existing water supply network

(a)	Tree Planting: These works are not measured nor paid under Section 2800: “Bio Engineering Works”, and consist basically of a particular measures for mitigation of impacts on the environment that maybe will be observed during the execution of the Works.

(b)	Relocation and repair of services: These works are not measured nor paid under other sections, and consist basically of a particular measures for relocation and/or repair of those unforeseen services or utilities that maybe will be found during the execution of the Works.

These works shall include, inter alia, the following:

  - Survey and inventory of the identified services or utilities

  - Method statement and drawings for the measures to be applied

  - Construction of the works

(c)	Emergency water supply to existing water supply network: These works are related to the works described on Sub-Section 103: “Publicly and Privately Owned Services” and other relevant specifications, but are not measured nor paid under that or any other section, and consist basically of a particular measures that might be necessary for rehabilitation of the system for Water Supply and recovery of water supply network, during the execution of the Works and after respective completion.

These works shall include, inter alia, the following:

  - Survey and brief study on the existing conditions

  - Design and drawings for the measures to be applied:

    - Temporary Water Supply system to be used during execution of the Works.

    - Permanent works for Water Supply system to be installed after completion of the Works.

  - Construction of the works

    - Civil works

    - Pipe & Fitting Works

    - Electrical Work/Mechanical

    - Environmental Mitigation Measure

(e)	Measurement and Payment: These works provided by the Particular Environmental Mitigation Measures, shall be measured in a Lump Sum supported by a breakdown submitted by the Contractor, for review and approval by the Engineer, when the mitigation measures are required to be applied.  A Provisional Sum for payment is established for each main item, and the payments shall be made for the related items included in the breakdown in conformity with the Clause 13.5: “Provisional Sums” of the Conditions of Contract and all other relevant clauses. The contract price and/or unit rate shall be the full and the final compensation to the Contractor as per Clause 112.

| Item No. | Description | Unit |
| --- | --- | --- |
| 109 | Environmental Protection Works |  |
| 109(15) | Provision for Particular Environmental Mitigation Measures |  |
| 109-4 | Tree planting | PS |
| 109-5 | Relocation and repair of services | PS |
| 109-6 | Emergency water supply and recovery to existing water supply network | PS |

110	PHOTOGRAPHS

(29)	Delete the full Sub-Section 110: “Photographs” and Sub-Section 111: “Supply of Video Cassettes” and replace both  with the following two Sub-Sections: “110 Project Safety” and “111 Supply of Project Record”:

110	PROJECT SAFETY

(1)	Description

This section consists of the requirements for planning, implementing, and maintaining the safety measures to be applied by the Contractor for the Project during the execution of the Works and Defects Notification Period.

The requirements for “Project Safety” described herein shall be interpreted, as supplemented by, and without detriment to the stipulations for safety described in Sub-Section 804 “Safety and Public Health Requirements” and other relevant Clauses of these Specifications, and the Conditions of Contract Sub-Clause 4.8: “Safety Procedures” and Sub-Clause 6.7: “Health and Safety”.

The following definitions shall be referred to, for proper interpretation of Specification Section:

| Contractor`s Health and Safety Plan: | The Contractor shall prepare the Contractor’s “Safety Plan” in accordance with “The Guidance for the Management of Safety for Construction Works in Japanese ODA Projects”, JICA, Sep. 2014. It shall consist of a complete plan for implementation and monitoring of safety measures applied in the project inclusive of the site and all the related areas such as the construction plant, quarries, roads for materials transportation, etc. |
| --- | --- |

(2)	Requirements

(a)	Scope of Work

The Contractor shall, throughout the period for execution and completion of the Works and the Defects Notification Period, take full responsibility for the protection of the Works, the health and safety of all persons entitled to be upon the Site and the public or others concerned to the Works, and the security of the Site. Lighting at night and underground sites including installation of floodlights shall be covered as a part of Safety.

The requirements established in this Specification Section shall also supplement the requirements and provisions stated in the Section 102: “Accommodation of Traffic”, Section 108: “Site Information”, Section 109: “Environmental Protection Works” and all sections regarding safety that are included in the Contract.

(b)	Relevant Statutes

The Contractor shall at all the time comply with all the existing and applicable statutes in Nepal concerning Safety, Security and Health for the implementation of the works.

(c)	Safety Plan and Security

The basic principle of PDCA (Plan, Do, Check, Act) required by the JICA’s guideline for Safety shall be established for implementation of a proper safety management.

Planning of Safety and Security:

The Contractor shall within twenty-eight (28) calendar days after the date of the Letter of Acceptance submit to the Engineer, for his approval, a health and safety plan or similar plan in order to comply with the requirements set out in Sub-Clause 4.8: “Safety Procedures” of the Conditions of Contract.

The so-called safety plan shall be developed with utmost care and vigilance in compliance with the requirements referred in the applicable statutes for safety (laws, regulations and authorities) in Nepal.

The construction work shall not begin until the safety plan has been approved by the Engineer. The Safety Plan shall cover the following aspects:

-	Statement of Contractor’s Safety and Security policy.

-	Senior management responsibility for Safety and Security.

-	Appointment, duties and responsibilities of Safety staff.

-	Policy for identifying Hazards.

-	Risk assessment and management plan for Safety and Security.

-	Promotion, awareness and training for Safety and Security.

-	Equipment for Safety and Security.

-	Safety and Security of the Contractor’s construction and office equipment.

-	Safety and Security of the staff and workers at Site

-	Safety and Security of the public and third parties at Site.

-	Safety and Security procedures for Subcontractors.

-	Measures to avoid traffic accident on public roads.

-	Disciplinary procedures.

-	Accident reporting.

-	Medical check-up to all project staff

-	First aid system.

-	Emergency evacuations plan

-	Others.

The safety and security plan for each section or installation shall be submitted in the Method Statement specified.

The approval of Health and Safety Plan by the Engineer shall not relieve the Contractor of any of his responsibility for Health and Safety at the Site. Furthermore, the Engineer shall have the right to request the Contractor to improve the approved Safety Plan whenever it is deemed necessary in the opinion of the Engineer.

Implementation of Health and Safety Plan:

The Contractor shall develop and implement a site-specific Health and Safety Plan which includes, but not limited to, a statement of health and safety policy organization and lines of responsibilities including Subcontractors and suppliers, training, methods for enduring health and safety incentive programs and compliance monitoring, accident reporting and record-keeping, medical support, and operating procedures. The Contractor shall be responsible and accountable for the actions and activities of his Subcontractors and suppliers, and for their compliance with the Safety Plan.

Notice of Contravention:

The Engineer will notify the Contractor in writing as Notice of Contravention of any observed noncompliance with relevant laws, regulations, permits and other elements of the Contractor’s Safety Plan.

The Contractor shall, after receipt of such notice, inform the Engineer of proposed corrective actions and take such actions, at his own cost, immediately when approved. If the Contractor fails to comply promptly, the Engineer may issue and order stopping, all or part of the work until satisfactory corrective action has been taken. No time extension therefore shall be granted to the Contractor for any such suspensions.

The Contractor may appeal to the Engineer for modification of the terms of any Notice of Contravention before the expiry of the notice. Upon such appeal the Engineer in his sole discretion may modify, withdrawal or confirm the notice.

(d)	Safety Management Committee (SMC)

For implementing the SMC, the Contractor shall constitute a duly empowered SMC which shall convene monthly under the chairmanship of the Occupational Health and Safety Manager.

The SMC shall be comprised of the following members at least and shall be subjected to the approval of the Engineer, who may, if in his opinion the committee is not carrying out its duties with due diligence, order that a new committee be reconstituted.

(i)	The Employer

(ii)	The Engineer

(iii)	The Contractor (project manager, Site managers, Safety manager and officers and so on)

(iv)	Representatives of Subcontractors

The Committee shall execute the site inspection regarding the health and safety at the Site before the Committee meeting.

The Committee shall have authority to cease work if unacceptable health and safety conditions exists, and instruct the Contractor to take necessary action to re-establish and maintain safe working conditions.

(e)	Safety Manager (SM)

The SM shall be a graduate with at least a bachelor course of environmental scientist/ engineering and have experiences of more than 15 years in health and safety works in construction. He or she shall have obtained a vocational qualification issued by the safety authorities or an equivalent institution.

The SM or his deputy or representative shall be available at Site on a 24H/ day basic and his or her deputies shall carry out regular and random checks of all parts of the Site where work is taking place.

The duties of the SM and his deputy are as follows, but not limited to:

(i)	Management and implementation of safety plan,

(ii)	Notification, investigation and recording of accidents,

(iii)	Ensuring healthy and safe working conditions and management of hazardous conditions in accordance with Section 109: “Environmental Protection Works”,

(iv)	Arrangement for frequency and occasions of routine and special meetings,

(v)	Arrangement for disseminating information, training and supervision,

(vi)	Participation in internal weekly inspection and auditing on safety measures at Site by the Contractor and Subcontractors,

(vii)	Participation in monthly safety inspection by the Employer and /or Engineer, and

(viii)	Other required duties in SM.

(3)	Special Requirements for Safety

(a)	Rules and Responsibilities of Project Stakeholders

Precedence:

The rules and responsibilities of Project Stakeholders specified in the contract documents shall take precedence over this safety specification.

Employer:

The rules and responsibilities of the Employer relating to the management of safety on construction sites are as follows:

(i)	The Employer shall endeavour to strictly comply with the relevant laws and regulations of Nepal to ensure the safety of the Project Stakeholders during construction works at site and protect nearby local residents, and any other third parties, from every potential accidental risk foreseen to arise from the construction works at site.

(ii)	The Employer shall, in collaboration with the Engineer, make sure the works carried out in accordance with the Safety Plan and the Method Statements on Safety prepared by the Contractor and provide notice, suggestion or guidance for improvement.

(iii)	The Employer shall endeavour to create an environment where all the project Stakeholders positively participate in activities to promote safety on construction sites.

(iv)	When two or more Contractors carry out work at the same construction site, the Employer shall establish an environment for mutual cooperation and coordination on safety management.

(v)	The Employer shall notify the Contractor of natural conditions, social conditions or any other factors that may affect the management of safety for construction works at site.

Engineer:

The rules and responsibilities of the Engineer relating to the management of safety on construction sites are as follows:

(i)	The Engineer shall have a full understanding of the roles and responsibilities of the Employer on the management of safety for construction works at site and, together with the Employer, appropriately implement activities to manage safety, including these obligations specified in the contract documents.

(ii)	The Engineer shall, in collaboration with the Employer, review the Safety plan and the Method Statements on Safety prepared by the Contractor and provide notice, suggestion or guidance for improvement to the Contractor if there are any risks to safety.

(iii)	The Engineer shall, in collaboration with the Employer, make sure the work is carried out as per the Safety Plan and the Method Statements on Safety prepared by the Contractor and provide notice, suggestion or guidance for improvement.

Contractor:

The rules and responsibilities of the Contractor relating to the management of safety on construction sites are as follows:

(i)	The Contractor shall be responsible for the operation and management of safety on construction sites.

(ii)	The Contractor shall appropriately prepare the Safety Plan at the appropriate time in the pre-construction stage in accordance with the relevant laws and regulations of the Nepal and Health and Safety Plan. In the construction stage, the Contractor shall appropriately prepare the Method Statement on Safety, which shall specify the details of safe methods to implement safety measures prior to the start of each item of work and submit the document to the Employer and Engineer for review and approval must be obtained.

(iii)	The Contractor shall make appropriate revision or correction whenever any insufficiency or suggestion for improvement relating to safety is raised following the review of the Safety Plan and the Method Statements on Safety.

(iv)	The Contractor shall undertake work according to the Health and Safety Plan and the method Statement on Safety they prepared.

(v)	The Contractor shall take into account the safety of nearly local residents and any other parties, as well as all Project Stakeholders of the construction project.

(vi)	The Contractor shall carry out construction works for the safety of nearly local residents and any other third parties, as well as Project Stakeholders of the project.

Subcontractor:

The rules and responsibilities of the Subcontractor relating to the management of safety on construction sites are as follows:

(i)	Each Subcontractors shall carry out construction works in compliance with the relevant laws and regulations of the recipient country applicable to the construction work and these Guidance’s.

(ii)	Each Subcontractors shall establish and maintain safe and sanitary site conditions according to the instructions of the Contractor.

(iii)	Each Subcontractors shall cooperate with other Subcontractors engaged on the construction site in accordance with the instructions of the Contractor.

(iv)	Each Subcontractors shall receive from the Contractor an explanation on the Safety Plan and the Method Statements on Safety prepared by the Contractor. Each Subcontractors shall pass on the explanation received from the Contractor to their Employees and have them fully comply with those explanations to ensure safety.

(b)	Safety of Public

The Contractor shall be responsible for the safety of the public passing through the Site. All excavations, plant or items of potential danger to the public must be barricaded and sign-posted to the satisfaction of the Engineer, and the Contractor must provide sufficient watchmen to ensure the safety of the public at all times. All existing pedestrian routes shall be maintained in a safe condition unless an alternative route is provided to the satisfaction of the Engineer.

(c)	Safety Barricade and Signboard

The Contractor shall provide and maintain adequate safety barricade and signboard around the dangerous area such as opening, excavate trench, heavy equipment, lifting work area, etc.

(d)	Handrail, Safety Belt and Safety Net

The Contractor shall provide and maintain adequate handrail and safety net.

Handrail shall be installed for all the working places higher than 2.0m. Handrail shall be strong, enough with adequate height. When working at higher than 2.0m without handrail, safety belt and safety rope shall be used. The Contractor shall use adequate safety belt and safety rope.

In case there is a risk of materials or tools falling down, the Contractor shall provide and maintain safety net around the stage. Type of the handrail, Safety Harness and Safety Net shall be subjected to the approval of the Engineer.

(e)	Evacuation Route and Place of Refuge

The Contractor shall provide and maintain the Evacuation route and place of the refuge in the site which can access to any location of the site by vehicle in case of emergency. Safety path for workers to access to their working location shall be provided in the site. The Contractor shall maintain safety path with safety condition. Emergency evacuation route and place of refuge shall be indicated by signboard.

(f)	Temporary Roads/Diversions/Construction Yard in the Site

The requirements described herein shall be applied supplementary to the requirements described in Sub-Clause 102(3)(a) “Provision of Diversions”.

Traffic lanes shall be clearly marked by safety cone, barricade and guide signs to prevent accidents and to ensure quality control of constructing roads.

The Contractor shall provide and maintain security fences around the site. Purpose of the fence is to prevent residence especially children from entering the site without any permission. Therefore, security fence shall be at least 2.4m high, dense and strong enough so that children cannot break it or slip through the fence. Type of the fence shall be approved by the Engineer prior to construction.

The Contractor shall check the fence condition every day and if any damage or opening is found, repair them immediately on his own responsibility.

Contractor shall install a detour information board to inform public vehicles and pedestrians of the need for diversions, and deploy flagmen as appropriate.

Notify local residents of the plan to work on public roads, so as to obtain their understanding and cooperation.

(g)	Workers Rest House

Labour’s rest house with first aid facility shall be provided and maintained by the contractor to prevent from lightening accident and sunstroke. Toilets, smoking corner and drinking water shall be set up in the worker’s rest house.

(h)	Prevention of Fall (scaffolding)

Where works are undertaken at a place more than two (2) meters above ground level, the Contractor shall construct scaffolding prior to the commencement of work. Where it is impossible to construct scaffolding, workers use protective gear to prevent themselves from falling, such as safety belts or fall arrestors. Where safety belts are used, the Contractor shall ensure that equipment is affixed.

(i)	Prevention of Public Accidents and Traffic Accidents

(i)	When work is undertaken on a public road, the Contractor shall adopt appropriate measures, such as barricading the work site and stationing the watch-personnel or traffic-control personnel, to prevent the entry of unauthorized personnel including third parties into the work area.

(ii)	Where buried utilities or facilities are located under the ground of a work site or where excavation is undertaken in the ground near a structure, then if damage to those utilities, facilities or structures by overturning or collapsing is likely, the Contractor shall take appropriate measures prior to the commencement of work, so as to prevent the risk, such as the relocation or reinforcement of the utilities, facilities or structures.

(iii)	When earth and sand is backfilled over buried utilities or facilities, the Contractor shall undertake backfilling according to the predetermined specifications, without applying unsymmetrical pressure or damaging the buried utilities or facilities.

(j)	Availability of Safety-related Documents

The Contractor shall comply with the Engineer’s requirements in so far as displaying in each of its site offices, workshops, and canteens a copy of such safety and industrial health posters, and keeping on Site copies of safety and industrial health regulations and documents. The minimum regulations and most important documents (selected by the Engineer from the approved Safety Plan) shall be translated into languages which are understood by the operators engaged by the Contractor or Subcontractors, and such translations shall be displayed or kept alongside with those in Nepal and English languages.

(k)	Safety Reports

The Contractor shall submit regular safety reports to the Engineer as a requirement of the Safety Plan. A summary of this report shall be included in each Monthly Progress Report. Safety reports shall comprehensively address all relevant aspects of site safety and industrial health regulations and, in particular, shall report on all site safety audits undertaken during the period covered by the report.

(l)	Safety Information

The Contractor shall ensure that safety, rescue, and industrial health matters are given a high degree of publicity to all persons, regularly or occasionally, at the Site. Posters, in both Nepal and English, drawing attention to site safety, rescue and industrial health regulations shall be made or obtained from the appropriate sources and shall be displayed prominently in relevant areas of the Site.

(m)	Safety Meeting and Patrol

The Safety Manager of the Contractor shall perform regular safety meetings and patrol in accordance with the Health and Safety Plan, with the participation of the Employer and Engineer (or a representative delegated by him for the purpose), and representatives of Subcontractors if so requested by the Engineer.  All safety meetings shall be notified to the Engineer at least three (3) days in advance.  Minutes of all safety meetings shall be taken and sent to the Engineer within three (3) days after the meeting.

(n)	Fire Extinguishers

In supplement to the safety equipment stated in the Safety Plan, the Contractor shall provide in particular wall-mounted hand-held fire extinguishers of multi-purpose, nitrogen-pressured, or dry chemical type at the Employer’s and Engineer’s facilities and the Contractor’s facilities as suitable in conformity o the law or the safety standard of the country.   Individual fire extinguishers shall also be provided in all the vehicles being used in the Project. Fire extinguishers shall be replaced before the expiry period.

(o)	First Aid Base

The Contractor shall, at his own expenses, establish and maintain a fully equipped first aid base as described in the approved Safety Plan, and satisfying the following conditions:

The first aid base shall be located at the Contractor’s principal work area and shall consist of a treatment room fitted with a hand wash basin, two treatment couches and sterilizing equipment, and lockable cupboards to contain sufficient medical supplies for the Contractor’s workforce, the Engineer’s site supervisory staff and any visitors to the Site.  In addition, three (3) stretchers shall be stored, available for instant use.

A qualified nurse and nurse aid shall be in attendance at the first aid base during all times when work is being undertaken at the Site, including work by any Subcontractor, and during periods when only emergency activities are being undertaken, such as periods of inclement weather.

All the costs for first aid shall be considered as a part of the unit price of the bid, and no additional payment will be made for it.

(p)	Notification of Accidents

The Contractor shall notify the Engineer immediately when any accident occurs whether on site or off site in which the Contractor, his personnel or construction plant, or those of his Subcontractors are directly or indirectly involved and which results in any injuries to any persons. Such initial notification may be verbal and shall be followed by a written comprehensive report within 24 hours after the occurrence of the accident.

(4)	Measures for Safety

The Contractor shall take the measures for health and safety at the Site listed below, but not limited to, in accordance with the Relevant Statutes and Code of Practices:

(a)	Personal Protective Equipment (PPE)

The Contractor shall ensure his and his Subcontractor’s personnel to wear appropriate PPE at the Site at all times and supply the Employer and the Engineer at his own cost with such safety equipment as requested.

No body without PPE is allowed to enter the Site.

Every person entering the Site in open air and underground shall wear PPE.  A sufficient number of electric lamps shall be equipped to the workers working underground and supplied by the Contractor to the Employer’s and Engineer’s persons entering underground.

The Contractor ensures that appropriate safety-toe footwear is worn by all persons engaged in work with an inherent danger to the foot.  Light footwear such as sandals, canvas shoes or the like shall not be permitted at any construction work site.

The Contractor shall ensure that ear protectors are worn by the workers engaged in the drilling work or in the work site where workers are exposed to harmful noise levels.

The Contractor shall ensure that protection glasses, goggles or masks are worn by the workers engaged in work with an inherent danger of eye or face injury.  Likewise, protective clothing shall be worn by the workers where irritant or toxic substances can come in contact with the skin or clothing.

The Contractor shall ensure that safety harnesses and lifelines are used by the workers working on steep slopes with an inherent danger of falls not protected by fixed guardrails or safety nets.

(b)	Sanitary Arrangement

The Contractor shall keep the Site in a clean and sanitary condition and shall provide and maintain sanitary conveniences for the use of persons working at the Site to the extent and in the manner and at such places as approved by relevant local authorities concerned and the Engineer. All persons concerned with the Works shall be obliged to use these conveniences when being at the Site.

(c)	Sewerage and Sanitation

The Contractor shall supply, install, operate, maintain and subsequently remove the adequate treatment process for disposal of sewerage from all temporary buildings including houses, offices, camps, etc. to be constructed by the Contractor.  The Contractor shall construct, operate and maintain at the Site temporary toilet facilities complete with adequate water closets, urinals and hand-basins, septic tanks, absorption trenches or other sewerage disposal installation for all of his personnel.  The temporary toilet facilities shall meet the requirements of the Government health authority.  The location of these facilities and their construction shall be as approved by the Engineer.  Sewerage from temporary facilities shall be disposed of in hygienic manner as approved by the Engineer.

The Contractor shall remove all rubbish and shall provide an adequate number of covered garbage bins/containers placed at convenient points around his camp quarters and work sites.  The Contractor shall establish, operate and maintain a regular garbage collection and disposal system, utilizing a covered vehicle for collection and transport of garbage to the place of disposal.  Garbage shall be disposed of by burning, by burial or by other means approved by the Engineer.  No waste of any kind shall be deposited in any watercourse

(d)	Epidemics and Hazardous Substances

The requirements established in this sub-clause shall also supplement the requirements and provisions stated in Section 109: “Environmental Protection Works”

In the event of any outbreak of illness of a highly contagious or epidemic nature the Contractor shall comply with and carry out such regulations, orders and requirements as may be made by the Country, or the local medical or sanitation authorities for the purpose of dealing with and overcoming the same.

The Contractor shall identify and keep records of all hazardous equipment, materials, or other substances and any other health hazards in his undertaking of the Contract. Newly created hazards or new hazardous equipment, materials or other substances brought on to Site shall be added to the record. The Contractor shall draft new, or adapt and modify existing codes of practice appropriate to the said hazards.

The Engineer shall be granted access to such records at all times.

The Contractor shall conduct and document at least weekly inspections of the storage areas for equipment, transport vehicles and hazardous materials and substances, especially for spillages and leaks. The Contractor shall take adequate precautions and make adequate provisions, to the satisfaction of the Engineer, to prevent such spillages of toxic materials and substances from entering natural stream or areas outside the Site.

(e)	Other Measures for Safety

Safety measures shall be taken for the following items (but not limited to):

(i)	Training

(ii)	Reporting

(iii)	Medical check-up and First Aid

(iv)	Emergency Plans

(v)	Fire Prevention and Protection

(vi)	Material Handling, Storage, and Disposal

(vii)	Electrical Safety

(viii)	Walking and Working Surfaces

(ix)	Confined Spaces

(x)	Control of Hazardous Energy (Lockout/Tag out)

(xi)	Fall Protection and Rope Supported Work

(xii)	Hand Tools, Power Tools, Pressure Vessels, Compressors, and Welding

(xiii)	Slings and Rigging Hardware

(xiv)	Hoisting and Pile Driving Equipment

(xv)	Equipment and operation for tunnel and road excavation

(xvi)	Mobile and Stationary Mechanized Equipment

(xvii)	Hazardous Waste Site Operations

(xix)	Drilling and Blasting Operations

(xx)	Concrete, Masonry Construction and Formwork

(xxi)	Steel Erection

(5)	Measurement and Payment

All provision for planning, implementing, and maintaining the safety measures described in this Section for “Project Safety” shall be quantified in months and shall be paid as per contract price on the basis of the split distribution of a lump sum.

The contract price and/or unit rate shall be the full and the final compensation to the Contractor as per Clause 112.

| Item No. | Description | Unit |
| --- | --- | --- |
| 110 | Project Safety |  |
| 110-1 | Project Safety during the construction period | month |
| 110-2 | Project Safety during the Defects Notification Period | month |

111	SUPPLY OF PROJECT RECORD

(1)	Scope and Description

This Clause covers the supply of digital record of project events in digital format (DVD/ Flash Drive) including the following:

(a)	Photographs

The Contractor shall take construction photographs by using digital cameras with an automatic setting on the records of the date-month-year, as described below:

| Items | Time to Shoot a Photo | Submission Time |
| --- | --- | --- |
| Photographs at intervals of no more than 50 meters or other intervals as agreed by the Engineer along the route of the Works | Each month throughout the Works before the commencement of the work | With monthly report or as otherwise required by the Engineer |
| Photographs of each entire site (each work front), all the structure sites, or pertinent features |  |  |
| Record of quarry sites, plant sites, camp sites including labour camps, haul roads, access road etc. with progress report |  |  |
| Record of all accidents |  |  |
| The same views from same points as above | Upon completion of all construction activities | With the Contractor’s application for final payment |

-	Photographs shall be consistent with the number of photographs to be specified by the Engineer.

-	The Contractor shall supply both “hard” (mounted on albums to serve as a permanent record of the work needed for an authentic documentation) and computerized copies of photographs as required by the Engineer. The Contractor shall provide the necessary software to the Engineer for computerized storage, editing, and reproduction of photographs.

-	Each photograph shall have suitable captions.

(b)	Video Recording

The Video Recording shall be taken by a professional with a digital camera capable to video images inside the tunnel or anywhere in the Project, having facilities to record the date and back ground commentary. The Contractor shall keep separate discs/ drives, one with the Engineer and the other with the Employer and update the data in these disc/drives as per instruction of Engineer or on monthly basics.

The Video Recording shall be edited and compiled, based on the following specifications:

| Video System: | DVD |
| --- | --- |
| Recording Time: | 90 minutes |
| Language: | Both in Nepali and English versions |
| Copies: | Five (5) for each version |
| Subjects: | - Preconstruction conditions of the Project area |
|  | - Mobilization and site preparation |
|  | - Installation of equipment and facilities |
|  | - Installation of plant and plant under operation |
|  | - Earthworks |
|  | - Tunnel works |
|  | - Pavement works |
|  | - Construction of bridges and other structures (Roadside Service Station) |
|  | - Electrical and telecommunication utilities |
|  | - Laboratory and testing for quality control |
|  | - Commencement and completion ceremonies |
|  | - Testing and inspection for acceptance of the Works |
|  | - Others as required by the Engineer |

The Contractor shall submit, for the acceptance of the Employer, the outline of the narration process prior to the compilation of the video record.  The final video record shall be produced by a specialized company acceptable to the Employer, and submitted to the Employer and the Engineer within thirty (30) days after the issuance of the Taking-Over Certificate

(2)	Measurement

This item shall be measured in number of sets. Each set consist of two copies of all digital records as described above and colour record photographs both in digital format as well as mounted in the albums and the video recording files.

(3)	Payment

The contract unit rate shall include all expenses for supply of project record including video recordings updated as per direction of Engineer and paid on lump sum monthly basis throughout the construction period shall be measured as single set.

This payment shall be the full and the final compensation to the Contractor as per Clause 112 and for the cost of all operations, materials, equipment, laboratories, transportation, and any incidentals involved for completion of this item.

| Item No. | Description | Unit |
| --- | --- | --- |
| 111 | Supply of Project Record |  |
| 111-1 | Supply of Project Record during construction period | month |
| 111-2 | Supply of Project Record final edition (28 days before issuance of Taking Over Certificate (TOC)). Mandatory Condition for TOC. | lump sum |

112	NOTES ABOUT MEASUREMENT AND PAYMENT

(1)	Measurement

(30)	Delete the full text of Clause 112(1): “Measurement” and replace with the following:

Unless specified, all measurements shall be made in metric system. Different items of work shall be measured in accordance with the procedures set forth in the relevant Sections read in conjunction with the General conditions of Contract. The same shall not, however, apply in the case of lump sum contracts.

All measurements and computations, unless otherwise indicated, shall be carried nearest to the following limits:

| (i) | Length and Width | 10 mm |
| --- | --- | --- |
| (ii) | Height, Depth or Thickness of: |  |
|  | - Earthwork, Subgrade | 5 mm |
|  | - Sub-base, Bases, Surfacing | 5 mm |
| (iii) | Structural Members | 2.5 mm |
| (iv) | Areas | 0.01 sq.m. |
| (v) | Volumes | 0.01 cu.m. |

In recording dimensions of work, the sequence of length, width and height or depth or thickness shall be followed.

The tolerances given in these Specifications are for the evaluation of the accuracy to which work has been constructed and for determining whether work should be accepted or rejected. The measurement of work done shall be based on the actual measured quantity of acceptable work executed except that, where the actual quantity is greater than the quantity which would have resulted from the precise designed or instructed dimensions and/or levels then the payable quantity shall be limited to the quantity determined on the basis of calculation using the designed or instructed dimensions and/or levels.

Measurement and payments shall be made after the contractor has executed the works and has been accepted in place to the full satisfaction of the Engineer.

The Works listed in the Bill of Quantities (BOQ) shall be measured, and valued for payment, in accordance with these Specifications and the Preambles to the BOQ.

113	NATIONAL SPECIFICATIONS

(31)	Delete the third (3rd) paragraph and list abbreviations of Clause 113 and replace it with the following:

In referring to Specifications, the following hierarchy shall be used:

i.	NS	: Nepal Bureau of Standards and Metrology

ii.	JIS	: Japan Industrial Standards

iii.	IRC	: Indian Road Congress

iv.	IS	: Indian Standards

v.	BS	: British Standards

vi.	ASTM	: American Society of Testing and Materials

viii.	AASHTO	: American Association of State Highway and Transportation Officials

ix.	ACI	: American Concrete Institute

ix.	BSCP	: British Standard Code of Practice

x.	ISO	: International Organization for Standardization

xi.	EN	: European Norm

xii.	Others as approved by the Engineer

115	UNITS OF MEASUREMENT, ABBREVIATION AND TERMINOLOGY

(32)	Modify the tittle of Section 115 “Units of Measurement, Abbreviation and Terminology” and replace it with the following:

115	UNITS OF MEASUREMENT, ABBREVIATION, TERMINOLOGY AND DEFINITIONS

(33)	Delete the full text of Clause 115(4) “The Sections, Clauses and/or …..etc.” and replace it with the following:

(4)	Definitions

The words such as Contract, Contractor, Engineer / Engineer in Charge, Drawings, Defect Liability Period (or Defect Notification Period), Employer, Government, Works and Site used in this Specification shall be considered to have the meaning as specified in definitions of these terms given in Section VII: “General Conditions of Contract”, and/or Section VIII: “Particular Conditions of Contract” of the Bidding Documents.

Words and expressions to which meanings are assigned in any section of the conditions of contract or the specifications shall have the same meanings in any other sections of the bidding documents except when the context otherwise requires.

(5)	The Sections, Clauses and/or Sub-Clauses mentioned in these Specifications deem to apply those of these Specifications only, if otherwise not specified.

116	PROGRAMME

(34)	Delete the full text of Section 116 “Programme” and replace it with the following:

(1)	Scope and Description

This Specification Section prescribes the form and contents of the Program of Work (hereafter referred as “the Program”). The Program described herein shall be interpreted, supplementary to, and without detriment to the stipulations referred to in Sub-Clause 8.3 “Program” of the Conditions of Contract.

The following definitions shall be referred to for proper interpretation of this Specification Section:

| Program: | The Program shall govern the execution of the whole Works, and shall consist of all the plans, schedules, statements on the methods and procedures for construction, assignments of equipment and personnel, testing and trials, drawings, submittals, and the like required or mentioned in the Specifications, or required by any other part of the Contract. |
| --- | --- |
| Time Schedule: | The time schedule that shall be incorporated in the Program to show, describe, and monitor the progress, using the Critical Path Method (CPM) presented in the form of bar chart and showing other data herein specified. |
| Cash Flow: | The cash flow schedule, in accordance with the requirements of Sub-Clause 14.4 “Schedule of Payment” of the Conditions of Contract, shall consist of non-binding estimate of the payment which the Contractor expects to become due during each quarterly period. |

(2)	Requirements

(a)	General

Failure of the Contractor to comply with the requirements of the Contract shall be considered as a serious default of the Contractor in meeting his contractual obligations, resulting in a delay in the commencement of the activities for execution of the Works.

Unless otherwise specified in the Contract, the delay caused by the Contractor and its effect on the work progress will be estimated by the Engineer and settled in accordance with the stipulations of Sub-Clause 8.7 “Delay Damages” of the Conditions of Contract, at the time of taking-over of the Works.

In the event that the Contractor requests a change in the sequence in the Program and such change is approved by the Engineer, the Contractor shall have no claim as per the Conditions of Contract for delay arising from such revisions to the program.

(b)	Requirements for the Program’s Composition and Contents

The Program’s Composition and Contents:

- The Program shall incorporate in detail all activities related to the Works from their commencement to their completion, taking into account all the main items introduced in the Bill of Quantities for each component of the Works.

- The Program shall be detailed enough to give, in addition to construction activities, detailed network activities for the submission and approval of materials, procurement of critical materials and equipment, fabrication of special products/equipment’s if any their installation and testing, and for all activities of the Engineer that are likely to affect the progress of work.

- The program shall also include the Contractor’s general requirements for any road closures pursuant to Section 102: “Accommodation of Traffic” to be agreed in principle with the Engineer. Such agreement shall not relieve the Contractor of his responsibility to obtain specific approval for each closure or series of closures.

- Any proposal for night working shall also be stated in the Program but discussed based on the Conditions of Contract.

- The Program shall include, but not be limited to, the following:

-	Schedule (showing the critical path);

-	Contractor’s mobilization and demobilization plan (the Contractor’s Mobilization described herein shall be interpreted, supplementary to, and without detriment to the stipulations referred to in these Specifications, the Conditions of Contract or any other related stipulations of the Contractual Documents);

-	Schedule of arrival of all construction equipment and facilities as well as the arrival of the Contractor and Subcontractors personnel as defined in Sub-Clauses 1.1.2.7 and 1.1.2.8 of the Conditions of Contract respectively.

-	Schedule for completion of all temporary facilities

-	All submittals and drawings;

-	All plans and schedules;

-	All activities for site investigation;

-	All surveys and laying out;

-	All key and milestone dates (such as the dates for submittals of schedule, the dates for mobilization, the commencement/completion dates for the works of soft soil treatment, bridge construction, sub-base, Engineers’ accommodation, etc.);

-	All interface dates between the Contractor and sub-contractors for the Works;

-	All activities associated with the procurement, purchase, fabrication and shipment of goods, materials, and equipment to be incorporated into the Works;

-	All public and site holidays; and

-	Other activities that the Engineer may require for monitoring.

- The construction schedule (the Program) prepared by the Contractor and progress by the charts, Critical Path Method (CPM) and S-curve shall be made using the latest version of software (tools) “Primavera P6: Professional Project Management”.  For this purpose the Contractor shall provide the required software and licenses at least for 3 users selected by the Engineer.

The Schedule’s Contents:

- The schedule shall include a narrative statement and a logical breakdown of major activities. The narrative statement shall comprehensively describe the work execution plan, the achievement of milestones and any intermediate dates, the outline method statements for the major work items, temporary works, and the like. It shall fully explain the reasons for the main logic links in the schedule, and shall include particulars on how activity durations are established.

- The details and number of activities incorporated in the schedule shall be sufficient to manage the total Works and to identify restraints, delays, and effects on the Works by dependent activities.

- All the activities shall be organized in a logical work breakdown structure, indicating work stages and phases, and shall have a code number for identification related to the work breakdown structure as well as a unique description.

- The schedule shall show activity durations, activity descriptions, early and late start and finish dates (all of them expressed in calendar days), and activity float and activity dependencies, applying the critical path method (CPM) of scheduling with all activities tied to only one start date and only one end date.

- All activities shall be cost-loaded (as appropriate) with schedule costs equivalent to the “Cash Flow” (S-Curve):

-	All activities shall be resource-loaded (as appropriate), indicating all manpower and/or equipment necessary to accomplish the activity within the given time duration.

-	Any activity having or creating an imposed constraint to the schedule shall be clearly defined and fully described.

-	The schedule shall be prepared by using a computer application approved by the Engineer for programming, and that has been provided in software by the Contractor with relevant licenses for both the Contractor and the Engineer.

(c)	Requirements for Submittal and Approval of the Program

- The Contractor shall comply satisfactorily and without delay to the requirements herein specified regarding the composition and contents of the Program.

- After the Program has been reviewed by the Engineer, the Contractor shall, within fourteen (14) days after receiving the Engineer’s comments on the Program, incorporate the required corrections or modifications into the Program and re-submit it to the Engineer for approval.

- No more than two (2) submittals of the Program are required for the Engineer’s review, provided however that:

-	If additional submittals are required by actions of the Contractor, any delays shall be the Contractor’s responsibility.

-	The Employer may, at any time, after advising the Contractor of such additional costs for submittal review, recover such costs from money due or will be due to the Contractor.

(3)	Requirements for Updating the Schedule and Cash Flow

Updating of the Work Program and Cash Flow (S-Curve) shall be made for monitoring of progress of works and shall be referred to the approved version of the Program but not substitute it.

The Work Program and Cash Flow shall be updated on monthly basis to reflect the actual work accomplished by the end of each month, and to facilitate the monitoring of payments.

In addition, an activity report, generated by the schedule computer program, shall be attached as an appendix to the Contractor’s monthly report.

The above referred report shall include the following information and data for each activity:

- Activity identification number and description;

- Activity duration expressed in calendar days;

- Early and late start and finish dates for each activity, as well as planned start and finish dates;

- Calculated total and free float for each activity;

- Predecessors(s) and successor(s), accompanying relationships and lag/lead duration(s) for each activity;

- Imposed time or date constraints on each activity;

- An activity calendar;

- Critical path delay; and

- All activities that have a negative float shall be identified and analysed to identify the impact on the timely completion of the Works.

(4)	Measurement and Payment

Unless stated otherwise, no separate measurement and /or payment shall be made for all materials and works required under this Section. All costs in connection with the work specified herein shall be considered to be included with other related items of the work in the Bill of Quantities.

(35)	Add the following new Sub-Section 117: “Contractor’s Drawings” and Sub-Section 118: Insurance and Warranty” at the end of Section 100, after Sub-Section 116: “Programme” and before Section 200: “Site Clearance”:

117	CONTRACTOR’S DRAWINGS

(1)	Description

This section consists of the requirements for preparation of drawings by the Contractor.

Without detriment to the stipulations shown in the Conditions of Contract, and this Section 6(b): Special Provisions to the Standard Specifications, The Contractor shall prepare and submit the following documents:

| Shop Drawings: | The Contractor shall submit to the Engineer, for his check and approval, Shop Drawings of relevant works or components at least 28 days before the intended start of works or fabrication of components. These drawings shall show all necessary details required for execution of the Works including (but not limited to) the following: (a) Details as per site conditions for construction of the Works based on the Drawings comprised in Part 2: “Works Requirements”, Section VI-D of the Bidding Documents, and in accordance with all pertinent stipulations of the General Conditions of Contract and Particular Conditions of Contract. (b) Fabrication details, including materials used for fabrication or workshops to use in fabrication of components necessary for the Works, Plant or other equipment to be installed. |
| --- | --- |
| Construction Drawings | Those Shop Drawings approved by the Engineer as the Drawings “good for construction” |
| Working Drawings: | (c) Detailed design calculations and drawings for all Temporary Works (such as from-work, staging, centring, specialized constructional handling and launching equipment and the like); (d) Detailed drawings for templates, support and end anchorage, details for per- stressing cable profiles, bar bending and cutting schedules for reinforcement, material lists for fabrication of structural steel, works for tunnelling construction, etc. |
| As-Built Drawings: | (e) Detailed drawings prepared based on the approved Shop Drawings and Working Drawings used for performing the Works, showing in detail all the works as actually constructed |

(2)	Requirements

(a)	General

(i)	The Contractor shall be responsible for the following:

־	Confirmation and correlation of all incorporated information;

־	Accuracy of all dimensions and quantities;

־	Information pertaining to the fabrication and construction method;

־	Handling and conservation of all the drawings in good condition; and

־	Delays in the submittals

(ii)	The Contractor shall provide all the necessary information to substantiate that all materials and/or plants incorporated into the Works, conform to and meet or exceed the requirements of the Specifications and/or the Drawings.

(iii)	The Contractor shall provide all necessary and/or required drawings, calculations, and certifications for all temporary works as required or described in the Specifications.

(b)	Requirements for the Contractor’s Drawings Preparation

(i)	All the Contractor’s Drawings shall be prepared using Auto-CAD 2016 or a later version, in a format previously approved by the Engineer.

(ii)	Notes shall be written in English.

(iii)	All dimensions shall be in the metric system.

(iv)	The format of the Contractor’s Drawings shall be as approved previously by the Engineer.

(v)	The Contractor’s Drawings shall be stamped “Submitted for Approval for Construction” and shall be signed by the Contractor’s expert designated as the Quality Control Manager.

(vi)	Design notes, calculations, lists, reports, descriptions, catalogue cuts, and other non-drawing submittals shall be submitted in A4 size sheets, in lieu of the size mentioned above.

(vii)	Unless otherwise specified, each submittal of the Contractor’s Drawings shall consist of three (3) copies and shall be submitted to the Engineer, one (1) set of which will be returned to the Contractor with revisions, if needed, noted thereon.

(viii)	Editable electronic files, in the approved application form for the Project, of the Contractor’s Drawings subject to the Engineer’s approval, shall be provided to the Engineer after issuance of the respective approval.

(ix)	Submittals without the required signatures and seal will not be accepted and will be returned to the Contractor without action.

(c)	Requirements for Submittal and Approval

(i)	Submittal Schedule (the Drawings’ Schedule)

-	The Drawings’ Schedule shall be prepared in close relation, and showing solid consistency with a program specified under Sub-Clause 8.1 of the Conditions of Contract and Sub-Section 116: “Programme” of these specifications.

-	Incorporating in a program, the Contractor shall prepare and submit a submittal schedule (hereafter referred as the Drawings’ Schedule), noting all required information and data required for the Contractor’s Drawings.

-	The Drawings’ Schedule shall be used as the document for monitoring the submittals, and to register the approvals, throughout the Contract.

-	The Drawings’ Schedule shall contain, but not be limited to, the following:

  - A list of the Contractor’s Drawings to be submitted well in advance of the construction activities shown on a program. Such drawings shall be classified by type (i.e., shop drawings, working drawings, as-built drawings or unclassified drawings);

  - A bar schedule of the main submittals showing the main work items of the critical path as the milestone of each submittal; and

  - A list and schedule of the main tests for materials and trials to be subject to approval by the Engineer for the main work items. This list shall include also the submittals of certifications from the suppliers.

-	The registry of each submittal shall contain the following:

  - A description of each submittal and its contents, including applicable specification identification;

  - Statement if the submittal is for information, or to receive the Engineer’s consent or approval;

  - The dates of the submittal: date of submittal to the Engineer and the expected date for return of the submittal;

  - In the event that a re-submittal is required, besides the new dates, it shall contain the status of the returned submittal (i.e., approved as noted, the comments, or rejected); and

  - In the event that a deviation regarding the requirements shown in the Drawings and the Specifications is detected, it shall be clearly stated by the Contractor in the respective submittal.

-	The Drawings’ Schedule shall be updated monthly or when required by the Engineer.

-	The Contractor shall include in his monthly report, a summary describing the existing conditions and the progress of his Drawings’ Schedule.

(ii)	Requirements and Procedure for the Approval

-	Submittals of the Contractor’s Drawings shall be made in complete packages according to the approved Drawings’ Schedule, and shall include all necessary information to allow a verification of conformity with the requirements of the Contract, the Drawings and the Specifications.

-	Upon receipt by the Engineer of a submittal package, it will be evaluated for completeness and the Contractor will be notified of any deficiencies therein.

-	The Engineer will review the submitted drawings to determine that general conformity with the design concept and general compliance with the information given in the Contract Drawings and the Specifications have been accomplished.

-	Any work done by the Contractor prior to the Engineer’s review of the submitted drawings shall be at the Contractor’s sole risk.

-	The Engineer may request additional details and require the Contractor to make changes in the submitted drawings which are necessary to conform to the provisions shown in the Contract Drawings and the Specifications (without additional cost to the Employer).

-	After review, the Engineer will return one (1) set of submitted drawings to the Contractor. Returned drawings will be stamped with the Engineer’s review stamp to indicate the following:

  - Approved:

The submitted Contractor’s Drawings have been reviewed and do not require re-submittal

  - Approved as Noted:

The submitted Contractor’s Drawings have been reviewed and the Contractor shall incorporate the comments as noted in the drawings.  The reviewed Contractor’s Drawings do not require re-submittal.

  - Reviewed and Resubmit:

The submitted Contractor’s Drawings require corrections or redrawing and shall be resubmitted for further review. If the submitted drawings are returned for correction or redrawing, corrections shall be made and the Contractor shall resubmit them in the same manner as the first submittal. Specific notation shall be made on the resubmitted drawings to indicate the revisions made.

-	The Contractor’s Drawings that have been evaluated by the Engineer as “Approved as Noted” shall be registered for the update of the Drawings’ Schedule and shall form part of the Construction Drawings that shall be available at the Site at all times.

-	Any approval, check, certificate, consent, examination, inspection, instruction, notice, proposal, request, test, or similar act by the Engineer (including absence of disapproval) shall not relieve the Contractor from any responsibility he has under the Contract, including responsibility for errors, omissions, discrepancies and non-compliances.

(d)	Particular Requirements for the “As-Built Drawings”

-	The requirements established in this Clause for completion of the Construction Drawings shall first be satisfied by the Contractor, and approved by the Engineer.

-	Based on the above condition, the Contractor shall prepare in consultation with the Engineer, the final format to be used for preparation of the as-built drawings (including relevant documents such as drawings, completed quantities, and backup data).

-	The completion and submittal of the “As-Built Drawings” is a mandatory condition for the issuance of the Taking Over Certificate. Therefore, it shall be made in accordance with these Specifications, and in accordance with Clause 4.1: “Contractor’s General Obligations”, Sub-Clause(d) of the Conditions of Contract, and all related procedures and stipulations of the contract.

The “As-Built Drawings” shall be submitted, for review and approval by the Engineer, 28 days before the commencement of Test on Completion (Clause 9: “Test on Completion” of the Conditions of Contract).

The procedure specified in Sub-Clause 10.1 of the Conditions of Contract for the issuance of the “Taking-Over Certificate”, may be suspended if the “As-Built Drawings” are not prepared and submitted as specified herein.

-	The Contractor shall initially submit two (2) sets of quality prints of the as-built drawings to the Engineer for review, comments, and approval. As-built drawings shall be submitted in turn according to the completed schedule.

-	After all corrections or comments as required by the Engineer have been addressed, and the “As-Built Drawings” have been approved, the Contractor shall submit to the Engineer the following:

  - One (1) original and six (6) clear photocopies, and

  - One (1) computer disk copy containing the electronic files of all drawings in an editable version by the approved application for the Project.

(3)	Drawing Copyrights

The rights for using, copying, or reproducing, partly or entirely, all or any of the drawings prepared by the Contractor as herein specified, are reserved and belong to the Employer at any time or in any situation.

(4)	Measurement and Payment

Excepting for the “As-Built Drawings” and unless stated otherwise, no separate measurement nor payment shall be made for all preparation and submittals of the “Contractor’s Drawings” required under this Clause. All costs in connection with the work specified herein shall be deemed included in the rates and prices of other related items of the work in the Bill of Quantities.

The preparation and submittal of the “As-Built Drawings” shall be measured and paid in “lump sum” (single instalment to be made upon the approval of the Drawings by the Engineer) which shall be the full and the final compensation to the Contractor as per Clause 112 and for the cost of all operations, materials and incidentals involved for completion of this item.

| Item No. | Description | Unit |
| --- | --- | --- |
| 117 | Contractor’s Drawings |  |
| 117-1 | As-Built Drawings (Mandatory Condition for TOC’s issuance). | lump sum |

118	INSURANCE AND WARRANTY

(1)	Description

-	These requirements shall be read and applied accordingly, and without detriment, to the requirements and provisions of Clause 18 “Insurance” of the Conditions of Contract.

-	The Contractor shall provide an evidence of insurance policies prior to the start of the Works.

-	With each application for Interim Payment Certificate under Sub-Clause 14.3 of the Conditions of Contract, the Contractor shall submit evidence consisting of receipts of payment or other verification as approved by the Employer that insurance policies are in effect.

-	The Contractor shall be responsible for all the costs for the warranty period of the Works in accordance with the related provisions of the Conditions of Contract.

-	The following securities shall be provided by the Contractor:

  - Performance Security

  - Insurance of the Works

  - Insurance of Contractor’s Equipment

  - Third-Party Insurance

The lump sum price for insurances includes all expenditures expected in providing and maintaining from Date of Commencement till end of Defect Liability Period, including expenditures resulting from administration work, dealing with and reporting of claim, stamps duties, taxes, etc.

In case of any damages to the works for which the payment has been made by the Employer, it’s the Contractor's obligation to reconstruct the same and claim for the payment from the insurance companies. No payment for the reconstruction of the damaged works shall be made by the Employer.

(2)	Measurement and Payment

Measurement and payment for provision and administration of the Insurances shall be on “lump sum” basis for all expenditure with regard to the provision of Insurances.

Payment of the lump sums shall be made under the relevant items in the Bill of Quantities on submission to the Engineer of acceptable Insurance policies, as required by the Conditions of Contract, with proof of payment of Insurance premiums. Where premiums are for one year only, 35% of the quoted lump sum shall be paid for the first year of the Contract period with a further 35% being paid on presentation of proof of payment of premiums for the rest of the construction period and the remaining 30% being paid on presentation of proof of payment of premiums for the Defects Notification Period.

| Item No. | Description | Unit |
| --- | --- | --- |
| 118-1 | Performance Security | lump sum |
| 118-2 | Insurance of the Works | lump sum |
| 118-3 | Insurance of Contractor’s Equipment | lump sum |
| 118-4 | Third-Party Insurance | lump sum |

SECTION 200: SITE CLEARANCE

201	CLEARING AND GRUBBING

(2)	Description of Work

(1)	Insert the following at the end of the Sub-Clause 201(2)(d) “Conservation of Flora”:

(e)	Clearing of Landslides

When instructed by the Engineer, the Contractor shall remove all debris from landslides as soon as possible after receiving instruction. The first priority will be the removal of material to maintain safe vehicular access and the second to remove material in order to continue construction of the works. The Engineer shall determine the amount of the material to be removed and/or any further work which may be required to stabilize the landslide.

Where a landslide has occurred in an area where the Contractor has commenced but not completed excavation works of any nature and such excavation works have, in the opinion of the Engineer, been the direct or indirect cause of the landslide, the responsibility for clearing and stabilizing the landslide shall rest entirely with the Contractor. Elsewhere the volume of material directed to be removed shall be measured in cubic meter.

(f)	Clearing of Debris

The Contractor shall remove all debris from causeways, culverts and side drains as per the instruction of the Engineer.

(4)	Measurement

(2)	Insert the following at the end of the Clause 201(4) “Measurement”:

Works related with Conservation of Topsoil and Conservation of Flora including excavation, conservation (or disposal if so instructed by the Engineer), haulage within 2.0Km including all incidentals for a proper stockpiling in an area approved by the Engineer, shall not be measured separately but deemed included in the respective main items for earthworks. In case that, further works for topsoil or planting can reuse these material, respective measurement and payment shall be made under respective clauses of Section 2800: “Bio-Engineering Works” as applicable and as approved by the Engineer. In such case, the measurement for reuse shall include hauling from the stockpiles, and related works for placing in Site and incidentals, but not the cost for soil as material itself to be reused, and excluding all cost and incidentals already included in the measured items of this Section.

Landslides and debris removed as per this specification shall be measured in cubic meter. Debris clearance from causeways, culverts and side drains shall be measured for one time only at the starting of the contract. Afterwards, debris clearance shall be covered by the Routine Maintenance.

Clearance of debris from box and pipe culverts and side drainages including haulage and disposal within 2.0Km including all incidentals for a proper completion, and approved by the Engineer, shall be measured in cubic metres.

(5)	Payment

(3)	Insert the following at the end of the Clause 201(5) “Payment”:

No payment shall be made in respect of clearing and grubbing carried out at the sites of quarries, borrow pits, Contractor’s facilities or any other area other than those areas specifically directed by the Engineer to be cleared and grubbed for the execution of the permanent works.”

Clearing of Landslide and Debris shall be paid at their respective contract unit rates which shall be full and final compensation to the Contractor as per Section 112 herein described.  However, for the works for “Cutting trees including stumps & roots removal of girth and compacted backfilling”, the Contractor shall do an estimation of total number based on the information available regarding to Environment, or by himself. These quantities shall be verified and confirmed by the Contractor, and approved by the Engineer, during execution of the Works.

No payment shall be made for the removal of the landslide of less than 20 cubic meters nor for the first 20 cubic meters in any one occurrence.

| Item No. | Description | Unit |
| --- | --- | --- |
| 200 | Site Clearance |  |
| 201 | Clearing and Grubbing |  |
| 201-1 | Clearing and Grubbing and Conservation of Top-Soil | sq.m. |
| 201-2 | Cutting trees including stumps & roots removal of girth and compacted backfilling above 300 mm to 600 mm | No. |
| 201-3 | Cutting trees including stumps & roots removal of girth and compacted backfilling above 600 mm to 900 mm | No. |
| 201-4 | Cutting trees including stumps & roots removal of girth and compacted backfilling above 900 mm | No. |
| 201-5 | Clearance of debris from box and pipe culverts and side drainages including haulage and disposal within 1.0Km | cu.m. |

202	DISMANTLING CULVERTS, BRIDGES, OTHER STRUCTURES AND PAVEMENT

(1)	Scope

(4)	Insert the following at the end of the Clause 202(1) “Scope”:

The Engineer shall designate all materials arising from dismantling as:

(i)	Scrap to be disposed of by the Contractor, or

(ii)	Salvageable material to be reused in the Works, or

(iv)	Salvageable material to be retained by the Employer

(3)		Dismantling Culverts and Bridges

(5)	Add the following new Clause (3): “Submittals” after the last paragraph of Clause 202(2): “General” and renumber all Clauses from Clause 202(4) “Dismantling Culverts and Bridges”:

(3)	Submittals

The Contractor, prior to the commencement of any work, shall prepare and submit to the Engineer, for respective approval, the following:

(a)	Shop Drawings in accordance with the requirements herein described in Sub-Section 117: “Contractor’s Drawings”, including detailed information regarding to all bridges, drainage structures, curbs, pavements, buildings, etc. to be dismantled and removed in accordance with the provisions and indications these Specifications.

(b)	Complete details of the method, equipment, and schedule to perform the dismantling and removal work pertinent to this item.

(c)	Inventory of the materials that could be recovered from demolition work and stockpiling of suitable materials from stripping that the Contractor intend to use for other items.

(d)	Detailed drawings showing disposal areas for non-reusable materials arising within the Site, or areas for stockpiling of the recovered materials planned to be used for other items.

(4)		Dismantling Culverts and Bridges

(6)	Insert the following after the last paragraph of the renumbered Clause 202(4) “Dismantling Culverts and Bridges”:

Particular Requirements:

(i)	Culverts, bridges and other drainage structures which still been used by traffic shall not be removed until satisfactory arrangements have been made to accommodate the traffic in accordance with the stipulations of Sub-Section 102: “Accommodation of Traffic” and other relevant specifications.

(ii)	The removal of existing culverts within embankment areas shall be required only as necessary for the installation of new structures. Abandoned culverts shall be broken down, crushed, and sealed or plugged.

(iii)	All culvert sections removed, which are not designated for stockpiling or relaying, unless otherwise directed by the Employer, shall become the property of the Contractor and shall be removed from the Project or disposed of in a manner approved by the Engineer.

(iv)	Unless otherwise directed, the substructures of existing structures shall be removed down to the natural stream bottom and those parts outside the stream shall be removed down to at least 300 mm below the natural ground surface.

(v)	Where such portions of the existing structures lie wholly or in part within the limits of a new structure, they shall be removed if necessary to accommodate the construction of the proposed structure. Where only a section of the existing structure is to be demolished, the Contractor shall execute the work in such a manner as to avoid damage to the section designated to remain. All details of the Contractor's proposed working methods shall be submitted to the Engineer for approval.

(vi)	Any structures designated to become the property of the Contractor shall be removed from the right-of-way.

(vii)		Existing bridges or other structures, when specified by the Engineer to be salvaged, shall be carefully dismantled without damage. Salvaged members shall be match-marked before dismantling, unless the Engineer waives such match marking. All salvaged materials shall be stored as requested by the Engineer.

(ix)	Unless otherwise waived by the Engineer in writing, all concrete removed that is of a suitable size for rip-rap and not needed for such use on the Project, shall be stockpiled at locations designated by the Engineer, for use by the Employer.

(5)		Dismantling Other Structures and Pavements

(7)	Insert the following new Clause after the last paragraph of the renumbered Clause 202(5) “Dismantling Other Structures and Pavements”, and renumber the Clauses for Back-filling, Measurement and Payment:

(6)	Removal of Buildings and Structures

In dismantling existing building and other structures (not included in previous Clause (5): “Dismantling Other Structures and Pavements”), the Contractor shall satisfy the following particular requirements:

Particular Requirements:

(a)	Upon receipt of notification by the Engineer, the Contractor shall take over all responsibility for the vacant buildings or buildings previously acquired, and shall proceed with the removal thereof as specified.

(b)	Unless otherwise indicated in the Drawings or directed by the Engineer, the Contractor shall complete the demolition and removal of buildings and structures together with all foundations and retaining walls, piers, partitions, and columns down to a plane 30 cm below the ultimate grade in the area.

(c)	Walls shall be broken into pieces not exceeding 60 cm2 for any area of surface, mixed with previous backfill materials and placed in basement areas.

(d)	Basement floor slabs shall be broken and displaced into pieces not exceeding 60 cm2 for any area of surface and shall be left in place. Excess debris shall be removed and disposed of. All other foundations, concrete floor slabs, sidewalks, signs, sheds, garages, and fences and all other incidental and collateral work necessary to fully complete the removal of buildings and appurtenances shall be removed and disposed of as directed.

(e)	The Contractor shall inspect each building to be removed and determine for himself the work involved and the equipment and materials required for such demolition work.

(f)	The Contractor shall arrange for the disconnection of all utility services that serve the building or buildings in accordance with the respective requirements and regulations of the relevant government authorities and utility agencies involved.

(g)	The Contractor shall disconnect and properly seal, in a manner approved by the relevant local government agency or agencies responsible and the Engineer, all sewer outlets that serve any building or buildings that the Contractor is directed to remove.

(h)	The Contractor shall keep the Engineer informed of his plans for the performance of any work in connection with the sealing off of such outlets in order that proper inspection may be provided at the time the work is performed.

(i)	The Contractor shall conduct his operations in such a manner as to avoid potential hazards to persons and vehicles. Once a work on any building has commenced, the work shall be continued promptly and expeditiously until completion.

(j)	All debris shall be removed from basement areas and concrete floors shall be broken. The area shall then be filled with previous backfill material simultaneously with the breaking of the foundation walls so that the previous backfill material will be well integrated with the pieces of broken concrete.

(k)	Concrete slabs resting on earth and forming walks, driveways, or the first floor slabs of buildings without basements, except as indicated in the Drawings, shall be broken up and removed from the Site.

(l)	The Contractor shall be responsible for the removal of any additional small buildings or miscellaneous structures which have not been indicated in the Drawings.

(m)	Existing underground storage tanks on the Site or within the buildings shall be removed and disposed of by the Contractor. The Contractor shall take all necessary measures and precautions during the removal and disposal of the liquid, sludge or other waste materials from the tanks as well as the removal of the existing tanks. The Contractor shall backfill with fine aggregate the areas where the tanks are removed. This backfill shall be in accordance with the specifications herein described.

(n)	Operations necessary for the removal of an existing structure or obstruction, which may damage new construction, shall be completed prior to placing the new work, unless otherwise stipulated in the Contract Documents.

(7)		Back-filling

(8)	After renumbering, delete the existing Clauses for Measurement and Payment, and insert the following revised Clause after Clause 202(7) “Back-filling”:

(8)	Measurement and Payment

Prior to commencement of dismantling, the work of dismantling structures shall be measured in the units given here below.

Associated works like disposal, stockpiling, marking and numbering, etc., shall not be measured separately but deemed included in the respective main items.

Payment for dismantling shall be made after the completion of all the works herein specified and successful disposal of cleared materials.

The various dismantling works shall be paid at their respective contract unit rates which shall be the full and the final compensation to the Contractor as per Clause 112 and for the cost of all operations involved for completion of this item.

| Item No. | Description | Unit |
| --- | --- | --- |
| 202 | Dismantling Culverts, Bridges, other Structures and Pavement |  |
| 202-1 | Dismantling brick masonry | cu.m |
| 202-2 | Dismantling stone masonry | cu.m |
| 202-3 | Dismantling concrete (Plain and reinforced) | cu.m |
| 202-4 | Dismantling gabion | cu.m |
| 202-5 | Dismantling steel structures | ton |
| 202-6 | Dismantling timber structures | cu.m |
| 202-7 | Dismantling pipes internal diameter less than 1200mm | lin.m |
| 202-8 | Dismantling kerbs and gutters | lin.m |
| 202-9 | Dismantling asphalt pavement and footpaths | cu.m |

Section 300: BLANK

(1)	Add in this Blank section, the following New Section 300:

SECTION 300: SOIL IMPROVEMENT

301	REINFORCED SOIL

(1)	Scope

The work covers construction of reinforced soil structures together with the construction of earthwork in layers, assembly and placing of reinforcing elements and facia elements during the construction process and all associated works.

The work shall include the design and construction of the reinforced soil structure and ground improvement measures required, if any.

The reinforced soil retaining structures can be used as, (i) Reinforced soil wall or (ii) Reinforced soil slope

Reinforced soil structures with slope face angles steeper than 70° are categorized as “reinforced soil walls” and those with slope face angle less than 70° are considered as “reinforced soil slopes”.

(2)	Design

The following standards for design and construction of reinforced soil walls shall be applied:

- Standards for Design and Construction of Reinforced Soil Walls. Japan Road Association 2004.

- Design and Construction of Mechanically Stabilized Earth Walls and Reinforced Soil Slopes. FHWA-NHI

or equivalent as approved by the Engineer.

(3)	Reinforcing Element

The following Geotextile, Geogrids and other Geosynthetic materials  might be used as Reinforcing Elements:

(a)	Reinforcing Geosynthetics

High strength high tenacity geotextile fabrics used as reinforcement in the construction of reinforced slopes or in the base of reinforced soil structure as reinforcement, shall be considered as reinforcing element and shall satisfy all the requirements stipulated for geosynthetic reinforcing elements, in Clause 2407 and Clause 2408.

Geotextile fabric used for separation, filtration and/ or drainage shall satisfy the requirements given in relevant Clauses of Section 2400: “Miscellaneous Structures”.

The manufacturer of geogrids, geotextiles, geostrips, polymeric strips or straps, polymeric ties or any other geosynthetic material, including any proprietary geosynthetic material, for use as reinforcing element shall fulfil the following requirements:

          - Shall have ISO (ISO-9001) or CE Certification for manufacturing process and quality control, and

          - The product shall have certification for use as soil reinforcing material from an agency accredited for certifying geosynthetic reinforcement products like: BBA or NTPEP.

          - The manufacturer shall provide test reports from an independent laboratory with valid accreditation, for all the tests needed to establish all the reduction factors listed below

- RFCR:	Reduction factor for creep.

- RFID:	Reduction factor for installation damage

- RFW:	Reduction factor for weathering

- RFCH:	Reduction factor for chemical/ environmental effects.

- fs:	Factor for the extrapolation of data

All the above factors shall be determined in accordance with the provisions of ISO/TR 20432- “Guide to the determination of long-term strength of geosynthetics for soil reinforcement.” No default reduction factors shall be used in design.

(b)	Test and Standard of Acceptance

Test for the ultimate tensile strength shall be carried out on a random sample for each grade of reinforcement as per ISO-10319: “Geosynthetics: Wide-width tensile test”. The test results shall be accompanied by stress-strain curves showing strength at 2% and 5% strain and strain/elongation at failure.

The manufacturer shall also provide the results of ultimate tensile strength for each lot and all grades of reinforcement proposed for use in the project.

Annual Average Daily Temperatures (AADT)/ design temperature of the project site shall be worked out and values of reduction factor for creep RFCR and for RFCH shall be provided as per procedures given in ISO/TR-20432: “Guidelines for the determination of the long-term strength of geosynthetics for soil reinforcement”.

The manufacturer / Supplier shall provide values of:

- Pull-out coefficient as per ASTM D6706: “Standard Test Method for Measuring Geosynthetic Pull-out Resistance in Soil” or  EN13738  “Geosynthetics- Determination of pull out resistance in soil” and

- Coefficient of interaction between reinforced fill soil and geogrids as per ASTM D5321: “Standard Test method for Determining the Coefficient of Soil and Geosynthetic or Geosynthetic and Geosynthetic Friction by the Direct Shear method” or as per IS: 13326: Part 1-1992 “Method of test for the evaluation of interface friction between geosynthetics and soil: Part 1 Modified direct shear technique” for all types of geogrids.

Each roll shall have at least one identification label with roll number and product type.

(4)	Earth fill

The fill in the reinforced soil zone shall have drained or effective angle of friction not less than 28°, measured in accordance with IS: 2720 (Part 13), by conducting a drained direct shear test. In case the fill material has 25 percent or more particles of 4.75 mm or larger, drained shear test using large shear box may be conducted (IS: 2720: Part 39: Section 1) . The gradation of fill soil shall be as per following limits.

| Sieve Size. | Percentage passing (AASHTO T27 & T11) |
| --- | --- |
| 40 mm | 100% |
| 425 micron | 0 – 60% |
| 75 micron | Less than 15% |
| PI | ≤6 |

Materials with more than 15 percent passing 75 micron sieve, but less than 10 percent of particles smaller than 15 microns are acceptable provided that PI is less than 6 and angle of friction is not less than 30°.

Fly ash may be used as fill material in reinforced soil walls provided its angle of internal friction is not less than 30° and PI is less than 6. Grading requirements need not be completely satisfied. Reference may be made to IRC Guide lines on Use of Fly-ash in Road Embankments (IRC SP-58).

The fill material used in the reinforced soil zone shall be free from organic or other deleterious materials and shall not react adversely (chemically, electrically or biologically) with the reinforcement material and / or facia material.

Properties of fill soil in the reinforced zone, unreinforced zone (or retained/back fill) soil and the foundation soil shall be determined accurately during the construction phase, as per quality assurance plans and directions of Engineer so as to ensure that these are the same as those considered in the design phase.

The fill soil in the unreinforced zone shall conform to the requirements specified in the design.

Environmental Conditions of Fill:

Where geosynthetic reinforcement is used for reinforcing elements manufactured from polyester yarn, pH value of the fill material shall be between 3 and 9, and for reinforcing elements manufactured from PVA, PP and HDPE, the pH value shall be greater than 3.

(5)	Material

(a)	Facia Material

The facing system shall consist of Gabion with integrated reinforcement or 	Metallic prefabricated in  different shapes including double twisted woven steel wire mesh, as shown in the Drawings or as approved by the Engineer.

Facing shall be sufficiently flexible to withstand any deformation of the fill and foundations.

- Gabion Facia:

Where gabion facia is used, it shall conform to the provisions of BS: 8006-1: 2010 and EN-14475 and made of mechanically fabricated and mechanically selvedge double twisted hexagonal mesh. Wire used for the double twisted mesh shall be hot dip galvanized as per NS-163, with wire and mesh properties in accordance with EN 10223 with minimum Zn or Zn-Al alloy coating as per EN 10244 and 0.5 mm nominal thick polymeric (e.g. PVC, PA6, etc.) coating as per EN 10245.

- Woven Steel Wire Mesh:

Where mechanically woven steel wire mesh in wrap around form is used for facing, the steel wire mesh as per IS: 16014, shall be with mechanical selvedging and bottom panel shall continue as an integrated tail mesh.

(b)	Connection between the Facia and Reinforcement

In case of gabion facia and woven steel wire mesh facia, where the reinforcement is held by friction between the facia block and the primary reinforcement, the results of pull-out test as per EN-13738 : “Geosynthetics- Determination of pull out resistance in soil”.

(6)	Construction Details

(a)	Depth of Foundation

Foundation shall be prepared as shown in the Drawings, and provided at founding level to receive the facia or the bottom most reinforcement.

For gabion around facia the strip footing is not required but the foot print shall be compacted to 95 % of maximum proctor density and/or a coarse gravel base shall be provided.

The depth of embedment below the finished ground level at the foot of the wall shall not be less than 1000 mm. In case rock is met above founding level, the depth of embedment shall be adjusted as per ground conditions.

(b)	Laying of Reinforcement

Reinforcing elements such as geogrids, shall be stretched and held taut by driving nails or pegs at the farther end.

(c)	Drainage

Drainage bay shall be provided as shown in the Drawings. The width of the drainage bay shall be 600 mm behind the facing element, or as shown in the Drawings. Alternatively geosynthetic drainage composite can be used which shall meet filter criteria and flow requirements.

The drainage material shall conform to the specifications of the filter media as per Clause 3110 of this Specifications.

Where reinforced soil retaining walls are provided to support hill cuts, the face of the hill cut is to be considered as a retained fill. To ensure that the runoff and surface water is drained, a drainage bay should be provided between the retained soil and the reinforced soil to ensure proper drainage. The drainage bay should be designed to carry the discharge and should be provided vertically or parallel to the face at the back of the retained fill and continued in a horizontal extent to a depth well below the toe of the Reinforced soil retaining wall and lead to a drain meant to carry the discharge away from the Reinforced soil retaining wall.

(d)	Laying and Compaction

The reinforcing elements shall be laid free from all kinks, damage and displacement during placing, spreading, levelling and compaction of the fill. The programme of filling shall be such that no construction plant moves directly on the reinforcement.

All construction plant having a mass exceeding 1500 kg shall be kept at least 2.0 m away from the face of slope or wall.

In the area up to 2.0 m from the face of slope or wall, the following compaction plant shall be used:

- Vibratory roller having a weight per metre width not exceeding 1300 kg with total weight not exceeding 1500 kg

- Vibratory plate compactor of maximum weight 1000 kg

- Vibro-tamper having a weight not exceeding 75 kg

Before allowing the movement of vehicles over the reinforcement, a minimum compacted thickness of 150 mm shall be provided over the reinforcement and the speed of the vehicles shall be restricted to 10 km/hr.

During construction of reinforced fill, the retained material beyond the reinforcement at the rear of the structure shall be maintained at the same level as reinforced fill.

Fill shall not be placed on surface that contains mud, organic soil or area that have not met compaction requirement.

The thickness of compacted layer shall not be more than 300 mm, compacted to 97 per cent of maximum laboratory density measured as per IS: 2720 (Part 8).

(e)	Rock Anchor and Connecting Fittings

- Rock Anchor shall consist of the construction of re-bar and grout as shown in the Drawings, and shall meet the requirements of Section 2014: “Reinforcement” of these Specifications.

- Connecting fittings shall have sufficient tensile strength and also shall have the function to connects the geotextile with panel skin wall/anchor bar properly

(7)	Measurement

Filling embankment shall not be measured nor paid under this Section. All works and materials for filling embankment for Soil Reinforcement shall be measured in cubic metres and paid under Section 900: “Earthworks”, Pay Item 900-4: “Filling in Reinforced Soil Zone”.

Sub-surface drains used in reinforced walls shall not be measured nor paid under this Section. All works and materials for sub-surface drains shall be measured and paid accordingly under Section 2404: “Sub-surface Drains”.

(a)	Reinforced Soil Wall

The measurement for payment for reinforced soil wall shall be in square metres of finished work of each face and shall be measured in the plane of final inclination specified in the drawings. The measurement of length shall be the finished work along the length of the road. The measurement of height along the slope shall be done from the top level of the footing on which the facia element is placed to the top of the capping beam.

(b)	Reinforced Soil Slope

The measurement for payment for reinforced soil slope shall be in square metres of finished work of each face and shall be measured in the plane of final inclination specified in the drawings. The measurement of length shall be the finished work along the length of the road. The measurement of height along the slope shall be done from the top of the levelling pad where provided, to the top of the embankment. Where levelling pad is not provided, the height shall be measured, in the final plane of inclination specified in the drawings, from the bottom of the slope face.

(8)	Payment

Reinforced Soil Wall and Reinforced Soil Slope actually completed and approved by the Engineer, measured as stated above, shall be paid at the respective contract unit rates. In addition to Clause 112, the Contract unit rate shall be full and final compensation for cost of investigation, design and construction of ground improvement measures, including all materials such as facia, reinforcing elements, any incidental to complete the works as shown in the Drawings and as specified herein.

| Item No. | Description | Unit |
| --- | --- | --- |
| 300 | Soil Improvement |  |
| 301 | Reinforced Soil |  |
| 301-1 | Reinforced Soil Wall, Type-1 (expanded metal 1:0.1) | sq.m. |
| 301-2 | Reinforced Soil Wall, Type-2 (gabion unit mesh 1:0.3) | sq.m. |
| 301-3 | Reinforced Soil Wall, Type-3 (reinforced concrete panel) | sq.m. |
| 301-4 | Rock Anchor, dia.25mm, L=2.0m (for expanded metal 1:0.1) | No. |
| 301-5 | Rock Anchor, dia.25mm, L=3.0m (for expanded metal 1:0.1) | No. |

Geosynthetics used for reinforced soil walls construction, shall be performed, measured and paid, respectively in accordance with the relevant Sub-Sections of these specifications such as:

- 2407 “Geosynthetics for Road and Bridges Works”

- 2408 “Geotextile for Filtration, Drainage and Separation”

- 2409 “Geosynthetics for Earth Reinforcement” (Geogrid)

- 2410 “Geo-composite drains”, etc.

As applicable, and as approved by the Engineer.

SECTION 400: FENCING

406	MEASUREMENT

(1)	Delete the full text of the Sub-Section 406 “Measurement” and replace with the following:

The Works for providing and installation of permanent fencing shall be measured in linear meters inclusive of all excavations, concrete Class D (M20/20), installation of posts and standards, barbed wires, painting if any, entry gates and other requirements necessary to complete the work as per this Specification, and as shown in the Drawings.

Sliding Gates, including all materials, railing, installation and incidentals shall be measured per number and classified by its width.

Temporary fencing shall not be measured nor paid separately but shall be deemed included in the rates and prices entered for the related main items.

407	PAYMENT

(2)	Delete the full text of the Sub-Section 407 “Payment” and replace with the following:

Permanent fencing, measured as stated above, shall be paid at the contract unit rate which shall be the full and the final compensation to the Contractor as per Clause 112.

| Item No. | Description | Unit |
| --- | --- | --- |
| 400 | Fencing |  |
| 400-1 | Fence type-A (Net, H=1.80m) | lin.m |
| 400-2 | Fence type-B (Grid, H=1.80m) | lin.m |
| 400-3 | Fence type-C (Net, H=1.80m with barbed wire) | lin.m |
| 400-4 | Fence type-D (Grid, H=1.80m with barbed wire) | lin.m |
| 400-5 | Fence type-E (Barbed wire, H=1.80m) | lin.m |
| 400-6 | Fence type-F (Net. H=0.80m on bridge guard wall) | lin.m |
| 400-7 | Fence type-G (Round barbed wire) | lin.m |
| 400-8 | Fence type-H (Movable, H=1.80m) | nos. |
| 400-9 | Fence type-I (Movable, H=0.90m) | nos. |
| 400-10 | Gate type-A (Sliding gate, W=8.00m) | nos. |
| 400-11 | Gate type-B (W=6.00m) | nos. |
| 400-12 | Gate type-C (W=5.00m) | nos. |

All costs in connection with the temporary fencing work shall be considered to be included with other related items of work in the BOQ.

SECTION 500: QUALITY CONTROL

501	SCOPE

(1)	Insert the following as per the last paragraph of Sub-Section 501 “Scope”:

This Section shall be interpreted, supplementary to, and without detriment to the stipulations of Sub-Clause 4.9: “Quality Assurance” and Sub-Clause 7.4: “Testing”, and all relevant stipulations of the Conditions of Contract.

502	CONTRACTOR RESPONSIBLE FOR THE QUALITY OF THE WORKS

(2)	Delete the last two (2) sentences of the second paragraph of the Sub-Section 502 “Contractor Responsible for the Quality of the Works” commencing at “The list of laboratory equipment …..” and replace with the following:

The Contractor shall obtain timely, and in accordance with the approved Programme, the approval by the Engineer for the laboratory facilities to be provided and the list of equipment to be purchased.

In designing the facilities and equipping the laboratory, the Contractor must himself make allowance for the need to be able to conduct all the tests required by these Specifications in regard to the works or materials itemized or implied in the drawings and the Bill of Quantities.

The Contractor remains responsible for the supply of all equipment of whatsoever nature and in whatsoever quantity required for carrying out all of the tests specified at the frequency specified.

(3)	Insert the following after the last paragraph of the Sub-Section 502 “Contractor Responsible for the Quality of the Works”:

Laboratory Check by the Engineer:

(a)	Rights to Check Testing Laboratories:

The Engineer shall at all times have full right and power to check the Laboratory installations and equipment for verifying their due compliance with the Specifications, and to confirm the adequacy of the Laboratory technicians’ testing procedures and techniques.

(b)	Engineer’s Access and Use of Laboratory Facilities:

The Engineer shall at all times have full right and power to access the Laboratory and the respective laboratory records related to the Project. The Engineer may, at any time, use the Laboratory facilities to conduct independent testing, or require the Laboratory personnel employed by the Contractor, to conduct such testing.

509	ACCEPTANCE TESTS FOR COMPLETED WORKS OR PARTS OF THE WORKS

(4)	Add the following Clause after the Clause 509(2): “Other Works and Equipment” and before Sub-Section 510: “Recapitulative Schedule of Tests”:

(3)	Testing and Commissioning

Testing and Commissioning of the completed installation of lighting system, transmission lines, facilities, etc., or those works indicated by the Engineer, shall include a working demonstration of satisfactory performance for the Engineer and Employer.

(a)	Without detriment of the stipulations of the Contractual Documents (Condition of Contract and Specifications), a Taking-Over Certificate may be issued by the Engineer when, in conformity with Clause 10: “Employer’s Taking-Over” of the Conditions of Contract:

- The installation has been satisfactorily completed.

- “As-built” documents including drawings and records have been submitted to an acceptable standard.

- Certification of any records has been made as required by the Engineer.

(b)	The Contractor shall provide a test and commissioning report to be approved by the Engineer. All tests and commissioning information shall be delivered in hard copy and soft copy together with advice on, software required for accessing the soft copy. All information delivered to the Employer has to be checked and approved by the Employer based on the recommendations by the Engineer.

(c)	The Contractor shall conduct testing and commissioning of the complete installation with good industrial practice. The Contractor shall carry out continuity tests on all cables and devices for termination and pair integrity. All records of tests shall be kept and submitted to the Engineer in the suitable tabulation for the inspection and acceptance.

(d)	Any item that found to be defective and damage during testing and commissioning shall be replaced by the Contractor at his expense.

511	LABORATORY

(4)	Ownership

(5)	Delete the full text of the Clause 511 (4):”Ownership” and replace with the following:

Unless otherwise stated in the Contract, excepting the laboratory facilities which shall be removed by the Contractor after the Project completion, the ownership of all laboratories and equipment shall revert to the Contractor.

(6)	Measurement

(6)	Delete the full text of Clause 511(6): “Measurement” (sub-clauses a, b and c) and replace with the following:

Unless otherwise stipulated in these Specifications or in the Conditions of Contract, measurement and payment for all Clauses of this Section 500 shall be made only and exceptionally for the items shown here below:

- Laboratory: which has been set up and equipped in accordance with the requirement of these Specifications, including the laboratory building, services, essential supplies like water, electricity, sanitary services and all equipment, tools, materials, labour and incidentals to perform tests and other operations of quality control according to the Specification requirements shall be measured for payment in number;

- Laboratory Equipment: The laboratory equipment provided to perform all testing required by these specifications shall be measured for payment in lump sum.

The salaries, allowances, accommodations, transportation and all incidentals related to the Contractor’s Laboratory Staff shall be deemed included in, or distributed among, all the rates and prices entered for the unit prices of the Bill of Quantities which will need the use of laboratory for testing and quality monitoring. Therefore, separate measurement shall not be made and shall not be listed in the BOQ.

(7)	Payment

(7)	Delete the full text of Clause 511(7): “Payment” (sub-clauses a, b and c) and replace with the following:

The Laboratory and Laboratory Equipment, measured as stated above, shall be paid at the contract unit rate which shall be the full and the final compensation to the Contractor as per Clause 112.

- Laboratory: shall be paid in number of the laboratory’s installations actually set-up for the Project and approved by the Engineer;

- Laboratory Equipment: shall be paid on a “lump sum” according to the following instalments for each laboratory:

(i)	Thirty percent (30%) upon completion of mobilization, installation, satisfactorily completion and equipping and upon occupation and use of the Laboratory to the extent that the staff can properly perform their duties under the Contract to the satisfaction of the Engineer

(ii)	Fifty percent (50%) during the execution of the Works split in partial payments in proportion to the work progress subject to certification issued by the Engineer.

(iii)	Twenty percent (20%) upon issuance of the Taking-Over Certificate and respective removal of installations, reinstated the site to the satisfaction of the Engineer and demobilization following the Engineer’s certification that these facilities are no longer required

| Item No. | Description | Unit |
| --- | --- | --- |
| 500 | Quality Control |  |
| 500-1 | Laboratory | No. |
| 500-2 | Laboratory Equipment | lump sum |

SECTION 600: MATERIALS AND TESTING OF MATERIALS

607	TRADE NAMES AND ALTERNATIVES

(1)	Insert the following before the first paragraph of the Sub-Section 607:”Trade Names and Alternatives”:

These specifications shall read and interpreted accordingly and without detriment to the requirements stated in Clause 13: “Variations and Adjustments”; Sub-Clause 13.1: “Value Engineering” of the Conditions of Contract.

612	SOIL AND GRAVELS

(2)	Standard Methods of Testing

(2)	Delete the tittle of Table 6.4 in Clause 612 (2): “Standard Methods of Testing” and replace with the following:

Table 6.4:	Tests Procedures Applicable to Disturbed and Undisturbed Samples of Soils and Gravels

613	STONE, AGGREGATE, SAND AND FILLERS

(1)	Sampling and Preparation of Samples

(3)	Delete the full text of Clause 613 (1): “Sampling and Preparation of Samples” and replace with the following:

Sampling and sample preparation shall comply with the standards IS: 2430 and the standards for test procedures shown in Table 6.5.

627	GEOTEXTILES

(4)	Delete the tittle of Sub-Section 627: “Geotextiles” and replace with the following:

627	GEOTEXTILES AND GEOGRIDS

(1)	Geotextiles

(the text for Clause 627(1): “Geotextiles” remains without modifications)

(5)	Add the following Clause after the last paragraph of Clause 627(1): “Geotextiles” and before Sub-Section 628: “Timber for Structural Works”:

(2)	Geogrids

Geogrids shall be made of high tenacity polymer yarn jointed at cross points by weaving. Unless otherwise shown on the Drawing, all geogrids shall meet the minimum strength in terms of Minimum Average Roll Value (MARV) as specified in Table 6.16

Table 6.16 Specification for Geogrids

| Property | Test Method | Unit | Requirement |
| --- | --- | --- | --- |
| Stiffness at 0.5% strain | ISO 10319 | KN/m | ≥350, both in machine and cross machine direction |
| Tensile strength @ 2% strain | ASTM D6637 | KN/m | ≥15% of Tilt : both in machine and cross machine direction |
| Tensile strength @ 5% strain | ASTM D6637 | KN/m | ≥20% of Tilt : both in machine and cross machine direction |
| Junction Efficiency for extruded geogrids | GRI-GG2-87 or ASTM- WK 14256 | - | 90% of rib ultimate tensile strength |
| Ultraviolet stability | ASTM D4355 | - | 70% after 500 hrs. exposure |

SECTION 700: PIPE DRAINS, PIPE CULVERTS AND CONCRETE CHANNELS

701	PIPE DRAINS AND PIPE CULVERTS

(9)	Pipes Bedding and Encasing

(1)	Delete the Sub-Clauses 701(9)(b) “Concrete Bedding” and (c) “Concrete Encasing” and replace with the following:

(b)	Concrete Bedding

Where indicated on the Drawing, concrete bedding shall be made on Concrete Class-E confirming the requirements of Section 2000 so that the bottom of the pipe rests on concrete of the specified thickness. The thickness of the concrete below the pipe and the height to which it extends upwards shall be as shown on the Drawing.

(c)	Concrete Encasing

As shown on the Drawing, encasing of pipe culverts shall be as shown in the Drawings for the following two types:

- 90ºEncasement: Partial encasement with Concrete Class D without steel reinforcement

- 360ºEncasement: Full encasement with Concrete Class D with steel reinforcement as show in the Drawings

In carrying out this work the Contractor shall take care to pack the concrete under and around the pipes to ensure even bedding and solidity in the concrete; in no instance shall the concrete be thrown directly on the pipes. The concrete shall be placed in such a way that all spaces around the pipe are completely filled with concrete. Concrete casing shall be cast in one continuous operation until completed. The upper surface of the concrete shall be struck off with a wooden screed and neatly finished off.

(15)	Measurement

(2)	Delete the full text of Clause 701(15):”Measurement” and replace with the following:

The diameter of pipe culverts shown in the BOQ for payment shall be measured by the internal surface of item of pipes.

Pipes shall be measured along their centre between the inlet and outlet ends in linear meters of individual pipe element of each sizes, completed and accepted in place.

The following shall not be measured nor paid separately but shall deemed be included in the rates and unit prices of the main item established for pipe drainages:

- Collar

- Concrete bedding

- Concrete for encasing

- Steel reinforcement

The works for excavation, backfilling and ancillary works such as head walls, wing walls, etc. shall be measured separately as provided for in the respective Sections of these Specifications.

(16)	Payment

(3)	Delete the full text of Clause 701(16):”Payment” and replace with the following:

Pipes, measured as provided above, shall be paid for at the contract unit rates per linear meter of each particular size measured by the internal surface of pipes which shall be the full and the final compensation for compliance of all requirements herein specified in addition to those specified in Clause 112, including furnishing, producing, all materials and for furnishing all labour, materials, tools, equipment and any incidentals to complete the work as shown in the Drawings, as required by these Specification Sections, and/or as directed by the Engineer..

Cost for cement jointing shall deemed to be included in the contract unit rate of pipe.

Ancillary works such as excavation, backfilling, granular bedding (if any) and masonry shall be paid for separately, as provided under respective Clause of these Specifications.

| Item No. | Description | Unit |
| --- | --- | --- |
| 700 | Pipe Drains, Pipe Culverts and Concrete Channels |  |
| 701 | Pipe Drains and Pipe Culverts |  |
| 701-1 | Concrete pipe class NP3 internal diameter 900mm with 90degree concrete foundation | lin.m |
| 701-2 | Concrete pipe class NP3 internal diameter 900mm with 360degree concrete foundation | lin.m |
| 701-3 | Concrete pipe class NP3 internal diameter 600mm with 90degree concrete foundation | lin.m |
| 701-4 | Concrete pipe class NP3 internal diameter 600mm with 360degree concrete foundation | lin.m |
| 701-5 | Concrete pipe class NP3 internal diameter 900mm (Re-used) with 90degree concrete foundation | lin.m |

702	CONCRETE CHANNELING

(4)	Insert the following as per the last paragraph of the Sub-Section 702(5):”Measurement”:

Concrete covers shall be measured as the number of covers satisfactorily installed by the Contractor as shown in the Drawings and approved by the Engineer.

(5)	Insert the following as per the last paragraph of the Sub-Section 702(6):”Payment”:

| Item No. | Description | Unit |
| --- | --- | --- |
| 702 | Concrete Channeling |  |
| 702-1 | Masonry U-shaped drain 475x450 type-1 (MU1-475x450) | lin.m |
| 702-2 | Masonry U-shaped drain 475x500 type-2 (MU2-475x500) | lin.m |
| 702-3 | Masonry U-shaped drain 450x500 (MU-450x500) | lin.m |
| 702-4 | Masonry U-shaped drain 775x450 (MU-775x450) | lin.m |
| 702-5 | Masonry U-shaped drain 775x500 (MU-775x500) | lin.m |
| 702-6 | Masonry U-shaped drain 1000x1000 (MU-1000x1000) | lin.m |
| 702-7 | Masonry vertical drain 600x700 (MV-600x700) | lin.m |
| 702-8 | Masonry vertical drain 400x400 (MV-400x400) | lin.m |
| 702-9 | Concrete U-shaped drain 300x300 (CU-300x300) | lin.m |
| 702-10 | Concrete U-shaped drain 350x350 (CU-350x350) | lin.m |
| 702-11 | Concrete U-shaped drain 400x400 (CU-400x400) | lin.m |
| 702-12 | Concrete U-shaped drain 450x450 (CU-450x450) | lin.m |
| 702-13 | Concrete U-shaped drain 500x500 (CU-500x500) | lin.m |
| 702-14 | Concrete U-shaped drain 500x700 (CU-500x700) | lin.m |
| 702-15 | Concrete U-shaped drain 600x600 (CU-600x600) | lin.m |
| 702-16 | Concrete U-shaped drain with cover for light vehicle 300x300 (CU(LC)-300x300) | lin.m |
| 702-17 | Concrete U-shaped drain with cover for light vehicle 400x400 (CU(LC)-400x400) | lin.m |
| 702-18 | Concrete U-shaped drain with cover for light vehicle 450x450 (CU(LC)-450x450) | lin.m |
| 702-19 | Concrete U-shaped drain with cover for light vehicle 500x500 (CU(LC)-500x500) | lin.m |
| 702-20 | Concrete U-shaped drain with cover for light vehicle 500x700 (CU(LC)-500x700) | lin.m |
| 702-21 | Concrete U-shaped drain with cover for heavy vehicle 300x300 (CU(HC)-300x300) | lin.m |
| 702-22 | Concrete U-shaped drain cover for heavy vehicle 400x200 (CU(HC)-400x200) | No. |
| 702-23 | Concrete U-shaped drain with cover for heavy vehicle 500x500 (CU(HC)-500x500) | lin.m |
| 702-24 | Concrete U-shaped drain with cover for heavy vehicle 500x600 (CU(HC)-500x600) | lin.m |
| 702-25 | Concrete U-shaped drain with cover for heavy vehicle 500x700 (CU(HC)-500x700) | lin.m |
| 702-26 | Concrete cover for pedestrian 500x1100x100 | No. |
| 702-27 | Sewage casting catch basin cover 600 diameter (Re-used) for pedestrian with concrete slab 1700x1700x250 | No. |
| 702-28 | Sewage casting catch basin cover 600 diameter for pedestrian with concrete slab 1700x1700x250 | No. |
| 702-29 | Concrete slab catch basin cover 2no.- 300x700 for pedestrian with concrete slab 1400x1400x200 | No. |
| 702-30 | Concrete slab catch basin cover 2no.- 300x700 for pedestrian with concrete slab 1700x1700x250 | No. |
| 702-31 | Steel grating catch basin cover 600x600 for heavy vehicle with concrete slab 1400x1400x250 | No. |
| 702-32 | Steel grating catch basin cover 600x600 for heavy vehicle with concrete slab 1700x1700x300 | No. |
| 702-33 | Sewage casting catch basin cover 600 diameter for heavy vehicle with concrete slab 1400x1400x250 | No. |
| 702-34 | HDP pipe 160mm | lin.m |
| 702-35 | Catch pit cover (500x500) of welded MS bar of dia. 20mm with opening 50mmx50mm | No. |

SECTION 800: QUARRIES, BORROW PITS, STOCKPILE AND SPOIL AREAS

802	DEFINITIONS

(1)	Insert the following as first item of the list shown in the Sub-Section 802:”Definitions”, and renumber the other items of the list accordingly:

(1)	Rock from Tunnel Excavation

The rock material extracted from the tunnel excavation, performed in accordance with the requirements of Section 4000, and stockpiled in an area approved by the Engineer.

This material shall be processed in appropriated crushing-plant(s) mobilized and installed by the Contractor for the Project exclusively, and approved by the Engineer. The produced material shall be for the following:

(i)	Concrete pavement (CRCP: Continuously Reinforced Concrete Pavement, JRCP: Jointed Reinforced Concrete Pavement and JCP: Jointed Concrete Pavement),

(ii)	Asphalt concrete pavement

(iii)	Concrete for structures

(iv)	Base Course and Sub-Base Course

(v)	Improved Subgrade: Capping layer (upper subgrade) and lower layer (lower subgrade)

(vi)	Road embankment

(vii)	Structural filling/backfilling

as shown in the Drawings or as instructed by the Engineer.

The rock material extracted from tunnel excavation is a property belonging to the Employer, and its use shall be reserved exclusively for the Project, or for the works authorized by the Employer.

The works for rock excavation and initial-stockpiling shall be measured and paid under the respective section for tunnel excavation. However, further operations for hauling the rock material from the stockpile to the embankment or to the crashing plant (for crashing properly in accordance with pertinent requirements), hauling and placing on its final destination, etc. shall not be measured nor paid separately but shall be deemed included into the rates and unit prices for the respective main pay item where it is finally used (refer to the above mentioned list of 7 items).

The ratio between the granular material obtained from tunnel or roadway excavation (with or without crushing) and those material procured from other approved sources located outside the Project area, which will be used for the above referred 7 items, shall not be measured. Therefore, the cost due to variations on such ratio shall not be paid separately but shall be deemed included into the rates for the unit price of the above referred 7 main items. The Contractor shall, at his risk and responsibility, to perform an estimation for establishment of respective unit prices.

[renumbering: (2) Rock Quarry; (3) Quarry; (4) Alluvial Deposit; (5) Stockpile Area; (6) Spoil Area]

803	LOCATING MATERIAL SOURCES

(2)	Delete the full text of Sub-Section 803: “Locating Material Sources” and replace with the following:

(1)	The Contractor shall be responsible of a proper extraction of rock material from tunnel excavation (without contaminating with unsuitable soils), processing of the rock in crushing plants properly installed and approved by the Engineer, and stockpiling the resulting material in proper locations approved by the Engineer.

In addition, the Contractor shall be responsible for locating all material sources and for the acquisition or renting of all land required for rock quarries (if required other than those material obtained from tunnel excavation), quarries, alluvial deposit, borrow pits, spoil and stockpile areas and for access thereto in accordance with the Conditions of Contract.

The location and size of rock quarries, quarries, alluvial deposit borrow pits, spoil and stockpile areas proposed by the Contractor shall be subject to the approval by the Engineer.

The Engineer shall withhold his approval, if in his opinion the rock quarry, quarry, alluvial deposit, borrow pit, spoil and stockpile area, or access into them:

(a)	will have a detrimental effect on the environment;

(b)	would be very difficult to acquire;

(c)	is in or near an urban centre;

(d)	will require an access road which is excessively long;

(e)	has excessively thick layers of overburden;

(f)	covers too large an area;

(g)	would constitute a danger to the public;

(h)	is an excessive distance from the location where the material is to be used or removed from; or

(i)	a source of suitable material is closer

(2)	Not later than 30 days after the issuance of order to commence the Works, the Contractor shall submit for the Engineer’s review and approval full information regarding the proposed location for stockpiling the processed rock material obtained from the tunnel excavation, and for all material sources of rock quarries, quarries, and alluvial deposits, borrow pits, spoil and stockpile areas that the Contractor will require for the whole the works.

The Contractor’s proposal shall include but not limited to the following;

(a)	A plan of the Site in a suitable scale of proposed location of material source showing, but not limited to, the following:

- plot boundaries;

- owners  names and addresses, and if appropriate identification numbers;

- the district, location, registration section and number for each plot;

- local details such as building, fences, types and areas of cultivation and services, all agreed with the land owners; and

- areas to be used for working areas, stockpile areas, etc.

(b)	Cadastral maps covering the areas to be acquired when available.

(c)	Detail of the proposed access road route.

(d)	Technical supporting information relevant to rock quarries, quarries, alluvial deposits, borrows pits, including:

- Summary of geophysical surveys (electric, and/or seismic), geotechnical survey, indicating for alluvial deposit the thickness of layers, thickness of overburden, lenticular beds depth and configuration of the bedrock, etc., and in addition for rock quarries, the discontinuity families, the intervals between discontinuities, thickness and nature of the infilling, the Weathering Index.

- Laboratory results including petrographic identification, and for alluvial deposit material, grading, tests on the fine fraction (PI, SE, Methylene Blue test, Mica content, Organic matters), tests on the coarse fraction (LAA, AIV, ACV, Micro Deval, FI, Adhesively test), CBR test, or for quarry material, LAA, AIV, ACV, Micro Deval, Adhesively test, Specific Gravity, Water Absorption.

- Conclusions on qualities and quantities.

(c)	Other relevant information

(3)	Where the Contractor uses a material herein defined, as identified or instructed by the Engineer, he shall obtain the Engineer’s approval of the areas required for the material and of the sitting of the access roads into the these respective locations. The Engineer, at any time during the construction period, may require the Contractor to modify his requirements for any of the reasons in accordance with the Conditions of Contract.

Where the material herein defined, available for inspection at the time of the Tender, are instructed by the Engineer, the Contractor shall satisfy himself as to the quality and quantity of materials available before providing the information required in this Clause. Should such investigations reveal that there is insufficient suitable material for the use for which such material sources were intended, the Contractor shall immediately inform the Engineer in writing and the Engineer shall either direct that such sources are extended or that new sources shall be used.

(4)	When the material has insufficient suitable quantity or area for the use for which it was intended, the Contractor shall propose in writing that either material be extended or that a new source shall be used. The approval and acquisition of such new or extended source shall be in accordance with all the above provisions of this Clause for the acquisition of the original material.

SECTION 900: EARTHWORKS

902	DEFINITIONS AND GENERAL REQUIREMENTS

(1)	Delete the full text of Sub-Clause 902(2)(ii) “Suitable Material” and replace with the following:

(ii)	“Suitable Material” shall comprise all material extracted from tunnel excavation, roadway excavation or borrow excavation (if required and approved by the Engineer) which satisfies the requirements shown here below and which is capable of being compacted in the manner specified in Clause 909 and 910 to form a stable fill having side slopes as indicated in the Drawing.

| Sieve Size | Percentage passing (AASHTO T27 & T11) |
| --- | --- |
| 75 mm | 100% |
| 25.0 mm | 70 – 100% |
| 4.75 mm | 30 – 70% |
| 75 micron | Less than 5% |
| Limit Liquid | 30 max. |

The material prepared by crushing rock material from tunnel excavation, or crushing rock material from roadway excavation should have the sole condition of maximum size of 75mm only.

High embankment performed using material from crashing plants shall conform the requirements of Sub-Section 909: “Forming of Embankment and others Areas of Fill”.

(2)	Add the following at the end of Sub-Section 902: “Definitions and General Requirements”:

(11)	Backfilling: The following types of backfilling works are defined:

- Structural Filling/Backfilling: Suitable, free draining material conforming to the following:

  - Maximum particle size: 40mm

  - Material passing sieve No. 200 (75-μm) AASHTO T27 and AASHTO T11: 15.0% max.

  - Plasticity index, AASHTO T 90:  6 max.

- Granular Backfilling (Crusher-run):

  - Maximum particle size: 40mm

  - Soil classification, AASHTO M 145 A-1, A-2, or A-3

- Granular Backfilling for reinforced soil slopes:

  - Maximum particle size: 40mm

  - Soil classification, AASHTO M 145 A-1, A-2, or A-3

- Backfilling (Sand) for Tunnel Inspection Passage:

  - Backfilling mainly composed by a sand approved by the Engineer

- Gradation and Properties for Structural and/or Granular Backfilling:

| Sieve Size. | Percentage passing (AASHTO T27 & T11) |
| --- | --- |
| 40 mm | 100% |
| 425 micron | 0 – 60% |
| 75 micron | Less than 15% |
| PI | ≤6 |

(12)	Prior to perform the earthworks herein defined (minimum 28 days before commencement of earthworks), the Contractor shall submit for review and approval by the Engineer, a “Plan for Earthworks” detailed by sections showing the cutting and embankment with respective hauling distances, quantities and location of the re-usable material and respective spoil-banks for wasting of unsuitable material, not re-usable material or exceeding material (from the balance between tunnel excavation, cutting excavation and fill embankment works), and respective time schedule.  The Plan for Earthworks shall be updated and reviewed periodically every 3 months or when required by the Engineer.

(13)	The following requirements shall be satisfied for the proper implementation of Spoil-Banks:

- The Contractor shall submit, at least 15 days before commencing the works for any Spoil-Bank, for review and approval by the Engineer a detailed plan for implementation of the Spoil-Banks that are being considered in his “Plan for Earthworks” showing the location, capacity, time schedule and method statement for construction

- The Spoil-Banks shall be constructed applying the same technical specifications used for road embankment construction regarding to clearing and grubbing, preparation of the ground, leveling, thickness and compaction of each layer, slopes arrangement, etc. as approved by the Engineer.

- The drainage to be implemented in the Spoil-Banks shall be constructed following the applicable standards for road drainage and in accordance with the drawings prepared by the Contractor and submitted to the Engineer for review and approval.

904	EXPLOSIVES AND BLASTING

(1)	General

(3)	Add the following at the end of Clause 904(1): “General”

Compliance with Regulations for Explosives: The Contractor shall comply with all relevant ordinances, instructions and regulations which the Government, or other person or persons having due authority, may issue from time to time regarding the handling, transportation, storage and use of explosives.

Permission for Blasting: The Contractor shall at all times maintain full liaison with and inform well in advance, and obtain such permission as is required from all Government authorities, public bodies and private parties whatsoever concerned or affected, or likely to be concerned or affected by blasting operations.

(6)	Account

(4)	Delete full Clause 904(6): “Account” and replace with the following:

(6)	Account and Records

A careful and day to day account of the explosive, including those brought to the Site and all explosives consumed in the Works, shall be maintained by the Contractor in a register and manner approved by the Engineer and with the acceptance of police and military authorities, which shall be open to inspection by the Engineer at all times.

Records must be kept by the Contractor of all drilling and blasting operations showing holes diameters and depths, drilling pattern, explosive charge and type per hole, detonator delay times and total charge per blast. These records must be submitted to the Engineer on completion of charging.

The Contractor shall remove all unused explosives from the Site on completion of the Works or shall dispose of such explosives in a manner acceptable to the police and military authorities.

905	EXCAVATION IN CUTTING

(5)	Delete the full text of Sub-Section 905: “Excavation in Cutting”, and replace with the following:

(1)	General Requirements for Excavation in Cutting

(a)	Cleaning and grubbing shall be performed as specified in Section 200.

(b)	While executing excavations, the Contractor shall take adequate precautions against soil erosion and water pollution.

(c)	All suitable excavated materials shall be used in construction of the roadway to the extent as required.

(d)	Hauling of material from cutting or borrow pits to embankments other areas of fill shall proceed only when plant or labour is operating at the place of deposition to ensure that adequate spreading and compaction of material can take place.

(e)	Over-excavation shall not be permitted. Any excess depth excavated below the formation levels as specified shall be made good by the Contractor at his own expense by backfilling with suitable material of similar characteristics to those of moved materials with compaction as specified in Clauses 909 and 910.

(f)	The slopes of cutting shall be cleared of all rock fragments which move when prized by a crowbar, unless otherwise directed by the Engineer. Where the Engineer considers that the slope, immediately after dressing, shall not be permanently stable, he shall direct the Contractor as to the stabilization measures required. The Contractor shall carry out these measures soon after Engineer’s instruction.

When completed, the excavation slopes shall be true to the lines and levels as shown on the Drawing or directed by the Engineer. When completed, no point on slopes shall vary from the designated slopes by more than 150 mm measured at right angles to the slope, except where excavation is in rock, no point shall vary more than 300 mm from the designated slope.

(g)	If slips, slides, over breaks or subsidence occur in cutting, they shall be removed. Adequate precautions shall be taken to ensure that during construction, the slopes are not rendered unstable or give rise to recurrent slides after construction.

(h)	If water is encountered in excavations due to seepage, springs, or other causes, it shall be removed by suitable diversions or bailing out and the excavation shall be kept dry. The drained water shall be discharged into suitable outlets as not to damage to the works, crops or any other property. If any such damage is caused due to any negligence of the Contractor, it shall be the sole responsibility of the Contractor to repair/restore to the original condition at his own cost or compensate for the damage.

(2)	Installation of Provisional Guard Fence

This work shall consist of the installation and, on-completion of the work, removal of provisional guard fence against rocks falling into the existing Road.

The Contractor shall calculate the required fences based on the following standards:

- Standards for Design and Construction of Rock-Fall Countermeasures. Japan Road Association 2004.

- Technical Manual for Design and Construction of Rock-Fall Countermeasures. Railway Technical Research Institute 2003, Japan.

Not later than 30 days prior to commence these works, the Contractor shall submit the above referred calculation book and shop drawings of the provisional guard fence to the Engineer for review and acceptance. All the drawings made by the Contractor shall be submitted to the Engineer, for review and approval, prior to the commencement of any work.

The Contractor shall maintain the provisional guard fence to be functional at any time. The Contractor shall remove any remaining material from excavation material and/or debris deposited behind the fence.

The provisional guard fence shall be installed to appropriate lines and grades under the Engineer’s instructions. The provisional guard fence shall be set plumb and in alignment. The posts shall be set in concrete to the planned depth.

The provisional guard fence shall remain during slope cutting and slope protection works.

907	EXCAVATION FOR FOUNDATION

(6)	Public Safety

(6)	Delete the first paragraph of the Clause 907(6): “Public Safety”, and replace with the following:

Where required, trenches and foundations pits shall be securely fenced, provided with proper caution signs and marked with red lights at night to avoid accident as per Clause 102(6): “Traffic Safety and Control”.

909	FORMING OF EMBANKMENT AND OTHERS AREAS OF FILL

(7)	Delete the full text of the Clause 909(7), and replace with the following:

(7)	Rock use in “rock fill embankments” shall be deposited in horizontal layers not exceeding 600 mm each extending up to the full width of the embankment. The maximum size of the rock material shall be 400 mm.

Material shall be spread, levelled and compacted in accordance with a compaction method approved by the Engineer based on the results obtained from a trial-section for compaction proposed and performed by the Contractor for determining the appropriated equipment and number passes. The compaction test shall be repeated one time every 5,000m3 or every time that the characteristics of the rock material varies.

Each layer shall consist of reasonably graded rock and all surface voids shall be filled with broken fragments before the next layer is placed. The top surface and side slopes of embankments so formed shall be thoroughly blinded with approved well graded material to seal the surface.

A transition layer of 300 mm shall be provided on the top of the rock fill embankment before placing the next layer composed by soil material in order to avoid losing of fine into the rock cavities.

910	COMPACTION OF EMBANKMENT AND OTHER AREAS OF FILL/BACKFILL

(8)	Delete the full text of the Clause 910(2), and replace with the following:

(2)	All fill shall be placed in layers’ thickness and compaction degree specified in Table 9.2, as described in Figure 9.1 or as shown on the Drawing. This requirement applies whether the specified zone is in fill or in existing ground, except for any part which may fall within rock or rock fill.

Formation Level shall mean the lower level of the Improved Subgrade.

Table 9.2: Compaction Requirements

| Location | Depth below Formation Level (mm) | Minimum Compaction Degree (%) (% of Minimum Dry Density (MDD). Heavy Compaction) |
| --- | --- | --- |
| Roadway Embankment | 300 | 95 |
| Roadway Cut | 300 | 95 |
| All other roadway fills and backfill not separately specified | (as instructed by the Engineer) | 93 |

Figure 9.1: Typical Cross Section of Subgrade Improvement

917		MEASUREMENT

(9)	Insert the following after the last paragraph of the Sub-Section 917:”Measurement”:

(5)	The earthworks to be measured for payment shall be classified into the following categories:

(i)	Roadway Cutting Excavation:

Cutting excavation for road construction including all types of soil with haulage and disposal up to 2km distance, all complete.

The works for “Installation of Provisional Guard Fence”, including all materials, labours, and any incidentals to complete the work as shown in the Drawings shall not be measured nor paid separately but shall be deemed included under Sub-Section 102: “Accommodation of Traffic”.

(iii)	Cutting Excavation for Improved Subgrade:

Excavation below embankments and below formation level in all types of soil with haulage and disposal up to 2km distance, all complete. Cutting excavation for Improved Subgrade shall not be measured nor paid separately, and shall deemed included in the relevant items for Earthworks.

(iv)	Structural Excavation

Excavation for foundation of structures (cross drainage, retaining wall, box-culvert and bridge) in all types of soil including hard rock, including dewatering

(v)		Filling Embankment:

Forming road embankment (beneath the improved subgrade level) using material from roadway cutting excavation and/or, if approved by the Engineer, borrowed material from quarries.

This includes also the following:

High Embankment using material from tunnel excavation:

Forming road high embankment (beneath the improved subgrade level) using material extracted from tunnel excavation (preferable).

(vi)		Filling in Reinforced Soil Zone:

Forming road embankment located on those sections or zones where Reinforced Soil is performed in accordance with the requirements of Sub-Section 301: “Reinforced Soil”.

(vii)	Structural Filling/Backfilling

Backfilling for structures, foundation pits etc. with suitable material including compaction, all complete.

(viii)	Granular Backfilling (Crusher-run)

Backfilling for structures, foundation pits etc. with granular material provided from Crushing, including compaction, all complete.

(ix)	Backfilling (Sand) for Tunnel Inspection Passage

Backfilling mainly composed by a sand approved by the Engineer, for tunnel inspection passage, with material provided from Crushing, including compaction, all complete.

Topsoil: Construction of topsoil shall be performed, measured and paid under Sub-Section 2813: “Topsoil” of these Specifications.

918		PAYMENT

(10)	Insert the following after the last paragraph of the Sub-Section 918:”Payment”:

(3)	All earthworks, excepting “Topsoil” (to be paid under Sub-Section 2813: “Topsoil”), shall be subject for payment only after completion and testing results approved by the Engineer.

(4)	Payment for temporary fence described in Clause 905(2) shall not be made separately but shall be deemed included under the pay Item 102-1: “Traffic Management at KM -0+240 to KM 0+240 along Tribhuvan Highway”

The Earthworks to be paid under this Section are the following:

| Item No. | Description | Unit |
| --- | --- | --- |
| 900 | Earthworks |  |
| 900-1 | Roadway Cutting Excavation | cu.m |
| 900-2 | Structural Excavation | cu.m |
| 900-3 | Filling Embankment | cu.m |
| 900-4 | Filling in Reinforced Soil Zone | cu.m |
| 900-5 | Structural Filling/Backfilling | cu.m |
| 900-6 | Granular Backfilling (Crusher-Run 40-0mm) | cu.m |

SECTION 1000: SUBGRADES

1002	DEFINITIONS

(2)	Subgrade

(1)	Delete the tittle and full text of Clause 1002(2): “Subgrade”, and replace with the following:

(2)	Improved Subgrade (Lower Subgrade)

(a)	Improved Subgrade (Lower Subgrade):

Subgrade layer constructed using same material and conforming all requirements for construction specified in Section 900: “Earthworks” for fill embankment.

(b)	Basic Requirements:

700mm thick layer, selected material with CBR>5% and compacted to 95% of MDD (heavy compaction), maximum particle size = 75mm, placed 1000mm below formation level (immediately beneath the capping layer).

The term “improved subgrade” shall be understood as replacing the word “subgrade” in all other sections of these Specifications.

Construction of the Improved Subgrade by using material from other sources such as borrow pits will not be allowed unless otherwise approved by the Engineer upon a request and technical justification by the Contractor.

(3)	Capping Layers

(2)	Delete the tittle and full text of Clause 1002(3): “Capping Layers”, and replace with the following:

(3)	Capping Layer (Upper Subgrade) for Improved Subgrade

Capping Layer (Upper Subgrade) constructed using material obtained from tunnel excavation and/or roadway cutting excavation, or from other approved source, including crashing and sieving, preparation of material, hauling, forming, watering and compaction, all completed as herein specified.

Basic Requirements:

300mm thick layer, selected material with CBR>15% and compacted to 95% of MDD (heavy compaction), maximum particle size = 40mm, and conforming to Clause 609, Table 6.2, laid on the completed and approved surface of the Improved Subgrade (“lower subgrade”).

Capping layer can be applied on weaker road accesses or other areas as required by the Engineer.

Construction of the Capping Layer by using material from other sources such as borrow pits will not be allowed unless otherwise approved by the Engineer upon a request and technical justification by the Contractor.

1007	MEASUREMENT

(3)	Capping Layer ….,etc.

(3)	Delete the full text of Clause 1007(3) and replace with the following:

(3)	Improved Subgrade (Lower Subgrade), defined in Clause 1002(2), shall be measured and paid specified under the Section 900: “Earthworks”, Pay Item No. 900-3: “Filling Embankment”.

Upper Subgrade (Capping Layer) shall be measured in cubic metre calculated by the “end area” method as described in Sub-clause 1007 (1), and shall be paid under this Section.

1008		PAYMENT

(4)	Insert the following after the last paragraph of the Sub-Section 1008:”Payment”:

(7)	Subgrades shall be subject for payment only after completion and testing results approved by the Engineer. The subgrades to be paid are the following:

| Item No. | Description | Unit |
| --- | --- | --- |
| 1000 | Subgrades |  |
| 1000-1 | Upper Subgrade (Capping Layer) | cu.m. |

SECTION 1100: ROADWORKS OVERALL REQUIREMENTS

1104	CARE OF WORKS

(1)	Delete the opening sentence of Sub-Section 1104: “Care of Works”, and replace with the following:

These Specification shall be applied without detriment, and supplementary to the stipulations of the Sub-Clause 17.2: “Contractor’s Care of the Works” of the Conditions of Contract, and shall include, inter alia, the following:

(2)	Insert the following after the last paragraph of the Sub-Section 1104: “Care of Works”:

(4)	The provision of temporary drainage works such as drains, open channels, banks, etc. and the furnishing and operation of temporary pumps and such other equipment as may be necessary to adequately drain, protect and de-water the Works and Temporary Works. This shall be considered as supplementary work for any permanent drainage works specified and installed, and to any temporary drainage works specifically paid separately.

(5)	Care shall be exercised not to allow material in borrow pits to become excessively wet, to keep all completed layers properly drained, to avoid dumps of material on completed layer work that shall inhibit surface drainage or form wet spots under and around dumps, and to protect all parts of the Works against erosion by floods and rain.

(6)	Fill and cut slopes shall be promptly repaired whenever damaged by surface water.

(7)	Excavations for pipe drains, culverts, service ducts and similar structures shall be adequately protected against possible ingress of water during rainstorms

(8)	The Contractor shall inform the Engineer of damage or defects to any work before repair or maintenance and the Engineer shall instruct the extent and method of repair. The previously constructed layer shall be thoroughly cleaned of all foreign material and the Contractor shall request its inspection by the Engineer before construction of the following layer or application of a prime coat, surfacing or surface treatment. In the case of a worked bituminous layer needs to be thoroughly broomed and all dung, clay, and other deleterious material completely removed. Where necessary the surface shall be sprayed with water before, during and after brooming to remove all foreign material.

SECTION 1200: SUBBASE, BASE, HARD SHOULDEr AND GRAVEL WEARING COURSE

1204	SHOULDERS

(6)	Measurement and Payment

(1)	Delete the full text of Clause 1204(6): “Measurement and Payment” and replace with the following:

(6)	Measurement

Granular shoulder shall be measured in cubic meter by taking cross Sections at intervals of 20 meters or as directed by the Engineer in the original position before the work starts and after its completion and computing the volumes in cubic meters by average end area method.

Granular shoulder shall be constructed as shown in the Drawings and with material extracted from the tunnel excavation (or road excavation if approved by the Engineer), crushed properly, sieved and graded, stockpiled (if necessary) and satisfies the requirements of these Specifications, including preparation, hauling, forming, watering and compaction, testing for approval, all complete and approved by the Engineer.

(7)		Payment

Granular Shoulder shall be paid at their respective contract unit rate. In addition to state in Clause 112, the contract unit rate shall be the final compensation for all costs required to complete the work complying with the requirements of these Specifications

Granular Shoulders shall be subject for payment only after completion and testing results approved by the Engineer:

| Item No. | Description | Unit |
| --- | --- | --- |
| 1204 | Shoulders |  |
| 1204-1 | Granular Shoulder (Subbase) | cu.m. |

(2)	Add the following new Sub-Section at the end of Section 1200: “Subbase, Base, Hard Shoulder and Gravel Wearing Course”:

1208	CRUSHED AGGREGATE SUBBASE AND BASE

(1)	Description

This work consists of supplying, processing, hauling, spreading, watering, and compacting crushed and graded aggregates on a prepared and accepted surface, in accordance with details shown in the Drawings or as directed by the Engineer. Processing shall include, crushing of rock material extracted from tunnel excavation, sieved and graded, stockpiled, hauling, forming, watering and compaction, testing for approval and any other operation necessary to produce a material conforming to the requirements of this Specification Section.

(2)	Material Sources

Base and Subbase courses shall consist of the graded aggregate prepared from the rock material extracted from the tunnel excavation, or from an approved source, and processed in an appropriate crushing-plant approved for the Project.

(3)	Crushing, Screening and Mixing

Crushing shall be carried out in accordance with the method statement prepared by the Contractor subjected to a review and approval by the Engineer. The statement shall include details for crushing, screening and proportioning of materials and their subsequent mixing.

Mixing shall be made exclusively with rock aggregates prepared in the approved crashing plants, and using the rock material extracted from tunnel excavation.

(4)	Storage, Mixing, and Handling of Materials

(a)	Materials shall be so stored and handled as to ensure the preservation of their quality and fitness for construction. Materials, even though approved before storage or handling, may again be inspected and tested prior to use. Stored materials shall be located to facilitate prompt inspection.

(b)	Handling and stockpiling of aggregates subjected to the approval by the Engineer, and always shall be made in a way to avoid contamination. To avoid segregation, graded crushed stone shall be moistened when being handled and shall not be stockpiled in heaps higher than 5 m.

(c)	Aggregate stockpiles for base course shall be protected from rain to prevent saturation of the aggregates, which would result in a reduction in the quality of the placed material or adversely affect the placement of the material.

(d)	Where materials of different sizes are to be blended to meet the gradation, such blending shall be as directed by the Engineer and shall be accomplished prior to delivery to the roadway. Mixing of separate materials on the roadway by motor grader will not be permitted by the Engineer.

(5)	Unacceptable Materials

Unless otherwise instructed by the Engineer, all materials that do not comply with the requirements of this Specification Section shall be rejected and removed immediately from the Site, and replaced if necessary, at the Contractor’s expense and without allowing any time extension to recover the delay due to this fault. Unacceptable materials of which defects can be corrected shall not be used until positive results of testing are obtained, and approval by the Engineer is provided.

(6)	Aggregate Material Requirements

The aggregate subbase and base shall consist of hard, durable particles of fragments of rock crushed to the size and conforming to requirements of this Specification Section. The crushed aggregate prepared for subbase and base shall be cleaned and free from vegetable matters, lumps or balls of clay and other deleterious substances. The materials shall be of such nature that it can be compacted readily to form a firm, and stable subbase or base.

The material for subbase and base shall conform to the grading requirements shown in Table 12-1:

Table 12-1 Grading Requirements for Aggregate

| Sieve Designation (mm) | Mass Percent Passing (*) |  |
| --- | --- | --- |
|  | Base | Subbase |
|  | Dmax = 25mm | Dmax = 50mm |
| 50 | - | 100 |
| 37.5 | - | 97-100 |
| 25 | 100 | - |
| 19 | 86-100 | - |
| 9.5 | 51-82 | - |
| 4.75 | 36-64 | 40-60 |
| 0.425 | 12-26 | - |
| 0.075 | 4-7 | 4-12 |

(*): Standard specifications for aggregate subbase and base courses by FHWA-FP2014.

The aggregate subbase and base shall conform, through wet sieve testing, to the properties given in Table 12-2.

Table 12-2 Aggregate Subbase and Base Properties

| Properties | Base | Subbase |
| --- | --- | --- |
| Abrasion of Aggregate retained on 2.00 mm (No.10) sieve (AASHTO T96) | 45% max | 50% max |
| Plasticity Index of Aggregate passing 0.425 mm (No. 40) sieve (AASHTO T90) | 6 max | 6 max |
| Percent by weight of Aggregate retained on 2mm (No.10) sieve with one fractured face | - | - |
| Liquid Limit of Aggregate passing 0.425 (No. 40) sieve (AASHTO T89) | 25 max | 35 max |
| Soaked CBR (AASHTO T193) at maximum dry density according to AASHTO T180 Method D | for material passing 19 mm sieve 80% min. | 30% min |

(7)	Construction Requirements

(a)	Provision on the Commencement of the Works

The works for construction of the subbase and base courses, as well as all the works for pavement construction, shall not commence if the works for slope formation (in cutting and/or embankment sections) and slope protection have not been properly completed by the Contractor, and accepted by the Engineer.

(b)	Submittals

At least twenty-one (21) days prior to the proposed date of first use of the materials prepared with crushed rock from tunnel excavation for the aggregate subbase and base, the Contractor shall submit to the Engineer the following:

    - Three (3) samples of 50 kg weight each of the material, one of which shall be retained by the Engineer for reference throughout the Contract Period;

    - A statement of the origin and composition of the material proposed for use as aggregate subbase and base, together with laboratory test data verifying that the material properties specified in this Specification Section are met.

    - The Contractor shall submit the following in written form to the Engineer immediately following the completion of each section of the work and before any approval is granted for placing other materials on top of the aggregate subbase and base:

- The results of testing as herein specified.

- The results of surface measurement tests and survey data verifying that the surface and thickness tolerances shown in Table 12-3 are met.

(c)	Trial Sections

    - Before commencement of construction of permanent works, the Contractor shall spread and compact trial sections as directed by the Engineer. The purpose of the trial section is to check the suitability of the materials and the efficiency of the equipment and construction method proposed by the Contractor. Therefore, the Contractor shall use the same materials, equipment, and procedures which he proposes to use for the main work. One trial section about 500 m2 shall be made for every type of material and/or construction equipment/procedure proposed for use.

    - Density tests and other tests required as directed by the Engineer.

    - If the materials are, in the Engineer’s opinion, not suitable for base as observed through the trial, the materials shall be removed at the Contractor’s expense, and a new trial section shall be constructed without consideration of time extension due to this delay.

    - If the basic conditions regarding the type of material or procedure change during the execution of the work, new trial sections shall be constructed.

(d)	Rectification of Unsatisfactory Material

    - Areas with a thickness or surface uniformity not satisfying the tolerances herein specified, or which develop irregularities in the surface during or after construction, shall be rectified by loosening the surface and removing or adding material as required, followed by reshaping and recompacting.

    - The aggregate subbase and base which is too dry for compaction, in terms of specified moisture content or as directed by the Engineer, shall be corrected by scarifying the material followed by sprinkling with an adequate quantity of water and thoroughly mixing by means of equipment approved by the Engineer, and recompacting.

    - The aggregate subbase and base which is too wet for compaction as defined by the specified moisture content or as directed by the Engineer, shall be rectified by scarifying the material followed by intermittently working using motor grader or other approved equipment to air-dry the material under dry weather conditions. Alternatively, or if sufficient drying cannot be achieved by reworking the loose material, the Engineer may instruct to remove that material and to replace it with a suitably dry material.

    - The aggregate subbase and base which becomes saturated by rain or flooding or otherwise after it has already been satisfactorily compacted according to this Specification Section will generally require no rectification work provided that the material’s properties and surface uniformity meet the requirements of this Specification Section.

    - Rectification of the aggregate subbase and base which does not meet the density or material property requirements of this Specification Section, shall be as directed by the Engineer and may include additional compaction, loosening followed by moisture content adjustment and recompacting, removal, and replacement of the material, or the application of an additional thickness of material.

(e)	Equipment

    - Equipment and method used shall place the aggregate subbase and base to the lines, levels, grades, dimensions, and cross-section shown in the Drawings and as directed by the Engineer.

    - The Engineer has the right, at any time, to stop the use of any equipment or plant which he deems to be inferior to the required quality, and to instruct the removal of such equipment, and to have it replaced by suitable equipment or to alter the method of operation. The Contractor shall immediately comply with such instructions without being entitled to any indemnities or time extensions as a result of such instructions.

(f)	Placing, Spreading, and Compacting the Aggregate Subbase and Base

The aggregate for subbase and base shall not be placed, spread or compacted while rain is falling, and no compaction shall be carried out when the moisture content of the material falls outside the specified range

- Placing:

- The aggregate subbase and base material shall be placed in accordance with the drawing requirements as a uniform mixture in a quantity which will provide the required compacted thickness.

- When more than one layer is required, each layer shall be compacted as specified in Subsection 3.6.3 of this Specification Section, before the succeeding layer is placed.

- The placing of material shall begin at the point designated by the Engineer.

- The area being prepared for laying the material shall be completed and the approval of the Engineer obtained for at least 200 m ahead of the placing.

- Hauling equipment may be routed over completed portions of the courses if no damage results and that such equipment is routed over the full width of the course to avoid rutting or uneven compaction. The Engineer has the right to stop all hauling over completed or partially completed subbase or base courses when in his opinion such hauling is causing damages

- Spreading:

- Where the required thickness is 150 mm or less, the material may be spread and compacted in one layer of subbase or base, respectively. Where the required thickness is more than the foresaid requirement, the aggregate subbase and base shall be spread and compacted into two (2) or more layers of approximately equal thickness, and the maximum compacted thickness of any one (1) layer shall not exceed for said requirement unless otherwise directed by the Engineer. All subsequent layers shall be spread and compacted in a similar manner. All aggregate subbase and bases shall be placed by a spreader.

- The aggregate subbase and base shall be delivered to the roadbed as a uniform mix and shall be spread at moisture content within the range specified in this Specification Section. The moisture shall be uniformly distributed throughout the material.

- The aggregate subbase and base shall be spread and shaped by any approved method, which does not cause segregation of the fine and coarse aggregate particles. Segregated materials shall be corrected or removed and replaced with a graded material.

- The aggregate subbase and base surfaces which are to receive a prime coat shall then be further compacted to produce a stable, tightly locked surface, without open pores, suitable for priming. All coarse aggregates shall be tightly locked in place. Under no circumstances shall any form of soil or fine aggregates be added to the surface to assist the locking process, which shall be achieved by compaction only. Vibration shall not be used in the final stages of the compaction of the surface. The Engineer may direct that pneumatic rollers be used for the final surface compaction, if static steel wheeled rollers are considered likely to cause excessive breakdown or degradation of the aggregate subbase and base.

(g)	Compaction

    - Compaction shall be carried out only when the moisture content of the material is within the range of 3% less than the optimum moisture content to 2% more than the optimum moisture content, where the optimum moisture content is as defined by the maximum modified dry density determined by AASHTO-T180, Method D.

    - The minimum dry densities to be achieved as a percentage of the Maximum Dry Density (MDD) determined in accordance with AASHTO-T180, Method D shall be:

- Base	:	98% of MDD

- Subbase	:	95% of MDD

    - Immediately following final spreading and smoothening, each layer shall be compacted to the full width by means of approved compaction equipment. Rolling operations shall begin along the edges and progress gradually towards the centre, in a longitudinal direction. On super-elevated sections rolling shall begin at the low side and progress towards the high side. The rolling operation shall continue until all roller marks are eliminated and the layer is uniformly compacted and the aggregates firmly keyed. Any irregularities or depressions that develop shall be corrected by loosening the material at these places and adding or removing material until the surface is smooth and uniform. Along kerbs, headers, and walls, and at all places not accessible to the roller, the material shall be compacted thoroughly with approved tampers or compactors. At the end at each shift, the surface of the base or subbase shall be shaped and sloped to prevent ponding of rain water.

    - When the underlying material is soft or yielding the layer course, compaction shall be stopped and the underlying materials rectified as directed by the Engineer.

    - During all the operations of spreading, compacting, and levelling of course material, care shall be taken so that the layers already compacted under the layer being executed are not affected, or that the finished subgrade, subbase or base surface is also not affected. Special attention shall be given in places where equipment makes turns in going back and forth. Any such damage resulting in mixing the various layers constituting the different subgrades, subbase and base courses shall be carefully made good by the Contractor and at his expense and to the satisfaction of the Engineer.

    - If the layer of subbase or base material, or part thereof, does not conform to the required finish, the Contractor shall, at his own expense, make the necessary corrections.

    - Compaction of each layer shall continue until a field density has been achieved. In-place density determination shall be made in accordance with AASHTO-T191. When aggregate subbase and base divides into layers for construction, the subsequent layers shall be placed only after execution of inspection and acceptance for compacted density of previous layer is done.

    - Proof rolling shall be conducted after finishing by more than 25 tons weight roller on the place directed by the Engineer. If the deflection is more than 3 mm, the place shall be replaced as instructed by the Engineer.

(8)	Control of Traffic on Base Surfaces

(a)	Construction traffic or other traffic shall not be permitted to travel on any aggregate layers, prior to the construction of the final surfacing, except where directed or permitted by the Engineer.

(b)	The aggregate subbase and base shall not be opened to traffic until shaping and compaction are substantially completed.

(c)	Pending the construction of the final surfacing, the aggregate subbase and base shall be maintained as follows:

    - The aggregate subbase and base shall be maintained always by drag blooming. The traffic shall be controlled by temporary speed restrictions and, during working hours, it shall be channelled by suitably defined traffic lanes, with frequent transverse shift of the defined lanes to obtain an even spread of traffic over the entire surface.

    - Where the pavement course is too dry, so that surface stability does not or is unlikely to improve under the effect of traffic and/or rolling and where rain is not imminent or has been insufficient, a uniform application of water shall be applied to the entire surface.

(9)	Testing

(a)	The amount of supporting test data required for initial approval of the quality of the material shall be as directed by the Engineer, and shall include all the tests specified in this Section on at least three (3) representative samples from the proposed material source, selected to represent the range of material quality likely to be obtained from the source.

(b)	Following approval of the quality of a proposed material, the full range of material quality tests performed shall be repeated subsequently, at the discretion of the Engineer, in the event of observed changes in the material or in its source or in its method of production.

(c)	A programme of routine material quality control testing shall be carried out to control variability of the material being brought on site. The extent of the testing shall be as directed by the Engineer, but following test shall be conducted for every 3000 m3:

    - Five (5) plasticity index tests

    - Five (5) particle grading tests

    - One (1) maximum dry density determination (AASHTO T180 Method D)

    - CBR Tests

- The density and moisture content of the compacted material shall be routinely determined, using AASHTO-T191. The test shall be made to the full depth of the layer at locations as directed by the Engineer, but not more than 200 m apart.

- All holes in the finished work made by density testing or otherwise shall be backfilled with the specified material by the Contractor without delay and compacted to the density and surface tolerance requirements of this Specification Section.

- Examination stage for acceptance of aggregate quality which was gathered at the Site for construction: tested sample is taken from material at the site and shall be conducted for every 1000 m3 from each material supply source or when material quality is extraordinary. Material shall meet all physical properties specified in Tables 12-1 and 12-2 and compacting test conducted inside the Laboratory.

(d)	Examination during construction period: During construction period, the contractor shall regularly conduct tests and check the following contents:

    - Moisture, aggregate material separation (observed by eyes and by checking sieve particle): Test on moisture and sieve particles shall be conducted for every 200 m3 aggregate or every shift;

    - Compacted density: test shall be conducted for every finished aggregate foundation layer, according to the testing process; base aggregate’s compacted density shall be certified by sand cone test according to equivalent process as required by the Engineer. At the final stage of compaction, test for checking compacted density shall be frequently performed and taken as basis for completion of compaction works; test for compactness shall be conducted for every 800 m2 at each random position; and

    - Flatness, geometric factors: shall be checked according to the requirements specified in Table 12-3. These testing data will be the basis for acceptance of works.

(e)	Checking and acceptance of construction works

    - As for compacted density, test shall be conducted for every 7000 m2 at two (2) random positions;

    - For flatness, geometric factors: checking density shall account for 20% of quantity regulated in Table 12-3.

(f)	During conducting of finishing works, all holes made by density testing shall be backfilled with the specified material by the Contractor without delay, and compacted to the density and surface tolerance requirements of this Specification Section.

(10)	Tolerances for Acceptance

The aggregate subbase and base shall be spread with the equipment that will provide a uniform layer which when compacted will conform to the designed thickness, level, and longitudinal grade and cross fall or camber as shown in the Drawings. The allowable tolerances after compaction shall be as shown in Table 12-3.

(a)	The surface of all aggregate subbase and base layers shall not have any irregularities which can hold moisture and the camber of all such surfaces shall comply with that shown in the Drawings.

(b)	When testing the irregularity of aggregate subbase and base surfaces to be treated with bituminous surfacing, all loose material shall be removed by hard blooming.

Note: Table 12-3 shows the tolerances for subbase and base courses.

Table 12-3: Tolerances for Aggregate Subbase and Base

| Item Checking | Permitted Variation comparing with the Drawings |  |  |
| --- | --- | --- | --- |
|  | Base | Subbase | Description |
| Layer Thickness | + 5 mm | + 10 mm | Measurement of cross-section shall be conducted for every 40-50 m for straight section and 20 – 25 m for horizontal or vertical curves |
| Surface Level | +5 mm | +10 mm |  |
| Width on Top | +50 mm | +100 mm |  |
| Cross Fall or Camber | + 0.2% | + 0.3% |  |
| Surface Irregularity by 3 m straight edge | < 5 mm | < 20 mm | One position for every 100 m along the route |

(11)	Measurement

Graded crushed aggregate Base and Subbase courses shall be measured in cubic metre of actually placed, compacted , tested and completed, and approved by the Engineer. The measurement shall be made by taking cross Sections at intervals of 10 metres, or as directed by the Engineer, in the original position before the work starts and after its completion and computing the volumes in cubic metres by average end area method.

(12)	Payment

The graded crushed aggregate Base and Subbase, as measured above, shall be paid at their respective contract unit rate. In addition to requirements of Clause 112, the contract unit rate shall be the full and the final compensation for all costs required to complete the work complying with the requirements of these Specifications, including crushing, stockpiling, hauling, supplying, placing, compacting, finishing and testing the materials, the supply and placing of running course, trials, maintenance of the surface under traffic, making arrangement for traffic control and for furnishing all labour, materials, tools, equipment, and any incidentals necessary to complete the work shown in the Drawings and as required in accordance with these Specifications and/or as directed by the Engineer.

The following shall not be measured nor paid separately, but deemed included in the related main items:

- The work for preparing and maintaining the subgrade or subbase on which the aggregate subbase or base is to be placed, respectively;

- Any running course gravel used for temporary protection of the surface of aggregate subbase and base exposed to traffic;

- Repairs to layers’ damages caused by traffic or natural conditions;

- Extra material provided in the next course due to deficiency of the base course.

The Crushed Aggregate Subbase and Base shall be subject for payment only after completion, and testing results approved by the Engineer:

| Item No. | Description | Unit |
| --- | --- | --- |
| 1200 | Subbase, Base, Hard Shoulder and Gravel Wearing Course |  |
| 1208 | Crushed Aggregate Subbase and Base |  |
| 1208-1 | Crushed Aggregate Subbase | cu.m. |
| 1208-2 | Crushed Aggregate Base | cu.m. |

SECTION 1300: BITUMINOUS SURFACE AND BASE COURSE

1302	PRIME COAT AND TACK COAT

(8)	Measurement

(1)	Delete the full text of the Clause 1302(8): “Measurement”, and replace with the following:

The applied Prime Coat and Tack Coat (both using MC30/MC70) shall be measured in square meters of work completed and accepted by the Engineer. Surface measurements shall be based on the width and length of the surface area, as shown in the Drawings or as approved by the Engineer.

The rates of binding material (litres per sq.m corrected to 15°C) shall be established based on the test results obtained from the trial sections, and approved by the Engineer.

(9)	Payment

(2)	Delete the full text of the Clause 1302(9): “Payment”, and replace with the following:

Payment shall be for full and final compensation for compliance of all requirements of these Specifications in addition to those specified in Clause 112, including furnishing, producing, mixing and placing, all materials including additives (if needed), testing and trial sections, and for furnishing all labour, materials, tests, tools, equipment and any incidentals to complete the work as shown in the Drawings and as required by these Specification Sections, and/or as directed by the Engineer.

(i)	Payment for the Prime Coat will be made at the Contract unit rate and shall include full compensation for cleaning the surface prior application, providing all materials, mixing where required, spraying and placement, labour equipment and all incidentals to complete the prime coat as specified in this Section.

(ii)	Payment for the Tack Coat will be made at the Contract unit rate and shall include full compensation for cleaning the surface prior application, providing all materials, mixing where required, spraying and placement, labour equipment and all incidentals to complete the prime coat as specified in this Section.

| Item No. | Description | Unit |
| --- | --- | --- |
| 1302 | Prime Coat and Tack Coat |  |
| 1302-1 | Prime Coat | sq.m. |
| 1302-2 | Tack Coat | sq.m. |

1308	ASPHALT CONCRETE

(1)	Scope

(3)	Delete the full text of the Clause 1308(1): “Scope”, and replace with the following:

This Clause covers the materials, method of construction and requirements for the construction of Asphalt Concrete and Asphalt Treated Base Course

Asphalt Concrete:

This work shall consist of construction of Asphalt Concrete, for use in wearing and profile corrective courses. This work shall consist of construction in a single layer of asphalt concrete on a previously prepared bituminous bound surface. A single layer shall be 40mm or 50mm thick.

Asphalt Treated Base (ATB) Course:

This work shall consist of construction of a ATB course using a bitumen type VG20, for the pavement works of the Project. This work shall consist of construction in a single layer of ATB on a previously prepared bituminous bound surface. A single layer shall be 100 mm thick.

(4)	Grading Requirements

(4)	Renumber the existing Table 13.18 as follows:

Table 13.18(a): Grading Requirements of the Mixture of Coarse and Fine Aggregates

(5)	Insert the following new table after the renumbered Table 13.18(a):

Table 13.18(b):	Grading Requirements of Aggregates for ATB Course (Asphalt Treated Base Course)

| Grading | 3 Asphalt Treated Base Course |
| --- | --- |
| Nominal aggregate size: | 37.5 mm |
| Layer Thickness: | 100 mm |
| Sieve (mm) | Cumulative % by weight of total aggregate passing |
| 37.5 | 100 |
| 25 | 93 - 100 |
| 12.5 | 27 - 58 |
| 6.3 | 0 - 10 |
| 0.075 | 0 - 2 |

(5)	Requirements for Asphalt Concrete

(6)	Renumber the existing Table 13.19 as follows:

Table 13.19(a): Requirements for Asphalt Concrete

(7)	Insert the following new table after the renumbered Table 13.19(a):

Table 13.19(b): Requirements for Asphalt of ATB Course

| Description | Requirement |
| --- | --- |
| Bitumen content % by mass of total mix | 1.5 – 4.0 |
| Mixing Temperature Range | 93°C - 135°C |
| Compaction Temperature Range | 60°C - 93°C |

(9)	Measurement

(8)	Delete the full text of the Clause 1308(9): “Measurement”, and replace with the following:

Asphalt Concrete of specified grading in contract document, and Asphalt Treated Base (ATB) course, shall be measured in square meters of work completed and accepted by the Engineer. Surface measurements shall be based on the width and length of the surface area, as shown in the Drawings or as approved by the Engineer.

(10)	Payment

(9)	Delete the full text of the Clause 1308(10): “Payment”, and replace with the following:

Asphalt Concrete of specified grading in contract document, and Asphalt Treated Base (ATB) course, shall be paid at the respective contract unit rate which shall be the full and the final compensation for compliance of all requirements herein specified in addition to those specified in Clause 112, including furnishing, producing, all materials including additives (if needed), placing, leveling and compaction, testing and trial sections, and for furnishing all labour, materials, tests, tools, equipment and any incidentals to complete the work as shown in the Drawings, as required by these Specification Sections, and/or as directed by the Engineer.

A bitumen tar-felt sheet shall be included as per water-proofing measure for bridge as shown in the Drawings, and as approved by the Engineer.

| Item No. | Description | Unit |
| --- | --- | --- |
| 1308 | Asphalt Concrete |  |
| 1308-1 | Asphalt Treated Base (ATB) Course (100 mm thick) | sq.m. |
| 1308-2 | Asphalt Concrete Binder Course (50 mm thick) | sq.m. |
| 1308-3 | Asphalt Concrete Wearing Course (50 mm thick) | sq.m. |
| 1308-4 | Asphalt Concrete Wearing Course (40 mm thick) | sq.m. |
| 1308-5 | Asphalt Concrete for Leveling | cu.m. |
| 1308-6 | Bitumen Tar Felt (for bridge) | sq.m. |

SECTION 1400: KERBS AND FOOTPATHS

1401	CONCRETE KERBS

(2)	Materials

(1)	Delete the first paragraph of the Clause 1401(2): “Materials”, and replace with the following:

The kerbs shall be provided in cement concrete Class C (M25/20) in accordance with Section 2000. All kerbs shall be pre-cast concrete blocks excepting the Cast-in-situ Kerb Type D shown in the Drawings.

(3)	Laying

(2)	Delete the full text of the Clause 1401(3): “Laying”, and replace with the following:

The kerbs shall be laid on cement concrete as shown in the Drawings. The concrete shall be of grade 25/20 in accordance with Section 2000 laid to the exact lines, dimensions and levels as shown in the Drawings and well compacted by ramming or by other method previously approved by the Engineer.

(5)	Measurement

(3)	Delete the full text of the Clause 1401(5): “Measurement”, and replace with the following:

All kerbs shall be measured for payment by the length in linear metres of the structures completed in place, as shown in the Drawings, and as approved by the Engineer. No deduction shall be made for drainage structures installed in the kerbs (if any), or for flattening of kerbs for entrances, or for kerb drainage inlets, or for flanged-end sections.

(6)	Payment

(4)	Delete the full text of the Clause 1401(6): “Payment”, and replace with the following:

The precast concrete kerbs measured as above shall be paid at the contract unit rate which shall be the full and the final compensation to the Contractor as per Clause 112 including cost for excavation and backfilling (if any), cutting of existing pavement (if any), leveling concrete and/or sand-gravel for bedding, mortar, labour, equipment, tools, and all incidentals to complete the work in accordance with the Drawings and Specifications, and as directed by the Engineer.

| Item No. | Description | Unit |
| --- | --- | --- |
| 1400 | Kerbs and Foothpaths |  |
| 1401 | Concrete Kerbs |  |
| 1401-1 | Pre-cast Concrete Kerb Type-A; Concrete Class C (M25/20) | lin.m. |
| 1401-2 | Pre-cast Concrete Kerb Type-A (Re-used) | lin.m. |
| 1401-3 | Pre-cast Concrete Kerb type-B (Concrete Class C (M25/20)) | lin.m. |
| 1401-4 | Cast -in-situ Concrete Kerb type-C (Concrete Class C (M25/20)) | lin.m. |

1402	CONCRETE/STONE FOOTPATHS

(6)	Payment

(5)	Insert the following at the end of Clause 1402(6): “Payment”:

| Item No. | Description | Unit |
| --- | --- | --- |
| 1402 | Concrete / Stone Footpaths |  |
| 1402-1 | Concrete block footpath (For service road, Roadside Service Station and O&M office) | sq.m. |
| 1402-2 | Natural stone footpath (For boundary parking area of Roadside Service Station and building) | sq.m. |

SECTION 1500: TRAFFIC SIGNS, ROAD MARKING, ROAD MARKER STONES AND DELINEATORS

1501	PERMANENT TRAFFIC SIGNS

(11)	Payment

(1)	Insert the following at the end of Clause 1501(11): “Payment”:

| Item No. | Description | Unit |
| --- | --- | --- |
| 1501 | Permanent Traffic Signs |  |
| 1501-1 | Traffic sign type A-1 (Single post with foundation block) | No. |
| 1501-2 | Traffic sign type A-2 (Single post without foundation block) | No. |
| 1501-3 | Traffic sign type B (Two posts with foundation block) | No. |

1503	ROAD MARKER STONE

(6)	Payment

(2)	Insert the following at the end of Clause 1503(6): “Payment”:

| Item No. | Description | Unit |
| --- | --- | --- |
| 1503 | Road Markers Stone |  |
| 1503-1 | Road Markers Stone (1 Km) | No. |
| 1503-2 | Road Markers Stone (5 Km) | No. |

(3)	Add the following new Sub-Sections after Sub-Section 1504: “Delineator Posts” and before Section 1600: “Piling for Structures”

1505	OVERHEAD SIGN

(1)	Scope

This Clause covers the supply and erection of overhead traffic signs along the roadside, over the carriageway and at the locations indicated on the Drawing or as directed by the Engineer.

The work shall consist of design review / updating,  fabrication, supply and installation of overhead traffic signs on the roads. The details of sign shall be as shown in the Drawings or indicated in the DOR’s Standard drawings for Road element- August 2014.

The Contractor shall carry out the design review of the structure and foundation of the conceptual design shown in the Drawings, and update it as necessary as approved by the Engineer.

(2)	Height

Overhead signs shall provide a vertical clearance of not less than 5.0m over the entire width of the pavement and shoulders except where a lesser vertical clearance is used for the design of other structures. The vertical clearance to overhead sign structures or supports need not be greater than 300 mm in excess of the minimum design clearance of other structures.

(3)	Lateral Clearance

The minimum clearance outside the usable roadway shoulder for expressway signs mounted at the road side or for overhead sign supports either to the right or left side of the roadway shall be 1.80 m. This minimum clearance of 1.80 m shall also apply outside of an un-mountable kerb. Where practicable, a sign should not be less than 3 m from the edge of the nearest traffic lane. Large guide signs should be farther removed preferably 9 m or more from the nearest traffic lane, unless otherwise specified.

Lesser clearances, but not generally less than 1.80 m, may be used on connecting roadways or ramps at inter-changes.

Where a median is 3.6 m or less in width, consideration should be given to spanning over both roadways without a central support. Where overhead sign supports cannot be placed at a safe distance away from the line of traffic or in an otherwise protected site, they should either be so designed as to minimize the impact forces or protect motorists adequately by a physical barrier or guard rail of suitable design.

(4)	Materials

Aluminium alloy or galvanized steel to be used as truss design supports shall conform to NS: 163/IS: 4826. These shall be of sections and type as per structural design requirements as shown on the drawings.

After steel trusses have been fabricated and all required holes punched or drilled on both the horizontal truss units and the vertical and support units, they shall be galvanized in accordance with NS: 163/ IS: 4826 Specifications.

Where aluminium sheets are used for road signs, they shall be of smooth, hard and corrosion resistant aluminium alloy conforming to IS: 736- Material Designation 24345 or 1900. The thickness of sheet shall be related to the size of the sign with minimum thickness of sheet as 1.5 mm.

High strength bolts shall confound, to IS: 1367 whereas precision bolts, nuts etc. shall conform to IS: 1364.

Plates and support sections for sign posts shall conform to IS: 226 and IS: 2062. The overhead signs shall be of micro prismatic retro-reflective sheeting.

(5)	Size, Locations. of Signs

The size of the signs, letters and their placement shall be as specified in the Contract drawings and Specifications.

In the absence of details or for any missing details in the Contract documents, the signs shall be provided as directed by the Engineer.

(6)	Installation

From safety and aesthetic considerations, overhead signs shall be mounted on overhead bridge structures. Where these are required to be provided at some other locations, the support system providing pleasing aesthetics, should be properly designed based on sound engineering principles, to safety sustain the dead load, live load and wind load on the completed sign system. For this purpose, the overhead signs shall be designed to withstand a wind loading of 150 kg/m2 normal to the face of the sign and 30 kg/m2 transverse to the face of the sign. In addition, dead load of structure, walkway loading of 250 kg concentrated live load shall also be considered for the design of the overhead sign structure.

The supporting structure and signs shall be fabricated and erected as per details given in the Drawings and locations directed by the Engineer.

Sign posts, their foundations and sign mountings shall be so constructed as to hold signs in a proper and permanent position to adequately resist swaying in the wind or displacement by vandalism.

The work of construction of foundation for sign supports including excavation and backfill, forms, steel reinforcement, concrete and its placement shall conform to the relevant Specifications given in these Specifications.

- The structures shall be erected with the specified camber and in such a manner as to prevent excessive stresses, injury and defacement.

- Brackets shall be provided for mounting signs of the type to be supported by the structure. For better visibility, they shall be adjustable to permit mounting the sign faces at any angle between a truly vertical position and three degree from vertical. This angle shall be obtained by rotating the front lower edge of the sign forward. All brackets shall be of a length equal to the heights of the signs being supported.

- Before erecting support structures, the bottom of each base plate shall be protected with an approved material which "will adequately prevent any harmful reaction between the plate and the concrete.

- The end supports shall be plumbed by the use of levelling nuts and the space between the foundation and base plate shall be completely filled with an anti- shrink grout.

- Anchor bolts for sign supports shall be set to proper locations and elevation with templates and carefully checked after construction of the sign foundation and before the concrete has set.

- All nuts on aluminium trusses, except those used on the flanges, shall be tightened only until they are snug. This includes the nuts on the anchor bolts. A thread lubricant shall be used with each aluminium nut.

- All nuts on galvanized steel trusses, with the exception of high strength bolt connections, shall be tightened only to a snug condition.

- Field welding shall not be permitted.

- After installation of signs is complete, the sign shall be inspected by the Engineer. If specular reflection is apparent on any sign, its positioning shall be adjusted by the Contractor to eliminate or minimize this condition.

(7)	Measurement

Aluminium or steel overhead sign structure shall be measured by the specific unit (each) complete in place as indicated in the Bill of Quantities and the Drawings.

Flat sheet aluminium signs with retro- reflective sheeting thereon shall be measured in square metre.

(8)	Payment

The quantities measured as above shall be paid at the respective contract unit rates. The contract unit rates shall be the full and the final compensation to the Contractor as per Clause 112. Rate shall be inclusive of painting of structural steel, fabrication, and installation, fixing in position, and furnishing of necessary test certificates, warranty and all other incidental cost to complete the work as specified.

| Item No. | Description | Unit |
| --- | --- | --- |
| 1505 | Overhead Sign |  |
| 1505-1 | Overhead Sign (W=17.50m) | No. |
| 1505-2 | Overhead Sign (W=25.00m) | No. |

1506	THERMOPLASTIC ROAD MARKING

(1)	Scope

This Clause covers the hot applied thermoplastic marking of the road surface with white or yellow paint as indicated on the Drawing or instructed by the Engineer.

The colour of the compound shall be white or yellow ( IS colour No 356) , width and layout of road marking shall be in accordance with the Traffic Signs Manual (latest publication), Department of Roads as per specification in the Contract.

The thermoplastic compound shall be screeded/extruded on to the pavement surface in a molten state by suitable machine capable of controlled preparation and laying with surface application of glass beads at a specific rate. Upon cooling to ambient pavement temperature, it shall produce an adherent pavement marking of specified thickness and width and capable of resisting deformation by traffic

Where the compound is to be applied to cement concrete pavement, a sealing primer as recommended by the manufacturer, shall be applied to the pavement in advance of placing of the stripes to ensure proper bonding of the compound. On new concrete surface any laitance and/or curing compound d shall be removed before the markings are applied

(2)	Materials

Requirements of Thermoplastic material:

(a)	Composition:

The pigment, beads and aggregate shall be uniformly dispersed in the resin. , The material shall be free from all skins, dirt and foreign objects and shall comply with requirements in the Table 15-1.

Table 15-1: Proportions of Constituents of making Material (% by weight)

| Component | White | Yellow |
| --- | --- | --- |
| Binder | 18 min. | 18 min |
| Glass Beads | 30-40 | 30-40 |
| Titanium Dioxide | 10 min | - |
| Calcium carbonate and inert Fillers | 42 max | See Note |
| Yellow Pigments | - | See note |

Note: 	Amount of yellow pigment, calcium carbonate and inert fillers shall be at the option of the manufacturer, provided all other requirements of this Specification are met.

(b)	Properties:

The properties of thermoplastic material, when tested in accordance with ASTM D36/ BS 3262 (part I) shall be as below:

- Luminance:

White: Daylight luminance at 45 degrees- 65 per cent min. as per AASHTO M249.

Yellow: Daylight luminance at 45 degrees -45 per cent mm. as per AASHTO M249.

- Drying time :

When applied at a temperature specified by the manufacturer and to the required thickness, the material shall set to bear traffic in not more than 15 minutes.

- Skid resistance: not less than 45 as per BS 6044.

- Cracking resistance at low temperature : The material shall show no cracks on application to concrete blocks

- Softening point: 102.50 C± 9.5o C as per ASTM D 36.

- Yellowness index (for white thermoplastic paint): not more than 0.12 as per AASHTO M 249

(c)	Storage Life:

The material shall meet the requirements of these Specifications for a period of one year. The thermoplastic material must also melt uniformly with no evidence of skins or non-melted particles for the one year storage period. Any material not meeting the above requirements shall be replaced by the manufacturer/ supplier/Contractor.

(d)	Reflectorizing :

Shall be achieved by incorporation of beads, the grading and other properties of the beads shall be as specified in: Reflectorizing Glass Beads” below.

(e)	Marking :

Each container of the thermoplastic material shall be clearly and indelibly marked with the following information:

- The name, trade mark or other means of identification of manufacturer

- Batch number

- Date of manufacture

- Colour (white or yellow)

- Maximum application temperature and maximum safe heating temperature.

(3)	Reflectorizing  Glass Beads

Type 1 beads are those which are constituent of the basic thermoplastic compound as per Table 15.1 and Type 2 beads are those which are to be sprayed on the surface. The glass beads shall be transparent, colourless and free from milkiness, dark particles and excessive air inclusions. These shall conform to the requirements spelt out in: application Properties of Thermoplastic material”.

Specific Requirements of Reflectorizing Glass beads:

(a)	Grading:

The glass beads shall meet the following requirements of grading for the two types as given in Table 15-2.

Table 15-2: Grading Requirements for Glass Beads

| Sieve Size | Percent Retained |  |
| --- | --- | --- |
|  | Type-1 | Typ-2 |
| 1.18 mm | 0 to 3 |  |
| 850 micron | 5 to 20 | 0 to 5 |
| 600 micron | -- | 5 to 20 |
| 425 micron | 65 to 95 | -- |
| 300 micron | -- | 30 to 75 |
| 180 micron | 0 to 10 | 10 to 30 |
| Below 180 micron | -- | 0 to 15 |

(b)	Roundness:

The glass beads shall have a minimum of 70 per cent true spheres.

(c)	Refractive Index:

The glass beads shall have a minimum refractive index of 1.50.

(d)	Free Flowing Properties :

The glass beads shall be free of hard lumps and clusters and shall dispense readily under any conditions suitable for paint striping. They shall pass the free flow test

(4)	Application Properties of Thermoplastic Material

The thermoplastic material shall readily get screeded/ extruded at temperatures specified by the manufacturers for respective method of application to produce a line of specified thickness which shall be continuous and uniform in shape having clear and sharp edges.

The material upon heating to application temperatures, shall not exude fumes, which are toxic, obnoxious or injurious to persons or property.

(5)	Preparation

The material shall be melted in accordance with the manufacturer's instructions in a heater fitted with a mechanical stirrer to give a smooth consistency to the thermoplastic material to avoid local overheating. The temperature of the mass shall be within the range specified by the manufacturer, and shall on no account be allowed to exceed the maximum temperature slated by the manufacturer. The molten material should be used, as expeditiously as possible and for thermoplastic material, which has natural binders or is otherwise sensitive to prolonged healing, the material shall not be maintained in a molten condition for more than 4 hours.

After transfer to the laying equipment, the material shall be maintained within the temperature range specified by the manufacturer for achieving the desired consistency for laying.

(6)	Application

Marking shall be done by machine. For locations where painting cannot be done by machine, approved manual methods shall be used with prior approval of the Engineer. The Contractor shall maintain control over traffic while painting operations are in progress so as to cause minimum in convenience to traffic compatible with protecting the workmen.

The thermoplastic material shall be applied hot either by screeding or extrusion process. After transfer to the laying apparatus, the material shall be laid at a temperature within the range specified by the manufacturer for the particular method of laying being used. The paint shall be applied using a screed or extrusion machine.

The pavement temperature shall not be less than 10°C during application. All surfaces to be marked shall be thoroughly cleaned of all dust, dirt, grease, oil and all other foreign matter before application of the paint.

The material, when formed into traffic stripes, must be readily renewable by placing an overlay of new material directly over an old line of compatible material. Such new material shall so bond itself to the old line that no splitting or separation takes place.

Thermoplastic paint shall be applied in intermittent or continuous lines of uniform thickness of at least 2.5 mm unless specified otherwise. Where arrows or letters are to be provided, thermoplastic compound may be hand sprayed. In addition to the beads included in the material, a further quantity of glass beads of Type 2, conforming to the above noted Specification shall be sprayed uniformly into a mono-layer on to the hot paint line in quick succession of the paint spraying operation. The glass beads shall be applied at the rate of 250 grams per square metre area.

(7)	Tolerance Requirement

Road marking shall be constructed to an accuracy within the tolerance given below:

(a)	Width

The width of the lines and other markings shall not deviate from the specified width by more than 5%.

(b)	Position

The position of lines, letters, figures, arrows, and other marking shall not deviate from the true position specified by more than 20 mm.

(c)	Alignment of Markings

The alignment of any edge or a longitudinal line shall not deviate from the true alignment by more than 10 mm in 15 m.

(d)	Broken Lines

The length of segments of broken longitudinal lines shall not deviate from the specified length by more than 150 mm.

In broken lines, the length of segment and the gap between segments shall be as indicated on the drawings; if these lengths are altered by the Engineer, the ratio of the lengths of the painted sections shall remain the same.

(8)	Tests and Standard of Acceptance

Sampling and Testing of Thermoplastic Material

The thermoplastic material shall be sampled and tested in accordance with the appropriate ASTM/BS method. The Contractor shall furnish to the Employer a copy of certified test reports from the manufacturers of the thermoplastic material showing results of all tests specified herein and shall certify that the material meets all requirements of this Specification

Sampling and Testing of Reflecting Glass beads

Free-flow test: Spread 100 grams of beads evenly in a 100 mm diameter glass dish. Place the dish in a 250mm inside diameter desiccator, which is filled within 25 mm of the top of a desiccator plate with sulphuric acid water solution (specific gravity 1.10).Cover the desiccator and let it stand for 4 hours at 20 to 29 degree C. Remove sample from desiccator, transfer beads to a pan and inspect for lumps or clusters. Then pour beads into a clean, dry glass funnel having a 100 mm stem and 6 mm orifice. If necessary, initiate flow by lightly lapping the funnel. The glass spheres shall be essentially free of lumps and clusters and shall flow freely through the funnel.

The requirements of gradation, roundness and refractive index of glass beads and the amount of glass beads in the compound shall be tested as per BS 6088 and BS3262 (Pan I).

The Contractor shall furnish to the Employer a copy of certified test reports from the manufacturer of glass beads obtained from a reputed laboratory showing results of all tests specified herein and shall certify that the material meets all requirements of this Specification. However, if so required, these tests may be carried out as directed by the Engineer.

The minimum thickness specified is exclusive of surface applied glass beads. The method of thickness measurement shall be in accordance with Appendices B and C of BS - 3262 (Part 3).

Properties of Finished Road Marking

The finished lines shall be free from ruggedness on sides and ends and be parallel to the general alignment of the carriageway. The upper surface of the lines shall be level, uniform and free from streaks.

- The stripe shall not be slippery when wet

- The marking shall not lift from the pavement in freezing weather

- After application and proper drying, the stripe shall show no appreciable deformation or discoloration under traffic and under road temperatures up to 60 oC

- The marking shall not deteriorate by contact with sodium chloride, calcium chloride or oil dripping from traffic

- The stripe or marking shall maintain its original dimensions and position. Cold ductility of the material shall be such as to permit normal movement with the road surface without chopping or cracking.

(9)	Measurement

The measurement of thermoplastic road marking shall be in square meter of actual area marked (excluding gap if any).

(10)	Payment

The quantities measured as provided above shall be paid at the respective contract unit rate for each type of marking signs which shall be the full and the final compensation to the Contractor as per Clause 112 for carrying out the work as specified in these Specifications.

| Item No. | Description | Unit |
| --- | --- | --- |
| 1506 | Thermoplastic Road Marking |  |
| 1506-1 | Thermoplastic road marking (Yellow and White) | sq.m. |

1507	REFLECTIVE PAVEMENT MARKERS (ROAD STUDS) AND SOLAR POWERED ROAD MARKERS (SOLAR STUDS)

(1)	Scope

The work shall consist of providing and fixing of Reflective Pavement Marker (RPM) or road stud, a device which is bonded to or anchored within the road surface, for lane marking and delineation for night time visibility, as specified in the contract.

(2)	Materials

Plastic body of RPM/ road stud shall be moulded from ASA (Acrylic Styrene Acrylonitrile) or HIPS (Hi –impact Polystyrene) or Acrylonitrile Butadiene Styrene (ABS) or any other suitable material approved by the Engineer. The markers shall support a load of 13,635 kg tested in accordance with ASTM D 2480.

Reflective panels shall consist of number of lenses containing single or dual prismatic cubes capable of providing total internal reflection of the light entering the lens face. Lenses shall be molded of methyl methacrylate confirming to ASTM D 788 or equivalent.

(3)	Design

The slope of retro reflecting surface shall preferably be 35 ±50 to base and the area of each retro – reflecting surface shall not be less than 13 sq. cm.

(4)	Optical Performance

(a)	Unidirectional and Bi Directional Studs

Each reflector or combination of reflector on each face of the stud shall have a Coefficient of Luminous Intensity (CIL) not less than that given in Table 15-3 or Table 15-4 as appropriate.

(b)	Omni- directional Studs

Each Omni directional stud shall have a CIL of not less than 2 mcd/lX

Table 15.3 Minimum CIL values for category “ A” Studs

| Entrance Angle | Observation Angle | CIL in mcd/lx |  |  |
| --- | --- | --- | --- | --- |
|  |  | White | Amber | Red |
| 0° U 5° L &R | 0.3° | 220 | 110 | 44 |
| 0° U 10° L &R | 0.5° | 120 | 60 | 24 |

Table 15.4 Minimum CIL values for category “ B” Studs

| Entrance Angle | Observation Angle | CIL in mcd/lx |  |  |
| --- | --- | --- | --- | --- |
|  |  | White | Amber | Red |
| 0° U 6° L &R | 0.3° | 20 | 10 | 4 |
| 0° U 10° L &R | 0.5° | 15 | 6.5 | 3 |

Note:

1)	The entrance angle of 0o U corresponds to the normal aspect of the reflectors when the reflecting road stud is installed in horizontal surface.

2)	The stud incorporating one or more corner cube reflectors shall be included in Category “A” The stud incorporating one or more bi-convex reflectors shall be included in Category “B”.

(5)	Tests

Co-efficient of luminance intensity can be measured by procedure described in ASTM E809 “Practice for Measuring Photometric Characteristics” or as recommended in BS: 873-part 4: 1973.

Under test conditions, a stud shall not be considered to fail the photometric requirements if the measured CIL at any one position of measurement is less than the values specified in Table 15.1 or 2 provided that

- The value is not less than 80 percent of the specified minimum and

- The average of the left and right measurements for the specific angle is greater than the specified minimum.

(6)	Rumble Strips

The road marking by Rumble Strips shall be made as shown in the Drawings, and as approved by the Engineer.

(7)	Fixing of Reflective Markers

(a)	Requirements:

The enveloping profile of the head of the stud shall be smooth and the studs shall not present any sharp edges to traffic The reflecting portions of the studs shall be free from crevices or ledges where dirt might accumulate. Marker height shall not be less than 10 mm and shall not exceed 20 mm. and its width shall not exceed 130 mm. The base of marker shall be flat within 1.3 mm. If the bottom of the marker is configured, the outermost faces of the configurations shall not deviate more than 1.3 mm from flat surface. All road studs shall be legibly marked with name, trade mark or other means of identification of the manufacturer.

(b)	Placement

The reflective marker shall be fixed to the road surface using the adhesives and the procedure recommended by the manufacturer. No nails shall be used to affix the marker so that they do not pose safety hazard on the roads. Regardless of the type and adhesive used, the markers shall not be fixed if the pavement is not surface dry and on new asphalt concrete surfacing until the surfacing has been opened to traffic for a period of not less than 14 hours. The portions of the highway surface, to which the marker is to be bonded by the adhesive, shall be free of dirt, curing compound, grease, oil, moisture, lose or unsound layers, paint and any other material which would adversely affect the bond  of the adhesive.

The adhesive shall be placed uniformly on the cleaned pavement surface or on the bottom of the marker in a quantity sufficient to result in complete coverage of the area of contract of the marker with no voids present and with a slight excess after the marker has been lightly pressed in place. For epoxy installations, excess adhesive around the edge of the marker, excess adhesive on the pavement and adhesive on the exposed surface of the markers shall be immediately removed.

(c)	Warranty and Durability

The Contractor shall submit a two year warranty for satisfactory field performance including stipulated retro reflectance of the reflecting panel. To the Engineer. In addition, a two year warranty for satisfactory infield performance of the finished road marker shall also be given by the Contractor who carries out of the work of fixing of reflective road markers. In case the markers are displaced, damaged, get worn out or lose their reflectivity compared to stipulated standards, the contractor would be required to replace all such markers within 15 days of the intimation from the Engineer, at his own cost.

(8)	Measurement

Reflective road markers/ solar powered road studs shall be measured in number. Supply and erection of marker and accessories shall not be measured. They are deemed included in the measurement of reflective road marker/ solar powered studs.

(9)	Payment

Reflective road marker/ solar powered studs measured as above shall be paid at the contract unit rate. The contract unit rate shall be the full and the final compensation to the Contractor as per Clause 112 and also for the all other incidental costs so as to complete the work as specified.

| Item No. | Description | Unit |
| --- | --- | --- |
| 1507 | Reflective Pavement Markers (Road Studs) And Solar Powered Road Markers (Solar Studs) |  |
| 1507-1 | Plastic reflective pavement markers (Road studs at madien edge in tunnel, shoulder of traffic island and nouse) | nos. |
| 1507-2 | Aluminum reflective pavement markers (Road studs at Center) | nos. |
| 1507-3 | Rumble strips (W=150mm, L=350mm, Depth=12mm @ 300mm) on concrete pavement | lin.m |
| 1507-4 | Rumble strips (W=150mm, L=350mm, Depth=12mm @ 300mm) on asphalt pavement | lin.m |

1508	STEEL CRASH BARRIER

(1)	Scope

The work shall consist of furnishing/providing of all components of Steel W-Beam Crash Barrier and installation of steel beam crash barrier of dimensions as shown on the drawing(s) or as directed by the Engineer.

Steel W-beam crash barrier shall generally be located on approaches to bridge structures, at locations where embankment height is more than 3 meters and at sharp horizontal curves or as directed by the Engineer.

(2)	Materials

The metal beam rail shall be corrugated sheet steel beam of the class, type, section and thickness indicated on the drawings. Railing post shall be made of steel of the section, weight and length as shown on the drawings. All complete steel rail elements, terminal sections, posts, nuts, hardware and other steel fittings shall be galvanized. All elements of railing shall be free from abrasions, rough or sharp edges and shall not be kinked, twisted or bent.

The design materials to be used and the location of metal beam crash barrier shall conform to relevant drawings or as otherwise directed by the Engineer.

(a)	Order for Materials and Manufactured Articles

The Contractor shall, before placing any order for materials and manufactured articles for incorporation in the Works, submit to the Engineer the names of the firms from whom he propose to obtain such materials and manufactured articles, giving for each firm a description of the materials and manufactured articles to be supplied, their origin, the manufacturer’s specification, quality, weight, strength and other relevant details ( details of projects where the product/ process/ system has been successfully used). The Contractor shall submit the samples of such materials and manufactured articles when requested by the Engineer and when appropriate, manufacturer’s certificates of recent test carried out on similar materials and manufactured articles shall also be submitted.

(b)	Test Certificates

When instructed by the Engineer, the Contractor shall submit to him all Test Certificates from the suppliers/manufacturers of the materials and/or manufactured articles to be used for the contract. Such certificates shall certify that the materials and/or manufactured articles concerned have been tested in accordance with the requirements of these Specifications. All Test results shall be enclosed along with such certificates. The Contractor shall provide adequate means of identifying the materials and/or manufactured articles delivered on the site with the corresponding certificates.

(c)	Technical Specification for Steel Crash Barrier and Accessories

The “W beam type safety barrier shall consist of steel post and a 3 mm thick “W” beam rail element. The steel post and blocking out spacer shall both be channel section of 75 mm x 150 mm & 5 mm thick. The rail shall be 70 cm above the ground level and post shall be spaced 2m centre to centre Double “W “beam barrier shall be as indicated in IRC 5-1988.

The Thrie Beam Safety Barrier shall have post and spacer similar to ones mentioned above for “W “beam type. The rail shall be placed at 85 cm above the ground level.

The “W” beam, thrie beam, the posts, spacers and fasteners for steel barriers shall be galvanised by hot dip process (zinc coated 0.55 kg per square metre; minimum single spot) unless otherwise specified. The Galvanizing on all other steel parts shall be confirm to the relevant NS/IS specifications. All fittings (bolts, nuts, washers) shall confirm to the IS: 1367 and IS: 1364. All galvanizing shall be done after fabrication.

| SN | Item | Requirement |
| --- | --- | --- |
| 1 | W-beam guard rail : | Base Metal: The beam, end sections shall consist of sheet made of open hearth, electric furnace, or basic oxygen steel and shall meet the mechanical properties specified below: - Length of rail -- 4.318 m/ 2.318 m - Yield stress, minimum, 310 MPa; and - Elongation, in gauge length (5.65X (sqrt of cross sectional area)) minimum, 15 percent. |
| 2 | C-Channel post: | - Length of Post – 1800 mm. - Yield stress , minimum 410 MPa; |
| 3 | C-Channel spacer | - Length of Spacer– 330 mm. - Yield stress , minimum 410 MPa |
| 4 | All fittings ( Bolt , Nuts, Washer) | - Confirm to IS 1364 and IS 1367 |

(d)	Packing of Steel W- Beam Crash Barrier and accessories

The Steel W-Beam Crash Barrier and accessories shall be well packed after the verification of compliance with specification requirement to the acceptable standard by the Engineer/Inspection agent before dispatching from the manufacturer’s premises. The packing shall facilitate safe transportation/shipment which ensures maintaining the true shape and zinc coating. The packing shall be tagged by the Engineer/Inspection Agent before dispatching from the manufacturer’s premises and shall remain intact upon arrival at designated storage site.

(e)	Storage of Metal Beam Crash Barrier and accessories

The Contractor shall be responsible for the safe storage of the Metal Beam Steel Crash Barrier and accessories in the storage yard. The Contractor shall be responsible for the safe keeping of the Metal Beam Crash Barrier and all accessories until the installation. The contractor shall submit details of storage facility for approval of the Engineer so as to ensure that the crash barriers imported will be stored satisfactorily.

(3)	Construction Operations

(a)	The line and grade of Steel Beam Crash Barrier shall be true to that shown on the plans. The railing shall be carefully adjusted prior to fixing in place to ensure proper matching at abutting joints and correct alignment and camber throughout their length. Holes for field connections shall be drilled in place in the structure at proper grade and alignment.

(b)	Splices and end connections shall be of the type and designs specified or shown on the plans and shall be of such strength as to develop full design strength of the rail elements.

(c)	Concrete for bedding and anchor assembly shall conform to Section 2000. The minimum size of the concrete foundation block for embedding guard posts and grade of concrete shall be as shown in the drawing or as directed by the Engineer.

(d)	Installation of Posts: Holes shall be dug or drilled to the depth indicated on the drawings or post may be driven by approved method and equipment, provided these are erected in proper position and are free from distortion and burring or any other damage.

All post holes that are dug or drilled shall of such size as will permit proper setting of the posts and allow sufficient room for backfilling or tapping. Alternatively, the guard posts may be embedded in the concrete footing of size and grade of concrete along with the depth of the embedment of post or as indicated in the drawing.

Posts and the end section for steel w-beam crash barrier on bridges shall be bolted to the structure as detailed on the plans. The anchor bolts shall be set to proper location and elevation with templates and carefully checked.

(e)	Erection: The Contractor shall engage a qualified person having experience in installing metal crash barrier.

All guardrail anchors shall be set and attachments made and placed as indicated on the plans or as directed by the Engineer.

All bolts or clips used for fastening the guardrail or fittings to the posts shall be drawn up tightly. Each bolt shall have sufficient length to extend at least 6 mm through and beyond the full nut, except where such extensions might interfere with or endanger traffic in which case the bolts shall be cut off flush with the nut.

All railings shall be erected, drawn and adjusted so that the longitudinal tension will be uniform throughout the entire length of the rail.

The reflective sticker shall be at the spacing of 4m interval or as directed by the Engineer. Retro reflective sheeting / tape for sticking on to guardrails shall meet the retro reflective properties. Tape should be 10 cm wide with red and white colour and shall be pasted as per the instruction of the Engineer.

(5)	End Treatment for Steel Barrier

End treatments shall from an integral part of safety barriers which should not spear, vault or roll a vehicle for head-on or angle impacts. The two end treatments recommended for steel barriers are "Turned- down- guardrail" and Anchored in back slope" as shown on the Drawings or as directed by the Engineer. Other treatment may be placed as shown in the drawing or as directed by the Engineer.

(6)	Tests and Standard of Acceptance

The Contractor shall be responsible for testing and conforming compliance of the quality of Metal Beam crash barrier and all related accessories as specified in above table upon arrival in storage yard. The Contractor shall bear the cost related to all sampling and testing cost including any transportation cost of sample to the laboratory.

The Contractor shall inform when the delivery of Metal Beam Crash Barrier and its accessories to the storage yard is completed. The Engineer can take assistance of Inspection Agent for the verification while the Contractor makes arrangement of sampling and testing of compliance with specification requirement of Metal Beam Crash Barrier and its accessories from an independent laboratory accredited by reputed accreditation agencies in the Employer’s Country and their quantity

The material shall be tested in accordance with the relevant standards specified and shall meet the prescribed criteria. The Contractor shall be responsible for testing and conforming compliance of the quality of Steel W-Beam Crash Barrier and all related accessories before dispatching them from manufacturer’s premises. The Contractor shall bear the cost related to all sampling and testing cost including any transportation cost of sample to the laboratory

Frequency of sampling:

One piece of Steel W-Beam Crash Barrier, end section, nut, bolt, washer, channel post, spacer from each 200 pieces in a lot or from each lot if less than 200 pieces shall be sampled for determination of compliance with specification requirement

From each lot the number of tests for the fulfilment of requirements for Nut and Bolts shall be as follows:

| Number of Pieces in Lot | Number of Supplies |
| --- | --- |
| 800 and under | 1 |
| 801 to 8000 | 2 |
| 8001 to 22000 | 3 |
| Over 22000 | 5 |

Tolerance:

The posts shall be vertical with a tolerance not exceeding 6 mm in a length of 3 meter. The railing barrier shall be erected true to line and grade.

(7)	Measurement

All steel crash barriers shall be measured by linear metres of completed and accepted length in place.

Unless otherwise stated in other sections of these specifications, Terminals/ anchors, Retro reflective Stricker, drilling, excavation, preparation of foundation and backfill, supply and erection C channel post, C- channel spacer Bolt, nuts and accessories, and all incidentals to complete the work in accordance with the Drawings and Specifications, and as directed by the Engineer, shall neither be measured nor paid separately. They are deemed included in the measurement of crash barrier.

(8)	Payment

Steel crash barrier measured as above shall be paid at the contract unit rate. The contract unit rate shall be the full and the final compensation to the Contractor as per Clause 112 and also for the cost of excavation, backfill, painting reflecting elements (where specified) including all other incidental costs so as to complete the work as specified.

| Item No. | Description | Unit |
| --- | --- | --- |
| 1508 | Steel Crash Barrier |  |
| 1508-1 | Steel crash barrier type-A set on earth shoulder | lin.m. |
| 1508-2 | Steel crash barrier type-B set on structure | lin.m. |

1509	POLYETHYLENE TANK CRASH BARRIER

(1)	Scope

This Clause covers the supply and installation of Polyethylene Tank Crash Barrier at locations as shown in the Drawing or as directed by the Engineer.

(2)	Materials

The Polyethylene Tank Crash Barriers shall be supplied from an approved source. If painting is required, the paint shall be non-reflectorized type and shall confirm NS: 112.

The following types shall be provided:

Polyethelene tank 1000 litre filed with sand

Polyethelene tank 1000 litre filed with water

Polyethelene tank 750 litre filed with water

Polyethelene tank 500 litre filed with water

(3)	Submittals

The Contractor shall submit all information regarding to the Polyethylene Tank Crash Barriers listed above to be supplied including fabrication catalogues and manufacturer's specifications, fabricator name and certifications, installation manuals, including details of procurement and transportation for the Engineer’s consent.

(3)	Construction

The Polyethylene Tank Crash Barriers, before be filled with water or sand as may correspond, shall first be located in its respective location in accordance with the Drawings, and as approved by the Engineer.

(6)	Measurement

All barriers shall be measured by linear metres as specified in contract for completed and accepted length in place.

(7)	Payment

The Polyethylene Tank Crash Barriers, measured as above, shall be paid at the contract unit rate. The contract unit rate shall be the full and the final compensation to the Contractor as per Clause 112 and also for painting reflecting elements (where specified) including all other incidental costs so as to complete the work as specified.

| Item No. | Description | Unit |
| --- | --- | --- |
| 1509 | Polyethylene tank crash barrier |  |
| 1509-1 | Polyethylene tank 1000 litre filed with sand | No. |
| 1509-2 | Polyethylene tank 1000 litre filed with water | No. |
| 1509-3 | Polyethylene tank 750 litre filed with water | No. |
| 1509-4 | Polyethylene tank 500 litre filed with water | No. |

SECTION 1600: PILING FOR STRUCTURES

(1)		Add the following new Sub-Section at the end of Sub-Section 1617: “Payment” and before Section 1700: “Well Foundation”

1618	SHINSO PILES (CAST-IN-SITU PILE BY MANUAL EXCAVATION)

(1)	Description

The cast-in-situ piles (using manual excavation) defined in this Section consist of the piles constructed applying the construction method of Shinso piles for deep foundation (a method which works for concrete pouring are made by labours entering inside the excavated hole).

(2)	Standards

The works for construction of the Shinso Piles shall conform with the following standards:

- Guideline for Management of Concrete Constructions. Nippon Expressway Company Limited. Japan 2017.

- Guideline for Management of Structures Constructions. Nippon Expressway Company Limited. Japan 2017.

- Standard Specifications for Roads and Bridges. Volume IV: Construction of Substructure. Japan Road Association.

(3)	Materials

The requirements for concrete and steel reinforcement of Section 2000: “Concrete for Structures” and Section 2014: “Reinforcement” of these specifications shall be satisfied.

The materials for Liner-Plate regarding to the steel shall meet the requirements of the JIS standards, or equivalent upon the approval by the Engineer:

- JIS G3101-SS400 (steel plate/sheet for general purpose structural steels)

- JIS G3131-SPHC (steel plate/sheet for stamping and cold forming steels)

- JIS G3101- SS330 (Japanese Standard Common Structural Steel)

- JIS B1180 (Hexagon Head Bolt).

Specifically the material for Liner-Plate and Steel shall meet the following specifications:

- Liner plate:	SS330 as specified in JIS G3101 or equivalent material.

- Bolt / Nut: 	Grades 4.6 and 8.8 specified in JIS B1180 or equivalent.

- Steel: 	SS400 as specified in JIS G3101 or equivalent material.

(4)	Construction Requirements

- Prior to all activities described in this Section, the Contractor shall perform the respective boring investigation to confirm the existing conditions of the bearing ground in accordance with the requirements of Section 3000: “Subsurface Geotechnical Investigation” and/or as directed by the Engineer.

- When a liner-plate (including reinforcement rings) is being used as a retaining plate for preventing the collapse of the excavated hole walls, it shall be buried and installed in accordance with the Drawings and as instructed by the Engineer, and omissions, deformities and loosening shall be avoided.

- In addition, when a mortar-lining is being used, the surface of the excavated hole shall be prepared carefully removing the loose material and the supports shall be installed properly as shown in the Drawings and as instructed by the Engineer, the construction shall be performed securing the use of adequate materials.

- The requirements for quality of the mortar for lining are the following:

| Compressive strength after 15hr of curing (1) | Compressive strength after 28 days of curing(1) | Cement Type |
| --- | --- | --- |
| More than 3 N/mm2 | More than 24 N/mm2 | Normal Portland Cement |

Notes:

(1): 	Testing method in accordance with the standards of JSCE (Japan Society of Civil Engineering) for testing cores of d=5cm: JSCE F561-2005.

- If shotcrete and rock-bolts are used, it shall refer to the relevant sections of these specifications.

- In the sections where the liner-plate will be applied, it is necessary to set-up previously the grouting-pipe and prepare the mortar, and to carry out the back filling grout. The following requirements shall be satisfied for preparation of the grout mortar for back filling:

| Material for Injection | Cement Content (2) | Mortar Flow | Air Content (3) | Compressive strength after 28 days of curing(1) |
| --- | --- | --- | --- | --- |
| Mortar for back-filling | 340 kg/m3 | 25±5 Seconds | 4±1 % | More than 2.5 N/mm2 |

Notes:

(1): 	Testing method in accordance with the standards of JSCE (Japan Society of Civil Engineering) for testing cores of d=5cm: JSCE F561-2005.

(2):	Cement type: Normal Portland Cement or Blast Furnace Cement Type-B

(3):	Tested in accordance with the standards of ASTM C231: “Standard Test Method for Air Content of Freshly Mixed Concrete by the Pressure Method”.

- The approval by the Engineer on the materials shall be based on the standards of the “Guideline for Management of Structures Constructions” by Nippon Expressway Company Limited (Japan 2017) to perform the testing and supervision.

- In the case of injection, the system shall set up an apparatus for measuring the flow rate and pressure, and the measurement results must be submitted to the Engineer.

- The concrete used in these works shall be referred to the requirements of Section 2000: “Concrete for Structures”. The concrete shall be of Class-B.

- The reinforcement used in these works shall be referred to the requirements of Section 2014: “Reinforcement”.

- Once the excavation is completed, the Contractor shall request for an inspection by Engineer.

- In the event that an unforeseen material such a huge boulder, etc. is found during the excavation, the Contractor shall request the instructions from the Engineer regarding to how to handle this situation. In this case, if the Engineer considers that it will be necessary a modification on the design drawings, a Variation Order to modify the design and respective unit price shall be discussed with the Engineer.

- The treatment applied for the excavation shall be in accordance with these Special Provisions to the Standards Specifications.

(5)	Measurement

- The works for “Shinso piles with steel lining” shall be measured for payment in cubic metres of excavation actually completed in Site as show in the Drawings for each diameter and plate thickness, and approved by the Engineer. The measurement of excavated volume shall be based on the nominals diameter and length of the liner-plate buried in Site and accepted by the Engineer.

- The Shinso Piles (Cast-in-Situ by manual excavation), of the sizes and types specified in the Drawings, shall be measured in cubic metres of excavation actually completed in Site, and approved by the Engineer. The measurement of excavated volume shall be based on the nominals diameter and length shown in the Drawings for each pile type, actually buried in Site and accepted by the Engineer.

- Quantities of Concrete Class B for Shinso Piles shall be measured in cubic metres of Concrete Class-B actually completed in Site, in accordance with the requirements of Section 2018(1): “Concrete” (Measurement), and as approved by the Engineer.

- Quantities of reinforcement shall be measured in tons of Reinforcement actually completed in Site, in accordance with the requirements Section 2018(4): “Reinforcement” (Measurement) and as approved by the Engineer.

(6)	Payment

- General: The Shinso Piles measured as described above shall be paid at the contract unit rates which shall be the full and the final compensation to the Contractor as per Clause 112, including excavation, cleaning, and all ancillary works, and furnishing, producing, mixing and placing mortar, additives (if any), all materials and tests including transportation, furnishing all labour, materials, tools, machinery/equipment and any incidentals to complete the work as shown in the Drawings, as required by these Specification Sections, and/or as directed by the Engineer.

- “Shinso piles with steel lining”: Payment shall include the cost for excavated quantities of the material for steel lining measured in accordance with the provisions of the preceding paragraph (general) and all works for installation of the Grouting-pipe and liner-plate in accordance with the design and Drawings, and as approved by the Engineer.

- “Shinso Piles (Cast-in-Situ by manual excavation)”: Payment shall include the cost for excavated quantities of the material for Cast-in-Situ by manual excavation measured in accordance with the provisions of the preceding paragraph (general) and all cost for performing the process of manual excavation and Cast-in-Situ of concrete, mortar lining for soil fastening (shotcrete), installation of lining thickness inspection rods, etc.

- “Concrete Class-B for Shinso Piles”: Payment shall include the cost for providing and placing Concrete Class-B for Shinso Piles in accordance with the requirements of Section 2000: “Concrete for Structures for the works and Section 2019(1): “Concrete” (payment)

- “Reinforcement”: Payment shall include the cost for providing and installing Reinforcement in accordance with the requirements of Section 2014: “Reinforcement” for the works and Section 2019(3): “Reinforcement” (payment).

Payment for the following items for “Shino” Piles (Cast-in-Situ Pile by Manual Excavation) shall be made under Section 7500: “Steel Bridge for Flyover” in accordance with the requirements established in this Section:

Pay Item 7503-1:	Excavation of Shinso with Steel Lining 
(D=2.50m ~ 5.00m)	cu.m.

Pay Item 7503-2:	Excavation of Shinso with Shotcrete (D=2.50m ~ 5.00m, 100mm thick with steel net)	cu.m.

Pay Item 7503-3:	Excavation of Shinso with Shotcrete (D=2.50m ~ 5.00m, 150mm thick with steel net)	cu.m.

Pay Item 7503-4:	Concrete Class-B for Shinso Piles (D=2.50m ~ 5.00m)	cu.m.

Pay Item 7503-5:	Reinforcement for Shinso Piles	ton

SECTION 1900: BEARINGS AND EXPANSION JOINTS

1901	EXPANSION JOINTS

(8)	Payment

(1)	Insert the following at the end of Clause 1901(8): “Payment”:

| Item No. | Description | Unit |
| --- | --- | --- |
| 1901 | Expansion Joints |  |
| 1901-1 | Expansion Joint Type-A (for Reinforced Concrete Bridge) | lin.m. |

1902	BEARINGS

(6)	Measurement

(2)	Delete the full text of Clause 1902(6): “Measurement” and replace with the following:

Regardless the material or type, all bearings shall be measured in numbers, inclusive of all ancillary and incidental works, according to their capacities and particular specifications given on the drawings.

(7)	Payment

(3)	Delete the full text of Clause 1902(7): “Payment” and replace with the following:

The bearings measured as above shall be paid at the contract unit rate which shall be the full and the final compensation to the Contractor as per Clause 112 including cost for bearing devices, material, labour, equipment, tools, and all incidentals used to complete the work in accordance with the Drawings and Specifications, and as directed by the Engineer.

| Item No. | Description | Unit |
| --- | --- | --- |
| 1902 | Bearings |  |
| 1902-1 | Elastomeric Bearing 320 mm x 500 mm x 56 mm (without pins) | No. |
| 1902-2 | Elastomeric Bearing 200 mm x 320 mm x 40 mm (without pins) | No. |
| 1902-3 | Anchor bar and cap (for approach slab) | set |

SECTION 2000: CONCRETE FOR STRUCTURES

2003	MATERIAL FOR CONCRETE

(2)	Cement

(1)	Delete the first paragraph of Clause 2003(2): “Cement” and replace with the following:

Cement shall be Ordinary Portland Cement (OPC) Grade-53 complying with the requirements of Clause 614, or other type of cement approved by the Engineer.

The cement to be used for construction of Continuously Reinforced Concrete Pavement (CRCP) shall be provided in the manufacturer bulk transported to the Project Site in containers.

2004	DESIGN OF CONCRETE MIXES

(1)	Classes of Concrete

(2)	Rename the Table 20.3 “Concrete Classes and Strength” as follows:

Table 20.3(a) Concrete Classes and Strength

(3)	Insert the following table after Table 20.3 as per supplementary description of specific concrete classes to be used in the Project:

Table 20.3(b) Composition and Strength of Specific Concrete Classes used in the Project

|  | Concrete Class |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Description | A | B | C | D | E (Lean Concrete) | P (CRCP, JRCP) |
| Maximum Size of Coarse Aggregate (4) (mm) | 20 | 20 | 20 | 20 | 20 | 20 |
| Slump (cm) (1) | 7.5±2.5 | 7.5±2.5 (15±2.5) (3) | 7.5±2.5 | 8.0±2.5 | 7.5±2.5 | 3.5±1.5 |
| Ratio of water/cement (%) (5) | 49.4 | 61.4 | 65.0 | 65.0 | 76.0 | 42 |
| Minimum Cement Content (kg/m3) | 450 | 350 | 350 | 310 | 180 | 450 |
| Minimum Strength at 28 days (MPa) (2) | Compressive Strength. Test method of AASHTO T22 using cylinders |  |  |  |  | Flexural Strength AASSHTO T97 (ASTM C78) |
|  | 30 | 25 | 20 | 15 | 13 | 4.25 |
| Minimum Strength at 28 days (MPa) (2) | Compressive Strength. Test method of IS 516.1959 using cubes |  |  |  |  |  |
|  | 35 | 30 | 25 | 20 | 15 |  |

Notes:

1) Slump shall be determined by AASHTO T119 or JIS A 1101.

2) Testing for concrete compressive strength using cylinders or cubes shall be subject of approval by the Engineer unless otherwise herein specified.

3) Requirements for Building Works

4) The aggregates for these Concrete Classes shall satisfy the requirements specified by AASHTO-M6 (for fine aggregates) and AASHTO M43 (for coarse aggregates)

5) The ratios shown for the w/c contents are as per initial reference. The Contractor shall determine the proper ratio based on the results obtained from the Mix Design approved by the Engineer.

Table 20.3(c) Concrete Classes and its Application in the Project

| Concrete Class (C strength/size) (note) | Equivalent Type (Table 20.3a) | Description |
| --- | --- | --- |
| A (C30/20) | M35/20 | Concrete for bridges and structures: - Footing of bridges - Abutment of bridges - Pier of bridges - Slab for bridges; RC Slab for Flyover - Railing and guard-wall for Flyover |
| B (C25/20) | M30/20 | Concrete for building and structures, - Box culverts (basement/wall/slab) - Abutment of flyover - Pier of flyover - Shinso piles - Tunnel portal structures If additives for Air Entrance (AE) are used, Concrete Class B-AE can be established. |
| C (C20/20) | M25/20 | - Gates (sliding and/or fixed) - Crib frames - Kerbs (precast or cast-in-place) - Footpaths - Transmission Line If additives for Air Entrance (AE) are used, Concrete Class B-AE can be established. |
| D (C15/20) | M20/20 | - Foundations footing - Drainage works - Channels - Backside of breast wall - Foot protection blocks for river-bed protection - KM posts - Basement for fences and railings - Stone masonry walls - Foundations for the foot/base of cast-in-place crib works - Support’s foundation for Traffic Information Boards (TIB). - Concrete pavement type JCP for tunnel |
| E (C13/20) | M15/20 | Lean concrete/leveling concrete. For all structures |
| P (f 4.25) | n/a | Concrete pavements: - CRCP - JRCP - Terminal Anchors for CRCP - and any other concrete pavements |

Note:	Example: A (C30/20) = Concrete Class-A (Compressive Strength of 30Mpa at 28 days tested by method of AASHTO T22 using cylinders, and maximum size of coarse aggregate of 20mm)

Before application of the designed mixes, the Contractor shall perform a complete set of testing of samples using cylinders (AASHTO T22) and cubes (IS 516.1959fdsf) in order to establish and confirm the ratio of correlation of the compressive strength between both methods.

The composition and strength of concrete herein specified shall be considered without detriment to the stipulations described in other sections of these Specifications. In case of any discrepancy, the Engineer will provide a clarification.

(5)	Quality Control of Concrete Production

(4)	Insert the following at the end of Clause 2004(5): “Quality Control of Concrete Production”:

(c)	Sampling and Testing for CRCP:

Sampling and testing of concrete for CRCP shall be in accordance with the specification standards of AASSHTO T97 (ASTM C78) for determining the Flexural Strength and AASHTO T22 for Compressive Strength.

The minimum frequency for sampling of concrete for CRCP shall be of 4 sets every 100 m3 of concrete (i.e. one set every 25.0m3).

The quantity of concrete to be placed shall be in accordance with the construction statement and the daily construction program.

The set for sampling is defined as follows:

- Sampling for Flexural Strength Testing:

1 Set = 9 samples of beams L=70cm x H=15cm x W=15cm

To be tested at 3 days (3 samples), 7 days (3 samples) and 28 days (3 samples)

- Sampling for Compressive Strength Testing:

1 Set = 9 samples of cylinders d=15cm x H=30cm

To be tested at 3 days (3 samples), 7 days (3 samples) and 28 days (3 samples)

Testing at 3 days can be modified depending upon the schedule for continuation of concrete placing after every transversal construction joint (minimum 1 day, maximum 3 days or as approved by the Engineer). Early testing is acceptable to verify the strength achieved at the time that the works can be continued.

2005	MIXING CONCRETE

(5)	Replace the existing second paragraph of Sub-Section 2005: “Mixing Concrete”, starting with “Concrete for the works……etc.” by the following:

Concrete mixes shall be proportioned according to the Design mix or Prescribed mix as specified. All mixing operations shall be supervised by experienced supervisor. Cement and aggregate shall be batched by weight or by volume as specified in the contract. Water may be measured by weight or volume. The quantity of cement, each size of aggregate and water as indicated by the mechanism employed shall be within a tolerance of plus or minus three percent of the respective weight/ volume per batch agreed by the Engineer. The water to be added to the mix shall be reduced by the amount of free water contained in the coarse and fine aggregates. This amount shall be determined by the Contractor by a method agreed by the Engineer.

Concrete for the works shall be batched and mixed exclusively only in the Plants installed by the Contractor for the Project.  The batching plants mobilized and installed for the Project shall be automatic and computerized of weighing (cement, aggregates, water, admixture) and mixing. All data for mixing (date, mixed volume, mixing time, weight of each materials, temperature) shall be recorded by computer hard disk and/or hard drive and shall be printed in daily basis to keep those documents properly compiled. The Contractor shall provide the mixing data to the Engineer before pouring operation is started.

(6)	Insert the following at the end of Sub-Section 2005(3): “Mixing Concrete”:

Hand mixing of concrete shall not be allowed. In any case that the Contractor requires to perform some specific small volumes of works using non-structural by hand-mixing may, the Contractor shall timely submit to the Engineer for review and approval all details of those works. These work shall not be measured nor paid separately, and it shall deemed included in the rates of the respective main items.

2007	PLACING OF CONCRETE

(7)	Insert the following paragraph immediately before of Clause 2007(1): “Consent of Placing”:

Prior to any work for placing concrete is performed, the Contractor shall submit to the Engineer, for review and consent, the sequence of construction, and a method statement on the arrangements to supply, shop-drawings, handle and finish each intended pour area. The method statement shall contain comprehensive details of the operation necessary to produce the construction joints.

(8)	Add the following new Sub-Section 2017: “Test and Standard of Acceptance” and renumber the remaining Sub-Sections as follows:

Renumbering:

2017: “Measurement” 	→ 2018: “Measurement”

2018: “Payment”	→ 2019: “Payment”

2017	TEST AND STANDARD OF ACCEPTANCE

The requirements and procedures of this sub-section shall be applied as supplement, and without detriment, to other sections of these Specifications. In any case of discrepancies, a clarification by the Engineer shall be requested.

Testing of samples for CRCP shall be referred to Section 2050: “Concrete Pavements”.

(1)	General:

Concrete shall conform to the surface finish and tolerance as prescribed in these specifications for respective components.

Random sampling and lot by lot of acceptance inspection shall be made for the 28 days’ cube or cylindrical strength of concrete.

Concrete under acceptance shall be notionally divided into lots for the purpose of sampling, before commencement of work. The basis of delimitation of lots shall be as follows;

(a)	No individual lot shall more than 30 cu.m. in volume

(b)	Different grades of mixes of concrete shall be divided into separate lots

(c)	Concrete of a lot shall be used in the same identifiable component of the structure

(2)	Sampling and Testing

Concrete for making 3 test cubes/cylinders shall be taken from a batch of concrete at point of delivery into construction, according to procedure laid down in IS: 1199 or AASHTO T22 accordingly.

A random sampling procedure to ensure that each of the concrete batches forming the lot under acceptance inspection has equal chance of being chosen for taking cubes 150 mm cubes shall be made, cured and tested at the age of 28 days for compressive strength in accordance with IS: 516. The 28-day test strength result for each cube shall form an item of the sample. Test at other age also shall also be performed, if specified.

Similar procedure shall be made for testing cylindrical samples in accordance with AASHTO T22.

(3)	Test Specimen and Sample Strength

Three test specimens shall be made from each sample for testing at 28 days. Additional specimens maybe required for various purposes such as to determine the strength of concrete at 7 days or for any other purpose.

The test strength of the sample shall be the average of the strength of 3 specimens. The individual variation should not be more that ±15 per cent of the average. If variation is more, the test results of the sample are invalid.

The minimum frequency of sampling of concrete of each grade shall be in accordance with Table 5-1: “Testing Schedule” of Sub-Section 510: “Recapitulative Schedule of Tests”.

(4)	Acceptance criteria

(a)	Compressive Strength

Cubes and/or Cylinders:

The concrete shall be taken as having the specified compressive strength when both the following conditions are met:

- The mean strength determined from any group of four consecutive non overlapping samples exceeds the specified characteristic compressive strength by 3 MPa.

- Strength of any sample is not less than the specified characteristic compressive strength minus 3 MPa.

The quantity of concrete represented by the test results include the batches from which the first and last samples were taken, together with all intervening batches

Cores:

When the concrete does not satisfy both the condition given in I above, representative cores shall be extracted from the hardened concrete for compression test in accordance with the method described in IS: 1199, or equivalent of AASHTO for cores’ testing, and tested to establish whether the concrete satisfies the requirement of compressive strength.

Evaluation of compressive strength by taking cores may also be done in case of doubt regarding the grade of concrete used either due to poor workmanship or based on result of cube strength tests the locations from which core samples.

(b)	Chloride and Sulphate Content

Where Choloride and Sulphate is specified. The total chloride and sulphuric anhydride (SO3) content of all the constituents of concrete as a percentage of mass of cement in the mix shall not exceed the values given in this Section of the specifications.

(c)	Density of Fresh Concrete

Where minimum density of fresh concrete is specified, the mean of any four consecutive non-overlapping samples shall not be less than the specified value and any individual sample result shall not be less than 97.5 per cent of the specified value.

(d)	Density of Hardened Concrete

Where minimum density of hardened concrete is specified, the mean of any four consecutive non – overlapping samples shall not be less than the specified value and any individual sample result shall not be less than 97.5 per cent of the specified value.

(e)	Permeability Test

Where Permeability of concrete is specified, water Permeability test as per DIN: 1048 Part 5 – 1991 shall be carried out as described below

- A cylindrical test specimen 150 mm dia and 160 mm high shall be prepared

- After 28 days of curing, the test will be conducted between 28 and 35 days. The test specimen shall be fitted in a machine such that the specimen can be subjected to a water pressure upto7 bars.

- The concrete specimen shall be subjected to a water pressure of 0.5 N/mm2 from the top for the period of 3 days. The pressure shall be maintained constant throughout the test period. If the water penetrates through to the underside of the specimen, the test may be terminated and the specimen rejected as fail.

- After 3 days, the pressure shall be released and the sample shall be taken out. The specimen shall be split in the middle by compression applied on two round bars on opposite sides above and below.

- When the split faces show signs of drying (after 5 to 10 minutes) the maximum depth of penetration in the direction of height shall be measured with the scale and extent of water penetration established.

- The mean of maximum depth of penetration obtained from three specimens thus tested, shall be taken as the test result and it shall not exceed 25 mm.

If the concrete is not able to meet any of the standards of acceptance as prescribed, the effect of such deficiency on the structure shall be investigated by the Contractor as directed by the Engineer. Any additional work required by the Engineer for such acceptance shall be carried out by the Contractor at his cost. In such case payment for such work shall be made at reduced rate proportional to reduced strength rate. In case the concrete is not found to be acceptable even after investigation, the Contractor shall remove the rejected concrete forthwith.

2019	PAYMENT

(1)	Concrete

(9)	Insert the following at the end of Clause 2019(1): “Concrete”:

(f)	The following items for concrete works (including those for reinforced concrete) shall be paid under this Section and shall include the works for providing and placing the concrete, all materials, forms, falseworks & scaffolding (when needed), compaction, curing, testing etc., and tools, equipment, plants and all incidentals used to complete the work in accordance with the Drawings and Specifications, and as directed by the Engineer:

| Item No. | Description | Unit |
| --- | --- | --- |
| 2000 | Concrete for Structures |  |
| 2000-1 | Concrete class A (M35/20, C30/20) | cu.m. |
| 2000-2 | Concrete class B (M30/20, C25/20) | cu.m. |
| 2000-3 | Concrete class C (M25/20, C20/20) | cu.m. |
| 2000-4 | Concrete class D (M20/20, C15/20) | cu.m. |
| 2000-5 | Concrete class E (Lean concrete) | cu.m. |

(2)	Reinforcement

(9)	Insert the following at the end of Clause 2019(3): “Reinforcement”:

| Item No. | Description | Unit |
| --- | --- | --- |
| 2000-6 | Reinforcement Bar | ton |

(10)	Add the following new Sub-Section at the end of Section 2000: “Concrete for Structures” and before Section 2100: “Pre-Stressing”

2050	CONCRETE PAVEMENTS

2051	CONTINUOUSLY REINFORCED CONCRETE PAVEMENT (CRCP) AND JOINTED REINFORCED CONCRETE PAVEMENT (JRCP)

This work includes the construction of Continuously Reinforced Concrete Pavement (CRCP) and Jointed Reinforced Concrete Pavement (JRCP) for the road inside the tunnel, and for the access roads out-side the tunnel respectively with required thickness and shape of the cross section shown on the Drawings or as instructed by the Engineer.

This work includes also the construction of the Reinforced Concrete Pavement (RCP) for the evacuation tunnel, with required thickness and shape of the cross section shown on the Drawings or as instructed by the Engineer.

These Specifications shall be interpreted as supplement of, and without detriment to, the Standards Specifications and respective Special Provisions herein described.

2052	MATERIALS

(1)	Aggregates, Cement, Water and others

Aggregates, Cement, Water and others such as admixtures, material for curing, etc. shall be in accordance with requirements of Section 2000 “Concrete for Structures”, Sub-Section 2003: “Material for Concrete”.

(2)	Concrete Class

CRCP and JRCP shall use the concrete Class-P of Section 2000 “Concrete for Structures”, Sub-Section 2004; Table 20.3(b): “Composition and Strength of Concrete used in the Project”.

(3)	Joint Sealer

(a)	Joint sealer shall be in accordance with ASTM D6690-06 “Joint and Crack Sealants”, Hot Applied for Concrete Pavements.

(b)	Each lot or batch of sealing compound shall be delivered to the job site in the manufacturer's original sealed container. Each container shall be marked with the manufacturer's name, batch or lot number, and the safe heating temperature, and shall be accompanied by the manufacturer's certification stating that the compound meets the requirements of this specification.

(4)	Steel Reinforcement

Steel reinforcement shall be placed as shown on the Drawings, in accordance with Section 2000 “Concrete for Structures”, Sub-Section 2014.

The Contractor shall not weld reinforcement unless otherwise permitted by the Engineer.

(5)	Dowels

Dowels are specifically used for JRCP pavement type, and CRCP’s terminal anchors.

Dowels shall be as shown on the Drawings and in accordance with Section 2000 “Concrete for Structures”. Dowel bars shall be round bars and shall be free from burring or other deformation. Plastic or epoxy coating shall be applied to the central part of the steel dowels as shown in the Drawings. Coated dowels shall conform to the requirements of AASHTO M254. Where called for on the Drawings, coated dowels shall be lubricated with MC-70.

Expansion joints shall be provided with sleeves for dowel bars as shown in the Drawings.

(6)	Epoxy Resin

Epoxy resin used to anchor dowel bars in pavements shall conform to the requirements of ASTM C881, Type I, Grade 3, Class C.

(7)	Material Acceptance

Material Acceptance shall be in accordance with the requirements of Section 500: “Quality Control”, Section 600: “Materials and Testing of Materials” and Sub-Section 2017: “Test and Standard of Acceptance” accordingly.

(8)	Concrete Mixes

In addition to, and without detriment to the requirements of Sub-Section 2004: “Design of Concrete Mixes”, the following requirements shall also be satisfied:

(a)	Prior to the start of paving operations and after approval of all materials to be used for concrete production, the Contractor shall submit a Mix Design to the Engineer, for review and approval, showing the proportions and flexural & compression strengths of concrete tested as herein specified.

(b)	The Mix Design shall include copies of test reports, including test dates, and a complete list of materials including type, brand, source, and amount of cement, coarse aggregate, fine aggregate, water, and admixtures. The fineness modules of the fine aggregate shall also be shown. Copies of the mix design shall be submitted to the Engineer at least 30 days prior to the start of operations.

(c)	The Contractor shall not begin the production of concrete until the Engineer approves the Mix Design in writing.

(d)	Should a change in sources be made, or admixtures added or deleted from the mix, copies of a new mix design must be submitted to the Engineer for validation.

(e)	Test specimens for Flexural and Compressive Strength shall be tested in accordance with Section 600: “Materials and Testing of Materials” and Sub-Section 2017: “Test and Standard of Acceptance”.

2052	EQUIPMENT

The Contractor shall furnish all equipment and tools necessary for handling materials and performing all parts of the work.

(1)	Mixers and Transportation Equipment

Concrete may be mixed at a central plant according to the approved Construction Program prepared by the Contractor.

(a)	Central Plant Mixer

The central plant mixer provided for construction of CRCP and JRCP shall be in accordance with the requirements of Clause 2005: “Mixing Concrete” and the special provisions herein specified.

Central plant mixers shall also conform to the requirements of ASTM C94. The mixer shall be examined daily for changes in condition due to accumulation of hard concrete or mortar or wear of blades. The pickup and throw over blades shall be replaced when they have worn down 19mm or more. The Contractor shall have a copy of the manufacturer's design on hand showing dimensions and arrangement of blades in reference to original height and depth.

(b)	Transportation of Concrete inside Project Site

The Contractor shall detail the transportation system of concrete inside the project site, including details of equipment for transporting and method for concrete placing.

Truck mixers for mixing and hauling concrete and truck agitators for hauling central mixed concrete (if used), shall also conform to the requirements of ASTM C94.

(2)	Finishing Equipment

The finishing equipment shall be of sufficient weight and power for proper finishing of the concrete. The finishing system shall be established and operated to strike off, screed and consolidate the concrete such that laitance on the surface is less than 3 mm thick.

(3)	Vibrators

Vibrator shall be of either internal type with immersed tube or multiple spuds, or surface type vibrating pan or screed. For 20cm thick pavements or thicker, internal vibrators shall be used. They may be attached to the spreader or the finishing machine, or they may be mounted on a separate carriage. Operating frequency for internal vibrators shall be between 8,000 and 12,000 vibrations per minute.  Average amplitude for internal vibrators shall be 0.06 to 0.13 cm.

(4)	Concrete Saws

The Contractor shall provide sawing equipment adequate in number of units and power to complete the sawing to the required dimensions in the established time according to the approved Construction Program.

(5)	Side Forms

Straight side forms shall be made of steel and shall be furnished in sections not less than 3m in length. Forms shall have a depth equal to the pavement thickness at the edge. Flexible or curved forms of proper radius shall be used for curves of 31 meters radius or less. Forms shall be provided with adequate devices for secure settings so that when in place they will withstand, without visible spring or settlement, the impact and vibration of the consolidating and finishing equipment.  Forms with battered top surfaces and bent, twisted or broken forms shall not be used.  Built up forms shall not be used. The top face of the form shall not vary from a true plane more than 3mm in 3 metres, and the upstanding leg shall not vary more than 6 mm. The forms shall contain provisions for locking the ends of abutting sections together tightly for secure setting.

Wood forms shall not be used.

(6)	Pavers

Pavers shall be fully energized, stable and designed for the specific purpose of placing, consolidating, and finishing the concrete pavement, true to grade, tolerances, and cross section. They shall be of sufficient weight and power to construct the maximum specified concrete paving lane width as shown on the Drawings, at adequate forward speed, without transverse, longitudinal or vertical instability or without displacement. Pavers shall be equipped with electronic or hydraulic horizontal and vertical control devices.

2053	CONSTRUCTION

(1)	Preparation of Surfaces to be Paved

The works for construction of pavement shall be started only after the previous layer (lean concrete base course) has been duly completed and the Engineer has authorized to proceed with the subsequent works accordingly with the approved Construction Program.

(2)	Preparation of Underlying Surface, Side Form and Fill in Lane Construction

(a)		Form Setting

-	Forms shall be set sufficiently in advance of the concrete placement to insure continuous paving operation securing the completion of daily progress stated in the Construction Program.

-	After the forms have been set to correct grade, the underlying surface shall be thoroughly cleaned at both the inside and outside edges of the base of the forms.

-	Forms shall be staked into place sufficiently to maintain the form in position for the method of placement. Form sections shall be tightly locked and shall be free from play or movement in any direction. The forms shall not deviate from true line by more than 3mm at any joint. Forms shall be so set that they will withstand, without visible spring or settlement, the impact and vibration of the consolidating and finishing equipment.

-	Forms shall be cleaned and oiled prior to the placing of concrete. The alignment and grade elevations of the forms shall be checked and corrections made by the Contractor immediately before placing the concrete.

(b)	Underlying Surface, Side Form and Fill in Lane Construction

-	In case of paving on a lean-concrete base course, the prepared surface shall be moistened with water, without saturating, immediately ahead of concrete placement to prevent rapid loss of moisture from the concrete.

-	Regardless the nature of the underlying surface, damage caused by hauling or usage of other equipment shall be corrected and retested. If damage occurs to the base course, the Contractor shall correct before starting procedure for paving and as directed by the Engineer.

-	A template shall be provided and operated on the forms immediately in advance of the placing of all concrete. The template shall be propelled only by hand and not attached to a tractor or other power unit. Templates shall be adjustable so that they may be set and maintained at the correct contour of the underlying surface. The adjustment and operation of the templates shall be such as will provide an accurate retest of the grade before placing the concrete thereon.  All excess material shall be removed and disposed off. The template shall be maintained in accurate adjustment, at all times by the Contractor, and shall be checked daily.

(c)	Placement of Reinforcement

Reinforcing steel, at the time concrete is placed, shall be free of mud, oil, or other organic matter that may adversely affect or reduce bond. Reinforcing steel with rust, mill scale or a combination of both will be considered satisfactory, provided that the minimum dimensions, weight, and tensile properties of a hand wire brushed test specimen are not less than the applicable ASTM specification requirements.

(d)	Handling, Measuring, and Batching Material

-	The batch plant site, layout, equipment, and provisions for transporting material shall assure a continuous supply of material to the work. Stockpiles shall be constructed in such a manner that prevents segregation and intermixing of deleterious materials.

-	Aggregates that have become segregated or mixed with soil or foreign material shall not be used. All aggregates produced or handled by hydraulic methods, and washed aggregates, shall be stockpiled or binned for draining at least 12 hours before being batched.

-	Batching plants shall be equipped to proportion aggregates and bulk cement, by weight, automatically using interlocked proportioning devices of an approved type.  When bulk cement is used, the Contractor shall use a suitable method of handling the cement from weighing hopper to transporting container or into the batch itself for transportation to the mixer, such as a chute, boot, or other device, to prevent loss of cement. The device shall be arranged to provide positive assurance that the cement content specified is present in each batch.

(e)	Mixing Concrete

-	Concrete may be mixed at the work site, in a central mix plant. The mixer shall be of an acceptable type and capacity. Mixing time shall be measured from the time all materials, except water, are emptied into the drum. All concrete shall be mixed and delivered to the site in accordance with the requirements of ASTM C94.

-	Mixed concrete from the central mixing plant shall be transported by a previously approved system. The elapsed time from the addition of cement material to the mix until the concrete is deposited in place at the work site shall not exceed 2 hours.

(f)	Limitations on Mixing and Placing

-	Visibility:  Concrete shall be mixed, placed, or finished under natural light or artificial lighting system approved by the Engineer.

-	Hot Weather: During periods of hot weather when the maximum daily air temperature exceeds 30°C, the following precautions shall be taken:

- Concrete placing shall be made in accordance with the requirements of Sub-Section 2011: “Concreting in Hot Weather”.

- The forms and/or the underlying surface shall be sprinkled with water immediately before placing the concrete.  The concrete shall be placed at the coolest temperature practicable, and in no case shall the temperature of the concrete when placed exceed 30°C.

- The aggregates and/or mixing water shall be cooled as necessary to maintain the concrete temperature at or not more than the specified maximum. This can be done by covering with wet burlap or cotton mats, fog spraying with water, covering with protective housing, or by other approved methods.

- Moreover, during placement, the concrete temperature can be maintained by using any combination of the following:

- Shading the material storage areas or production equipment;

- Cooling the aggregate by sprinkling; and

- Cooling the aggregate and water by refrigeration or replacing a portion or all of the mix water with flaked or crushed ice to the extent that the ice completely melts during mixing of the concrete. For this purpose testing of the procedure shall be practiced previously by trials approved by the Engineer.

- The finished surfaces of the newly laid pavement shall be kept damp by applying a water fog or mist with approved spraying equipment until the pavement is covered by the curing medium.

- When conditions are such that problems with plastic cracking can be expected, and particularly if any plastic cracking begins to occur, the Contractor shall immediately take such additional measures as necessary to protect the concrete surface.  Such measures shall consist of windscreens, more effective fog sprays, and similar measures commencing immediately behind the paver.  If these measures are not effective in preventing plastic cracking, paving operations shall be immediately stopped.

(g)	Placing Concrete

-	The Contractor shall prepare and submit to the Engineer, for review and approval, an specific plan and method statement properly supported by shop-drawings including all necessary details for concrete placing for Continuously Reinforced Concrete Pavement (CRCP) and showing details on the equipment (batching plant, trucks for transportation, finisher and compaction, etc.); materials (concrete: cement, coarse and fine aggregates, sand, water approved in terms of quality and quantity); approved reinforcement placed on-site, and preparation and location of the construction joints, materials and equipment for curing, etc.

The Contractor has the option of placing the concrete with either side (fixed) forms or slip forms. At any point in concrete conveyance, the free vertical drop of the concrete from one point to another or to the underlying surface shall never exceed 1 metre.

Side Form Method:

-	For the side form method, the concrete shall be deposited on the surface prepared for underlying to require as little re-handling as possible. Unless truck mixers, truck agitators, or non-agitating hauling equipment are equipped with means for discharge of concrete without segregation of the materials, the concrete shall be placed and spread using an approved mechanical spreading device that prevents segregation of the materials.

-	Placing shall be continuous without the use of intermediate bulkheads (unless transverse joints approved by the Engineer or shown in the Drawings).  Necessary hand spreading shall be done with shovels not rakes. Workmen shall not be allowed to walk in the freshly mixed concrete with boots or shoes coated with earth or foreign substances.

-	Concrete shall be deposited as near to the joints as possible without disturbing them but shall not be dumped from the discharge bucket or hopper onto a joint assembly unless the hopper is centered above the joint assembly.

-	Concrete shall be thoroughly consolidated against and along the faces of all forms and previously placed concrete and along the full length and on both sides of all joint assemblies by means of vibrators inserted in the concrete.

-	Vibrators shall not be permitted to come in contact with a joint assembly, the grade, or a side form.  In no case shall the vibrator be operated longer than 20 seconds in any one location, nor shall the vibrators be used to move the concrete.

Slip Form Method:

-	For the slip form method, the concrete shall be placed with an approved crawler mounted, slip form paver designed to spread, consolidate and shape the freshly placed concrete in one complete pass of the machine so that a minimum of hand finishing will be necessary to provide a dense and homogeneous pavement in conformity with requirements of the Drawings and Specifications.

-	The concrete shall be placed directly on top of the joint assemblies to prevent them from moving when the paver moves over them. Side forms and finishing screeds shall be adjustable to the extent required to produce the specified pavement edge and surface tolerance. The side forms shall be of dimensions, shape, and strength to support the concrete laterally for a sufficient length of time so that no edge slumping exceeds the requirements for acceptance. Final finishing shall be accomplished while the concrete is still in the plastic state.

-	In the event that slumping or sloughing occurs behind the paver or if there are any other structural or surface defects which cannot be corrected within permissible tolerances, paving operations shall be immediately stopped until proper adjustment of the equipment or procedures have been made. In the event that satisfactory procedures and pavement are not achieved after one day of paving works, the Contractor shall complete the balance of the work using standard metal forms and its respective method of placing and curing.

Any concrete not corrected by the Contractor to permissible tolerances shall be removed and replaced at his own expense, and the time required for this correction shall be counted and registered to be taken into account when the contractual period is near to last.

(3)	Joints

-	Joints shall be constructed as shown on the Drawings and in accordance with these requirements. All joints shall be constructed with their faces perpendicular to the surface of the pavement and finished or edged as shown on the Drawings.

-	Joints shall not vary more than 13mm from their designated position and shall be true to line with not more than 6mm variation in 3 meters.

-	The surface across the joints shall be tested with a 3 metre straightedge as the joints are finished and any irregularities in excess of 6 mm shall be corrected before the concrete has hardened.

-	All joints shall be so prepared, finished, or cut to provide a groove of uniform width and depth as shown on the Drawings.

(a)	Construction Joints

-	Longitudinal construction joints shall be slip formed or formed against side forms with or without keyways, as shown on the Drawings.

-	Transverse construction joints shall be installed in the places specified in the Drawings, or included in the approved Construction Program, or where instructed by the Engineer.

-	In case of concrete pavement type CRCP, transverse construction joints shall be installed strictly as shown in the Drawings including the additional bar reinforcement. Any unplanned joint installation shall be rejected and the correction shall be made at the Contractor’s own expense.

In case of concrete pavement type JRCP, transverse construction shall be planned taking into account each day’s placing operations and at any other points within a paving lane where concrete placement might be interrupted for more than 30 minutes.

In no case the installation of transverse joints due to delay in providing fresh concrete will be approved.  In case a late concrete arrives, it shall be wasted in a determined area and it shall not be subject for payment and the expended time shall be counted as a delay without any compensation.

(b)	Keyways

-	Keyways shall not be applied.

(c)	Tie Bars

-	Tie bars shall consist of deformed bars installed in joints as shown on the Drawings.

-	Tie bars shall be placed at right angles and symmetrically spaced along the longitudinal joint at intervals shown on the Drawings. They shall be held in position parallel to the pavement surface and in the middle of the slab depth, or as shown in the Drawings.

-	The Contractor shall submit for approval a detailed procedure for installation of tie bars, including particular details for tie bars that will be installed on a joint where the concrete will be placed at different time for each contiguous slab.

(d)	Dowel Bars

-	Dowels are specifically used for JRCP pavement type, and CRCP terminal anchors.

-	Dowel bars or other load transfer units shall be placed across approved transverse joints with the dimensions and spacing, and placed in the manner, as shown on the Drawings, and shall be held rigidly in the middle of the slab depth in the proper horizontal and vertical alignments by an approved assembly device to be left permanently in place.

-	The dowel or load transfer and joint devices shall be rigid enough to permit complete assembly as a unit ready to be lifted and placed into position.

-	Dowel bars shall be painted with rust preventative paint and coated as indicated in the Drawings.

(e)	Installation of Joints

-	All devices used for the installation of joints shall be as specified.

-	The top of an assembled joint device shall be set at the proper distance below the pavement surface and the elevation shall be checked. Such devices shall be set to the required position and line and shall be securely held in place by stakes or other means to the maximum permissible tolerances during the pouring and finishing of the concrete.

-	Dowel bars and assemblies shall be checked for position and alignment. During the concrete placement operation, it is advisable to place plastic concrete directly on dowel assemblies immediately prior to the passage of the paver to help maintain dowel position and alignment within maximum permissible tolerances.

-	When concrete is placed using slip form pavers, dowels and tie bars shall be placed in longitudinal construction joints by bonding the dowels or tie bars into holes drilled into the hardened concrete.  Holes approximately 3 to 6 mm greater in diameter than the dowel or tie bar shall be drilled with rotary type core drills that must be held securely in place to drill perpendicularly into the vertical face of the pavement slab. Rotary type percussion drills may be used provided that spalling of concrete does not occur. The Contractor shall repair any damage of the concrete. Dowels or tie bars shall be bonded in the drilled holes using an epoxy resin material.  Installation procedures shall be adequate to insure that the area around dowels is completely filled with epoxy grout.  Epoxy shall be injected into the back of the hole and displaced by the insertion of the dowel bar. Bars shall be completely inserted into the hole and shall not be withdrawn and reinserted creating air pockets in the epoxy around the bar.

-	The Contractor shall furnish a template for checking the position and alignment of the dowels.

(f)	Sawing of Joints

-	Joints shall be cut as shown on the Drawings.

-	Equipment shall be as described in this section. The circular cutter shall be capable of cutting a groove in a straight line and shall produce a slot at least 3 mm wide and to the depth shown on the Drawings. The top portion of the slot shall be widened by sawing to provide adequate space for joint sealers as shown on the Drawings. Sawing shall commence as soon as the concrete has hardened sufficiently to permit cutting without chipping, spalling, or tearing and before uncontrolled shrinkage cracking of the pavement occurs. Sawing shall be carried out both during the day and night as required. The joints shall be sawed at the required spacing, consecutively in sequence of the concrete placement.

(4)	Final Strike Off, Consolidation and Finishing

(a)	Sequence

The sequence of operations shall be the strike off, floating and removal of latency, straight edging, and final surface finish. The addition of superficial water to the surface of the concrete to assist in finishing operations shall not be permitted.

(b)	Finishing at Joints

The concrete adjacent to joints shall be compacted or firmly placed without voids or segregation against the joint material; it shall be firmly placed without voids or segregation under and around all load transfer devices, joint assembly units, and other features designed to extend into the pavement. Concrete adjacent to joints shall be mechanically vibrated. After the concrete has been placed and vibrated adjacent to the joints, the finishing machine shall be operated in a manner to avoid damage or misalignment of joints. If uninterrupted operations of the finishing machine to, over, and beyond the joints, cause segregation of concrete, damage to, or misalignment of the joints, the finishing machine shall be stopped when the screed is approximately 20cm from the joint. Segregated concrete shall be removed from the front of and off the joint; and the forward motion of the finishing machine shall be resumed. Thereafter, the finishing machine may be run over the joint without lifting the screed, provided that there is no segregated concrete immediately between the joint and the screed or on top of the joint.

(c)	Machine Finishing

-	The concrete shall be spread as soon as it is placed, and it shall be struck off and screed by a finishing machine. The machine shall go over each area as many times and at such intervals as necessary to give proper consolidation and to leave a surface of uniform texture. Excessive operation over a given area shall be avoided.

-	When side forms are used, the tops of the forms shall be kept clean by an effective device attached to the machine, and the travel of the machine on the forms shall be maintained true without lift, wobbling, or other variation tending to affect the precision finish. During the first pass of the finishing machine, a uniform ridge of concrete shall be maintained ahead of the front screed for its entire length. When in operation, the screed shall be moved forward with a combined longitudinal and transverse shearing motion, always moving in the direction in which the work is progressing, and so manipulated that neither end is raised from the side forms during the striking off process. If necessary, this shall be repeated until the surface is of uniform texture, true to grade and cross section, and free from porous areas.

(d)	Hand Finishing

-	Hand finishing methods will not be permitted, except under the following conditions:

        - in the event of breakdown of the mechanical equipment, hand methods may be used to finish the concrete already deposited on the grade;

        - in areas of narrow widths or of irregular dimensions where operation of the mechanical equipment is impractical.

-	Concrete, as soon as placed shall be struck off and screed. An approved portable screed shall be used. A second screed shall be provided for striking off the bottom layer of concrete when reinforcement is used. The screed for the surface shall be a least 0.6 meters longer than the maximum width of the slab to be struck off. It shall be of approved design, sufficiently rigid to retain its shape, and shall be constructed either of metal or of other suitable material covered with metal. Consolidation shall be attained by the use of suitable vibrators.

(e)	Floating

After the concrete has been struck off and consolidated, it shall be further smoothed and trued by means of a longitudinal float using one of the following methods:

- Hand Method:

Long handled floats shall not be less than 3.6 meters in length and 15 cm in width, stiffened to prevent flexibility and warping. The float shall be operated from footbridges spanning but not touching the concrete or from the edge of the pavement. Floating shall pass gradually from one side of the pavement to the other. Forward movement along the centerline of the pavement shall be in successive advances of not more than one half the length of the float. Any excess water or latency in excess of 3 mm thick shall be removed and wasted.

- Mechanical Method:

The Contractor may use a machine composed of a cutting and smoothing float(s), suspended from and guided by a rigid frame and constantly in contact with the side forms or underlying surface. If necessary, long handled floats having blades not less than 1.5 meters in length and 15cm in width may be used to smooth and fill in open textured areas in the pavement. When the crown of the pavement does not permit the use of the mechanical float, the surface shall be floated transversely by means of a long handled float. Care shall be taken not to work the crown out of the pavement during the operation. After floating, any excess water and latency in excess of 3mm thick shall be removed and wasted. Successive drags shall be lapped one half the length of the blade.

(f)	Straight Edge Testing and Surface Correction

After the pavement has been struck off and while the concrete still being plastic, it shall be tested for trueness with a 5-meter straightedge swung from handles 1 meter longer than one half the width of the slab. The straightedge shall be held in contact with the surface in successive positions parallel to the centerline and the whole area gone over from one side of the slab to the other, as necessary. Advancing shall be in successive stages of not more than one half the length of the straightedge. Any excess water and latency in excess of 3 mm thick shall be removed from the surface of the pavement and disposed of. Any depressions shall be immediately filled with freshly mixed concrete, struck off, consolidated, and refinished. High areas shall be cut down and refinished.

(5)	Surface Texture

The surface of the pavement shall be finished with either a brush or broom finish applied when the water sheen has practically disappeared. The equipment shall operate transversely across the pavement surface, providing corrugations that are uniform in appearance and approximately 2mm in depth. It is important that the texturing equipment does not tear or unduly roughen the pavement surface during the operation. Any imperfections resulting from the texturing operation shall be corrected.

(6)	Curing

The construction requirements for curing shall conform to requirements of Sub-Section 2009: “Curing of Concrete” and this Sub-section.

Immediately after finishing operations are completed and marring of the concrete does not occur, the entire area of the newly placed concrete shall be cured in accordance with the methods described below.  Failure to provide sufficient cover material of whatever kind the Contractor may elect to use, or lack of water to adequately take care of both curing and other requirements, shall be cause for immediate suspension of concreting operations. The concrete shall not be left exposed for more than an hour during the curing period.

The Contractor shall utilize one of the following types of curing methods:

(a)	Impervious Membrane Method

-	The entire surface of the pavement shall be sprayed uniformly with white pigmented curing compound immediately after the finishing of the surface and before the concrete has set.

-	The curing compound shall not be applied during rainfall. Curing compound shall be applied by mechanical sprayers under pressure at the rate of 4 litres to not more than 14 square metres.

-	The spraying equipment shall be of the fully atomizing type equipped with a tank agitator. At the time of use, the compound shall be in a thoroughly mixed condition with the pigment uniformly dispersed throughout. During application the compound shall be stirred continuously by mechanical means. Hand spraying of odd widths or shapes and concrete surfaces exposed by the removal of forms will be permitted.

-	The curing compound shall be of such character that the film will harden within 30 minutes after application. Should the film become damaged from any cause, including sawing operations, within the required curing period, the damaged portions shall be repaired immediately with additional compound or by other approved means.

-	Upon removal of side forms, the sides of the exposed slabs shall be protected immediately to provide a curing treatment equal to that provided for the surface.

(b)	Water Method

-	Keep the concrete surface continuously wet by ponding, spraying, or covering with material that is kept continuously and thoroughly wet.

-	Covering material may consist of cotton mats, multiple layers of burlap, or other approved material that does not discolour or otherwise damage the concrete.

-	Entirely cover the surface of the pavement and the edges of the slab with water saturated mats. Extend mats at least twice the thickness of the pavement beyond the edges of the slab.

-	Place the mats in complete contact with the surface. Use weights or other approved methods to maintain contact.

(c)	Waterproof Cover Method

-	Thoroughly wet the surface using a fog mist applicator.

-	Entirely cover the surface with a waterproof cover. Lap the cover at least 460 mm.

-	Extend the cover beyond the edges of the slab at least twice the thickness of the pavement. Place the cover in complete contact with the surface.

(7)	Removing Forms

Unless otherwise specified, forms shall not be removed from freshly placed concrete until it has hardened sufficiently to permit removal without chipping, spalling, or tearing.  After the forms have been removed, the sides of the slab shall be finished, cleaned and protected. Major honeycombed areas shall be removed and replaced in accordance with respective specifications or as directed by the Engineer.

(8)	Sealing Joints

(a)	Time of Application

Joints shall be sealed as soon after completion of the curing period as feasible and before the pavement is opened to traffic, including construction equipment.

(b)	Preparation of Joints

-	Immediately before sealing, the joints shall be thoroughly cleaned of all latency, curing compound, and other foreign material.

-	Sandblasting and/or wire brushing shall be used for cleaning. Upon completion of cleaning, the joints shall be blown out with compressed air.

-	The joint faces shall be surface dry when the sealant is applied. Prior to resealing joints, any existing joint material shall be removed.

(c)	Installation of Sealant

-	Joints shall be inspected and approved by the Engineer for proper width, depth, alignment, and all preparation before sealing is authorized.

-	Sealant shall be installed in accordance with the following requirements:

        - Hot Poured Sealant shall be applied uniformly solid from bottom to top and shall be filled without formation of entrapped air or voids.

        - The heating kettle shall be of an indirect heating type, constructed as a double boiler. A positive temperature control and mechanical agitation shall be provided.

        - The sealant shall not be heated below the safe heating temperature.

        - The safe heating temperature shall be obtained from the manufacturer's shipping container.

        - A direct connecting pressure-type extruding device with nozzles shaped for insertion into the joint shall be provided.

        - The concrete surface adjacent to the joint locations shall be protected from spillage of the sealant by adhesive tape or other approved means. Any sealant spilled on the surface of the pavement shall be removed immediately.

(9)	Protection of Pavement

Protection of Pavement shall be made in accordance with the requirements of Sub-Section 2010: “Protection of Fresh Concrete” and this Sub-Section.

The Contractor shall protect the pavement and its appurtenances against all kind of traffic including traffic caused by the Contractor's employees and agents. This shall include watchmen to direct traffic and the erection and maintenance of warning signs, lights, crossovers, and protection of unsealed joints from intrusion of foreign material, etc.

Any damage to the pavement occurring prior to final acceptance shall be repaired or the pavement replaced at the Contractor's expense. For any pavement areas that are uncovered, the Contractor shall have available at all times, materials for the protection of the edges and surface of the unhardened concrete. Such protective materials shall consist of rolled polyethylene sheeting at least 0.1 mm thick of sufficient length and width to cover the plastic concrete slab and any edges. The sheeting may be mounted on either the paver or a separate movable bridge from which it can be unrolled without dragging over the plastic concrete surface. In any uncovered areas, when rain appears imminent, all paving operations shall stop and all available personnel shall begin covering the surface of the unhardened concrete with the protective covering.

(10)	Opening to Traffic

The pavement shall not be opened to traffic until test specimens molded and cured in accordance with ASTM C31 have attained the required strength when tested in accordance with these Specifications. If such tests are not conducted, the pavement shall not be opened to traffic until 14 days after the concrete was placed.  Prior to opening to traffic, the pavement shall be cleaned.

(11)	Rejection of Concrete Batches

The Engineer may at any time, notwithstanding previous plant acceptance, reject and require the Contractor to dispose of any batch of concrete mixture which is rendered unfit for use due to contamination, segregation, improper temperature of concrete or improper slump.

(12)	Acceptance of Pavement

(a)	Acceptance Sampling and Testing

-	Acceptance Sampling and Testing shall be made in accordance with the requirements of Section 500: “Quality Control”, Section 600: “Materials and Testing of Materials”, Sub-Section 2017: “Test and Standard of Acceptance” and this Sub-Section. The Contractor’s testing Laboratory shall perform all acceptance sampling and testing necessary to determine conformity with the requirements specified in this Specification Section.

-	The Engineer will have the authority to direct locations for sampling and testing and will at all times have access to the Contractor’s Laboratory and testing equipment for observing and checking results.

-	Acceptance sampling and testing will be based on lot quantities. A lot will consist of 1500 m2 of pavement or one day’s placement (whichever is less).

-	Testing organizations performing these tests shall meet the requirements of ASTM C1077. The Contractor shall bear the cost of providing curing facilities for the strength specimens and coring and filling operations.

(b)	Tested Strength

The strength registered for tested samples shall satisfy the requirements of these Specifications for compressive and flexural strength.

(c)	Pavement Smoothness

-	As soon as the concrete has hardened sufficiently, the pavement surface shall be tested with a 5m straightedge or other specified device. Surface smoothness deviations shall not exceed 6 mm from a 5m straightedge placed in any direction, including placement along and spanning any pavement joint edge.

-	Areas in a slab showing high spots of more than 6 mm but not exceeding 13 mm in 5 m shall be marked and immediately ground down with an approved grinding machine to an elevation that will fall within the tolerance of ±6 mm. Where the departure from correct cross section exceeds 13 mm, the pavement shall be removed and replaced at the expense of the Contractor when so directed by the Engineer.

(d)	Tie and Dowel Bars Alignment:

Tie and dowel bars and assemblies shall be checked for position and alignment. The maximum permissible tolerance on bar alignment in each plane, horizontal and vertical, shall not exceed 2 percent or 20mm per meter of the tie or dowel bar.

(13)	Trial length

The Contractor shall submit to the Engineer at least one month prior to the date proposed for the initial trial length, a detailed description of the plant, equipment and method of construction.

The Contractor shall demonstrate the plant, equipment and method of construction by laying an initial trial length of 150m long at a location provided by the Contractor outside the permanent works, or where approved by the Engineer. Subsequent trial lengths may be instructed by the Engineer if any aspect of the initial trial proves unsatisfactory.

The Contractor shall not start the works for pavement construction if comprehensive trial has not been approved the Engineer.

(14)	Settlement Risks for Upper Layers (Sub-base, Base Course and Concrete Pavement)

The Contractor shall be responsible for all risks for leveling and proper compaction of the sub-base, base course and concrete pavement to the level shown in the Drawings. Any regulation of the final surface to the level of sub-base course, base course and/or pavement due to settlement shall be made on the Contractor’s own expenses.

(15)	Construction Method for the Reinforced Concrete Pavement (RCP)

The construction of Reinforced Concrete Pavement (RCP) shall conform all the requirements herein specified for CRCP and JRCP pavements.

2054	MEASUREMENT

Concrete pavement (CRCP: Continuously Reinforced Concrete Pavement, JRCP: Jointed Reinforced Concrete Pavement and RCP: Reinforced Concrete Pavement) shall be measured by the square metres of the completed pavements at respective thicknesses in accordance with the Drawings, and as directed by the Engineer.

Terminal Anchors for CRCP shall be measured by the number of the completed terminal anchors in accordance with the Drawings, and as directed by the Engineer. Terminal Anchors deemed include the construction of two concrete approach slabs, three transversal joints, all required steel reinforcement, tie and dowel-bars, respective chairs and steel supports, and all related incidental works.

Measurement shall be made only after all procedure and requirements for testing and acceptance are satisfied and the results are approved by the Engineer.

The steel reinforcement for pavements CRCP and JRCP type shall not be measured nor paid separately but shall be deemed included in the rates and unit price established for the main item of pavement CRCP and JRCP.

The wire mesh for RCP (Reinforced Concrete Pavement) shall not be measured nor paid separately but shall be deemed included in the rates and unit price established for the main item of RCP.

Therefore, Clause 2018(4): “Reinforcement” for measurement of reinforcement doesn’t applies to any type of concrete pavements.

2055	PAYMENT

Concrete pavement (CRCP: Continuously Reinforced Concrete Pavement, JRCP: Jointed Reinforced Concrete Pavement and RCP: Reinforced Concrete Pavement) measured as above shall be paid at the contract unit rate which shall be the full and the final compensation to the Contractor as per Clause 112 including cost for all materials labour, equipment, tools, equipment, plants and all incidentals to complete the work in accordance with the Drawings and Specifications, and as directed by the Engineer.

The concrete pavement type JCP [t=150 mm], Concrete Class-D,  used in tunnel construction for “Inspection Path with Kerbs” and “Emergency Walkway with Handrails” shall meet the requirements of these Specifications.  However, respective payment shall be made under the Special Provisions for tunnel’s technical specifications Section 4800: “Drainage System, Walkway & Path”, Item 4800-4: “Inspection Path with Kerbs” and Item 4800-5: “Emergency Walkway with Handrails”.

The unit rates shall deemed include, inter alia and in addition to other items previously stated (e.g. concrete, etc.), the following:

      - preliminary trials and trial sections

      - preliminary trial lengths not accepted as part of permanent works

      - trials by the Contractor to demonstrate compaction methods

      - supplying, laying and compaction of concrete

      - forms

      - protection of mixed materials in transit and while waiting tipping

      - grading, measuring, mixing and depositing materials

      - masking and unmasking of exposed concrete surfaces, etc.

      - jointing works

      - curing and protection of materials

      - maintenance of surface and cleaning or removal and replacement of contaminated layers

      - making good up to design surface level

      - sealing surfaces

      - anchorages necessary for joints

      - all necessary tools and material for surface finishing

      - longitudinal and transversal reinforcement for CRCP and JRCP

      - longitudinal construction joints for CRCP including tie bars, chairs, additional bars for supporting, etc.

      - transversal construction joints for CRCP including tie bars, additional bars for load transfer, etc.

      - transversal joints for JRCP including all dowels and material for stable installation

      - wire mesh for RCP

      - all expenses for material and placing of supplementary construction joints (i.e. joints that are not shown in the drawings, etc.), but approved by the Engineer upon the appropriated Contractor’s proposal and supplementary drawings.

Therefore, Clause 2019(3): “Reinforcement” for payment of reinforcement doesn’t applies to any type of concrete pavements and separate payment for reinforcement shall not be made, but shall be deemed included in the main items listed below:

| Item No. | Description | Unit |
| --- | --- | --- |
| 2050 | Concrete Pavements |  |
| 2050-1 | Continuously Reinforced Concrete Pavement (CRCP) [t=300 mm] (Concrete Class P) | sq.m. |
| 2050-2 | Terminal Anchors for CRCP (Concrete Class P) | No. |
| 2050-3 | Jointed Reinforced Concrete Pavement (JRCP) [t=300 mm] (Concrete Class P) | sq.m. |
| 2050-4 | Jointed Reinforced Concrete Pavement (JRCP) [t=200 mm] (Concrete Class-P) | sq.m. |

2060	CEMENT TREATED BASE COURSE (CTBC)

(1)	General

The work provided shall include the furnishing, placement and compaction of one course of plant mixed Portland Cement Treated Base Course (CTBC) to the lines, grades, dimensions, moisture, density and typical sections as specified in the Drawings and in accordance with the specifications, and as directed by the Engineer. The Contractor shall be solely responsible for the CTBC either batched at and/or delivered to the Site. A design mix for CTBC shall be certified in accordance with the requirements of these specifications.

Each design mix submitted and authorized for use under this specification shall be identified by a number, unique to that design mix and aggregate production plant/pit. If a change in material(s) from that specified in the design  mix occur during a project, the Contractor shall submit a new design mix to include the changed materials for authorization by the Engineer. A design mix shall not be used on the Project without authorization by the Engineer.

(2)	References

The following standards of American Society for Testing and Materials (ASTM), (Latest Edition) (or equivalent subjected to the Engineer’s approval) shall be applied:

- C136	Standard Test Method for Sieve Analysis of Fine and Course Aggregates

- C150	Standard Specifications for Portland Cement

- D75	Standard Practice for Sampling Aggregates

- D559	Standard Test Methods for Wetting and Drying Compacted Soil-Cement Mixtures

- D1632	Standard Practice for Making and Curing Soil-Cement Compression and Flexure Test Specimens in the Laboratory

- D1633	Standard Test Method for Compressive Strength of Molded Soil-Cement Cylinders

- D2419	Standard Test Method for Sand Equivalent Value of Soils and Fine Aggregate

- D2922	Standard Test Methods for Density of Soil and Soil-Aggregate in Place by Nuclear Methods

- D2940	Standard Specification for Graded Aggregate Material for Bases or Subbases for Highways or Airports

- D3017	Standard Test Method for Water Content of Soil and Rock in Place by Nuclear Methods

- D4318	Standard Test Methods for Liquid Limit, Plastic Limit, and Plasticity Index of Soils

(3)	Materials

Portland cement to be used or furnished under this specification shall comply either with the requirements of ASTM C150, Type II, Low Alkali (LA) cement, or as specified herein, or as specified in the Drawings, or as approved by the Engineer. The Contractor shall submit certification of compliance signed by the cement manufacturer, identifying the cement Type and source (plant location), stating the Portland cement used in the cement treated base delivered to the project complies with this specification. If required, certification of the Portland cement used for each day's concrete placement, shall be submitted to the Engineer. Portland cement specified in an authorized design mix shall be of the same source and type for all cement treated base under that design mix identification number, as specified in the design mix.

Aggregates for CTBC shall consist of a combination of aggregates and sand prepared for a Base Course by crushed rock obtained from tunnel excavation conforming to the requirements of Section 1200: “Subbase, Base, Hard Shoulder and Gravel Wearing Course”, and Portland cement,

Water used for preparation of CTBC shall be clean and free from injurious amounts of oil, acids, alkalis, salts, organic materials, or other deleterious  substances. Non-potable water shall not be used unless the requirements of ACI 318.3.4.3.2 are met.

Prime coat for surface sealing of compacted Cement Treated Base Course (CTBC) shall comply with the requirements of Section 1300: “Bituminous Surface and Base Course”.

(4)	Proportioning

The Contractor shall be solely responsible for the CTB design mix proportions and material batched and delivered to the site.

A CTB design mix shall be a blend of Portland Cement, aggregate base course and water. The design mix shall be prepared in a laboratory under the supervision by the Engineer.

- M	Optimum moisture maximum dry density relationship and graph.

- N	Compressive strength test results, average of three briquettes (each), at 2 days, 7 days, and 28 days

Portland cement shall be proportioned by percent of dry weight of aggregates. The amount of  Portland cement shall be the minimum cement content that will provide an aggregate cement water mixture, when compacted at optimum moisture, as determined  by ASTM D558, and that will comply with the following requirements:

Table 20-4 Cement Treated Base Properties

| Description | Requirement |
| --- | --- |
| Weight Loss After 12 Cycles: Wetting and Drying Tests (ASTM D559) | 15% max. |
| Compressive Strength (ASTM D558; ASTM D1632; ASTM D1633) | min. 5.2 MPa at 7 days |

(5)	Mixing

(a)	Batching

When the CTB is proportioned and mixed in a central mixing plant, either weigh batching or volume batching; the plant shall be equipped with feeding and metering devices which will introduce the aggregates, cement and water into the mixer in the proportions specified in the authorized design mix. The plant shall be calibrated at the particular configuration, to include but not be limited to scales, belt speeds, gate settings, dispenser rates and mixing time, to proportion a specified mix. The production configuration required for a design mix shall be maintained on file at the plant for review by the Engineer. Certification by the supplier that the required configuration was used in the production of cement treated base for each days production shall be maintained at the batch plant for review by the Engineer. Batch weights of materials and/or daily production weights will be recorded at the batch plant and maintained on file for review by the Engineer.

(b)	Mixing In-Place Method

- Equipment for Construction:

CTBC should be constructed by mix-in-place method of construction or as otherwise approved by the Engineer.

Manual mixing may be permitted only where the width of laying is not adequate for mechanical operations, as in small-sized jobs or corners.

The equipment used for mix-in-place construction shall be a rotavator/road bed aggregate mixer mounted to a tractor, or similar, approved equipment capable of pulverizing and mixing the granular material with additive and water to specified degree to the full thickness of the layer being processed, and of achieving the desired degree of mixing and uniformity of the stabilized material. If so instructed by the Engineer, trial runs with the equipment shall be carried out to establish its suitability for work. The equipment (and the number of machines for pulverizing and mixing) provided for these works shall have a capacity to perform as minimum a total length of 300m/day per lane.

The thickness of the layer to be stabilized shall be not less than 150mm after compaction.

- Mix-in-Place Method of Construction:

Before deploying the equipment, the granular material after full process is completed in the Crashing-Plants shall be spread uniformly on the prepared subbase/subgrade in a quantity sufficient to achieve the desired compacted thickness of the stabilized layer. Where single-pass equipment is to be employed, the granular soil shall be lightly rolled at the discretion of the Engineer.

The equipment used shall either be of single-pass or multiple-pass type. The mixers shall be equipped with an appropriate device for controlling the depth of processing and the mixing blades shall be maintained or reset periodically so that the correct depth of mixing is obtained as all times.

With single-pass equipment the forward speed of the machine shall be so selected in relation to the rotor speed that the required degree of mixing, pulverization and depth of processing is obtained. In multiple-pass processing, the prepared base layer shall be pulverized to the required depth with successive passes of the equipment and the moisture content adjusted to be within prescribed limits mentioned hereinafter. The blending or stabilizing material shall then be spread uniformly and mixing continued with successive passes until the required depth and uniformity of processing have been obtained.

(6)	Placement and Compaction

(a)	Batching Construction Method

Material shall be transported in suitable vehicles with a cover. Loads of material shall be covered immediately after loading and remain covered until unloading.

The Contractor shall provide to the  Engineer with each load of material batched and/or  delivered to the job site, before unloading at the site, a copy of a delivery ticket on which is printed, stamped or written, all the information of each batch.

Material shall be placed on prepared subgrade, prepared in accordance with the requirements of Section 1000: “Subgrades”, in lifts to provide a compacted thickness of not less than 100mm and not more than 150mm, to the required limits and sections specified in the Drawings and specifications or as authorized by the Engineer.

Compaction of the material shall be initiated within sixty (60) minutes from the time of mixing. The material shall be compacted to an equivalent dry density greater than ninety eight (98) percent of maximum dry density, at a moisture content range of optimum moisture to optimum moisture plus four (4) percent, as determined in accordance with ASTM D558. Compaction shall be completed within three (3) hours from the time of mixing as directed by the Engineer.

Upon completion of compaction, the surface of the compacted cement treated base shall be sealed with a prime coat as instructed in Section 1300: “Bituminous Surface and Base Course”, and as approved by the Engineer. The prime coat shall be applied as required to provide a uniform coverage of the surface. Application shall be between 0.05 and 0.15 gallons per square yard of surface. If final surfacing is placed within 24 hours after completion of compaction, the prime coat may be waived as authorized by the Engineer. The surface shall be kept at compaction moisture until the next lift of material is placed in the event the prime coat is waived.

CTB shall not be placed on  frozen subgrade or subgrade with a surface temperature less than 4°C. The material shall not be placed when the ambient temperature is less than 4°C.

Cement treated base shall be protected from freezing for a minimum of 7 days after placement. The material shall be cured during 7 days before opening to traffic, as approved by the Engineer.

(b)	Mix-in-Place Construction Method

The moisture content shall not be less than the optimum moisture content.

Immediately after spreading, grading and levelling of the mixed material, compaction shall be carried out with approved equipment preceded by a few passes of lighter rollers if necessary. Rolling shall commence at edges and progress towards the centre, except at super-elevated portions where it shall commence at the inner edge and progress towards outer edge.

During rolling the surface shall be frequently checked for grade and cross-fall (camber) and any irregularities corrected by loosening the material and removing/adding fresh material. Compaction shall continue until the density achieved is at least ninety eight (98) per cent of the maximum dry density for the material.

Care shall be taken to see that the compaction of cement stabilized material is completed within two hours of its mixing or such shorter period as may be found necessary in dry weather.

The base course shall be suitably cured for 7 days. Subsequent pavement course shall be laid soon after to prevent the surface from drying out and becoming friable. No traffic of any kind shall ply over unless permitted by the Engineer.

(5)	MEASUREMENT

Cement Treated Base Course (CTBC) shall be measured by the square metres of the completed base course of 150mm thicknesses in accordance with the Drawings, and as directed by the Engineer.

Measurement shall be made only after all procedure and requirements for testing and acceptance are satisfied and the results are approved by the Engineer.

Measurement of the Prime Coat shall be made in square metres as required in Section 1300: “Bituminous Surface and Base Course”.

(6)	PAYMENT

The CTBC measured as above shall be paid at the contract unit rate which shall be the full and the final compensation to the Contractor as per Clause 112 including cost for all materials labour, equipment, tools, and all incidentals to complete the work in accordance with the Drawings and Specifications, and as directed by the Engineer.

The unit rate input into the BOQ shall be for the Mix-in-Place construction method. However, the Contractor should be free to use the method of Batching Plant for expediting time and securing the quality.  Compensation for the differences in price shall not be considered.

Payment of the Prime Coat, as measured above, shall be paid under Section 1300: “Bituminous Surface and Base Course”.

| Item No. | Description | Unit |
| --- | --- | --- |
| 2060 | Cement Treated Base Course (CTBC) |  |
| 2060-1 | Mix-in-Place Cement Treated Base Course [t=150 mm] (for tunnel) | sq.m. |

SECTION 2400: MISCELLANEOUS STRUCTURES

2401	GABIONS

(5)	Measurement

(1)	Delete the full text of Clause 2401(5): “Measurement” and replace with the following:

Gabions and/or mattresses shall be measured on the basis of number of cubic metres of completed gabions in place satisfy all tolerances for boxes and mattresses physically measured, and are verified and approved by the Engineer.

Stone filling in gabions including fixing of gabion in position, tying with binding wires and tension wires as specified, and the binding wires, selvedge wire and tension wires, diaphragms every 1.00m, gabion ends, etc. shall not be measured nor paid separately; they shall be deemed included in the measurement of the gabion and/or mattresses itself.

The following gabions will be used in the Project in accordance with these specifications:

| Item | Mesh Type | Box Height (m) | Wire Type | Mesh Wire Diam. (mm) | Edge/ Selvage Wire (mm) | Lancing Wire Diam. (mm) |
| --- | --- | --- | --- | --- | --- | --- |
| Gabion (H=1.0m) | 10x12 Manually or mechanically woven. Double twisted hexagonal mesh | 1.00 | Heavy Zn Coated GI | 3.00 | 3.9 | 2.40 |
| Gabion (H=0.5m) |  | 0.50 |  | 3.00 | 3.9 | 2.40 |
| Reno Mattress (W : H) |  | H |  | 3.00 | 3.9 | 2.40 |

Geosynthetics used for gabion construction, shall be performed, measured and paid, respectively in accordance with the relevant Sub-Sections of these specifications such as:

- 2407 “Geosynthetics for Road and Bridges Works”

- 2408 “Geotextile for Filtration, Drainage and Separation”

- 2409 “Geogrid”

- 2410 “Geo-composite drains”, etc.

As applicable, and as approved by the Engineer.

(6)	Payment

(2)	Delete the full text of Clause 2401(6): “Payment” and replace with the following:

Gabion boxes and gabion mattresses shall be paid as per respective contract unit rate which shall be the full and the final compensation to the Contractor as per Clause 112 to complete the work in accordance with these Specifications which shall include the costs for furnishing and producing materials, transporting, placing, stone filling, etc. and for furnishing all labour, tests, tools, equipment and any incidentals to complete the work as shown in the Drawings, as required by these Specifications, and/or as directed by the Engineer.

The unit prices established for these items will be applied regardless the mesh has been woven manually or mechanically.

| Item No. | Description | Unit |
| --- | --- | --- |
| 2400 | Miscellaneous Structures |  |
| 2401 | Gabions |  |
| 2401-1 | Gabion (box height H=1.0m) | cu.m. |
| 2401-2 | Gabion (box height H=0.5m) | cu.m. |

2402	CRIB WALLS

(3)	Rename the Sub-Section 2402: “Crib Walls” as follows, delete the full text and replace with the following:

2402	CAST-IN-PLACE CRIB WORKS

(1)	Description

These works shall be carried out in accordance with the requirements of these specifications and to the lines, grades, dimensions and other particulars shown on the Drawings or as directed by the Engineer.

This work shall consist of the construction of cast-in-place concrete crib and filling the cavities between the crib frames as described in this section.

(2)	Materials

(a)	Concrete:

The concrete for the frames shall be in accordance with the relevant requirements of Section 2000: “Concrete for Structures”, Concrete Class C, of these Specifications, and as directed by the Engineer. The concrete for the foundations shall be of Class-D.

(b)	Reinforcement and Anchor Bars

Reinforcement bars for concrete and the anchor bars for slope protection shall be in accordance with the relevant requirements of Sub-Section 2014: “Reinforcement” of these Specifications, and as directed by the Engineer.

(c)	Wire Mesh

Wire mesh shall meet the relevant requirements of the Standard Specifications, as shown in the drawings and as approved by the Engineer. In addition, the following basic requirements shall be satisfied:

| Item | Requirement |
| --- | --- |
| Wire diameter | φ2.00mm |
| Reticulation | 50mm |
| Tension strength | 500N/m2 |
| Galvanization volume | More than 23g/m2 |

(d)	Filling Material

Depending on the slope conditions, the cavities in between the crib frames shall be filled with random boulders or cobbles, or with appropriate seeded soil, or seeded jute bags provided with weep-holes with Polyvinyl Chloride (PVC) pipes, as shown in the Drawings and as directed by the Engineer.

(2)	Construction Method

(a)	Preparation of Slope Surface

All loose rock, sharp protruding edges as well as dirt, grease, oil scale and other contamination shall be carefully removed.  Any seepage of water entering locally shall be dealt with suitable methods approved by the Engineer, such as local sealing, drainage, etc.  Weep holes with PVC pipes of 40 mm diameter shall be provided in random boulder set in concrete.

(b)	Foundation

Prior to placing the foundation, the Contractor shall report the bedding conditions and obtain the direction of the Engineer.

(c)	Crib Works

The concrete crib works shall be constructed complying duly the requirements of Section 2000: “Concrete for Structures” and Sub-Section 2014: “Reinforcement” of these Specifications, as shown in the Drawings and as directed by the Engineer.

The Contractor shall drill holes on the rock surface and install anchor bars as shown in the Drawings or as directed by the Engineer. The Contractor shall use suitable machines depending on geological conditions. The machines shall have enough ability to drill to the required depth to form holes of the required diameter.

(d)	Filling of the Cavities in Between the Crib Frames

The cavities in between the crib frames shall be filled as follows:

- Type-1: Crib Frames installed on a wet slope “With Spring” water

After installation of the wire  mesh, the cavities shall be filled with boulder stones set in a concrete Class D. The stones shall satisfy the requirements for material specified in Sub-Clause 2401(2)(a): “Stones”, or cobbles, as approved by the Engineer.

- Type-2: Crib Frames installed on a dry slope “Without Spring” water

After installation of the wire  mesh, the cavities shall be filled with appropriate seeded soil, or seeded jute bags (vegetation bags) as shown in the Drawings.  The jute bags shall be filled with suitable soil for vegetation and sowed with approved grass or shrub seeds complying the relevant requirements of Section 2800: “Bio Engineering Works”, as directed by the Engineer.  The soil shall be placed and compacted to the specified thickness containing grass and/or shrub seeds as directed by the Engineer.

(3)	Measurement and Payment

Measurement of this item shall be in square meters based on the Drawings on which field survey data for measurement purposes are plotted.

The Cast-in-Place Crib Works shall be paid as per respective contract unit rate which shall be the full and the final compensation to the Contractor as per Clause 112 to complete the work in accordance with these Specifications which shall include the costs for furnishing and producing materials, transporting, placing, boulders or cobbles, vegetation bags, etc. and for furnishing all labour, tests, tools, equipment and any incidentals to complete the work as shown in the Drawings, as required by these Specifications, and/or as directed by the Engineer.

The works and materials for filling the cavities between crib frames with vegetation bags (or sowing) shall not be measured nor paid separately, and shall not be included in the pay items of Section 2800: “Bio Engineering Works”.

| Item No. | Description | Unit |
| --- | --- | --- |
| 2402 | Cast-in-Place Crib Works |  |
| 2402-1 | Cast-in-Place Crib Work Type-A (with Cobble) | sq.m. |
| 2402-2 | Cast-in-Place Crib Work Type-B (with Boulder mixed with concrete) | sq.m. |
| 2402-3 | Cast-in-Place Crib Work Type-C (with Vegetation bags) | sq.m. |

2404	SUB-SURFACE DRAINS

(5)	Measurement

(4)	Delete the full text of Clauses 2404(5): “Measurement” and 2404(6): “Payment” and replace with the following:

(5)	Measurement

Unless otherwise specified, Sub-Surface Drains shall be measured shall be measured on the basis of the number of lineal metres of its actual length for the works completed in place as per planned dimensions as shown in the Drawings.

Overlapping at joints shall not be measured nor paid separately.

Excavation, back fill material, bedding or cover material, and all other material shall not be measured separately for payment.

(6)	Payment

The following items for Sub-Surface Drains shall be paid under this Section and shall include all the works for providing and installing the several types of Sub-Surface Drains, HDPE perforated pipes, geosynthetics, all materials, equipment and tools, and all incidentals used to complete the work in accordance with the Drawings and Specifications, and as directed by the Engineer:

| Item No. | Description | Unit |
| --- | --- | --- |
| 2404 | Sub-Surface Drains |  |
| 2404-1 | Sub-surface drain 600x750 with perforated HDPE 350mm | lin.m. |
| 2404-2 | Sub-surface drain 1400x750 with two cell perforated HDPE 350mm | lin.m. |
| 2404-3 | Sub-surface drain 500x400 with perforated HDPE 150mm | lin.m. |
| 2404-4 | Sub-surface drain 620/350 x 450 with perforated HDPE 150mm | lin.m. |
| 2404-7 | Sub-surface drainage filter (9.5mm to 19mm well graded aggregate) back side reinforced soil wall type-3 concrete panel | cu.m |
| 2404-8 | Sub-surface drain by perforated PVC 160mm wrapped with geotextile | lin.m |

(5)	Add the following new Sub-Sections at the end of Sub-Section 2406: “Scour Protection and River Training Works” and before Section 2500: “Brickworks for Structures”:

New Sub-Sections:

- 2407 “Geosynthetics for Road and Bridges Works”

- 2408 “Geotextile for Filtration, Drainage and Separation”

- 2409 “Geogrid”

- 2410 “Geo-composite drains”

- 2411 “Riverbed Protection”

- 2412 “Shotcrete for Slope Protection”

- 2413 “Dry Stone Packing and Soling”

2407	GEOSYNTHETICS FOR ROAD AND BRIDGES WORKS

(1)	Scope

The specification covers the various applications of geosynthetic materials for use in road and bridge works including supplying and laying as per contract specifications.

(2)	Terminology

Geosynthetic is a general classification for all synthetic materials used in geotechnical engineering application. It includes geotextile, geogrids, ego-trips, geomembrane, geonets, geocomposites, geocells, geosynthetic mats, paving fabric and glass grid etc. Textiles made from natural fibers such as jute, and coir referred to herein under natural Geotextiles may also be used in different geotechnical engineering applications.

(a)	Geotextile: Any permeable synthetic textile used with foundation, soil, rock, earth, or any other geotechnical engineering-related material as an integral part of a human-made project, structure, or system.

The geotextile fabric shall be a woven or non-woven or knitted fabric consisting of long-chain polymeric filaments or yarns such as polypropylene, polyethylene or polyester or any combination thereof, formed into a stable network such that the filaments or yarns retain their relative position to each other.

There are several application areas for Geotextile requiring specific functions namely separation, filtration, drainage, reinforcement, protection or a combination thereof.

(b)	Geogrids: A deformed or non-deformed netlike polymeric material used with foundation, soil, rock, earth, or any other geotechnical engineering-related material as an integral part of human-made project, structure, or system.

Geogrids have relatively high strength, high modulus, and low-creep-sensitive polymers with apertures varying from 10 to 100 mm in size or more. The openings/holes in geogrids are either elongated ellipse, near squares with rounded corners, squares or rectangles. Geogrids can be of uni-axial grid, bi-axial grid or three dimensional grids. Geostrip is another form of Geogrid, which is used in reinforced soil structures. It is primarily made of synthetic material in strips and is made from high tenacity polyester yarn and contained in a suitable low density polyethyelen sheath.

These are used as reinforcement in pavements and reinforced soil walls and slopes.

(c)	Geomembrane: An essentially impermeable membrane (liner or barrier) used with foundation, soil, rock, earth, or in any other geotechnical application as an integral part of human-made project, structure, or system, used to control fluid migration.

Geomembrane are mostly made from HDPE, LDPE, PVC or polyethylene sheets, which are duly protected from ultraviolet exposure by carbon black or any antioxidants and thermal stabilizers.

Geo-clay-liners are also, in most cases, consider as geo membrane due to their low permeability performances and can be typically used as an alternative to Polymeric Geo-membrane under specific circumstances. These are used as capillary cut off in roads in water logged areas.

(d)	Geosynthetic Clay Liner: Geosynthetic clay liners (GCLs) are geocomposites that are prefabricated with a bentonite clay layer typically incorporated between a top and bottom geotextile layer or bonded to a geomembrane or single layer of geotextile. Geotextile-encased GCLs are often stitched or needle-punched through the bentonite core to increase internal shear resistance. When hydrated they are effective as a barrier for liquid or gas and are commonly used in landfill liner applications often in conjunction with a geomembrane.

(e)	Geonets: Geonets are used in combination with other types of geosynthetics. These are usually formed by continuous polymeric ribs or filament at acute angle to one another. When the ribs are opened relatively large size apertures are formed in a net like configuration

These are typically used in combination with other geosynthetic materials to form a composite material.

(f)	Geocomposite: A manufactured material, which could be a combination of any two or more synthetic materials like Geotextile, geogrids, geomats, geonets and geomembrane etc., in laminated or composite form. One of the popular forms of geocomposite is Drainage Composite. Drainage Composites are formed by combining geotextile or geomembrane with a core of geonet, geomat or seratted/corrugated polymeric materials.

Prefabricated Vertical Drains (PVD)/Band Drains and Fin Drains come under the category of geocomposites

(g)	Geocell: It is a three dimensional structure with interconnected cells. The geocells are made of polyester/polypropylene/high density polyethylene stabilized with carbon black.

Geocells may be used in for soil containment purpose.

(h)	Geosynthetic Mats: These are two dimensional or three dimensional mats with specified thickness, made of multi-filaments, with apertures to allow vegetation growth for erosion control application. Geosynthetic mat consists of UV stabilized non-degradable polypropylene/polyethylene or similar polymer fibers that are extruded or heat bonded to provide a dimensionally stable matrix. A tension element like steel wire mesh or Geogrid shall be included in these mats as reinforcement, where these mats are required to posses more strength against erosive forces, like in steep slopes or in heavy rainfall areas.

These are used for erosion protection of slopes or when reinforced with geogrids or woven double twist wire mesh as reinforcement materials in soil veneer applications.

(i)	Natural Geotextile: These geotextiles are made of natural fibres like jute or coir. The blankets/mats/mesh made of these fibres are sometimes further reinforced with polymeric nettings to enhance its tensile strength and for holding the fibres intact. The polymer netting is securely stitched on both sides of the fabric to form a strong quilted mat. These fabrics have excellent drapability and aid in quick growth of vegetation and are used for erosion control applications.

(j)	Paving Fabric and Glass Grids: The paving fabrics are non-woven heat set material, consisting of at least 85% by weight of polyolefin, polyester or polyamides. They are heat bonded only on one side.

Glass grids are either a composite glass fiber reinforced Geogrid with continuous filament nonwoven geotextile chemically /mechanically bonded to the grid, or bituminous coated glass fibre geogrids with or without adhesive on one side of the grid.

The glass grids and composite of fabric and glass grids are used in bituminous pavements to act as stress relieving membrane and crack retarding layer within the pavement structure. The paving fabric also serves the function of water barrier.

(3)	Marking

Geosynthetic rolls shall be marked with the following information:

(a)	Manufacturer's name

(b)	Roll number

(c)	Grade

(d)	Length

(e)	Date of manufacture; and

(f)	Product identification details

(4)	Packing, Storage and Handling

(a)	Each geosynthetic roll shall be wrapped with a material that will protect the geosynthetic from damage due to shipment, water, sunlight and contaminants. The protective wrapping with a tarpaulin or opaque plastic sheet shall be maintained during periods of shipment and storage.

During storage, geosynthetic rolls shall be elevated off the ground and adequately covered to protect from site construction damage, precipitation, prolonged ultra-violet radiation including sunlight, chemicals that are strong acids or strong bases, flames including welding sparks, temperatures in excess of 71°C, and any other environmental condition that may damage the physical properties of the geosynthetic.

(b)	If the outer layer of the geosynthetic is damaged, or exposed to sunlight for a period beyond that is permitted the outermost wrap of the rolls shall be discarded, and only the remaining undamaged/unexposed material shall be used. If the geosynthetic rolls become wet, the water proof cover shall be removed, the rolls shall be elevated off the ground and exposed to wind in order to dry the fabric. The paving fabric used with bitumen overlays shall be completely dry prior to installation.

(5)	Testing and Acceptance

(a)	Geosynthetic Materials Shall is Tested and Certified in the following Manner.

  - The manufacturer shall have ISO or CE certification for manufacturing process and quality control.

  - The manufacturer shall provide manufacturer’s test certificate for every lot supplied from the factory.

  - The supplier shall provide third party test reports from an independent laboratory with valid accreditation for all the test values in Manufacturer’s test certificate

  - Geosynthetic shall be tested in accordance with tests prescribed by BIS. In absence of NS codes, tests prescribed either by IS, ASTM, EN, or ISO shall be conducted.

  - The material shall meet the requirements as specified in the contract.

2408	GEOTEXTILE FOR FILTRATION, DRAINAGE AND SEPARATION

(1)	Scope

The work covers the use of geotextile materials for drainage, separation/filtration and erosion control works including supplying and laying as per design, drawing and these specifications.

For drainage/filtration function, geotextile shall be able to convey water across the plane of the fabric throughout its design life.

For separation function the geotextile shall prevent intermixing of two layers of dissimilar materials, throughout the design life of the structure.

The geotextile as a filter material below erosion control measures like stone pitching or stone filled mattresses over the slopes, shall allow the water to flow out and at the same time prevent the loss of soil under the protective measures.

(2)	Material

(a)	Strength Requirement

The minimum strength of geotextile in terms of MARV under different installation conditions shall be as specified in Table 24-5.

Table 24-5: Minimum Geotextile Strength Property Requirements

| Installation Conditions | Type | Strength Property Requirement (MARV) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Grab Strength in Newton (N) as per IS: 13162, Part 5 |  | Tear Strength in Newton (N) as per IS: 14293 |  | Puncture Strength in Newton (N) as per IS: 13162, Part 5 |  | Burst Strength in Newton (N) as per IS: 1966 |  |
|  |  | Elongation at Failure |  |  |  |  |  |  |  |
|  |  | <50% | >50% | <50% | >50% | <50% | >50% | <50% | >50% |
| Hard | I | 1400 | 900 | 500 | 350 | 500 | 350 | 3500 | 1700 |
| Moderate | II | 1100 | 700 | 400 | 250 | 400 | 250 | 2700 | 1300 |
| Less Severe | III | 800 | 500 | 300 | 180 | 300 | 180 | 2100 | 950 |

Note:

(1)	All numeric values in the above table represent Minimum Average Roll Value (MARV) in weaker principal direction. The MARV is derived statistically as the average value minus two standard deviations.

(2)	When the Geotextile are joined together by field sewing, the seam strength shall be at least 60 percent of the material’s tensile strength. All field seams shall be sewn with thread as strong as the material in the fabric.

(3)	The puncture strength if determined in accordance with ASTM D 6241, the minimum requirement in terms of “Newton (N)” shall be as follows:

| Installation Conditions | Strength Property Requirement (MARV) |  |
| --- | --- | --- |
|  | Puncture Strength in Newton (N) as per ASTM D6241 |  |
|  | Elongation at Failure |  |
|  | <50% | >50% |
| Hard | 2800 | 2000 |
| Moderate | 2250 | 1400 |
| Less Severe | 1700 | 1000 |

(b)	Ultraviolet Stability Requirements

The material shall conform the following requirements for ultraviolet stability:

Table: 24-6: Requirements for Ultra Violet Stability

| S.No. | Properties of Fabric | Requirement (Retained Strength) |
| --- | --- | --- |
| 1 | Grab Strength | Not less than 70% after 500 hours of exposure |
| 2 | Tear Strength |  |
| 3 | Puncture Strength |  |
| 4 | Burst Strength |  |

(c)	Hydraulic Requirements for various applications

            - Subsurface Drainage and filtration

The geotextile shall conform to the physical requirements specified in Table 24-7.

Table: 24-7: Geotextile Requirements for Subsurface Drainage

| In-situ passing 0.075 mm sieve (%) | Permittivity, per sec ASTM D4491 / IS 14324-1995 | Maximum Apparent opening size (mm) ASTM D4751 / IS 14294-1995 |
| --- | --- | --- |
| <15 | 0.5 | 0.43 |
| 15 to 50 | 0.2 | 0.25 |
| >50 | 0.1 | 0.22 |

The type of geotextile shall be decided by the Engineer depending upon the installation conditions.

            - Separation

The geotextile for different subgrade soil conditions shall conform to the requirements given in Table 24-8 and Table 24-9.

Table: 24-8: Geotextile Requirements for Separation (Subgrade Soaked CBR>3)

| Sl. No. | Geotextile Property | Requirement |
| --- | --- | --- |
| 1 | Permittivity as per ASTM D4491 | 0.02/sec |
| 2 | Maximum Apparent Opening Size as per ASTM D4751 | 0.60mm |

Table: 24-9: Geotextile Requirements for Separation (Subgrade Soaked CBR ≤3)

| Sl. No. | Geotextile Property | Requirement |
| --- | --- | --- |
| 1 | Permittivity as per ASTM D4491 | 0.05/sec |
| 2 | Maximum Apparent Opening Size as per ASTM D4751 | 0.43mm |

            - Erosion Control

The geotextile for erosion control shall conform to requirements of Table 24-10.

Table: 24-10: Geotextile Requirements for Erosion Control

| In-situ passing 0.075 mm sieve (%) | Permittivity, per sec ASTM D4491 | Maximum Apparent opening size (mm) ASTM D4751 |
| --- | --- | --- |
| <15 | 0.7 | 0.43 |
| 15 to 50 | 0.2 | 0.25 |
| >50 | 0.1 | 0.22 |

(3)	Construction

(a)	General Requirement

Exposure of Geotextile to the elements between lay down and cover shall be a maximum of 14 days to minimize damage potential.

In trenches, after placing the backfill material, the geotextile shall be folded over the top of the filter material to produce a minimum overlap of 300mm for trenches greater than 300mm wide. In trenches less than 300mm wide, the overlap shall be equal to the width of the trench. The geotextile shall then be covered with the subsequent course.

Overlap at roll ends and at adjacent sheets shall be a minimum of 450mm, except when placed under water. In such instances, the overlap shall be a minimum of 1 m. Where seams are required in the longitudinal trench direction, they shall be joined by either sewing or overlapping. All seams and overlaps shall be subject to the approval of the Engineer.

Care shall be taken during installation so as to avoid any damage to the geotextile. Damages, if any, during installation shall be repaired by placing a geotextile patch over the damaged area and extending it 1m beyond the perimeter of the tear or damage, or as approved by the Engineer.

(b)	Subsurface Drainage

Construction shall conform to Clause 2404 of these specifications.

(c)	Separation

After preparation of subgrade as per the specifications along the road alignment, geotextile shall be rolled out as indicated in the drawings. The entire roll shall be placed on the subgrade and unrolled as smoothly as possible. Wrinkles and folds in the fabric shall be removed by stretching as required.

Adjacent rolls of Geotextile shall be overlapped, sewn, or joined as required. For curves, the geotextile shall be folded or cut and overlapped in the direction of construction. Folds in the geotextile shall be stapled or pinned approximately 0.6m centre-to-centre. Before covering, the condition of the geotextile shall be checked for damage (i.e., holes, nips, tears, etc.) by the Engineer.

Before placing the first lift of granular sub-base on the geotextile, a trial stretch of 100m shall be laid as per roll width to establish a proper construction methodology of placing and compacting the sub-base in a manner that no damages are caused to the separation layer of geotextile.

(d)	Filter Layer Under Stone Pitching for Erosion Control

The geotextile shall be placed in intimate contact of soil ensuring slight tension, to avoid wrinkles or folds and shall be anchored on a properly shaped surface as indicated in drawings and approved by the Engineer. It shall be ensured that the placement of the overlying material be placed in such a manner that it does not tear/puncture the geotextile. Anchoring of the terminal ends of the geotextile shall be accomplished as per drawings through the use of key trenches or aprons at the crest and toe of slope.

The geotextile shall be placed with the machine direction parallel to the direction of water flow. Adjacent geotextile sheets shall be joined by either sewing or overlapping.

The pitching shall begin at the toe and proceed up the slope. Big sized boulders shall not be allowed to roll down the slope.

Any geotextile damaged shall be either replaced or repaired with a patch, as directed by the Engineer, at the cost of the contractor.

(4)	Measurement

The geotextile for separation and for filter layer shall be measured on the basis of the number of square metres of the works completed in place as per planned dimensions as shown in the Drawings. Overlapping at transverse and longitudinal joints shall not be measured nor paid separately. Excavation, back fill, bedding and cover material shall be measured separately as per relevant Clauses of the Contract.

(5)	Payment

Geotextiles for filtration, drainage, and separation shall be paid as per respective contract unit rate which shall be the full and the final compensation to the Contractor as per Clause 112 to complete the work in accordance with these Specifications which shall include the costs for furnishing and producing materials, transporting, placing, etc. and for furnishing all labour, tests, tools, equipment and any incidentals to complete the work as shown in the Drawings, as required by these Specifications, and/or as directed by the Engineer.

| Item No. | Description | Unit |
| --- | --- | --- |
| 2408 | Geotextiles for Filtration, Drainage, and Separation |  |
| 2408-1 | Geotextile sheets | sq.m. |

2409	GEOGRID

(1)	Scope

The work covers the use of geogrids in sub-base of pavement, erosion control of slopes, reinforced soil slopes and reinforced soil walls including supplying and laying as per design, drawing and these specifications.

The use of geogrids as a component for reinforced soil slopes and walls shall be as per Sub-Section 301: “Reinforced Soil”.

(2)	Materials

(a)	General

Geogrids shall be either made from high tenacity polyester yarn jointed at cross points by weaving or knitting or thermal bonding process with appropriate coating or from polypropylene or polyethylene or any other suitable polymeric material by an appropriate process. Geogrids manufactured by extrusion process are integrally jointed, mono or bi-directionally oriented or stretched meshes, in square, rectangular, hexagonal or oval mesh form. The geogrids manufactured by weaving/ knitting/ bonding process shall be formed into a stable network such that ribs, filaments or yarns retain their dimensional stability relative to each other including selvages.

(b)	Earth Reinforcement

Geogrid for earth reinforcement shall meet the requirement as per the design subject to the minimum requirements as given in Table 24.11.

Table: 24-11: Minimum Requirements for Geogrids

| Property | Test Method | Unit | Requirement |
| --- | --- | --- | --- |
| Stiffness at 0.5% strain | ISO 10319 | kN/m | ≥350; both in machine and cross- machine direction |
| Tensile strength@2% strain | ASTM D6637 | kN/m | ≥15 % of Tult; both in machine and cross-machine direction |
| Tensile strength@5% strain | ASTM D6637 | kN/m | ≥20 % of Tult; both in machine and cross-machine direction |
| Junction Efficiency for extruded geogrids | GRI-GG2-87or ASTM- WK 14256 | - | 90 % of rib ultimate tensile strength |
| Ultraviolet stability | ASTM D4355 | - | 70% after 500 hrs exposure |

Note:

1)	All numerical values in the Table represent MARV in the specified direction.

2)	All geogrids shall be placed along machine direction parallel to the centre line of roadway alignment.

(c)	Erosion Control

The Geogrid for erosion control application shall have the minimum tensile strength of 4kN/m, when tested as per ASTM D5035 (Minimum Average Roll Value in Machine Direction). The aperture opening size shall be minimum 20mm x 20mm and average grid thickness shall be minimum 1.0mm. Geogrid for erosion control application shall be UV stabilized. The Geogrid shall have ultraviolet stability of 70 percent after 500hrs exposure as per ASTM D4355.

(d)	Reinforced Soil Slopes and Walls

The strength and other requirements shall be as per Sub-Section 301: “Reinforced Soil”.

(3)	Installation and Construction Operations

(a)	Sub-base Reinforcement

Prior to laying of Geogrid, the surface shall be properly prepared, cleaned and dressed to the specified lines and levels as shown on the drawings.

The Geogrid shall be laid within the pavement structure as shown on the drawings.

Geogrid reinforcement shall be placed flat, pulled tight and held in position by pins or suitable means until the subsequent pavement layer is placed.

No vehicle shall be allowed on Geogrid unless it is covered by at least 150mm thick sub-base material.

(b)	Erosion Control

The Geogrid for erosion control applications shall be installed in accordance with the manufacturer’s recommendation and relevant requirements of these specifications.

(c)	Reinforced Slopes and Walls

The Geogrid for reinforced slopes and walls shall be installed in accordance with the

manufacturer’s recommendation and as per Section 300.

(4)	Measurement

The geogrids for reinforced soil shall be measured on the basis of the number of square metres of the works completed in place as per planned dimensions as shown in the Drawings. However, these quantities shall be included in the unit rates measured for the pay items of Section 300: “Soil Improvement”.

Overlapping at joints and anchoring at toe and crest of the slope shall not be measured nor paid separately. Excavation, back fill, bedding and cover material shall be measured separately as per relevant Clauses of the Specifications.

(5)	Payment

Geogrids for reinforced soil shall not be paid separately but shall be deemed included in unit rates of the respective pay items of Section 300: “Soil Improvement”, which shall be the full and the final compensation to the Contractor as per Clause 112 to complete the work in accordance with these Specifications which shall include the costs for furnishing and producing materials, transporting, placing, etc. and for furnishing all labour, tests, tools, equipment and any incidentals to complete the work as shown in the Drawings, as required by these Specifications, and/or as directed by the Engineer.

2410	GEO-COMPOSITE DRAINS

(1)	Scope

The work covers the use of geocomposite drainage system for construction of “Fin Drains”. These works shall be carried out in accordance with the requirements of these specifications and to the lines, grades, dimensions and other particulars shown on the Drawings or as directed by the Engineer.

Fin drains with plastic core shall be installed for affecting vertical and/or horizontal drainage.

(2)	Materials

Fin Drains shall be made of light weight, three dimensional high compressive strength polyethylene cores; and heat bonded or needle punched polypropylene/polyester geotextile provided on one side or both sides of the core as per the requirements. Geotextile used in drainage composite shall meet the requirements as specified in Table 24-12.

Table 24-12: Geotextile Requirements for Fin Drains

| In-situ soil passing 0.075 mm sieve (%) | Permittivity, per sec ASTM D 4491/ IS 14324 - 1995 | Maximum Apparent Opening Size, mm as per ASTM D 4751/IS 14294-1995 |
| --- | --- | --- |
| < 15 | 0.5 | 0.43 |
| 15 to 50 | 0.2 | 0.25 |
| > 50 | 0.1 | 0.22 |

The properties of the core material shall meet the requirements as indicated in Table 24-13.

Table 24-13: Properties for Fin Drain Composite

| Property |  | Test method | Units | Minimum Average Roll value |
| --- | --- | --- | --- | --- |
| Tensile strength |  | EN ISO 10319 | kN/m | 16 |
| CBR Puncture Resistance |  | EN ISO 12236 | N | 3000 |
| Mass per unit area |  | EN ISO 9864 | g/m2 | 650 |
| Thickness of Composite |  | EN ISO 9863 | mm | 4.5 |
| In-plane permeability | Hydraulic Gradient, i=1 at 100 kPa pressure | EN ISO 12958 | l/m | 0.55 |
|  | Hydraulic Gradient, i=1 at 200 kPa pressure |  |  | 0.45 |

(3)	Installation

The installation of fin drains shall be as per drawings. Where fin drains are assembled on site, the assembly area shall be clean and dry. No geotextile or core material shall be exposed to daylight (or any source of ultraviolet radiation) for a period exceeding 50 hours. Where fin drains are laid in a trench, the bottom of the trench shall be free of irregularities and shall be brought to the required level. Rock and other hard protrusions shall be removed and any excess cut in the trench bottom shall be filled and compacted back to the required grade with suitable excavated or imported material as directed by the Engineer. Fin drains shall be capable of being connected longitudinal or laterally into pipe systems or chambers for inflow and outflow purposes. Joints parallel to the direction of flow and any exposed edge shall be protected from the ingress of soil by a geotextile wrapping with a minimum overlap of 150mm or other measures as approved by the Engineer. The splicing of lengths of geotextile and minimum overlap shall be as per the drawing or as approved by the Engineer.

(4)	Measurement

The Geo-composite sub-surface drain 8mm x 300mm, as shown in the Drawings, shall be measured on the basis of the number of lineal metres of its length of the works actually completed in place and as approved by the Engineer.

Geo-composite planar drainage composed of drainage core material with one filtering non-woven geotextile, as shown in the Drawings, shall be measured in square metres of the actual covered surface, and as approved by the Engineer.

Overlapping at joints shall not be measured nor paid separately. Excavation, embankment, back fill, bedding or cover material shall be measured separately as per relevant Clauses of the Specifications.

The geocomposite measured for payment under this Section shall not be counted in any other sections such as Section 301: “Reinforced Soil”, etc.

(5)	Payment

Geo-composite, measured as described above, shall be paid as per respective contract unit rate which shall be the full and the final compensation to the Contractor as per Clause 112 to complete the work in accordance with these Specifications which shall include the costs for furnishing and producing materials, transporting, placing, etc. and for furnishing all labour, tests, tools, equipment and any incidentals to complete the work as shown in the Drawings, as required by these Specifications, and/or as directed by the Engineer.

| Item No. | Description | Unit |
| --- | --- | --- |
| 2410 | Geo-Composite Drains |  |
| 2410-1 | Geo-composite sub-surface drain 8mm x 300mm | lin.m |
| 2410-2 | Geo-composite planar drainage composed of a drainage core material with one filtering non-woven geotextile | sq.m |

2411	RIVERBED PROTECTION

(1)	Description

These works shall be carried out in accordance with the requirements of these specifications and to the lines, grades, dimensions and other particulars shown on the Drawings or as directed by the Engineer.

This work shall consist of the construction of the foot protection blocks installed in areas adjacent to culverts, piers and/or abutments, at outfalls of drainage, along the road side and elsewhere, where protection is required for ensuring safety of structures against damage by flood/flow of water as shown in the Drawing and as directed by the Engineer.

(2)	Materials and Construction Method

(a)	Stones and Boulders

The stones or boulders shall satisfy the requirements for material specified in Sub-Clause 2401(2)(a): “Stones”, and Section 613: “Stone, Aggregate, Sand and Fillers”, as approved by the Engineer.

(b)	Concrete

Concrete for the foot protection blocks shall be in accordance with the requirement of Section 2000: “Concrete for Structures” of these Specifications. Concrete Class-D.

(c)	Reinforcement Bar

Reinforcement bar shall be in conformity with Sub-Section 2014: “Reinforcement” of these Specifications.

(3)	Measurement

Measurement of the item for the concrete Foot Protection Blocks shall be in number of blocks of 11,000mm x 1,000 mm x 500 mm constructed as shown in the Drawings, in accordance to these Specifications, counted and accepted by the Engineer at Site.

Measurement of the item for the boulders for Foot Protection Blocks shall not be measured nor paid separately but deemed included in the rates and unit price established for the main item.

(4)	Payment

The works for Foot Protection Blocks shall be paid as per respective contract unit rate which shall be the full and the final compensation to the Contractor as per Clause 112 to complete the work in accordance with these Specifications which shall include the costs for furnishing and producing materials, transporting, placing, boulders or stones, concrete Class-D (M20) including reinforcement bars (connection bars and metal fitting), etc. and for furnishing all labour, tests, tools, equipment and any incidentals to complete the work as shown in the Drawings, as required by these Specifications, and/or as directed by the Engineer.

The concrete and reinforcement bars (connection bars and metal fitting) included in this section for payment shall not be included in the pay items of Section 2000: “Concrete for Structures” and Section 2014: “Reinforcement”. The reinforcement bars for connection bars and metal fitting shall be deemed included in the price for Foot Protection Blocks.

| Item No. | Description | Unit |
| --- | --- | --- |
| 2411 | Riverbed Protection |  |
| 2411-1 | Foot Protection Blocks (1,100mm x 1,000 mm x 500 mm) | No. |

2412	SHOTCRETE FOR SLOPE PROTECTION

(1)	Description

These works shall be carried out in accordance with the requirements of these specifications and to the lines, grades, dimensions and other particulars shown on the Drawings or as directed by the Engineer.

This work shall consist of the spraying of shotcrete on the prepared slope surfaces for strengthening the slope protection.

(2)	Materials

The type of shotcrete to be used in a particular location shall be determined by the Engineer. The required mix proportions of cement, aggregates and suitable admixture for each type shall be proposed by the Contractor and shall be approved by the Engineer. Minimum compressive strength at 28 days (cylinder specimen tested in accordance with AASHTO T22) shall not be less than 15 N/mm2 (Concrete Class D).

Shotcrete mixtures shall not be used prior to the Engineer’s approval. Mix proportions shall be varied as directed to maintain minimum rebound.

(a)	Cement

The range of cement content in the shotcrete shall be from 320 kg/m3 to 400 kg/m3. For the cement content, a tolerance limit of +5% shall be allowed.  Cement shall be Portland cement of approved type. The Contractor shall submit his proposal for the type of cement to be used to the Engineer for the approval.

(b)	Aggregate

Aggregate for shotcrete shall be uniformly graded, dense, with hard particles which are required to avoid crumbling and forming fine powder of the grains by passing the same through the nozzle. The size gradation of Concrete Class D (measured by weight), for the fine and coarse aggregates shall be modified as shown below:

| Sieve Size (mm) (Square Mesh) | Fine Aggregate | Coarse Aggregate (Maximum size: 15mm) |
| --- | --- | --- |
| 15 | - | 90 - 100 |
| 10 | 100 | 20 - 55 |
| 5 | 90 - 100 | 0 - 10 |
| 2.5 | 80 - 100 | 0 - 5 |
| 1.2 | 50 - 85 | - |
| 0.6 | 25 - 65 | - |
| 0.3 | 10 - 35 | - |
| 0.15 | 2 - 10 | - |

Both cleanliness and uniformity of the fractions in accordance with appropriate standards shall be guaranteed through. The aggregate shall be stored in adequate and appropriate manner so that aggregate cleanliness and composition remain unchanged.

(c)	Additives

Additives are able to use for the improvement of workability and the development of strength and hardening pattern of the concrete.  If the Contractor used additives, the Contractor shall report the category of additives and the purpose of use and obtain the Engineer’s approval.

(d)	Water

The water used in shotcrete shall be fresh, clean and free from injurious amounts of sewage, oil, acid, alkali, salts or organic matter.

(e)	Wire Mesh

Wire mesh shall meet the relevant requirements of the Standard Specifications, as shown in the drawings and as approved by the Engineer. In addition, the following basic requirements shall be satisfied:

| Item | Requirement |
| --- | --- |
| Wire diameter | φ2.00mm |
| Reticulation | 50mm |
| Tension strength | 500N/m2 |
| Galvanization volume | More than 23g/m2 |

(f)	Anchor bar and Anchor pin

Anchor bar and Anchor pin shall meet the requirements of SR295 of JIS G 3112 or equivalents. The dimensions are shown below:

Anchor bar: D16 x 400 x 30             Anchor pin:  D9 x 200 x 30

(g)	Concrete Nail

The Nail used for shotcrete shall meet the requirements for materials described in Section 302: “Soil Nail” and as shown in the Drawings or as instructed by the Engineer. The dimensions are shown below:

| Item | Requirement |
| --- | --- |
| Nail diameter | φ10 mm |
| Length | 200 mm |
| Quantity to be installed | 2 nails per each m2 of shotcrete surface |

(h)	PVC drain pipes for weep holes

The Contractor shall furnish and install polyvinyl chloride (PVC) drain pipes and necessary fittings as mentioned in the drawings or as directed by the Engineer. The PVC drain pipes to be used shall be 50 mm in diameter and un-elasticized polyvinyl chloride pipes or as approved by the Engineer.

(3)	Construction Method

(a)	Preparation of slope surface

All loose rock, concrete, sharp protruding edges as well as dirt, grease, oil scale and other contamination shall be carefully removed. Any seepage of water entering locally shall be dealt with suitable methods approved by the Engineer, such as by drainage, boring with grouted pipe nipples connected to pipe, or local sealing. Fresh shotcrete shall not be exposed to water until it has sufficiently set.

Weep holes with PCV drain pipes of 50 mm in diameter, if required as shown in the drawings or directed by the Engineer, shall be made in every 3 square meters of slope surface prepared for shot concreting.  The slope surface to be treated shall be covered by wire mesh as approved by the Engineer. The wire mesh shall be fastened with anchor bars and anchor pins as shown in the drawings. Anchor bars shall be set in every 3 square meters. Anchor pins shall be set in 3 pins per 2 square meters. Where wire mesh is to be covered, it shall be held properly in place by expansion bolts or dowels anchored firmly in place.

(b)	Mixing

Aggregate, cement, additive and water shall be thoroughly mixed for all shotcrete. The optimum mix shall contain water that is free to cause any sloughing, and just enough cement as required per desired water-cement ratio.  The water-cement ratio of fresh shotcrete shall generally be between 0.35 and 0.50. The non-lift mixing process shall be used until it is applied. During handling, the mixed concrete shall be protected against dripping water. Concrete that has been prepared for more than one hour shall not be used.

(c)	Placing

Shotcrete shall be sprayed using proper nozzle spacing and at correct spray angle. Shotcreting shall not be executed to any surface prior to the approval of the Engineer.

The Contractor shall develop operating procedures and operations to the satisfaction of the Engineer, to give:

Minimum rebound

Rebound-free finished concrete

Smooth finished surface as little as possible

Minimum shrinking cracks and good adherence of the shotcrete to slope surface.

The quantities of shotcrete to be discharged by the nozzle shall be determined on the basis of the average thickness of shotcrete as mentioned in the drawings or as required by the Engineer, and also taking due account of rebound. Once the procedure for the placement of shotcrete has been established, subsequent work shall be carried out accordingly. When shotcreting is to be performed near existing structures, the Contractor shall protect the surface of the existing structure before injecting. Construction joints or stop joints shall be provided as approved or required by the Engineer, and shall be unlined at 45 degrees to the adjacent shotcrete surface in a clean and regular edge. Before placing to an adjoining work, the sloped portion and adjacent shotcrete shall be prepared as specified herein.

Before another layer of shotcrete is placed, the preceding layer shall be checked by the Engineer to maintain better quality. The Contractor shall repair any defective, sandy, cracked or spalled areas and any other areas, as directed by the Engineer.  The Contractor shall remove faulty shotcrete and, carry out surface preparation as specified herein and re-spray over that area.

(d)	Curing

For a certain period, depending on local conditions and as approved by the Engineer, the freshly placed shotcrete shall be protected against sunshine, cold, rain, running water, chemical attacks, and vibrations until it hardens.

(4)	Measurement and Payment

Measurement of the item shall be in square meters based on the drawings on which field survey data for measurement purposes are plotted.

The “shotcrete for slope protection” shall be paid as per respective contract unit rate which shall be the full and the final compensation to the Contractor as per Clause 112 to complete the work in accordance with these Specifications which shall include the costs for furnishing and producing materials including wire mesh, nails, transporting, placing, etc. and for furnishing all labour, tests, tools, equipment and any incidentals to complete the work as shown in the Drawings, as required by these Specifications, and/or as directed by the Engineer.

| Item No. | Description | Unit |
| --- | --- | --- |
| 2412 | Shotcrete for Slope Protection |  |
| 2412-1 | Shotcrete for Slope Protection (100 mm thick) | sq.m. |

2413	DRY STONE PACKING AND SOLING

(1)	Description

These works shall be carried out in accordance with the requirements of these specifications and to the lines, grades, dimensions and other particulars shown on the Drawings or as directed by the Engineer.

This work shall consist of the construction of the foundations for protection works installed in areas adjacent to culverts, piers and/or abutments, at outfalls of drainage, existing structures to be affected, along the road side and elsewhere, where protection is required as shown in the Drawing and as directed by the Engineer.

(2)	Materials and Construction Method

(a)	Stones and Boulders

Stones or boulders from road cutting excavation, or river stones and/or boulder from approved borrow pits, can be used and shall be stockpiled safely at locations previously approved by the Employer.  The Contractor shall investigate borrow area to obtain suitable materials for filling and shall submit the Construction Plan showing location, excavation area and demolishing plan to the Engineer for review and approval.

The stones, boulders or gravel shall satisfy the requirements for material specified in Sub-Clause 2401(2)(a): “Stones”, and Section 613: “Stone, Aggregate, Sand and Fillers”, as approved by the Engineer.

The material may be placed and spread out without compaction.

(b)	Excavation

The excavation shall be done in a way that any public utilities or properties are not damaged.  Any damage to a Third Party outside of the construction area caused under this work shall be compensated by Contractor at his own expenses.

(3)	Measurement

Measurement of the item for the “Dry Stone Packing and Soling” shall be in cubic metres of foundations actually constructed as shown in the Drawings, in accordance to these Specifications, and approved by the Engineer at Site.

The excavation for these works shall not be measured nor paid separately but deemed included in the rates and unit price established for the main item.

(4)	Payment

The works, measured as described above, shall be paid as per respective contract unit rate which shall be the full and the final compensation to the Contractor as per Clause 112 to complete the work in accordance with these Specifications which shall include the costs for furnishing materials, transporting, placing, boulders, stones, excavation, etc. and for furnishing all labour, tests, tools, equipment and any incidentals to complete the work as shown in the Drawings, as required by these Specifications, and/or as directed by the Engineer.

| Item No. | Description | Unit |
| --- | --- | --- |
| 2413 | Dry Stone Packing and Soling |  |
| 2413-1 | Dry Stone Packing and Soling | cu.m. |

SECTION 2600: MASONRY FOR STRUCTURES

2611	MEASUREMENT

(1)	Delete the full text of Sub-Section 2611: “Measurement” and replace with the following:

Stone masonry for breast wall (Concrete Class D, M20/20) shall be measured in square metres, and Stone masonry for gravity wall (Concrete Class D, M20/20) and Stone masonry (MM7.5) shall be measured in cubic metres.

The works pointing shall not be measured nor paid separately; but shall be deemed included in the measurement of the masonry.

2612	PAYMENT

(2)	Delete the first paragraph of Sub-Section 2612: “Payment” and replace with the following:

The Stone Masonry shall be paid at its respective contract unit rate which shall be the full and the final compensation to the Contractor as per Clause 112 to complete the work as per these Specifications including the works for pointing, scaffolding (if needed), curing, preparation of mortar, and the costs for providing and laying random rubble stone masonry in cement sand mortar MM7.5 (Mortar Mix with 7.5 Mpa compression strength at 28 days) for structure work, other materials, transporting, etc. and for furnishing all labour, tests, tools, equipment and any incidentals to complete the work as shown in the Drawings, and/or as directed by the Engineer.

| Item No. | Description | Unit |
| --- | --- | --- |
| 2600 | Masonry for Structures |  |
| 2600-1 | Stone masonry for breast wall (Concrete Class D, M20/20) | sq.m |
| 2600-2 | Stone masonry for gravity wall (Concrete Class D, M20/20) | cu.m |
| 2600-3 | Stone masonry (MM7.5) | cu.m |

SECTION 2800: BIO ENGINEERING WORKS

2801	SCOPE

(1)	Rename and delete the full text of Sub-Clause 2801 and replace with the following as follows:

2801	GENERAL

(1)	Scope

This Section covers all component of bio-engineering works such as provision of seed and plant cutting, nursery construction and operation, slope preparation for planning, site planting and sowing, jute netting, gabion wire bolsters and wire bolsters and wire netting, site protection and aftercare/maintenance.

(2)	Contractor’s Personnel

The Contractor shall provide, in a full time assignment, an expert in Bio-Engineering who shall take responsibility of management of execution of works related to Bio-Engineering during the whole project execution period, including periodical assignment during the Defects Notification Period.

The expert shall have similar experience for five (5) years or at least in two roads projects.

2806	FINAL SLOPE PREPARATION FOR BIo-ENGINEERING

(1)	Cut Slope Preparation for Grass Planting

(2)	Rename this Clause as follows:

(1)	Cut Slope Preparation for Grass Planting – Slope Trimming

(3)	Delete the full text of Sub-Clause 2806(1)(c) and replace with the following:

(c)	Trimming Works:

This work shall consist of the trimming of slopes, site protection and site aftercare in accordance with these specifications.  The area to be trimmed shall be part of cut slopes or embankments of the road as shown in the Drawings and as directed by the Engineer. The classification of soil shall be made by the Engineer based on the results of test performed by the Contractor.

Trimming shall be as follows:

- new sites:	trimming to straight plan section;

retaining wall to be filled behind.

- old sites:	minor trimming only required on part of site;

keeping rill or gully pattern in plan section.

(4)	Delete the full text of Sub-Clause 2806(1)(f): “Measurement and Payment:” and replace with the following:

(f)	Measurement and Payment:

The works for trimming shall not be measured and nor paid separately. All costs related to this requirements shall be deemed included in the Contractor’s Unit Prices for the applicable works (earthworks, slope protection, etc.) and shall be the full and final compensation to the Contractor as per Clause 112 to complete the work as herein specified including the arrangements for traffic control, removal of loose materials, all labour, tools equipment, safety harness and incidentals to complete the work as per these Specifications.

2807	SITE PLANTING AND SOWING

(5)	Delete the full text of Sub-Clauses 2807(2)(b), 2807(3)(b), 2807(4)(b), 2807(5)(b), 2807(6)(b) and replace with the following:

The Contractor shall prepare the surface for planting in accordance with the Drawings, or as instructed by the Engineer. Areas to be planted shall be given a layer of topsoil at least 75 mm thick unless, due to the presence of suitable sub-soil, the Engineer orders that the topsoil can be omitted.

(2)		Direct Seed Sowing of Shrubs and Trees on Site

(6)	Delete the full text of Sub-Clause 2807(2)(h): “Measurement and Payment”, and replace with the following:

(h)	Measurement and Payment: The works for this item shall be measured in number of seedlings planted in accordance to these Specifications, counted and accepted by the Engineer at Site.

The payment shall be the full and the final compensation to the Contractor as per Clause 112 to complete the work as herein specified including making arrangements for traffic control, supplying and storage of seeds, collection and application of mulch, all labour, tools, equipment, safety harness and incidentals to complete the work as specified herein.

| Item No. | Description | Unit |
| --- | --- | --- |
| 2807 | Site Planting and Sowing |  |
| 2807-1 | Orange trumpet creeper (Pyrostegia venusta) | No. |
| 2807-2 | Wall creeper (Ficus pumila) | No. |

(3)	Site Planting of Grass Slips and Cuttings

(7)	Delete the full text of Sub-Clause 2807(3)(j): “Measurement and Payment”, and replace with the following:

(j)		Measurement and Payment: The measurement shall be the actual area of grass planted and covered surface in square meters for the actual completed works in accordance with the Drawings and as directed by the Engineer.

The payment shall be the full and final compensation to the Contractor as per Clause 112 to complete the work as herein specified including making arrangements for traffic control, providing grass slips or rhizome, collection of mulch, all labour, tools, equipment, safety harness and incidentals to complete the work as per these Specifications. If the grass slips and mulch are supplied through a separate contract, costs for the supply of the same shall not be included herein. However, if the Contractor is responsible for the supply slips and mulch, no separate payment shall be made for these items.

| Item No. | Description | Unit |
| --- | --- | --- |
| 2807 | Site Planting and Sowing |  |
| 2807-3 | Site Planting of Grass Slips | sq.m. |

(4)	Site Planting of Shrubs and Trees raised in Polythene Pots

(8)	Rename the Sub-Clause 2807(4)(a) as follows:

(4)	Tree Planting

(9)	Delete the full text of Sub-Clause 2807(4)(a) and replace with the following:

(a)	The planting of trees and shrubs is intended to replace or restore something of the natural vegetation on the slope to be treated.

The Contractor shall perform the works as herein Specified and in accordance with approved Program.

The work shall be carried out as designated in the drawings and as directed by the Engineer.

(10)	Delete the full text of Sub-Clause 2807(4)(i): “measurement and payment…” and replace with the following:

(i)	For at least six months after completion of planting, the work will be inspected and evaluated by the Engineer. The area, where planting and straw mats are damaged and/or the covering rate of the grass is less than the average result, shall be repaired and/or re-seeded by the Contractor at his own expenses, and as directed by the Engineer.

(j)	Measurement: The measurement shall be made in number of seedlings planted in accordance to these Specifications, counted and accepted by the Engineer at Site.

Topsoil (75mm thick) shall be applied in accordance with the requirements of Sub-Section 2813: “Topsoil”, and as approved by the Engineer

(k)	Payment: The quantities measured shall be paid at the unit rates shown in the Bill of Quantities. This payment shall be the full and the final compensation to the Contractor as per Clause 112 to complete the work as herein specified including supplying seedlings and mulch, and the works for making arrangements for traffic control, providing seedlings, collection and application of mulch, all labour, tools, equipment, safety harness and any other incidentals to complete the work as per these Specifications.

| Item No. | Description | Unit |
| --- | --- | --- |
| 2807 | Site Planting and Sowing |  |
| 2807-4 | Lahre pipal tree | No. |
| 2807-5 | Jacaranda tree | No. |
| 2807-6 | Dhupi | No. |
| 2807-7 | Kaphal (Mountain peach) tree | No. |
| 2807-8 | Induru kumal | No. |
| 2807-9 | Beli chameli | No. |

2811	SITE AFTERCARE AND MAINTENANCE

(11)	Delete the first paragraph of Clause 2811(12): “measurement and payment” and replace with the following:

(12)	Measurement and Payment: The works for Sub-Section 2811: “Site Aftercare and Maintenance” shall be carried out as specified herein; however, these shall not be measured nor paid separately. All costs related to these requirements shall be deemed included in the Contractor’s Unit Prices for the applicable works for Bio-Engineering and shall be the full and final compensation to the Contractor as per Clause 112 to complete the work as herein specified including the arrangements for traffic control, removal of loose materials, all labour, tools equipment, safety harness and incidentals to complete the work as per these Specifications.

(12)	Add the following new Sub-Sections at the end of Sub-Section 2811: “Site Aftercare and Maintenance” and before Section 2900: “Maintenance Works”:

New Sub-Sections:

- 2812 “Turfing with Sods”

- 2813 “Topsoil”

2812	TURFING WITH SODS

(1)	General

This work shall consist of furnishing and laying of the live sod of perennial turf forming grass on embankment slopes, verges (earthen shoulders) or other locations shown on the drawings or as directed by the Engineer. Unless otherwise specified, the work shall be taken up as soon as possible following construction of the embankment, provided the season is favourable for establishment of the sod.

(2)	Materials

The sod shall consist of dense, well-rooted growth of permanent and desirable grasses, indigenous to die locality where it is to be used, and shall be practically free from weeds or other undesirable mailer. At the time the sod is cut, the grass on the sod shall have a length of approximately 50 mm and the sod shall have been freed of debris. Thickness of the sod shall be as uniform as possible, with some 50 - 80 mm or so of soil covering the grass roots depending on the nature of the sod, so that practically all the dense root system of the grasses is retained in the sod strip. The sods shall be cut in rectangular strips of uniform width, not less than about 250 mm x 300 mm i n size but not so large that it is inconvenient to handle and transport these without damage. During wet weather, the sod shall be allowed to dry sufficiently to prevent rearing during handling and during dry weather shall be watered before lifting to ensure its vitality and prevent the dropping of the soil in handling.

(3)	Construction Operations

(a)	Preparation of the earth bed

The area to be sodded shall have been previously constructed to the required slope and cross section. Soil on the area shall be loosened, freed of all stones larger than 50 mm size, sticks, stump s and any undesirable foreign matter, and brought to a reasonably fine granular texture to a depth of not less than 25 mm for receiving the sod.  Where required, topsoil shall be spread over the slopes. Prior to placing the topsoil, the slopes shall be scarified to a depth which, after settlement, will provide the required nominal depth shown on the plans. Spreading shall not be done when the ground is excessively wet. Following soil preparation and top soiling, where required, fertilizer and ground limestone when specified shall be spread uniformly at the rate indicated on the plans. After spreading, the materials arc incorporated in the soil by digging or other means to the depths shown on the Drawings.

(b)	Placing the sods

The prepared sod bed shall be moistened to the loosened depth, if not already sufficiently moist, and the sod shall be placed thereon within approximately 24 hours after the same had been cut. Each sod strip shall be laid edge to edge and such that the joints caused by abutting ends are staggered. Every strip, after it is snugly placed against the strips already in position, shall be lightly lamped with suitable wooden or metal tampers so as to eliminate air pockets and to press it into the underlying soil. On side slopes steeper than 2 (horizontal) to 1 (vertical), the laying of sods shall be started from bottom upwards. At points where water may flow over a sodded area, the upper edges of the sod strips shall be turned into the soil below the adjacent area and a layer of earth placed over tin's followed by its thorough compaction.

(c)	Staking the sods

Where the side slope is 2 (horizontal) to 1 (vertical) or steeper and the distance along the slope is more than 2m, the sods shall be staked with pegs or nails spaced approximately 500mm to 1000mm along the longitudinal axis of the sod strips. Stakes shall be driven approximately plumb through the sods to be almost flush with them.

(d)	Top dressing

After the sods have been laid in position, the surface shall be cleaned of loose sod, excess soil and other foreign material. Thereafter, a thin layer of topsoil shall be scattered over the surface of top dressing and the area thoroughly moistened by sprinkling with water:

(e)	Watering and maintenance

The sods shall be watered by the Contractor for a period of at least four weeks after laying. Watering shall be so done as to avoid erosion and prevent damage to sodded areas by wheels of water tanks. The Contractor shall erect necessary warning signs and barriers, repair or replace sodded areas failing to show uniform growth of grass or damaged by his operations and shall otherwise maintain the sod at his cost until final acceptance,

(4)	Measurement

The measurement of Turfing with Sods shall be made square metres of the actual areas performed in accordance to these Specifications and accepted by the Engineer at Site, in accordance with the following requirement:

- Turfing with Sods (50%): Sods for turfing are placed on the prepared surface on strips of 50cm spaced 50cm each other covering approximately 50% of the total area subject for payment under this item.

Topsoil  applied in accordance with these specifications and approved by the Engineer shall satisfy the requirements of Sub-Section 2813: “Topsoil”. However, the measurement of Filling for Topsoil for turfing works shall not be measured nor paid separately but deemed included in the rates established for turfing works.

(5)	Payment

The quantities, as measured above, shall be paid at the unit rates shown in the Bill of Quantities. This payment shall be the full and the final compensation to the Contractor as per Clause 112 to complete the work as herein specified including supplying sods, topsoil-filling, and the works for making arrangements for traffic control, all labour, tools, equipment, safety harness and any other incidentals to complete the work as per these Specifications.

| Item No. | Description | Unit |
| --- | --- | --- |
| 2812 | Turfing with Sods |  |
| 2812-1 | Turfing with Sods (50%) | sq.m. |

2813	TOPSOIL

(1)	General

This work consists of the preparation of the ground surface for topsoil application, hauling the topsoil material stockpiled for reuse as stipulated in Sub-Section 201: “Clearing and Grubbing”, Clause (4): “Measurement” and placing and spreading in accordance with these Specifications..

(2)	Materials

(a)	Topsoil shall be the surface layer of soil with no admixture of refuse or any material toxic to plant growth, and it shall be reasonably free from subsoil and stumps, roots, brush, stones (50 mm or more in diameter), clay lumps or similar objects.

(b)	Brush and other vegetation that will not be mixed into the soil during handling operations shall be cut and removed.

(c)	Ordinary sods and herbaceous growth such as grass and weeds are not to be removed but shall be thoroughly broken up and intermixed with the soil during handling operations.

(d)	Topsoil or soil mixture, unless otherwise specified or approved, shall have a pH range of approximately 5.5 to 7.6.

(e)	The organic content shall not be less than 3% but not higher than 20%.

(f)	The material passing the 0.075mm sieve, as determined by the wash test in accordance with ASTM C117, shall not be less than 20% and not higher than 80%.

(g)	Upon the request by the Contractor, approval on the materials for topsoil may be issued by the Engineer upon inspection and evaluation of the existing conditions of topsoil material stockpiled in accordance to Clause (4) of Sub-Section 201: “Clearing and Grubbing”.

Obtaining Topsoil:

(a)	The material for topsoil shall be exclusively such material stockpiled as result of the works performed for Clearing and Grubbing in the Project.

(b)	Heavy sod or other cover, which cannot be added into the topsoil by disking or other means, shall not be transported to the Site were topsoil will be applied, and shall be spoiled accordingly with respective specifications.

(c)	The sites of all stockpiles and areas adjacent thereto which the Contractor has disturbed, shall be graded as required by these Specifications.

(d)	Topsoil shall be hauled to the site of the work and placed for spreading, or spread as required. Any topsoil hauled to the site of the work and stockpiled shall be re-handled and placed without additional compensation.

Filling Material for Topsoil:

The filling material, when required for those areas to be covered with topsoil but requiring some filling materials to be in accordance with the designed levels, shall be, as described for embankment construction, reasonably compacted as instructed by the Engineer.

(3)	Construction Operations

(a)	Preparing Ground Surface

-	Immediately prior to dumping and spreading the topsoil on any area (except directly on a rock surface), the surface shall be loosened by discs or spike tooth harrows, or by other approved means, to a minimum depth of 50 mm to facilitate bonding of the topsoil to the covered subgrade soil.

-	The surface of the area to be covered with topsoil shall be cleared of all stones larger than 50 mm in any diameter and all litter or other materials which may be detrimental to proper bonding, the rise of capillary moisture, or the proper growth of the desired planting.

-	Limited areas as shown in the Drawings, which are too compact to respond to these operations, shall receive special scarification.

-	Grades on the area to be covered with topsoil, as shown in the Drawings, shall be maintained in a true and even condition. Where grades have not been established, the areas shall be smooth graded and the surface left at the prescribed grades in an even and properly compacted condition to prevent, insofar as practical, the formation of low places or pockets where water will stand.

-	Areas to be covered with topsoil that require filling material to reach the lines and levels described in the Drawings, shall be filled up to a level that topsoil can be applied properly as specified in this Specification Section.

(b)	Placing Topsoil

-	Topsoil shall be evenly spread on the prepared areas to a uniform depth of 150 mm after compaction, unless otherwise shown in the Drawings. Spreading shall not be done when the ground or topsoil is excessively wet, or otherwise in a condition detrimental to the work.  Spreading shall be carried out so that seeding operations can proceed with a minimum of soil preparation or tilling.

-	After spreading, any large, stiff clods and hard lumps shall be broken with a pulverizer or by other effective means, and all stones or rocks (50 mm or more in diameter), roots, litter, or any foreign matter shall be raked up and disposed of by the Contractor. After spreading is completed, topsoil shall be satisfactorily compacted by rolling or by other approved means. The compacted topsoil surface shall conform to the required lines, grades, and cross sections. Any topsoil or other dirt falling upon pavements as a result of hauling or handling of topsoil shall be promptly removed.

(4)	Measurement

The measurement of Topsoil shall be made square metres of the actual areas performed in accordance to these Specifications and accepted by the Engineer at Site.

The Topsoil-filling used for the item of Turfing shall not be measured nor pair under this Section.

(5)	Payment

The quantities, as measured above, shall be paid at the unit rates shown in the Bill of Quantities. This payment shall be the full and the final compensation to the Contractor as per Clause 112 to complete the work as herein specified including hauling Topsoil from Project’ stockpiles, conservation, pre-preparation to meet specification requirements, ground surface preparation, placing, spreading and preparing final topsoil surface and all necessary clean-up and remedial work and the works for making arrangements for traffic control, and providing all labour, tools, equipment, safety harness and any other incidentals to complete the work as per these Specifications.

| Item No. | Description | Unit |
| --- | --- | --- |
| 2813 | Topsoil |  |
| 2813-1 | Topsoil (150mm thick) | sq.m. |

SECTION 3100: MISCELLANEOUS WORKS

3105	RAILINGS

(1)	Insert the following additional requirement at the end of Sub-Clause 3105(1)(f):

(g)	Galvanizing Repair:

              - In the event that minor damage to the galvanized coating of the guardrail occurs during handling before or during installation, the Engineer may, after evaluation of the situation, require removal and replacement, or allow the Contractor to make repairs, at his own cost, by three applications of zinc anticorrosive paint as approved by the Engineer. The Contractor shall provide to the Engineer, for review and approval, all details and information, including the manufacturer’s application and surface preparation requirements for the repair application coating that he is proposing.

              - Repair of the coating application shall not proceed until such information and data have been approved by the Engineer.

(7)	Payment

(2)	Insert the following additional requirement at the end of Clause 3105(7): “Payment”:

| Item No. | Description | Unit |
| --- | --- | --- |
| 3100 | Miscellaneous Works |  |
| 3105 | Railings |  |
| 3105-1 | Steel railings Type A (H=1.10m for pedestrians) | lin.m. |

(3)		Add the following new Sub-Sections at the end of Sub-Section 3110: “Filter Materials”:

New Sub-Sections:

- 3111 “Bridge Drainage”

- 3112 “PVC Water-Stops”

3111	RC BRIDGE DRAINAGE

(1)	Description

This work consists of the construction of bridge drainage including drainage pipes, catch pit with grating and strainer.

(2)	Material Requirements

(a)	Catchpit with Grating and Strainers

Catch Pit with gratings, strainers and chains shall be as shown in the Drawings, complying with the requirements for types, shape, sizes, galvanizing and connections.

(b)	Drain Pipes

Pipes and pipe fittings for drains shall be as shown in the Drawings, complying with the requirements for types and sizes, and conforming to the standards of JIS K 6741, ASTM D1785 - 15 or equivalent for Polyvinyl Chloride (PVC) pipes (VP pipes).

(c)	Delivery, Storage and Handling

    - Delivery and Storage: Materials delivered to site shall be inspected for detecting damages, and shall be unloaded and stored with minimum handling.  Materials shall not be stored directly on the ground.  The inside of pipes and fittings shall be kept free of dirt and debris. Plastic pipes shall be installed within six (6) months from the date of manufacture unless otherwise approved.

    - Handling: Materials shall be handled in such a manner as to ensure delivery to the point of installation in sound, undamaged condition.  Pipes shall be carried and not dragged along the ground to the installation location.

(d)	Submittals

The Contractor shall submit for the approval of the Engineer method statements, schedule, materials, all certifications from the manufacturers, attesting that materials meet the specification requirements.  Certificates are required for all drain pipes, fittings, catch pit and slab drain.

(3)	Construction Requirements

(a)	Installation of Catch Pit

Catchpit shall be installed completing with gratings, strainers, chains and connections at the locations indicated in the Drawings. The catchpit shall be firmly fixed with deck slab reinforcement before concreting, so as not to be moved during the concreting operation.

(b)	Pipe Placement

Each pipe shall be carefully inspected before it is placed.  Any defective or damaged items shall be rejected and replaced immediately. The pipes shall be jointed fully with leak-resistant seals using gasket, bell and spigot joints. The adhesive for securing piping and fittings shall be suitable for the application and approved by the Engineer.

The pipes shall be firmly fixed with the bridge structures using metal accessories and appropriate gradient. After completion of the catch pit and the drain pipe installation, water leak test shall be carried out and any defects shall be corrected and repaired.

(4)	Measurement

(a)	Catch Pit

Catch Pit shall include all the materials described in the Drawings.  Catch Pit shall be measured for payment by the actual quantity of drainage devices constructed and installed in accordance with the requirements of the Drawings, these Specifications and instructions by the Engineer.

(b)	Drainage Pipe

Drainage piping shall be measured for payment by the actual length (in linear meters) of the straight PVC drain piping in the diameter and type required in the Drawings.

No separate measurement shall be made for the bolts, nuts, washers, accessories, protective coatings and metal supports for PVC Pipes.

(c)	Drainage Pipe for Elbows and Branch Connections

Elbows and Branch connections shall not be measured and nor paid separately but shall be deemed included in the price rates for Drainage Pipes.  These include pipes processed for connection, flexible joints, protective coatings, constructed and installed in accordance with the requirements of the Drawings, these Specifications and instructions by the Engineer.

(5)	Payment

Payment shall be made in accordance with the applicable unit prices of the pay items of the Bill of Quantities listed below, and shall be the full and final compensation to the Contractor as per Clause 112 to complete the work as herein specified including all materials, fabrication, installation, all labors, equipment, tools and incidentals necessary to install catch-pits, elbows and branch connections, and piping to complete the work as per these Specifications. and accepted by the Engineer at Site.

| Item No. | Description | Unit |
| --- | --- | --- |
| 3111 | Bridge Drainage |  |
| 3111-1 | Casting Catch Pit | No. |
| 3111-2 | PVC Drain Pipe (VP) φ150mm | lin.m. |

3112	PVC WATER-STOPS

(1)	Description

This work consists of the preparation, fabrication and placing of PVC Water Stops as shown on the Drawing to this Specification or as directed by the Engineer.

PVC Water Stops are defined as joints placed in construction joints between the continuous concrete floor slab or the deck and the wall.  The purpose of the PVC Water Stops is to permit free movement of the structure due to thermal effect, vibration and rotational articulation, to prevent cracking in the surface layer by effectively supporting the surfacing and to prevent water and water-borne contaminants from invading the superstructure, substructure and foundation.

(2)	Material

The water-stop shall be fabricated from a plastic compound, the basic resin of which shall be polyvinyl chloride. The compound shall contain additional resins, plasticizers, inhibitors or other materials such that when the material is compounded, it shall meet the requirements given in these Specifications.

(3)	Test Requirements

PVC water-stops shall meet the requirements specified in Table 31-1.  The procedure for sampling and the criteria for conformity shall met the requirements of the of IS 15058: 2002 (Annex-D).

Table 31-1 Requirement for PVC Water-Stops

| Sl. No. | Characteristic | Requirement | Test Method (Note 1) |
| --- | --- | --- | --- |
| i) | Tensile Strength, Min | 13.8 Mpa | IS 8543 (part 4/sec I) |
| ii) | Elongation, Min | 285% | IS 8543 (part 4/sec I) |
| iii) | Hardness (Shore A), Min | 65 | IS 8543 (part 5/sec II) |
| iv) | Water absorption, percent by mass, Max | 0.6 | Annex A of IS 15058: 2002 |
| v) | Cold bend temperature at which samples does not crack, Min | -25° C | Annex G of IS 9766 |
| vi) | Accelerated extraction test: |  | Annex B of IS 15058: 2002 |
| a) | Tensile strength, Min | 10.3Mpa |  |
| b) | Elongation, Min | 280% |  |
| vii) | Stability in effect of alkalis test |  | Annex C of IS 15058: 2002 |
| a) | Weight increase at 7 days, percent by mass, Max | 0.25 |  |
| b) | Weight decrease at 7 days, percent by mass, Max | 0.10 |  |
| c) | Change in hardness at 7 days, (Shore A) | ±5 |  |
| d) | Weight increase at 28 days, Max | 0.40% |  |
| e) | Weight decrease at 28 days, Max | 0.30% |  |
| f) | Dimension change | ±1% |  |

Note-1: Equivalent test methods applied by standards of ASTM, JIS or AASHTO may be applied upon the approval by the Engineer.

The surface of water-stop shall be ribbed. The outside edges shall be at the level of the central bulb.

The water-stops shall be free from blisters, pinholes, cracks and embedded foreign matters.

(4)	Shape and Dimensions

The cross-section of PVC water-stops may vary depending upon the water head and site conditions. A typical cross section of PVC water-stops is shown in Figure 31.1 as per reference.

Water stops shall be furnished full length for each straight portion of the joint, without field splices. Water stops shall be cut and spliced at changes in direction as may be necessary to avoid buckling or distortion. All field splices shall be performed by heat sealing the adjacent surfaces in accordance with the manufacturer's recommendations to form continuous watertight joints.

The water-stop shall be packed in coil form, the inside diameter of which shall not be less than 300 mm.

Note: All dimensions are in millimeters.

Figure 31.1 Typical Cross Section of PVC Water-Stops (for reference)

(5)	Tolerances on Dimensions

The permissible to tolerance on width and thickness shall be as follows:

Width: 	± 10mm

Thickness 	+2mm

(6)	Marking

The water-stops shall be marked with the following information:

Size of the water-stops;

Name of manufacturer or trade-mark, if any; and

Month and year of manufacture

(7)	BIS Certification Marking

The PVC water-stops may also be marked with the Standard Mark. The use of the Standard Mark is governed by the provisions of the Bureau of Indian Standards Act, 1986 and the Rules and Regulations made there under. The details of conditions under which the license for the use of the Standard Mark may be granted to manufacturers or producers may be obtained from the Bureau of Indian Standards.

(8)	Installation

The PVC Water Stops shall be held firmly in place to prevent displacement during concreting. If after placing concrete water stops are materially out of position or shape, the surrounding concrete shall be removed, the water stop reset, and the concrete replaced, all at the Contractor's expense.

The following procedure may be applied for the installation:

- The PVC Water Stop shall be carefully and accurately fixed in position with the bulkhead of the forms slotted and shaped to hold the PVC Water Stop in alignment, and to prevent leakage of mortar or damage to the PVC Water Stop. Provision shall also be made to hold that portion of the PVC Water Stop being built accurately in position during concrete pouring and compaction.

- After removing forms and before placing the second part of the concrete joint, PVC Water Stop shall be cleaned of mortar or other debris.

- Where required, joint filler or removable inserts for joint filler shall be placed and fixed as shown on the Drawings.

- The second part of the joint concrete shall then be placed. Provision shall again be made to hold that portion of PVC Water Stop being built in accurately in position during placing and compaction of concrete.

- Particular care shall be taken to ensure that no air is trapped between the concrete and PVC Water Stop. In locations where the PVC Water Stop is installed horizontally or near horizontally and the top face is not formed, the PVC Water Stop shall be bent upwards while concrete is placed in the lower part of the joint to within 10 mm of the underside of the PVC Water Stop and compacted. An excess of 1:2 cement:fine aggregate mortar shall then be placed under the turned up PVC Water Stop and the PVC Water Stop then straightened into position on full bedding of mortar. No further concrete shall be poured on top of the PVC Water Stop until the bedding of the PVC Water Stop has been inspected and passed as satisfactory by the Supervising Engineer. Excess mortar squeezed out when straightening the PVC Water Stop shall be spread on top of the PVC Water Stop and the balance of the concrete for the joint placed and compacted.

(9)	Jointing

- Use of its specialized welding equipment for on-site welding.

- This consists of thermostatically controlled welding blades and welding jigs.

- On-site joining is a critical application and must be completed correctly to guarantee to quality of the system.

- The ends of the PVC Water Stop are cut square and placed into the adjustable jig.

- The ends of the PVC Water Stop then slide up against the welding iron and bring the two ends together until the molten ends of the PVC fuse.

- Splices, directional changes and intersections are achieved with premolded unions, fittings and a rubber adhesive kit.

- Welding requirements shall in all respects conform to the standards of AWS D1.1-98.

(10)	Measurement

PVC Water - Stop shall be measured in nominal linear meter measured by the Drawings.  Measurement shall be made along the center line of the concrete joint where the water-stop has actually been installed.

(11)	Payment

PVC Water - Stop measured as provided above shall be paid at the contract unit rate which shall be the full and final compensation to the Contractor as per Clause 112 to complete the work as herein specified including all materials, supplying, installation, all labors, equipment, tools and incidentals necessary to install the PVC Water Stops to complete the work as per these Specifications. and accepted by the Engineer at Site.

| Item No. | Description | Unit |
| --- | --- | --- |
| 3112 | PVC Water Stops |  |
| 3112-1 | PVC Water Stops (W=300mm) | lin.m. |

(4)		Add the following new Sub-Sections at the end of Sub-Section 3112: “PVC Water-Stops”:

New Sub-Sections:

- 3120 “Street Lighting System (SLS)”

- 3130 “Traffic Information Boards (TIB)”

3120	STREET LIGHTING SYSTEM (SLS)

(1)	Description

This works shall include the following:

Supplying, construction and installation of the Street Lighting System (SLS)

Ancillary Works and Testing

The street light system consists of the specified electrical hardware, mast arm poles, and footing which includes excavation, breaking of concrete or any other hard surface such as road pavement, concreting, erecting poles to lines and levels and curing the concrete and re-instatement of the surface to its original condition and disposal of excess spoil materials: on an all complete net basis.

The works shall include furnishing all materials, construction or installation of the above referred items, and all ancillary works and any other incidental necessary to complete the Works in conformity with the Drawings and these Specifications, or as directed by the Engineer.

This work shall also include the Design Review and Updating of the Street Lighting System (SLS) by the Contractor based on the Conceptual Design provided by the Employer. The report shall include also all detailed drawings, applicable technical specifications and requirements for construction and materials.

The Contractor shall be responsible for providing all design, fabrication and installation details necessary to provide work and operations as intended under and required by the Drawings and Specifications.

(2)	Compliance with Authorities and Standards

All works under this Contract shall comply with the applicable regulations and requirements of Statutory Authorities.

All equipment and materials shall conform to the requirements of the codes and standards specified herein and the Drawings, or certified by other internationally recognized standard institutions or testing laboratories that maintains periodic inspection of production of listed equipment and materials meet nationally recognized standards or has been tested and found suitable for use in the specified manner.

All cabling work shall be carried out by licensed sub-contractors and certified installers for the cabling system being installed.

Certification of the system shall be provided by the Contractor.

Codes and Standards: Unless otherwise indicated in the Drawings or in the Specifications, the design, materials, manufacture, testing, inspection and performance of all electrical and electromechanical equipment shall comply with the latest revisions of relevant standards listed in Section 113: “National Specifications” and the following standards:

JIS:	Japanese Industrial Standard

JEC:	Japanese Electro-technical Committee

JEM:	The Standards of the Japan Electrical Manufacturer’s Association

IEC:	International Electro-technical Commission (authorized standards)

When local regulatory bodies, have jurisdiction over installation, the following Nepalese standards shall be applied:

Nepal Electricity Authority

Nepal National Building Code NBC207: 2003 – Electrical Design Requirements

(3)	Construction Method and Design Review and Updating Report

Before commencement of the works, the Contractor shall prepare and submit to the Engineer, for review and approval, a statement method for construction and installation of the Street Lighting System (SLS) describing the labour, materials and equipment to be used and the method of work execution. The Statement shall also describe the safety precautions to be adopted and all measures for compliance with environmental requirements of these specifications. In addition to the statement, the Contractor shall submit a detailed report of the Design Review and Updating. These documents shall be attached to the Program which requirements are described in Section 116: “Programme”.

The Street Lighting System (SLS) shall be installed with the height as shown on the Drawings or as directed by the Engineer.

(4)	Submittals

Prior to commencement of the Electrical Works, the Contractor shall prepare and submit the following for the Engineer’s review and approval:

(a)	Details of the manufacturer and subcontractor(s), including description of work experience, major equipment, labour, methods of quality control and safety control, etc.

(b)	Manufacturer’s detailed technical data, specifications and installation instructions for all materials, equipment, components and accessories required for this Section.

(c)	Manufacturer’s Certification for all materials, equipment and/or systems shall be provided by the Contractor (prior to installation) and indicating compliance with the requirements of the Specification. Where so instructed by the Engineer, the Contractor shall obtain independent certification for materials, equipment and/or systems from independent and approved testing agencies, all at the expense of the Contractor.

(d)	Guarantee:

- The guarantee for the Electrical Works shall be at least for a full two (2) year period and shall cover the Employer for the complete responsibility of the Contractor for design, suitability and performance of the Electrical Works.

- The Guarantee shall cover replacement by the Contractor or manufacturer, free of charge, of any components which fail to perform (intentional damage, fair wear and tear excepted) for the guarantee period stated above.

- The Engineer may request that the Guarantees are provided in writing and in a form approved by the Engineer and by the Employer, for the due and faithful observance and performance of this obligation. In this case the guarantee shall be delivered to the Engineer for approval at the same time as the “Request For Approval” (RFA) of the relevant System, equipment, materials or manufacturer.

Materials and equipment shall not be ordered or fabricated until the above submittals have been approved by the Engineer.

Moreover, in addition to the above, the following submittals are required:

(a)	Schedule of equipment, cables and other materials.

(b)	Shop-Drawings for all work required under this Section.  The Shop Drawings shall clearly indicate the particular manufacturer’s document number and fitting details used as reference in the development of the shop drawings.

Shop drawings shall include scale plans, and details of the equipment, structures, piping, cable and wiring and shall show the method of installation with all conduits, trays and other cable and pipe support and conveying systems and all foundations supports and accessories.

All Field dimensions shall be verified and included showing exact locations and dimensions.

(c)	Samples and test certificates of materials and equipment as specified or as requested by the  Engineer. Any materials delivered thereafter that the  Engineer considers not equal to the approved samples, shall be rejected and removed from the Site.

Samples should be the same as the permanent items to test the performance and show the workmanship, colour and finishes.

Samples are not returnable, nor included in quantities listed for payment of the Works.

Approved samples shall be kept on Site, under the Engineer’ supervision, for reference.

(d)	Listing of all signs and nameplates required, giving full text for each item.

(e)	Certification of materials and equipment shall be provided for each system of the Electrical Works (after installation) by the Manufacturer of the System indicating that the complete System as installed complies fully with the requirements of the Contract. Certification shall be complete with all necessary test reports, and data sheets.

(f)	Maintenance Instructions: The Contractor shall be responsible for obtaining from the manufacturer, for each type of fitting, control and accessory, the recommended operation and maintenance manual including tools required and types of cleaners to be used.

The Contractor shall incorporate the manufacturer’s information within the Operation and Maintenance Manuals prepared by the Contractor and submitted to the Engineer for review and approval.

(5)	Materials, Equipment and Systems

(a)	Unless otherwise specified in the Drawings or the contract, products shall be Manufacturer’s first quality line of standard and/or series of factory fabricated items.

(b)	Materials and equipment specified shall conform standards herein established or equivalent if approved by the Engineer and tested satisfactorily. The approved material shall be certified by an approved independent agency.

(c)	Comparable materials, assemblies and systems of manufacturers other than those specified may be proposed subject to prior approval by the Engineer.

(d)		Electrical Works shall be designed, fabricated and supplied by a single manufacturer for all work under this Contract.

(6)	Requirements for the Street Lighting System

The following basic requirements shall be satisfied for the Street Lighting System (non-solar system):

- Type of the Street Lighting is High-Pressure Sodium Lamp. Power of the lamp is 250W. Luminous flux of the road lighting shall be 28,000 lumen minimum, and shall be the product of Phillips or equivalent as approved by the Engineer

- The lighting poles shall be made from octagonal steel plate with the thickness of 3.2mm minimum and also protected by hot dip galvanize.

- Lamp Power:	250W

- Reliability / MTBF:	5 years (product life cycle)

- Gear	SGR (SON gear)

- Ingress Protection	IP 65

- Housing material	Aluminum

- Voltage/Hz	220V / 50 Hz

- Dimension	755 x 380 x 275 mm

(7)	Requirements for the Solar Street Lighting System

(a)	Definition

A standalone Solar Photo-Voltaic (SPV) Street Lighting System (SLS) is an outdoor lighting unit used for illuminating a street or an open area. It consists of Photo-Voltaic (PV) module(s), Compact Fluorescent Lamp (CFL), lead acid battery, control electronics, inter-connecting wires/cables, module mounting Pole including hardware and battery box. The CFL is fixed inside a luminaire which is mounted on the pole. The PV module is placed at the top of the pole at an angle to maximize incident solar radiation, and a battery is placed in a box attached to the pole. The module is mounted facing south, so that it receives solar radiation throughout the day, without any shadow falling on it. Electricity generated by the PV module will charge the battery during the day time. This system operates from dusk to dawn.

(b)	Requirements

- Duty Cycle:

The system should automatically switch ON at dusk, operate throughout the night and automatically switch OFF at dawn.

- PV Modules (s):

    - Both crystalline and thin film technology modules are allowed in the system. The PV module should have a certificate of testing conforming to IEC 61215 Edition II / BIS 14286 or IEC 61646 for crystalline and thin film PV modules respectively. The certificate should be from an NABL or IECQ accredited Laboratory.

    - The power output of the module(s) under STC should be a minimum of 74 Wp. Either two modules of minimum 37 Wp output each or one module of 74 Wp output should be used. In case of thin film technology PV modules, the specified values refer to the stabilized power output after the initial degradation. The module efficiency should not be less than 12%.

    - The operating voltage corresponding to the power output mentioned above should be 16.4 ± 0.2 V.

    - The open circuit voltage of the PV modules under STC should be at least 21.0 Volts.

    - The terminal box on the module should have a provision for opening for replacing the cable, if required.

    - Each PV module must use a RF identification tag (RFID), which must contain the following information:

    - Name of the manufacturer of PV Module.

    - Model or Type Number

    - Serial Number

    - Month and year of the manufacture

    - I-V curve for the module

    - Peak Wattage of the module at 16.4 volts

    - Im, Vm and FF for the module

    - Unique Serial No and Model No of the module Until March 2013, the RFID can be inside or outside the module laminate, but must be able to withstand harsh environmental conditions.

    - A distinctive serial number starting with NSM will be engraved on the frame of the module. The distinctive number starting NSM will also be screen printed on the tedlar sheet of the module.

- Battery:

    - Lead Acid, tubular positive plate flooded electrolyte or Gel or VRLA Type.

    - The battery will have a minimum rating of 12V, 75 Ah (at C/10 discharge rate).

    - 75 % of the rated capacity of the battery should be between fully charged and load cut off conditions.

- Lamp:

    - The lamp should be 11 Watt compact fluorescent lamp (CFL) with 4 pins along with proper pre- heating circuit.

    - The light output from the lamps should be around 900±5 % lumens (for 11 W CFL).

    - The lamp should be housed in an assembly suitable for outdoor use, with a reflector on its back.

    - No blackening or reduction in the lumen output by more than 10%, should be observed after 1000 ON/OFF cycles (two minutes ON followed by four minutes OFF is one cycle).

- Electronics:

    - The inverter should be of quasi sine wave/ sine wave type, with frequency in the range of 20 - 30 KHz. Half-wave operation is not acceptable.

    - The total electronic efficiency should be not less than 85 %.

    - The idle current consumption should not be more than 10 mA.

    - The PV module itself should be used to sense the ambient light level for switching ON and OFF the lamp.

- Electronic Protections:

    - Adequate protection is to be incorporated under no load conditions e.g. when the lamp is removed and the system is switched ON.

    - The system should have protection against battery overcharge and deep discharge conditions.

    - Fuses should be provided to protect against short circuit conditions.

    - Protection for reverse flow of current through the PV module(s) should be provided.

    - Electronics should have temperature compensation for proper charging of the battery throughout the year.

- Mechanical Hardware:

    - A metallic frame structure (with corrosion resistance paint) is to be fixed on the pole to hold the SPV module(s). The frame structure should have provision to adjust its angle of inclination to the horizontal between 0 and 45, so that the module(s) can be oriented at the specified tilt angle.

    - The pole should be made of mild steel pipe with a height 6 m above the ground level, after grouting and final installation. The pole should have the provision to hold the weather proof lamp housing. It should be painted with a corrosion resistant paint.

    - All poles will be of slip base type. This type of special purpose Street Lighting Pole addresses the universal problem of high-speed road accidents (over 60km/h) involving roadside collisions with light poles. These poles shall comply with Australian Standards 1158.1.3 or equivalent Indian Standards. Slip Base Poles differ in design which features the two base plates bolted together with the facility of freely coming apart if a crash occurs. This separation lessens resistance and mitigates the overall effects of the impact (and subsequent damage to the vehicle or injury to its occupants).

    - All poles shall be tapered round of specified diameter and consist of standard base-plate mounted with either in-ground section or connection spool on rag bolts, bolts, slip washer and rectangular washers. All poles shall be hot dip galvanized to Australian Standards 4680:2006 or equivalent Indian Standards and shall be either powder coated or painted as approved by the Engineer. The slip base poles shall be in a single and double outreach arm arrangement, curved or straight as approved by the Engineer. Poles shall have security or tamper proof screws for access door covers.

    - A vented, acid proof and corrosion resistant painted metallic box for outdoor use should be provided for housing the battery with a provision of lock and key.

- Other Features

    - The system should be provided with 2 LED indicators: a green light to indicate charging in progress and a red LED to indicate deep discharge condition of the battery.

    - There will be a Name Plate on the system, which will give: (a) Name of the Manufacturer or Distinctive Logo. (b) Serial Number.

    - Components and parts used in the solar street lighting systems should conform to the latest BIS specifications, wherever such specifications are available and applicable.

    - The PV module(s) will be warranted for a minimum period of 25 years from the date of supply and the street lighting system (including the battery) will be warranted for a period of two years from the date of supply. PV modules used in Solar Street Lighting System must be warranted for their output peak watt capacity, which should not be less than 90% at the end of Twelve (12) years and 80% at the end of Twenty five (25) years. The Warranty Card to be supplied with the system must contain the details of the system.

    - Necessary lengths of wires/cables and fuses should be provided.

    - An Operation, Instruction and Maintenance Manual, in English should be provided with the Solar Street Lighting System. The following minimum details must be provided in the Manual: basic principles of photovoltaics; a small write-up (with a block diagram) on the Solar Street Lighting System - describing its components (PV module, battery, electronics and luminaire) and their expected performance; a section on charging and significance of indicators; together with clear instructions about erection of poles and mounting of PV module (s) and lamp housing assembly on the pole.

(7)	Testing and Commissioning

Testing and Commissioning of the completed installation of Street Lighting System (SLS) shall be made in accordance with the requirements of Clause 509(3): “Testing and Commissioning”.

(8)	Measurement

The measurement for these works shall be made by the Number of Lights actually installed, satisfactorily tested according to Clause 509(3): “Testing and Commissioning” and conforming the requirements of Clause 9: “Test on Completion” of the Conditions of Contract, confirmation of proper function of the established system, satisfying all requirements of these specifications, and approved by Engineer. All materials, equipment and ancillary works or incidentals shall not be measured separately but shall be deemed included in the rates of the main items for lighting system.

(9)	Payment

The works for the Street Lighting System (SLS) measured as described above shall be paid at the unit rates shown in the Bill of Quantities. This payment shall be the full and the final compensation to the Contractor as per Clause 112 including the cost for Design Review and Updating, provision of all materials, transportation, all operations required for construction and installation as shown in the Drawings and described in this Item including drawings, tests and all other ancillary works, and any incidental work needed to complete the work as per these Specifications and/or directed by the Engineer.

| Item No. | Description | Unit |
| --- | --- | --- |
| 3120 | Street Lighting System (SLS) |  |
| 3120-1 | Street Lighting System for West Portal and West Toll-Gate Area | No. |
| 3120-2 | Street Lighting System for East Portal and East Toll-Gate Area | No. |
| 3120-3 | Solar Street Lighting System for Balambu Intersection | No. |

3130	TRAFFIC INFORMATION BOARDS (TIB)

(1)	Description

This works shall include the following:

Supplying, construction and installation of the Traffic Information Boards (TIB) including, but not limited to, the following:

- Complete system for information display with data connection to the data center

- Solar power system

- Display board and a control system

- Design for the provision of off-grid solar power system for operation of the TIB

- Design of Data Communication system with data center

- Preventions measures against rain or water and anti-theft

Ancillary Works and Testing

Figure 35.1 shows as per reference the system for installation of the Traffic Information Boards (TIB) that shall be implemented for the Project.

The works shall include furnishing all materials, construction or installation of the above referred items, and all ancillary works and any other incidental necessary to complete the Works in conformity with the Drawings and these Specifications, or as directed by the Engineer.

This work shall also include the Design Review and Updating of the Traffic Information Boards (TIB) by the Contractor based on the Conceptual Design provided by the Employer. The report shall include also all detailed drawings, applicable technical specifications and requirements for construction and materials.

Figure 35.1: System Structure of the System for Traffic Information Boards (TIB)

The Contractor shall be responsible for providing all design, fabrication and installation details necessary to provide work and operations as intended under and required by the Drawings and Specifications.

(2)	Compliance with Authorities and Standards

All works under this Contract shall comply with the applicable regulations and requirements of Statutory Authorities.

All equipment and materials shall conform to the requirements of the codes and standards specified herein and the Drawings, or certified by other internationally recognized standard institutions or testing laboratories that maintains periodic inspection of production of listed equipment and materials meet nationally recognized standards or has been tested and found suitable for use in the specified manner.

All cabling work shall be carried out by licensed sub-contractors and certified installers for the cabling system being installed.

Certification of the system shall be provided by the Contractor.

Codes and Standards: Unless otherwise indicated in the Drawings, the design, materials, manufacture, testing, inspection and performance of all equipment shall, unless otherwise specified in the Specifications, comply with the latest revisions of relevant standards listed in Section 113: “National Specifications” and the following standards:

JIS:	Japanese Industrial Standard

JEC:	Japanese Electro-technical Committee

JEM:	The Standards of the Japan Electrical Manufacturer’s Association

IEC:	International Electro-technical Commission (authorized standards)

When local regulatory bodies, have jurisdiction over installation, the following Nepalese standards shall be applied:

Nepal Electricity Authority

Nepal National Building Code NBC207: 2003 – Electrical Design Requirements

(3)	Construction Method and Design Review and Updating Report

Before commencement of the works, the Contractor shall prepare and submit to the Engineer, for review and approval, a statement method for construction and installation of the Traffic Information Boards (TIB) describing the labour, materials and equipment to be used and the method of work execution. The Statement shall also describe the safety precautions to be adopted and all measures for compliance with environmental requirements of these specifications. These documents shall be attached to the Program which requirements are described in Section 116: “Programme”.

The Traffic Information Boards(TIB) shall be installed with the height and dimensions as shown on the Drawings or as directed by the Engineer.

The TIB shall be installed along the roadside of Tribhuvan Highway Intersection as shown in the Drawings. The battery and inverter shall be installed inside each control house (Control House of Eastern and Western portals). Similarly, the solar panels shall be installed above each control house.

(4)	Submittals

Prior to commencement of the Electrical Works, the Contractor shall prepare and submit the following for the Engineer’s review and approval:

(a)	Details of the manufacturer and subcontractor(s), including description of work experience, major equipment, labour, methods of quality control and safety control, etc.

(b)	Manufacturer’s detailed technical data, specifications and installation instructions for all materials, equipment, components and accessories required for this Section.

(c)	Manufacturer’s Certification for all materials, equipment and/or systems shall be provided by the Contractor (prior to installation) and indicating compliance with the requirements of the Specification. Where so instructed by the Engineer, the Contractor shall obtain independent certification for materials, equipment and/or systems from independent and approved testing agencies, all at the expense of the Contractor.

(d)	Guarantee:

- The guarantee for the Electrical Works shall be at least for a full two (2) year period and shall cover the Employer for the complete responsibility of the Contractor for design, suitability and performance of the Electrical Works.

- The Guarantee shall cover replacement by the Contractor or manufacturer, free of charge, of any components which fail to perform (intentional damage, fair wear and tear excepted) for the guarantee period stated above.

- The Engineer may request that the Guarantees are provided in writing and in a form approved by the Engineer and by the Employer, for the due and faithful observance and performance of this obligation. In this case the guarantee shall be delivered to the Engineer for approval at the same time as the “Request for Approval” (RFA) of the relevant System, equipment, materials or manufacturer.

Materials and equipment shall not be ordered or fabricated until the above submittals have been approved by the Engineer.

Moreover, in addition to the above, the following submittals are required:

(a)	Schedule of equipment, cables and other materials.

(b)	Shop-Drawings for all work required under this Section.  The Shop Drawings shall clearly indicate the particular manufacturer’s document number and fitting details used as reference in the development of the shop drawings.

Shop drawings shall include scale plans, and details of the equipment, structures, piping, cable and wiring, installation with all conduits, trays and other cable and pipe supports and conveying systems and all civil works and steel works for construction of the supports and accessories.

All Field dimensions shall be verified and included showing exact locations and dimensions.

(c)	Samples and test certificates of materials and equipment as specified or as requested by the  Engineer. Any materials delivered thereafter that the  Engineer considers not equal to the approved samples, shall be rejected and removed from the Site.

Samples should be the same as the permanent items to test the performance and show the workmanship, colour and finishes.

Samples are not returnable, nor included in quantities listed for payment of the Works.

Approved samples shall be kept on Site, under the Engineer’ supervision, for reference.

(d)	Listing of all signs and nameplates required, giving full text for each item.

(e)	Certification of materials and equipment shall be provided for each system of the Electrical Works (after installation) by the Manufacturer of the System indicating that the complete System as installed complies fully with the requirements of the Contract. Certification shall be complete with all necessary test reports, and data sheets.

(f)	Maintenance Instructions: The Contractor shall be responsible for obtaining from the manufacturer, for each type of fitting, control and accessory, the recommended operation and maintenance manual including tools required and types of cleaners to be used.

The Contractor shall incorporate the manufacturer’s information within the Operation and Maintenance Manuals prepared by the Contractor and submitted to the Engineer for review and approval.

(5)	Materials, Equipment and Systems

(a)	Unless otherwise specified in the Drawings or the contract, products shall be Manufacturer’s first quality line of standard and/or series of factory fabricated items.

(b)	Materials and equipment specified shall conform standards herein established or equivalent if approved by the Engineer and tested satisfactorily. The approved material shall be certified by an approved independent agency.

(c)	Comparable materials, assemblies and systems of manufacturers other than those specified may be proposed subject to prior approval by the Engineer.

(d)		Electrical Works shall be designed, fabricated and supplied by a single manufacturer for all work under this Contract.

(e)	Substantial increase in size(s) of items of equipment or major components shall not be permitted, unless approved in advance by the Engineer.

(f)	The following basic requirements shall be satisfied:

- LED board size of approximately 192 cm x 96 cm with steel frame, Module 32x16, with pitch size P10 capable to display messages in both Nepali and English

- Proposed Solar Power System: 3610 watt.hr considering maximum three rows of text with font size of 16 cm.

- Solar Plant with 3 days autonomy: 	600A/48V Battery System, 6 solar panels (Senersun SSP72-285-C) and 3KVA Inverter (Axpert MKS-2K-48) with built-in MPPT charger of 50 amps

- Reliability / MTBF:	5 years (product life cycle)

- Voltage/Hz	220V / 50 Hz

(g)	The materials for construction of the supports shall conform the requirements of the following sections:

- Section 400: 	"Fencing"

- Section 2000: 	"Concrete for Structures"

- Section 2014:	“Reinforcement”

- Section 2200: 	"Structural Steel Work"

(6)	Testing and Commissioning

Testing and Commissioning of the completed installation of Street Lighting System (SLS) shall be made in accordance with the requirements of Clause 509(3): “Testing and Commissioning”.

(7)	Measurement

The measurement for this Section 3130 shall be made by the Number of TIB actually installed, satisfactorily tested according to Clause 509(3): “Testing and Commissioning” and conforming the requirements of Clause 9: “Test on Completion” of the Conditions of Contract, confirmation of proper function of the established system, satisfying all requirements of these specifications, and approved by Engineer. All materials, equipment and ancillary works or incidentals shall not be measured separately but shall be deemed included in the rates of the main item of TIB.

(8)	Payment

The works for the Traffic Information Board (TIB) system measured as described above shall be paid at the unit rates shown in the Bill of Quantities. This payment shall be the full and the final compensation to the Contractor as per Clause 112 including the cost for Design Review and Updating, provision of all materials, transportation, all operations required for construction and installation as shown in the Drawings and described in this Item including drawings, tests and all other ancillary works, and any incidental work needed to complete the work as per these Specifications and/or directed by the Engineer.

| Item No. | Description | Unit |
| --- | --- | --- |
| 3130 | Traffic Information Board (TIB) |  |
| 3130-1 | Traffic Information Board (TIB) for West Portal | No. |
| 3130-2 | Traffic Information Board (TIB) for East Portal | No. |

Add the following new Sections at the end of Sub-Section 3130: “Traffic Information Boards”:

New Sections:

- 6000 “Supply of O&M Equipment”

- 6100 “O&M Facilities and Buildings”

- 6300 “Toll Gate Facilities”

- 6500 “Transmission Line”

- 7000 “Roadside Service Station”

- 7500 “Steel Bridge for Flyover”

SECTION 6000: SUPPLYING OF O&M EQUIPMENT

6001	DESCRIPTION

(1)	Scopes

Without detriment to requirements of the Conditions of Contract, Clause 4.1: “Contractor’s General Obligations”, or other relevant conditions or specifications of the contractual documents, this item shall include the following:

- Supplying of the following O&M equipment:

- Pickup truck

- Lift truck

- Movable sign-board truck

- Light towing vehicle

- Heavy towing vehicle

- Small size fire fighting

- Ambulance car

- Weight scale (Mat type)

- Height restriction device (Gate type)

- Rescue machinery and tools

- Inspection machinery and tools

- Each item of equipment that is delivered by the Contractor shall be furnished with its manufacturer’s Operation & Maintenance Manual Book. And for the overall system operation manuals, the Contractor shall provide the whole system operation manual to be submitted to the Engineer as per the As-Built Documents, for onward transmission to the Employer.

- The Contractor shall prepare and submit the Instruction Manuals for operating and maintenance to the Engineer, including all modifications that should be required. The manuals shall be resubmitted before the date of issue of the Taking-Over Certificate.

- The schedule for procurement and mobilization of the O&M plant shall be submitted to the Engineer for review and approval. This approval shall be the basic condition for respective measurement and payment.

The works shall include furnishing all materials, and all ancillary works and any other incidental necessary to complete scopes of this Item in conformity with these Specifications, and as directed by the Engineer.

(2)	Workmanship and Standards

The works comprehended on supplying of the O&M equipment shall conform in all respects to highest standards of engineering, design, workmanship, this specifications and the latest revisions of relevant standards applicable in Nepal.

(3)	Submittals

The Contractor shall submit all information regarding to the equipment to be supplied including fabrication drawings, catalogues and manufacturer's specifications, fabricator, installation manuals, including details of procurement, and details at fabrication shop, etc. for the Engineer’s consent.

The Contractor shall allow the Engineer four (4) weeks from receipt of any submittal or re-submittal, for his review and acceptance prior to commencing the procurement for supplying the O&M equipment to which this submittal refers.

(4)	Basic Requirements

The equipment supplied for O&M works shall conform with the following basic requirements (all items shall be of the latest model as approved by the Engineer):

| Item No. | Description | Requirement |
| --- | --- | --- |
| 1 | Pickup truck | - 4WD Type - Turbo-diesel engine - Capacity: 3000 cc - Gearbox//No. of Gears: 5 Forward, 1 Reverse - Fuel Tank//Capacity: 65 liters - Performance Speed: 120 km/h - Gross Vehicle Weight (kg): 3000 - Seating Capacity: 4+1 |
| 2 | Lift-mounted truck | - Product Details: Operating Height = 10m - Material: Strength Steel - Other details: platform capacity = 225kg - 450 kg; and height ranges up to 8.76m, (working height 10.76m) - Mounted on a chassis of 7.5 ton |
| 3 | Movable sign board-mounted truck | - Turbo-diesel engine - Capacity: 3783 cc - Gearbox//No. of Gears: 5 Forward, 1 Reverse - Fuel Tank//Capacity: 160 liters - Max Speed (km/h): 110 - GVW / GCW (Kgs): 7490 - Kerb Weight (Kgs): 3690 - Payload (Kgs): 3800 - Moving Message Displays: 3.0m x 1.8m - Color: Amber |
| 4 | Light towing vehicle | - 2017 Tow Truck Wrecker, 8 Ton - Fuel Type: Diesel, Emission Standard: Euro 4 - Transmission Type: Manual - Foton 4*2 180HP - 8Ton road recovery wreck truck with 2 winch 8ton - Unload weight: 3600kg, Engine: futon cummins |
| 5 | Heavy towing vehicle | - 2017 Tow Truck Wrecker, 30 Ton - Fuel Type: Diesel, Emission Standard: Euro 3 - Transmission Type: Manual - Horse Power: 270HP - Maximum under-pulling, capacity: 30000kg - Maximum hoisting-pulling capacity: 24000kg |
| 6 | Small size fire fighting | - 4WD Type - Provided with Hydraulic Lift Equipment for Pump - Gross Vehicle Weight (kg): 1220kg - Pump Capacity: 0.6 m3/min - Seating Capacity: 2 |
| 7 | Ambulance car | - Engine: inline 4-cylinder, DOHC - Cylinder capacity: 2.694 cc - Fuel: Gasoline - Fuel tank capacity: 70 litters |
| 8 | Weight scale (portable weigh-pad: Mat type) | - The weigh pads shall be of a complete portable weighing solution for load control - Weigh pads shall not be used as weighbridge to invoice weights - Each pad shall have a capacity of 15Ton. Calibrated as 30ton x 10kg. - These pads don’t work on abnormal sized vehicles. |
| 9 | Rescue machinery and tools | (as approved by the Engineer) |
| 10 | Inspection machinery and tools | (as approved by the Engineer) |

6002	MEASUREMENT

The works for this item shall be measured respectively in accordance with the specified units established here below of the specific units (each) provided and tested in place as indicated in the Bill of Quantities and the Drawings.

6003	PAYMENT

The works for supplying the O&M equipment shall be paid under this Section at the contract unit rate which shall be the full and the final compensation to the Contractor as per Clause 112 including the related cost of all materials, all operations required for supplying the equipment as described in this Item and incidentals needed to complete the work as per these Specifications and/or directed by the Engineer.

| Item No. | Description | Unit |
| --- | --- | --- |
| 6000 | Supplying of O&M Equipment |  |
| 6000-1 | Pickup truck | No. |
| 6000-2 | Lift-mounted truck | No. |
| 6000-3 | Movable sign board-mounted truck | No. |
| 6000-4 | Light towing vehicle | No. |
| 6000-5 | Heavy towing vehicle | No. |
| 6000-6 | Small size fire fighting | No. |
| 6000-7 | Ambulance car | No. |
| 6000-8 | Weight scale (portable weigh-pad: Mat type) | No. |
| 6000-9 | Rescue machinery and tools | PS |
| 6000-10 | Inspection machinery and tools | PS |

SECTION 6100: O&M FACILITIES AND BUILDINGS

6101	DESCRIPTION

This work shall include the Design Review and Updating by the Contractor based on the Conceptual Design provided by the Employer. The report of the Design Review shall include also all detailed drawings, applicable technical specifications and requirements for construction and materials.

This works shall include all the necessary civil works, water supply and sanitation works, and electrification works as per the specification, the Drawings and instruction by the Engineer.

The works shall consist of the following:

West Portal: Block-1, Block-2 and Block-3

- O&M office building

- Garage

- Power house building

- Water tank

- Security guard office building

East Portal: Block-1, Block-2 and Block-3

- O&M office and power house building

- Warehouse and staff canteen building

- Police, toll and emergency staff office building

- Water tank and pump house

- Fuel tank

- Security guard office building No.1

- Security guard office building No.2

The works shall include furnishing all materials including all required for flooring, doors, windows, roofing, waterproofing, etc., all for construction of the above referred buildings and systems, and all ancillary works and any other incidental necessary to complete the Works in conformity with the Drawings and these Specifications, or as directed by the Engineer.

6102	STANDARDS

The works for design’s verification and construction of O&M Facilities and Buildings shall conform in all respects to highest standards of engineering, design, workmanship, this specifications and the latest revisions of relevant standards applicable in Nepal (Specifications of Building Construction Works and Nepal National Building Code NBC206: 2003 - Architectural Design Requirements)

6103	CONSTRUCTION METHOD, DESIGN REVIEW AND UPDATING

Before commencement of the works, the Contractor shall prepare and submit to the Engineer, for review and approval, a construction method statement and the detailed report of the Design Review and Updating for O&M Facilities and Buildings. These documents shall be attached to the Program which requirements are described in Section 116: “Programme”.

6104	MEASUREMENT

The works for this item shall be measured in “lump sum” but summarized by respective Item No. in accordance with the specified works completed in place as indicated in the Bill of Quantities and the Drawings including all subsidiary items related to relevant sections of these specifications.

6105	PAYMENT

The works for the construction of O&M Facilities and Buildings, as measured above, shall be paid under this Section at the contract unit rate which shall be the full and the final compensation to the Contractor as per Clause 112 including the cost for Design Review and Updating, provision of all materials, all operations required for construction as shown in the Drawings and described in this Item including drawings, tests and all other ancillary works and any incidental work needed to complete the work as per these Specifications and/or directed by the Engineer.

| Item No. | Description | Unit |
| --- | --- | --- |
| 6100 | O&M Facilities and Buildings |  |
| 6100-1 | West portal side O&M office building | sq.m |
| 6100-2 | West portal side garage | sq.m |
| 6100-3 | West portal side power house building | sq.m |
| 6100-4 | West portal side water tank | lump sum |
| 6100-5 | West portal security guard office building No.1 | sq.m |
| 6100-6 | West portal security guard office building No.2 | sq.m |
| 6100-7 | East portal side O&M office and power house building | sq.m |
| 6100-8 | East portal side warehouse and staff canteen building | sq.m |
| 6100-9 | East portal police, toll and emergency staff office building | sq.m |
| 6100-10 | East portal side water tank and pump house | lump sum |
| 6100-11 | East portal side fuel tank | lump sum |
| 6100-12 | East portal security guard office building No.1 | sq.m |
| 6100-13 | East portal security guard office building No.2 | sq.m |
| 6100-14 | O&M office facility, furniture and tools | PS |

SECTION 6300: TOLL GATE AND FACILITIES

6301	DESCRIPTION

(1)	General

(a)	This work consists of the Design Review & Updating, supply, construct and install of toll gate islands, booths, roof, lighting, name board, lane IP incident cameras, traffic lights, automatic lane barriers and toll gate equipment including civil works and works for electricity installation conforming to the Specification, the Drawings and instruction by the Engineer.

(b)	This shall include all works needed for furnishing and installing all materials and equipment, manufacturer’s manual/instructions, latest accepted standards of the industry and the Drawings, or as directed by the Engineer.

(c)	The locations of toll gate facilities shown in the Drawings are approximate, and the exact locations may be modified to fit in with surrounding structures. Any such modifications shall be made in accordance with the Engineer’s instructions.

(2)	Scope of Work

The scope of this work consists of supply and delivery to the site, installation, testing and commissioning as required of all materials, equipment and labor for the works of the following:

    - Toll Gate Facilities

Roof, Toll Island, Toll Booth, Related Equipment

(3)	Quality Assurance

(a)	For the actual fabrication, installation, and testing of the work described in this Specification Section, the Contractor shall use only thoroughly trained and experienced personnel who are completely familiar with the requirements for this work and with the installation recommendations of the manufacturers of the specified items.  In acceptance or rejection of the installed electrical system, no allowance will be made for lack of skill on the part of the installers.

(b)	Installers shall hold the relevant valid certificates complying with the regulations of the concerned Nepal’s authorities.

(4)	Drawings and Documents

The Contractor shall refer to all relevant Drawings and ascertain for himself the location and routes of all other utility services so as to maintain adequate clearance between the services. The Drawings issued for construction indicate the general arrangement of the works, and the Contractor shall provide working drawings (in accordance with Sub-Section 117: “Contractor’s Drawings”) showing the exact routes of all pipes, overhead and underground installations, the exact run of conduits and trunk-connection, the location of manholes or control holes, draw-in and junction units, the number and size of cables in each conduit, etc.

The Contractor shall submit a schedule showing lists of intended events and times such as the date on which concreting in different sections will take place, together with the submission of the working drawings.

Upon completion of the work, and as a condition of its acceptance, the Contractor shall provide to the Employer copy of instructions/manuals for the operation and maintenance of all equipment and a parts list sufficient for the ordering of parts.

6302	MATERIAL

(1)	Standards and Regulations

(a)	The work covered by this Contract shall be carried out in accordance with the regulations issued by the local authority and with the following applicable standards and codes:

  - IEC (International Electrotechnical Commission)

  - JIS (Japanese Industrial Standards Committee)

  - JEAC (Japan Electric Association Code)

  - JEM (Standards of the Japan Electrical Manufactures’ Association)

(b)	The Contractor shall carefully examine at his own expense all the regulations issued by the local power company, and ensure that selected materials and method of installation are in accordance with such regulations.

The Contractor shall include in his unit price for any reasonable minor changes, modification, or supplementation of engineering drawings and specifications to ensure conformance with local regulations.

- General Requirements

All works shall be done under the supervision of the Engineer and in accordance with the Drawings and the requirements herein described.  Any changes made thereof shall only be made upon obtaining previous consent from the Engineer.

All electrical installations shall be done in accordance with the applicable ordinances, rules and regulations or standards of the local power company and with requirements of the relevant authority. The construction works shall be carried out under the supervision of a licensed engineer of the Contractor.

The materials shall comply with the following general requirements:

(a)	All the materials used in the works must be brand-new which satisfies the requirements of quality and performance denoted in the drawings. However, this does not apply to the case of temporal construction.

(b)	The Contractor must submit the documents certifying the quality and performance of the materials to the Engineer. However, in case of using materials which has been specified to use JIS in the drawings and approved by the Engineer in advance to use the materials marked “JIS”, submission of this documents can be omitted.

(c)	The Contractor must report to the Engineer in each time of carrying materials into the site. However, this does not apply to the case where the Contractor obtains approval from the Engineer.

(d)	Material Inspection

- Material inspection shall be carried out by each type of materials when these materials are carried into the Site.

- As the result of above inspection, the same types of material which passed the inspection can generally change to sampling inspection. However, this does not apply to the case where the Engineer gives instructions.

- Carrying material into the site which is designated as an inappropriate material due to the deterioration, etc. by the Engineer must be conveyed out from the site immediately.

(e)	Testing Associated with Material Inspection

Without detriment to the requirements of Sub-Clause 7.4 “Testing” of the Conditions of Contract, or other relevant requirements of these Specifications, the Contractor shall perform the following:

- In case of conducting material testing to certificate its quality or performance, test shall be performed based on the designated method in the Drawings. In the event that there is no designation in the Drawings, the test shall be performed based on the method which is approved by the Engineer.

- The Contractor shall submit the testing plan to the Engineer before the testing.

- The test shall be performed at the appropriate site such as Examination Institution, construction site, etc. For the selection of testing site, approval by the Engineer shall be required.

- Generally, the test shall be performed in the presence of the Engineer. However, this does not apply to the case where the Contractor obtains approval from the Engineer.

(f)	Material Storing

- Material which is carried into the site shall be stored not to be deteriorated till the time to use in the construction work.

6303	CONSTRUCTION

(1)	Construction Works

(a)	Construction works shall be conducted according to the Drawings, work plan, method statement, working/shop drawings.

(b)	In case of the work which shall make a hided area such as Concrete placing work, the Contractor must not conduct the work till required inspection is completed. However, this does not apply to the case where the Contractor obtains previous approval from the Engineer.

(2)	Inspection of Construction Works

(a)	When the instruction from the Engineer received, the inspection of the construction work shall be conducted.

(b)	As the results of the above inspection, the same types of material and work method which passed the inspection of construction work can generally change to sampling inspection. However, this does not apply to the case where the Engineer gives instructions.

(3)	Equipment Installation Works

(a)	Equipment installation work must be carefully performed in terms of horizontal and vertical position so as not to interfere with its performance after the installation of the equipment due to strain, etc.

(b)	Used ironmongery shall be manufactured after the adequate site survey.

(4)	Management of Construction Site

(a)	Appropriate curing shall be performed not to deface existing facilities, already completed or installed facilities, etc.

(b)	When construction works are completed, the Contractor shall conducts clean up the site both inside and outside of the architecture.

6304	WORKS’ REQUIREMENTS FOR TOLL FACILITIES

Construction and installation of the Toll Facilities shall comply with the following requirements of the relevant sections of these Specifications:

| Works | Material / Description | Specification and Requirements |
| --- | --- | --- |
| Concrete and Reinforcement for main structure | - Concrete Class B - Concrete Class D | - Section 2000: “Concrete for Structures” main structure: Concrete Class-B; other ancillary works: Concrete Class-D. |
|  | - Reinforcement Bar | - Section 600: “Materials and Testing of Materials” - Section 2014: “Reinforcement” |
| Asphalt Concrete Pavement | - Asphalt Concrete Wearing Course (50 mm) | - Section 1300: “Bituminous Surface and Base Course” |
|  | - Asphalt Concrete Binder Course (50 mm) |  |
|  | - Asphalt Treated Base (ATB) Course (100 mm) |  |
| Concrete Pavement | - Jointed Reinforced Concrete Pavement (JRCP) [t300mm] (Concrete Class P) | - Section 2050: “Concrete Pavements” |
| Steel Roofing and columns | - Structural steel for Frame structure for roof and columns | - Section 600: "Materials and Testing of Materials" - Section 2200: “Structural Steel Work” - Steel SM90YA (or equivalent); SM400A (or equivalent); |
|  | - High Tension Bolts (HTB) | - Section 2200, IS: 961 (HTB) with minimum tensile strength of 58 kg/mm2 |
|  | - MS Purlins and MS Pipes - Corrugated metal sheet | - SS400, SN400B, SN400C, STKR400: JIS or equivalents - SSC400: JIS or equivalents |
|  | - Painting | - Sub-Section 2205: “Painting” |
|  | - Others | - Design Review shall be made by Contractor to confirm the capacity of steel frame before starting factory production. - Testing of Welding Section: UT (Ultrasonic Test) AOQL: Shop Welding 4.0% Field Welding 4.0% |
| Guard Rail | - φ100 GI pipe Guard Rail - Other Steel Guard Rail | - Sub-Section 3105: "Railing" |
| Lighting Facility | - Illuminance: (a) Under Roof = 150(Lx) at road surface level (b) Inside Toll Booth = 500(Lx) at 85cm from floor | - JIS: Japanese Industrial Standard - JEC: Japanese Electro-technical Committee - JEM: The Standards of the Japan Electrical Manufacturer's Association - IEC: International Electro-technical Commission (authorized standards) - Nepal Electricity Authority - Nepal National Building Code NBC207: 2003 - Electrical Design Requirements |
| Conduit and Wiring Works | - Material for Conduit and Wiring, including theφ150mm PVC pipes, etc., as approved by the Engineer | - Specifications: same than above - Terminal processing to the cable termination and jointed perfectly. - All cables must be marked manufacture’s name, year and month of manufacture and testing laboratory on outer surface of the cable or wire. Name plate, which indicates connected place from/to, must be attached on the cable. - Prior to the construction work, the Contractor shall conduct the calculation of conduit and wiring and obtain the approval by the Engineer. - Cable in handhole must secure the extra length. |
| Distribution Panel | - Material for Panels as approved by the Engineer | - Specifications: As approved by the Engineer - Work space for repairing and maintenance must be secured sufficiently. - The wiring diagram must be provided on the inside of panel door. - Prior to the construction work, the Contractor shall conduct the calculation of piping and wiring and obtain the approval by the Engineer. |
| Truck Scale Facility | - Weight scale (portable weigh-pad: Mat type) (to be used on some particular cases only) | - As described in Section 6000: “Supplying of O&M Equipment” |
| Single Toll Booth | - W=1400mm x L=3300mm x H=2400mm | - Specifications of Building Construction Works and Nepal National Building Code NBC206: 2003 - Architectural Design Requirements |
| Lane IP incident Camera, Traffic Lights, Automatic Lane Barrier, new Jersey barriers | - As approved by the Engineer | - The Contractor shall submit complete details of the material from the suppliers including catalogs and specifications |

The works for providing and installation of the Polyethylene tank crash barriers shall be in accordance with the requirements of Section 1509: Polyethylene Tank Crash Barrier.

6305	MEASUREMENT

The quantity of Toll Gate Facilities described in this item shall be measured in Lump Sum of each Toll Gates Facilities provided for each portal (Eastern and Western side) furnished and installed in accordance with this Specification Section, as shown in the Drawings, and in accordance with the instructions of the Engineer.

The works for providing and installation of the Polyethylene tank crash barriers shall be measured and paid under the items of Section 1509: Polyethylene Tank Crash Barrier.

6306	PAYMENT

The Toll Gates Facilities measured as above shall be paid at the contract unit rate which shall be the full and the final compensation to the Contractor as per Clause 112 including cost for all materials labour, equipment, tools, and all incidentals to complete the work in accordance with the Drawings and Specifications, and as directed by the Engineer.

The unit rates shall deemed include, inter alia and in addition to other items previously stated (e.g. concrete, reinforcement steel, structural steel works, etc.), the following:

- Construction of Toll islands

- Providing and installation of lane IP incident camera, traffic lights, automatic lane barrier

- Construction and installation of booths

- Handholes and respective connections with other handholes and with the main “Operation Center” of the Toll-Gate Office

- Drainages

- Equipment, materials, labor, transportation, installation, and testing

- Related works, which are not specifically described in this section, such as excavation and backfilling, markers for underground cables, buried cable protectors, etc. shall be deemed included in the rates of pay items specified hereunder.

| Item No. | Description | Unit |
| --- | --- | --- |
| 6300 | Toll Gate and Facilities |  |
| 6300-1 | West portal side toll gate | lump sum |
| 6300-2 | East portal side toll gate | lump sum |

SECTION 6500: TRANSMISSION LINE

6501	DESCRIPTION

These works shall include the following:

Supplying and Installation of Transmission Line (11kV), XLPE Power Cable.

Cable Termination Kit and Joint Kit for XLPE

Supplying and Installation of HDPE Pipe and PPR Pipes

Sub-station and Transmission Line:

  - Vacuum Circuit Breaker (VCB) Switch Gear - 2sets - 11kV

  - Adaptation Panel - 2sets

  - Power Cable (11kV) for underground cabling - 1.95 km length

  - Civil Works for Cable Laying : RCC Channel Box (Pre-cast)

For East Portal Switch Yard:

  - Transformer (11kV/400V), Dry, Indoor Type, Capacity 750kVA x 2 nos.

  - Bus coupler - 2 nos.

  - Air Circuit Breaker (ACB) Switch Gear

Supplying and Installation of Ground Rods and Clamps, and Grounding Conductor.

Ancillary Works and Testing

The works shall include furnishing all materials, construction or installation of the above referred items, and all ancillary works and any other incidental necessary to complete the Works in conformity with the Drawings and these Specifications, or as directed by the Engineer.

This work shall also include the Site’ Survey for confirmation and preparation of drawings, and a full Design Review and Updating by the Contractor based on the Conceptual Design provided by the Employer. The report shall include also all detailed drawings, applicable technical specifications and requirements for construction and materials.

6502	STANDARDS

All works under this Contract shall comply with the applicable regulations and requirements of Statutory Authorities.

All equipment and materials shall conform to the requirements of the codes and standards specified herein and the Drawings, or certified by other internationally recognized standard institutions or testing laboratories that maintains periodic inspection of production of listed equipment and materials meet nationally recognized standards or has been tested and found suitable for use in the specified manner.

Certification of the system shall be provided by the Contractor.

Codes and Standards: Unless otherwise indicated in the Drawings or in the Specifications, the design, materials, manufacture, testing, inspection and performance of all electrical and electromechanical equipment shall comply with the latest revisions of relevant standards listed in Section 113: “National Specifications” and the following standards:

JIS:	Japanese Industrial Standard

JEC:	Japanese Electro-technical Committee

JEM:	The Standards of the Japan Electrical Manufacturer’s Association

IEC:	International Electro-technical Commission (authorized standards)

When local regulatory bodies, have jurisdiction over installation, the following Nepalese standards shall be applied:

Nepal Electricity Authority

Nepal National Building Code NBC207: 2003 – Electrical Design Requirements

6503	CONSTRUCTION METHOD AND DESIGN REVIEW AND UPDATING REPORT

Before commencement of the works, the Contractor shall prepare and submit to the Engineer, for review and approval, a construction method statement and the detailed report of the Design Review and Updating. These documents shall be attached to the Program which requirements are described in Section 116: “Programme”.

6504	GENERAL OPERATING CONDITIONS

The following are the general operating conditions for the system:

Ambient temperature:	-5°C to 45°C

Altitude: 	Approximately 1420 m above MSL

Humidity:	100% (max.)

Main voltage: 	11 kV for Distribution HT and 400/230 V for LT

System frequency	50 Hz

Note: The variation of frequency and voltage for design and operation is as per actual system operation i.e. ± 5 %.

6505	MATERIALS AND CONSTRUCTION REQUIREMENTS

(A)	XLPE CABLE & ACCESSORIES

(A1)	11 kV XLPE Power Cable

(1)	Cable Construction Details

(a)	The XLPE insulated cables shall conform to the requirements of IEC 60502-2 (applicable clauses only) for construction and IEC 60840 and other relevant standards for testing. The terminating accessories shall conform to IEC 60840/ IEC 62067 (as applicable). The offered cables and its terminating accessories shall be compatible with each other.

(b)	The 11kV XLPE cable shall be three core, armoured, stranded, compacted  Aluminum conductor of 300 sq.mm size, core screening by a layer of semiconducting XLPE, treeing resistant XLPE insulation, insulation screening by a layer of semiconducting XLPE. The core screening, insulation and insulation screening to be triple extruded and dry cured. Helically wound copper wire screening with equalizing tape, shall be provided on each conductor. Each core shall have a Polyethelene sheath. The over-sheath shall be black HDPE. Bidder may offer necessary layers such as water blocking, separation tape, binder tapes etc. additionally as per their manufacturing practices for meeting required performance of the offered cable.

(c)	The cable shall be suitable for installing in the climate conditions (as specified in this chapter)

(d)	Cable shall be designed to withstand all mechanical, electri¬cal and thermal stresses under steady state and transient operating conditions.

(e)	Cable sheath shall be embossed “ELECTRIC CABLE 11000V”. The size of the lettering shall not less than 15% of the approximate cable diameter and shall not be larger than 13mm. The gap between the end of one set of embossed characters and the beginning of the next set shall be not greater than 250mm. In addition the manufacturer and year of manufacture shall be embossed on the cable at regular intervals and shall not affect the spacing of the other characters.

(f)	Each core shall be identifiable throughout the cable either by colour or numbering.

(g)	Repaired cables shall not be accepted.

(h)	Allowable tolerance on the overall diameter of the cables shall be + 2 mm.

(i)	Conductor: The conductor shall be of Aluminium as specified in the Bid Price Schedule (BPS). The shape of conductor shall be compacted having high compactness and smooth surface finish.

(j)	Conductor Screen: The conductor screen shall consist of extruded semi-conducting XLPE. Semi-conducting separator tapes may be applied between conductor and the extruded semi-conductor XLPE. The conductor screen (non-metallic semi-conductive) shall be extruded in a single one-time process to ensure homogeneity and absence of voids. The aluminum conductor shall comply with requirements specified in relevant IEC with latest amendments.

(k)	Insulation: The conductor screen shall consist of extruded semi-conducting tape or insulation. Semi-conducting separator tapes may be applied between conductor and the extruded semi-conductor insulation. The conductor screen (non-metallic semi-conductive) shall be extruded in a single one-time process to ensure homogeneity and absence of voids.

(l)	Insulation Screen: The insulation screen shall consist of extruded semi-conducting XLPE. Suitable bedding tapes shall be applied over the extruded semi-conducting XLPE in combination with 1 non-magnetic metallic shield.

(m)	Insulation: and Semi Conducting SCREENs The XLPE insulation and semi conducting screens shall be triple extruded and dry 	cured.

(n)	Moisture Barrier: Longitudinal water barrier:

The longitudinal water barrier shall be applied over insulation screen by a layer of non-woven synthetic tape with suitable water swellable absorbent.

(o)	Metallic Screen: The metallic screen shall be of plain copper wires, helically applied over the radial moisture barrier. A binder tape of annealed plain copper shall be applied in the form of an open helix over the copper wire screen. The combination of the metallic sheath (lead sheath) in combination with wire screen shall be designed to meet the requirement of the system short circuit rating as specified in the bidding documents. The copper wire screens shall be applied to each core.

(p)	Outer Sheath: The outer sheath shall consist of extruded black colored HDPE. The outer sheath shall be suitably designed by the addition of chemicals in the outer sheath for protection against termite and rodent attack.

Armouring:

The armour of cables shall consist of aluminum wires or strips. The armoring shall be applied such that the minimum area of coverage shall be 90% and the gap between any two armour strips/ wire shall not be more than the width of strip/ diameter of armour wire. The galvanized steel strips/wire shall comply with the requirements of IEC with latest amendments.

In case of Single core cable armouring shall be of Non-magnetic material with dimensions as per relevant IEC with latest amendments. No tolerance on the negative side shall be acceptable.

(q)	Rating: The contractor/ manufacturer shall declare current rating of cable for maximum conductor temperature of 90°C under continuous operation and 250°C during short-circuit condition. The contractor/ manufacturer shall also declare over load curve with duration for conductor temperature of 105°C. A complete set of calculation made in arriving at the current rating shall be furnished, for laying condition envisaged under the project, during detailed engineering for Employer/Employer’s reference.

(r)	Cable Jointing Accessories:

- The cable jointing accessories shall include all the straight through joints. Bidder shall arrange all special tools and tackles required for making these joints at his own cost. Unless specified separately in BPS, cable end terminating kits shall be deemed included as part of cable jointing accessories.

- The joints shall be suitable for tropical conditions as specified in this chapter. Straight joints shall be heat shrink type with compression ferrules. Each joint shall include all necessary material and components for effecting a reliable and durable joint. Particularly importance is attached to the proper provision of stress relieving materials and tubing.

- The straight through joints and cable end terminations shall be of proven design and should have been type tested as per relevant IEC standard. A list shall be furnished indicating supply of such cable jointing accessories which are successfully operating in other projects.

- The detailed description on jointing procedure shall be furnished during detailed engineering.

- The cable end terminations shall be of anti-fog type and shall be of Polymeric type suitable for withstanding the climatic conditions with required Creepage distance as specified in bidding documents. The cable end terminals for terminating the cables shall be complete with accessories & fully compatible with the cables to be supplied. The terminations shall also be capable to withstand mechanical forces during normal and short circuit operations.

(2)	Cable Drums

(a)	Cables shall be supplied in wooden/steel drums of heavy construction of suitable size and packed conforming to applicable standards. Maximum drum length shall be 500m.

(b)	Standard drum lengths for manufacturing shall be finalised during detailed engineering. Each drum shall carry the manufacturer's name, the employer's name, address and contract number and type, size and length of the cable, net and gross weight stencilled on both sides of drum.  A tag containing the same information shall be attached to the leading end of the cable.  An arrow and suitable accompanying wording shall be marked on one end of the reel indicating the direction in which it should be rolled.

(c)	Packing shall be sturdy and adequate to protect the cables from any injury due to mishandling or other conditions encountered during transportation, handling and storage. Both cable ends shall be sealed with heat shrink caps so as to eliminate ingress of water during transportation and erection.

(3)	Tests on Cables

All XLPE insulated cables shall conform to all Type, Routine and Acceptance tests listed in the relevant IEC & shall submit the type test reports for Employer’s approval. If specified in Section-Project, Type tests shall be carried out on the EHV cable as per relevant standard.

(4)	Tests on Accessories

Contractor shall submit type test reports for accessories, as per IEC 60840:1999/ IEC 62067 for Employer’s acceptance. Contractor shall submit type test reports as per respective Technical Specification, GTR for Employer’s acceptance.

(5)	Tests After Installation

All tests on cable system as prescribed in IEC 60840:1999/IEC 62067 (as applicable) shall be performed after installation in the presence of Employer. The result of the tests shall be well documented and submitted with the as built drawings.

(6)	Laying and Installation

(a)	The bidder is advised to visit the site and acquaint themselves with the topography, infrastructure etc. The contractor shall be fully responsible for providing all equipment, materials, system and services specified or otherwise which are required to complete the erection and successful commissioning of XLPE cables in all respects. In general the cable laying operation shall be in accordance with best internationally accepted practices and procedures and in accordance to manufacturers recommendation.

(b)	Cable drum shall be positioned in such a way that the axle of the drum is perpendicular to the cable trench. The cable shall come off the drum always at the lower part in order to minimize stress. If the laying is done by the winch it has to be equipped with a tension release in order to avoid any excess stresses on the cable. The cable tension as well as the continuity of the laying operation shall be monitored by instruments and shall not exceed limits given by the manufacture’s recommendation. Winching equipment which doesn’t fulfil above requirements will be rejected.

(c)	Cable rollers shall be used to roll off the cables from the drums. The rollers shall ensure that the cable does not scuff on the ground during pulling operation. The rollers must be cleaned and lubricated properly.

(d)	Cables shall be laid in the cable duct throughout the route. Further, as per requirement of the field, the cables shall also have to be laid in the followings (with prior approval of employer):

                - In ducts

                - In HDPE ducts (wherever necessary)

                - In air at terminations

                - At varying depths due to obstructions

                - As per approved drawings

(e)	At places where the cables cross roads, the cables can be laid in HDPE pipes of adequate strength.

(f)	Concrete trenches with precast covers may be used in exceptional cases in smaller portions, wherever bending of cables are involved and HDPE pipes can’t be laid.

(g)	The arrangement of laying the cable en-route shall be submitted by contractor during detailed engineering for Employer’s acceptance. The Bidder shall also submit inception report indicating detail description of procedure (Method Statement for Construction) to be deployed for laying of underground and fibre optic cable. The contractor shall propose optimum cable sizing based on load current, voltage drop, short circuit current and protective device requirement and also to maintain reliable ring main 11 kV supply.

(h)	Any damage occurred to existing utility services during cable laying shall be brought into notice of relevant authorities and rectified immediately.

(7)	Trenching

(a)	Trenches shall in general, unless impeded, be in accordance with the dimensions indicated in the cable installation specification included with this document.

(b)	The cable trench work involves earth excavation for cable trench, back filling and removal of excess earth from site. The work site shall be left as clean as possible.

(c)	Wherever possible sidewalks shall be used for the installation of cable. The trench shall be excavated using manual /mechanical modes as per field conditions. Most main roads are of asphalt surface and some of the roads with cement concrete surface. The sides of the excavated trenches shall wherever required, be well shored up.

(d)	Where paved footpaths, black topped roads or concrete slabs are encountered, the pavement slabs, roads shall be properly reinstated in original condition or better. Identification markers of other services shall be properly stored and reinstated. The excavated material shall be properly stored and filled back within 24 hours of excavation to avoid obstruction to public and traffic movement.

(e)	Suitable barriers should be erected between the cable trench and pedestrian/ motorway to prevent accidents. The barriers shall be painted with yellow and black or red and white coloured cross stripes. Warning and caution boards should be consciously displayed. Red lights as warning signal should be placed along the trench during the nights.

(f)	The bottom of the excavated trench should be levelled flat and free from any object which would damage the cables.  Any gradient encountered shall be gradual. All cables shall only be laid directly onto the bottom of the trench, if the surface is unlikely to cause damage to the outer sheath. Where a sand bedding material needs to be used for the base of a trench, the depth of the trench shall be increased by the appropriate amount to ensure that the required minimum depth of cover is achieved.

(g)	Following installation, cables shall be blinded to a compacted depth of 100mm above the cable or cable joint with soil taken from the excavated material or imported material.  The blinding shall be free from materials that may damage the cable. Imported material for blinding (sand) shall only be used if all the excavated material is unsuitable.

(h)	The Contractor shall remove from site all rubbish and salvaged materials. The whole area shall be left in a neat and tidy condition to the satisfaction of Employer.

(8)	Trefoil/Flat Formation

Cables shall be laid in flat formation for entire route.  The Contractor shall submit drawings and arrangements for Employer’s approval. If a single three core/triplex cable is installed, then the cable shall just be laid direct. If two or more 3 core cables are laid in the same trench then each cable needs to be horizontally separated by a minimum of 400mm.

(9)	Cable Handling

The inspection of cable on receipt, handling of cables, paying out, flaking, cushioning with sand or sieved compacted soil, back-filling, reinstatement of road surfaces, providing and fixing joint markers, route indicators, precautions of joint holes, sump holes and all necessary precautions that are required shall be carefully planned and in accordance with acceptable standard practices/statutory requirements.

(10)	Damage to Property

The Contractor shall take all precautions while excavation of trench, trial pits etc., to protect the public and private properties and to avoid accidental damage. Any damage so caused shall be immediately repaired and brought to the notice of the concerned and to the Employer. The contractor shall bear all responsibilities and liabilities and shall bear all costs of the damages so caused by him or by his workman or agents.

(11)	Cable Route Markers/Cable Joint Markers

Permanent means of indicating the position of joints and cable route shall be fabricated supplied and erected as per approved drawings.

Markers provided shall be as per the field requirement, if the route passes through open fields, markers should be conspicuously visible and above ground surface.

The marker should incorporate the relevant information such that the name of the Employer, voltage, circuit and distance of cable from the marker.

(12)	Depth of Laying of Cables

Depth of laying shall be as per drawing enclosed with Specification. Laying at varying depths due to obstructions/site conditions may be accepted in extreme cases with prior approval of Employer during detailed engineering.

(13)	Paying Out the Cable

The excavated cable trench shall be drained of all water and the bed surface shall be smooth, uniform and fairly hard before paying out the cable. The cable shall be rolled in the trench on cable rollers, spaced out at uniform intervals. The paying out process must be smooth and steady without subjecting the cable to abnormal tension. The cable on being paid out shall be smoothly and evenly transferred to the ground after providing the cushion.  The cables shall never be dropped. All snake bends shall be straightened. Suitable size cable stocking pulling eye shall be used for pulling the cable. While pulling the cable by winches or machines, the tension/ loading shall be monitored by tension indicator and shall not exceed the permissible value for the cable. The cable laying shall be performed continuously at a speed as recommended by manufacturer.

The cable end seals shall be checked after laying and if found damaged shall immediately be resealed.  Sufficient number of heat shrinkable cable end sealing caps shall be stocked at site stores for testing and jointing work. The integrity of the outer sheath shall be checked after the cable is laid in position.

(14)	Sand Bedding

The cable shall be completely surrounded by well-compacted cable sand to such a thickness and of such size that the cable is protected against damage (applicable where cables are not to be laid in pipes).

(15)	Snaking

Snaking shall be done at necessary places recommended by manufacturer with prior approval of Employer.

(16)	Thermal Backfill

If specifically mentioned in Section-Project, Thermal Backfilling shall be carried out based on the evaluation of soil thermal resistivity along the cable route and after approval from the Employer the contractor shall design, specify, supply, lay and monitor the installation of thermal backfill surrounding the cables. Thermal back fill shall be of thermal resistivity of 1.20 K.m/W or better.

(17)	Immediate Envelope to Cable

The option on the use of the material that immediately envelopes the cable viz., thermal backfill or sand or sieved native soil rests with the Employer/Employer.  The contractor shall seek prior approval on the use of the envelop material from the Employer before execution of the works.

(18)	Back Filling

Normally back filling shall consist of the material earlier excavated. However, bigger stones or pieces of rock shall be removed.

(19)	Warning Tape

A pre-warning, Red colour plastic/ PVC tape, of at least 250 mm wide 100 microns thick, shall be laid at approx. 0.4 m above the cable specified depth, throughout the cable route.  The tape shall carry the legend printed in black continuously as under Caution; Employer, Voltage Class Of Cables. Protection concrete tiles shall also be used all along the route of the cable to protect the cables.

(20)	Prevention of Damage due to Sharp Edges

After the cables have been laid in the trench and until the cables are covered with protective covering, no sharp metal tool shall be used in the trench or placed in such a position that may fall into the trench. Straight and curved rollers used shall have no sharp projecting parts liable to damage the cable. While pulling through pipes and ducts, the cable shall be protected to avoid damage due to sharp edges. The cables shall never be bent, beyond the specified bending radius recommended by the manufacture.

(21)	Road, Rail & Canal Crossings

(a)	The road cutting, whether cement concrete asphalt or macadam road surface; canal crossing shall be taken after obtaining approval for cutting/crossing from the concerned authorities i.e. office of the municipality, traffic police, telephone authorities, roads and water supply department etc., and work should be planned to be completed in the shortest possible time.  Where necessary the work shall be planned during night or light traffic periods.  HDPE pipes shall be used for crossing. HDPE pipes diameter shall be as specified in BPS.

(b)	Trenchless Digging: It is envisaged that trenchless digging shall be used for crossing the National highways, and Canals etc. and the same shall be in the scope of bidder.  Trenchless digging shall also be used where the concerned authorities do not permit open cut method and it is essentially required to carry out for installation of underground cables.  The trenchless digging methods shall generally conform to ITU-T L.38. The various methods of trenchless digging such as hand/ manual auguring (up to 15m), impact moling (from 16m to about 40-50m), HDD (above 40-50m) shall be adopted based on the soil/site conditions and the requirement. The exact method for trenchless digging shall be finalised during detail engineering as per actual site/soil condition.  The equipment used for HDD shall be capable of drilling at least 100m at one go.  The contractor shall propose the exact methods and procedures for implementation of trenchless digging at various crossings taking into consideration the following guidelines, for approval by the Employer.

          - Excavation and backfilling of trial pits and verification of soil condition

          - Excavation of entry and Exit pits

          - Erection of drill machine for Drilling of pilot hole

          - Placement and driving hand augur

          - Placement and carrying out impact moling

          - Reaming and widening of bore holes in steps (if required)

          - Pulling of product pipe

(22)	Footpath Cutting

The slabs, kerbstones, on the roads shall be removed and reinstated without damage. The Contractor shall ensure that all surfaces make smooth junction with existing works. In case of damages they have to be replaced by new ones.

(23)	Reinstatement

After the cables and pipes have been laid and before the trench is backfilled all joints and cable positions should be carefully plotted and preserved till such time the cable is energized and taken over by the Engineer in charge.  The protective covers shall then be provided, the excavated soil riddled, sieved and replaced.  It is advisable to leave a crown of earth not less than 50 mm and not more than 100 mm in the centre and tapering towards the sides of the trench.

The temporary reinstatement of roadways should be inspected at regular intervals, more frequently in rainy season and immediately after overnight rain for checking settlement and if required the temporary reinstatement should be done.

After the subsidence has ceased the trench may be permanently reinstated and the surface restored to the best possible condition.

(24)	Manholes

Manholes shall be provided at every proposed joint location for jointing bays and as directed by the Employer. The bidder shall identify the location of the joint bays after carrying out detailed survey of the cable route and excavation of the trial pits.  The delivery lengths of the cables shall match the location.

The Contractor shall get inspected, by a representative of the Employer, all manholes before carrying out the backfilling.  Pipe & cable sealing, installation of joint box and cable service loops as per approved drawings shall be visually inspected and checked for tightness.

The contractor shall submit design and drawing of joint bay including manholes for withstanding a live load of 20 ton vehicle plus 30% for impact from moving vehicle.  The Contractor shall propose a suitable procedure for testing the manhole for approval by the Employer.  Manholes type approved by the Employer only shall be acceptable. The manhole shall include sufficient number of suitable entries. I seriously caution against using joint pits with manholes. They become a major source of maintenance headaches with no benefit to the utility. With XLPE cables and heat shrink jointing there is no reason to have permanent access to a joint position. Joint holes can be backfilled with sand and given extra mechanical protection above the sand and then completely buried. The important aspect is to make sure joint positions are recorded and if considered necessary to place a marker somewhere near the joint position.  Clearly it is best to avoid making joints in roadway, should this be unavoidable then extra depth and mechanical protection will prevent possible damage from traffic. In the event of cable faults, modern fault location techniques will enable accurate location of faults, therefore the will be no necessity to have access to any joints during fault finding. Should the fault location be in a joint, excavation will be a simple matter.

(25)	Tools and Plants

The successful bidder shall arrange, at his own cost, all necessary tools, plant and equipment to carry out the survey and cable installation work. The bidders are instructed to give all the details of equipment at their disposal, to carry out the work successfully and speedily.

(26)	Bending Radius

The cables shall be treated with according to manufacturer’s recommendation. Any overstress due to over-bending, excess pulling must be avoided. The minimum bending radius of XLPE insulated cables shall be 20XD where “D” means the Outer diameter of the cable.

(27)	Jointing and Termination of Cables

The cable jointing personnel and his crew shall have good experience in the type of joints and terminations that are used.  The jointing work shall commence as soon as two or three lengths of cables have been laid.  All care should be taken to protect the factory-plumbed caps/ seals on the cable ends. The cable ends shall be protected all the times with heat shrink end caps.

Jointing of cables in carriage ways, drive ways under costly pavings, under concrete or asphalt surfaces and in proximity to telephone cables and water mains should be avoided whenever possible.

Cable overlap of at least 1.5 m shall be allowed for making the joints.

The joint bay should be of sufficient dimensions to allow the jointers to work with as much freedom of movement and comfort as possible.  Sufficient space should be kept below the cable to be jointed.

The joints of different phases shall be staggered in the jointing bay.

(a)	Sump-holes

When jointing cables in water logged ground or under unforeseen rainy conditions, a sumphole should be made at one end of the joint bay, in such a position so that the accumulated water can be pumped or taken out by buckets, without causing interference to the jointing operation.

(b)	Tents/Covers

An enclosure or suitable protection cover shall be used in all circumstances wherever jointing work is carried out in the open, irrespective of the weather conditions.  Joints shall be made in as clean an environment as practicable.

(c)	Precautions Before Making a Joint

The cable end seals should not be opened until all necessary precautions have been taken to prevent circumstances arising out of rainy/ inclement weather conditions, which might become uncontrollable.

If the cable end seals or cable ends are found to have suffered damage the cables should not be jointed, without tests and rectification.

(d)	Measurement of Insulation Resistance

Before jointing, the insulation resistance of both sections of cables shall be checked.

(e)	Identification

The identification of each phase, shall be clearly and properly noted.  The cables shall be jointed as per the approved design.   Each cable shall have identification for phase at joint bays.

(f)	Making a Joint

Comprehensive jointing instructions should be obtained from the manufacture of jointing kits and meticulously followed.

The materials used in the joints like ferrules, screen/sheath continuity bonds, lugs etc., shall be of good quality and conform to standards.

The jointing tools shall be appropriate and as per the requirement of jointing XLPE cables.

All cable joints and terminations must be carried out by qualified personnel. The cable jointers must be in possession of a valid trade certificate for jointing works from a recognized institution. CVs of cable jointers shall be approved by Employer prior to commencement of works.

(28)	Cable Laying & Terminations

The preparation of the cable end for installing the terminations and the precautions to be taken before fixing the terminations shall be followed as in the case of the cable jointing procedures. The instructions furnished by the termination manufacturer shall be strictly followed. Cables shall be terminated at RMU, Distribution Transformers, Switches or LT distribution boxes.

At cable terminating end, the following provisions for supply and erections are to be included:

(a)	A sufficient length of spare cable shall be left in the ground, for future needs.

(b)	The rise of the cable immediately from the ground shall be enclosed in PVC/PE pipe of suitable diameter to protect against direct exposure to the sun.

(c)	The cable shall be properly fastened using non-metallic clamps.

(d)	Appropriate labels shall be fixed identifying the phase circuit, voltage and date of commissioning etc., on the cable supporting structure.

(29)	Bonding of Screen

The cable earth screens shall be bonded to earth at the termination positions.

As indicated previously, cross bonding is not necessary on 11kV systems.

(30)	Connection of Radial Water Barrier ond Cable Screen

If the metallic radial water barrier is insulated from the metallic wire screen, a connection suitable to carry the currents occurring during operation must be installed between metallic radial water barrier of the cable and metallic wire screen in joints and sealing ends.

(31)	Measurement (for Civil Works)

The buried cable trench shall be measured in the running meters including excavation, back filling, thermal back filling (if applicable),  compaction, laying of concrete/ reinforcement, placing of warning tape markers, dewatering as required as per the drawing & specification & any other job required for successful completion of work.

(32)	Optical Fibre Cable (For Communication Equipment)

Optical fibre cable required for Communication Equipment shall also be laid in the same cable trench in separate HDPE pipe.

(33)	Miscellaneous

All the civil works associated with laying of underground cable are described in Civil Works

(34)	Inspection and Tests

Through the work execution, various inspections and tests on the progressing works will be ordered to the Contractor by the Employer.

Following inspections and tests will be carried out after completion of the works section by section. The Contractor shall perform all the inspections and tests in accordance with IEC Standard.

Underground Power Cables:

- Visual inspection of the underground cable lines

-	Back filling and grade

-	Cable and joint marks

-	Cable termination and connection

- Measurement insulation resistance of the lines

- H. V. test

(a)	Tests for 11 Kv Power Cables, Straight Joints and Accessories

Test at works

The following tests shall be carried out at the manufacturer's plant before shipment as far as applicable for type of conductors and cables:

- 11 kV XLPE power cable

i)	Appearance check

ii)	Conductor resistance measurement

iii)	Capacitance measurement

iv)	Insulation resistance measurement

v)	A.C. withstand voltage

vi)	A.C. long duration withstand voltage

vii)	Impulse withstand voltage

viii)	A.C. long duration breakdown voltage

ix)	Impulse break-down voltage

x)	Dielectric loss tangent

xi)	Temperature-voltage characteristic

xii)	Dimension

-	Conductor outermost diameter

-	Insulation thickness

-	Sheath thickness

-	Over-sheath thickness

-	Thickness of each tape

-	Interval of tape lapping

-	Total diameter

xiii)	Bending withstand characteristic

xiv)	Over-sheath, tensile strength

xv)	- do -	, thermal aging

xvi)	- do -	, oil-proof

xvii)	- do -	, non-inflammability

xviii)	- do -	, thermal deformation

xix)	- do -	, hardness

- Sealing/termination ends or straight joints

i)	Construction

ii)	A.C. long duration withstand voltage

iii)	Impulse withstand voltage

iv)	A.C. long duration breakdown voltage

v)	Impulse breakdown voltage

vi)	Porcelain or epoxy insulator, construction

vii)	- do -	, power frequency flash-over voltage (dry)

viii)	- do -	, power frequency flash-over voltage (wet)

ix)	- do -	, 50% impulse flash-over voltage

x)	- do -	, cantilever strength

xi)	- do -	, thermal mechanical performance

Test on Completion:

After completely installing the 11 kV XLPE power cables, sealing ends and other conductors at site, the following test shall be carried out by the Contractor:

- D.C. High voltage test

(b)	Special Requirement

Small cut piece lengths of cables will not be accepted. Cables up to 500 meters in length or as approved by Employer/Employer's Representative shall be of one length shipped in a drum of adequate size. For higher quantities, multiple lengths/drums may be shipped subject to the approval of Employer/Employer's Representative.

(c)	Drawings, Data & Manuals

The following information shall be furnished along with the Tender.

- Manufacturer’s leaflets giving constructional details, dimensions and characteristics of different cables.

- Current rating of cables including de-rating factor due to grouping, ambient temperature and type of various installation.

- Write-up with sketches illustrating the manufacturer’s recommendation for splicing, jointing and termination of different types of cables.

- Type test report of power cables. The Bidder shall clearly describe the type and routine tests to be performed on cables.

- Drum length for each of cable.

(d)	Performance Guarantee

The performance figures quoted on schedule of Technical Data shall be guaranteed within the tolerance permitted by relevant standards and shall become part of the Contract. In case of failure of the cables to meet the guarantees, the Employer reserves the right to reject the item. The Contractor shall have to rectify/replace the defect/defective part at no extra cost to the Employer and without delaying the commissioning schedule.

(e)		Spare Parts, Tools & Instruments

The bidder shall also furnish Manufacturers recommended spare parts, tools & Instruments with price. The Contractor shall submit an itemized list of such equipment.

(A2)	Cable Termination Kit and Joint Kit for XLPE

(1)	Scope

This specification covers the design, manufacture, factory test, supply and delivery of heat shrinkable, push on type termination and joint kits for 11 kV, 3 core screened, armoured, with aluminium conductor XLPE cables suitable for earthed system and confirming to relevant standard with latest amendment, if any.

The termination kits for the use on 50 Hz, 3 phase system with earthed neutral for working voltage of 11 KV.

(2)	Description

(a)	All high voltage terminations and jointing kits shall be standard quality type. They shall be factory engineered kits containing all the necessary components to reinstate the cable insulation, metallic shielding of each core, together with the reinstatement of the sheath, of the cable being terminated.

(b)	The heat shrinkable termination and joint kit shall be manufactured and tested in accordance latest version of IEC, or any other national or international standard that ensures at least a substantially equal quality to the standard mentioned above will also be acceptable.

(c)	The manufacturer of the termination and joint kit (Indoor, outdoor and straight- through) must have been accredited with ISO 9001 quality certification. The heat shrinkable/push on type terminations offered shall be of proven design and make, which have already been extensively used and fully type tested.

(d)	The entire termination and joint kit shall be environmentally sealed and capable of preventing the ingress of external moisture and contamination.

(e)	Kits shall contain sufficient cleaning solvents and cleaning clothes for the proper making of the joint or termination.

(f)	Voltage stress relief shall be provided and this may be inherent in the heat recoverable polymeric material.

(g)	The terminating or jointing materials shall not be subjected to storage limitations such as controlled temperature or humidity restrictions, nor have self-life limitations.

(3)	Other Requirements

(a)	The outdoor termination kits shall be suitable for terminating the cable at steel cross arm complete with brackets, terminals, saddles and all necessary materials for fixing the termination. The heat shrinkable termination kit to be supplied and installed under this scope of work shall be capable enough to cope with all the weather change. Terminations that do not require manually built stress relief cones or field pouring compound are preferred.

The term heat shrinkable refers to extruded or molded polymeric materials which are cross-linked to develop elastic memory and supplied in expanded or otherwise deformed size/shape, subsequently heating in a non-constrained state to a temperature above the shrink temperature resulting in the material recovering or shrinking to its original shape

(b)	Since the sealant or adhesives (to be used for environment sealing) between the heat shrinkable materials and XLPE cables shall be exposed to high electrical stresses, they must be track resistant.

(c)	The heat shrinkable polymer materials being used for external leakage insulation between the high voltage of conductors and grounds should be weather resistant.

(d)	All cuts/nicks inadvertently occurred to XLPE insulation must be rendered discharge free by using suitable discharge suppression compound.

(e)	The heat shrinkable tubing may be either extruded or moulded type.

(f)	Higher thickness of heat shrinkable sleeves shall be preferable to counter erosion due to pollution.

(g)	Push on type (Terminations only):

(h)	Rubber components should be made from proven quality of tuber with tested curing properties.

(i)	The semi conducting portion of the stress cone should be vulcanized with insulation so that both semi conducting and insulation portion becomes an integrated part.

(j)	The stress cone must be of proven design of stress control.

(k)	The moulding of rubber components should be aimed to achieve a smooth finish on interior and exterior of the components.

(l)	The stress cone should probably be reusable type.

(m)	In case of outdoor terminations, the suitable provision for covering the cable cores with re-useable protective system from the crotch seal to the bottom of stress cone should be made.

Accessories shall match the cable test ratings in all respects. The material and components not specifically stated in the specification, but which are essential for satisfactory operation of the equipment shall be included without any extra cost.

The termination and joint kits shall be of suitable for following cables:

- Indoor use for 11 kV, 3-cores Aluminium XLPE insulated Power cables.

(4)	Testing

The routine tests of the kits shall be done at manufacturer's plant in accordance with IEC or other equivalent national or international standards including following tests.

- Appearance Test

- Construction Test

- High Voltage Test

(5)	Bid Documentation

(a)	The Bidder shall provide with the Bid two (2) clear copies of the manufacturer governing Standard of the termination and joints and kits two (2) clear copies of all other relevant standards referenced herein.

(b)	The Bidder shall provide certified type test results of the termination and joint kits as required by governing standards.

(c)	The Bidder shall provide complete description, catalogue and drawings of the termination and joint kits.

(d)	A clause-by-clause commentary on specification, specifying compliance and deviation, if any.

(e)	All data, drawing, catalogue and other technical documents supplied shall be bound separately from the Bid Document.

(B)	HDPE PIPE AND PPR PIPES

(1)	Scope

This Specification covers the fabrication and supply of 160 mm (outer diameter with inner dia 152mm) Flexible HDPE Pipe and PPR Pipes used for XLPE power cable in underground 11kV distribution system.

(2)	Description

The HDPE pipes are black and should suitable for inserting cable. The flexible conduit pipe shall be corrugated hard polyethylene pipe shall be used for installation of XLPE power cable. The flexible pipe shall be buried before the cable installation and, then the cables shall be pulled in. The HDPE pipe material should be Fire Retardant or non-Flammable.

The HDPE pipes shall be fabricated and tested in accordance with BS: 3412, Class N HDPE or any revision thereof or other equivalent national or international standard provided that ensure at least equal or better quality to the standard mentioned above will also be acceptable. The minimum tensile strength of the pipe shall be 240 kg/cm2.

The HDPE pipe should be suitable for 4kg/cm2 pressures with thickness not less than 6.5mm and weight of the HDPE pipe should not be less than 2.5kg per meter. The HDPE pipe shall have a minimum tensile strength of 3200 kg/mm2. The HDPE pipes should have design at 27°C for a stress of over 50kg/cm2 with safety factor of 1.3.

The flexible conduit pipe shall be strong enough to withstand the compression force from heavy trucks or lorries when it is buried more than 80cm below the ground level and temperature rise up to 80°C.

The PPR pipes shall be fabricated and tested in accordance with EN ISO 15874, Class PN20 and PN25 or any revision thereof or other equivalent national or international standard provided that ensure at least equal or better quality to the standard mentioned above will also be acceptable. The specific chemical structure of green PPR should be present to provide the well balanced mechanical properties and superior long term heat resistance.

(a)	For HDPE Pipe 160mm diameter

| SI. No. | Description | Unit | Requirement |
| --- | --- | --- | --- |
| 1 | M.F.R. ( 190°C, 5kg load) | gm/10 mins | 0.20 to 1.10 |
| 2 | Specified base densiiy |  | 940 to 958 |
| 3 | Material Grade | kg/m3 | PE-63 |
| 4 | Wall Thickness | mm | 7.7 - 8.7 |
| 5 | Carbon Black | % | 2.5 ± 0.5 |
| 6 | Ant1ox 1dant | % by mass | <0.3% by mass |
| 7 | Overall Migration | mg/dm2 | 10 Max |
| 8 | Reversion | % | <=3% |
| 9 | Hydraulic Characteristics |  | No sign of localized swelling, leakage or weeping (at 80°C for 48 & 165 hrs.) |
| 10 | Continuous Temperature withstand capacity |  | 120°C |

b)	Accessories

The flexible conduit pipe shall be provided with necessary accessories, such as joints and sealing material etc. The straight joint sleeve shall be made of high density polyethylene black coloured and to be so designed as to be screwed on to flexible pipe.

Bell mouth shall be fixed to the end of corrugated pipe to facilitate cable pulling in. The bell mouth shall be so designed as to screw into the pipe. It shall be made of hard density polyethylene and colored black.

Water proof materials for pipes in manhole shall be mounted to an outlet of duct to keep the water tightness. The waterproof materials shall be comprised the components such as sand-proof seal, sealing tape, neo seal compound, VUL-CO tape, PVC tape and other necessary materials to complete the specified scope of works.

(3)	Tests

The type test and routine tests shall be carried out for the materials to be supplied according to the above mention technical specification in accordance with the governing standard.

(4)	Packaging

The packing should be done for the materials to be supplied accordingly.

(5)	Bid Documentation

(a)	The Bidder shall provide a complete description, and catalogue of materials.

(b)	The Bidder shall provide certified test report as required by governing standards for materials.

(c)	A clause-by-clause commentary on specification, specifying compliance and deviations, if any.

(d)	All data, drawings, catalogues and other technical documents shall be bound separately from the Bid documents.

(C)	MISCELLANEOUS

(C1)	Ground Rods and Clamps

(1)	Scope

This Specification covers the fabrication and supply of galvanized steel ground rods and clamps for use in overhead power line construction.

(2)	Description

Ground Rod:

(a)	The ground rod shall be made of high carbon, open-hearth steel so as to achieve maximum strength. It shall be hot dip galvanized.

(b)	The ground rod shall be 19mm in diameter and 34,000 mm in overall length.

(c)	The driven end of the ground rod shall have a truncated cone point. The cone point shall be approximately 13mm long, measured along the axis of the ground rod. The driving head of the ground rod shall have an approximate 3 mm, 45 degrees chamfer.

(d)	The manufacturing process shall assure that ground rod does not bend when driven into hard soils.

Ground Rod Clamp:

(a)	The ground rod clamp shall be heavy duty forged steel clamp provided with a hex head cup point set screw of high strength steel with machine cut threads. It shall be so manufactured that it gives low resistance connection. The ground rod clamp shall be galvanized.

(b)	The clamp shall suitably accommodate and clamp a 19 mm. ground rod and a stranded grounding conductor of 7/12 SWG size (SPECIFICATION: SP 4.5.2).

(3)	Galvanizing

The galvanization of ground rod and clamp shall be in accordance with IS: 2629-1985 or any revision thereof or other equivalent national or international standard provided that ensure at least equal or better quality to the standard mentioned above will also be acceptable.

(4)	Tests

Grounds rods and clamps shall undergo type and routine tests in accordance with the relevant governing standard.

(C2)	Grounding Conductor

(1)	Scope

This specification covers the fabrication and supply of galvanized stranded steel grounding conductor for use in the neutral grounding of distribution transformers and body grounding of electrical equipment.

(2)	Description

(a)	The conductor shall be 7 wire stranded conductor and shall conform to the characteristics as specified in Table 1 contained herein. Stranded conductor shall be galvanized.

(b)	The manufacturer of ground Conductor must have been accredited with ISO 9001:2000 with design and manufacturing quality certification.

(3)	Galvanizing

The grounding conductor shall be galvanized after fabrication in accordance with IS: 2629-1985 or any revision thereof or any other national or international standards that ensure at least equal or better quality to the standard mentioned above will also be acceptable.

(4)	Tests

The grounding conductor shall undergo type and routine tests in accordance with the relevant governing standard.

Grounding Conductor Minimum Requirements:

7 No. 12 SWG:

Diameter of single strand	2.67 mm

Weight	299 kg/km

Short time fusing current	12 kA

Resistivity	15 Micro-Ohm-cm

(D)	CIVIL WORKS

(1)	Site Works

(a)	Work Included

Furnishing all materials equipment and labor and performing all operations required for constructing access roads so indicated on the drawings, specified herein and as evidently necessary to complete to work as given in the price schedule.

(b)	Laying of underground cable

Contractor shall furnish all construction work for under grounding of 12 kV cables. This work shall include excavation, sand filling from the top.

(2)	Foundations

(a)	Work Included

Furnishing all materials, equipment and labor and perform all operations required for the design and construction of all the concrete foundations for equipment and other structures, oil containment and cable trenches, as specified herein and as evidently necessary to complete the work.

(b)	Foundation Design

- General

The Contractor shall design all foundations as shown in the Drawings. The design shall be based on assumed soil parameters. Upon completion of the detail soil test, actual soil conditions shall be investigated and shown to be in compliance with the assumed condition.

- Submittals

Before commencement of the works, the Contractor shall submit design calculations, detailed drawings and reinforcement steel schedules to the Engineer for review and approval. Review of the foundation design by the Engineer shall not relieves the Contractor of his responsibility for an adequate foundation design. Upon receiving the Engineer observations (if any), the Contractor shall submit the full set of final drawings of all foundation details, including reinforcement steel schedules as established in Sub-Section 117: “Contractor’s Drawings”.

- Design Load

The structure design loads are defined on the structure outline drawings and the loads used to design the foundation shall be actual working loads applied to the foundations by the equipment and structures. The foundations shall be designed to resist all vertical and lateral forces, uplift forces and overturning moments with a minimum factor of safety of 1.5.

- Bearing Loads

The Contractor shall use an allowable soil bearing pressure of 1.0kg/cm2 for the design of the foundation for the purpose of bidding, but this is only reference value. After award of contract the Contractor shall carry out detail soil test and detail design of foundation based on the soil test result. There may be variation in the volume of work in final design compared to the bidding design, for which the Contractor will not get any extra payment.

- Uplift and Overturning Loads

The uplift and overturning resistance of concrete spread footing shall be assumed as the weight of a volume of earth in the form of an inverted frustum of cone or pyramid. The cone of pyramid height shall be 30cm less than the depth from finish grade to the top of the concrete mat, the base area shall be the top area of the mat and the top area shall be determined by the intersection of planes starting at the mat edges and sloping outward at a 20 degree cone angle from the vertical and the horizontal plane 30 cm below finish grade.

- Unit weight for overturning resistance

The following unit weight shall be used for design:

- Soil	 12,000kg/m3

- Concrete	 16,00kg/m3

(c)	Ground bearing tests

The Contractor shall carry out the required tests in accordance with Section 3000: “Sub-Surface Geotechnical Investigations” of these Specifications to determine the ground bearing capacity by means of a Standard Penetrations Test and Auger Boring or a method approved by the Engineer.

Soil sampling shall be made in accordance with ASTM D-1587-63T and the Standard Penetration Tests in accordance with ASTM-D-1586-64T.

Every undisturbed sample collected from thin-walled tube shall be subject to the following series of test:

- Natural moisture content

- Atterberg limits

- Sieve analysis

- Unit weight

- Specific gravity determination

- Unconfined compression tests

Accurate log of all soil strata penetration resistance test unconfined compressive strength, soil classification, ground water table and other tests result shall be recorded in the reports submitted.

(b)	Excavation and Backfill

Excavation and Backfilling works shall be made in accordance with the requirements of Section 900: “Earthworks” and other pertinent and relevant sections of these Specifications as applicable, and as approved by the Engineer.

(c)	Concrete

The concrete for these works shall met the requirements of Concrete Class C described in Section 2000: “Concrete for Structures” of these Specifications.

When placing concrete in hot weather, the recommendations of the American Concrete Institute’s publication “Recommended, Practice for Hot Weather Concreting”(ACI 605) shall be followed as directed by the Engineer. Concrete with a temperature higher than 30°C before placement shall be rejected and shall be wasted at the Contractor’s expense.

(d)	Steel reinforcing bar

Steel reinforcing bar shall conform to the requirements of Sub-Section 2014: “Reinforcement” and other relevant sections of these Specifications.

(e)	Grouting

Grouting for seating structural steel members and equipment on foundations shall be made conforming the requirements of relevant sections of these Specifications.

6506	MEASUREMENT

The works for this item shall be measured in lump sum but summarized in each linear metre of the Transmission Line actually constructed/installed, satisfactorily tested according to Clause 509(3): “Testing and Commissioning” and conforming the requirements of Clause 9: “Test on Completion” of the Conditions of Contract, confirmation of proper function of the established system, satisfying all requirements of these Specifications, in accordance with the Drawings, and approved by Engineer.

6507	PAYMENT

The works for the Transmission Line shall be paid under this Section in lump sum which shall be the full and the final compensation to the Contractor as per Clause 112 including the cost for Design Review and Updating, provision of all materials, all operations required for construction as shown in the Drawings and described in this Item including drawings, tests, cabling, civil works and all other ancillary works and any incidental work needed to complete the work as per these Specifications and/or directed by the Engineer.

| Item No. | Description | Unit |
| --- | --- | --- |
| 6500 | Transmission Line |  |
| 6500-1 | Transmission Line | lin.m. |

SECTION 7000: Roadside Service Station

7001	DESCRIPTION

This work shall include the Design Review and Updating by the Contractor based on the Conceptual Design provided by the Employer. The report of the Design Review shall include also all detailed drawings, applicable technical specifications and requirements for construction and materials.

This works shall include all the necessary civil works, water supply and sanitation works, and electrification works as per the specification, the Drawings and instruction by the Engineer.

The works shall consist of the following:

- Water Supplying System

- Parking Area Lighting System

- Restaurant Building

- Shopping Building

- Toilets Building

- Sewage System

- Security Guard Office Building

The works shall include furnishing all materials, construction of the above referred buildings and systems, and all ancillary works and any other incidental necessary to complete the Works in conformity with the Drawings and these Specifications, or as directed by the Engineer.

7002	STANDARDS

The works for design’s verification and construction of the Roadside Service Station shall conform in all respects to highest standards of engineering, design, workmanship, this specifications and the latest revisions of relevant standards applicable in Nepal (Specifications of Building Construction Works and Nepal National Building Code NBC206: 2003 - Architectural Design Requirements)

7003	CONSTRUCTION METHOD AND DESIGN REVIEW AND UPDATING REPORT

Before commencement of the works, the Contractor shall prepare and submit to the Engineer, for review and approval, a construction method statement and the detailed report of the Design Review and Updating for the Roadside Service Station. These documents shall be attached to the Program which requirements are described in Section 116: “Programme”.

7004	MEASUREMENT

The works for this item shall be measured respectively in accordance with the specified units and method established in the relevant sections of these Specifications.

7005	PAYMENT

The works for the construction of the Roadside Service Station shall be paid under this Section at the contract unit rate which shall be the full and the final compensation to the Contractor as per Clause 112 including the cost for Design Review and Updating, provision of all materials, all operations required for construction as shown in the Drawings and described in this Item including drawings, tests and all other ancillary works and any incidental work needed to complete the work as per these Specifications and/or directed by the Engineer.

| Item No. | Description | Unit |
| --- | --- | --- |
| 7000 | Roadside Service Station |  |
| 7000-1 | Park area lighting system | nos. |
| 7000-2 | Restaurant building | sq.m |
| 7000-3 | Shopping building | sq.m |
| 7000-4 | Toilet building | sq.m |
| 7000-5 | Security guard office building | sq.m |
| 7000-6 | Water supply system including pipeline from western portal water tank to water supply tank | lump sum |
| 7000-7 | Sewage system including sewage pipeline, manhole with casting cover (Heavy duty) and treatment system | lump sum |
| 7000-8 | Roadside Service Station restaurant tables and chairs | PS |

SECTION 7500: STEEL BRIDGE FOR FLYOVER

7501	DESCRIPTION

This works shall include the following:

Superstructure

Reinforced Concrete Substructure

Shinso Piles (Cast-in-Situ Pile by Manual Excavation) Foundation

Boring (100mm Dia.) with SPT (@1m) and Corresponding Laboratory Tests and Reporting

The works shall include furnishing all materials, fabricating, transporting, erection, painting structural steel, rivet steel, cast steel, steel forgings, cast iron, providing all materials and construction of reinforced concrete substructure and Shinso Piles (Cast-in-Situ pile by manual excavation) for foundation, and all ancillary works and any other incidental necessary to complete the Works in conformity with the Drawings and these Specifications, or as directed by the Engineer.

This work shall also include the Design Review and Updating by the Contractor based on the Conceptual Design provided by the Employer. The report shall include also all detailed drawings, applicable technical specifications and requirements for construction and materials.

7502	MATERIALS

(1)	Superstructure

The materials for construction of the steel superstructure shall meet with the following requirements:

| Superstructure Component | Material | Specification and Requirements |
| --- | --- | --- |
| Main Girder | Structural Steel | - Section 600: “Materials and Testing of Materials” - Section 2014: “Reinforcement” - Steel SM90YA (or equivalent); SM400A (or equivalent) |
|  | High Tension Bolts | - Section 2200, IS: 961 with minimum tensile strength of 58 kg/mm2 |
|  | Erection of Girder | - Sub-Section 2204: “Erection” |
|  | Painting of Girder | - Sub-Section 2205: “Painting” |
| RC-Slab | Concrete Class A | - Section 2000: “Concrete for Structures” |
|  | Reinforcement Bar | - Section 600: “Materials and Testing of Materials” - Section 2014: “Reinforcement” |
| Railing and Guard Wall | Concrete Class A | - Section 2000: “Concrete for Structures” |
|  | Reinforcement Bar | - Section 600: “Materials and Testing of Materials” - Section 2000: “Concrete for Structures” |
|  | Railing; Fencing Type-F | - Sub-Section 3105: “Railing” - Section 400: “Fencing” |
| Wearing Surface | Waterproof sheets | - Section 1300: “Bituminous Surface and Base Course” - Section 2000: “Concrete for Structures” |
|  | Asphalt Concrete (50mm thick) |  |
|  | Kerbs (Concrete Class-C) |  |
| Elastomeric Bearing | Maximum reaction from superstructure: A1 (1,500kN to 3,500kN) P1 (3,500kN to 4,000kN) P2 (3,500kN to 4,000kN) P3 (3,500kN to 4,000kN) A2 (1,500kN to 3,500kN) | - Section 600: “Materials and Testing of Materials” - Section 1900: “Bearings and Expansion Joints” - Sub-Section 1902: “Bearings”, (4) Elastomeric Bearing |
| Expansion Joint | Expansion Joint Type-B | - Section 600: “Materials and Testing of Materials” - Section 1900: “Bearings and Expansion Joints” Sub-Section 1901: “Bearings” |
| Drainage | Catch-Pit | - Section 600: “Materials and Testing of Materials” - Section 3111: “Bridge Drainage” |
|  | PVC Drain Pipe (VP) φ200mm |  |

(2)	Reinforced Concrete Substructure

The materials for construction of the reinforced concrete substructure shall meet with the following requirements:

| Substructure Component | Material | Specification and Requirements |
| --- | --- | --- |
| RC-Pier and Abutments | Concrete Class B | - Section 2000: “Concrete for Structures” |
|  | Reinforcement Bar | - Section 600: “Materials and Testing of Materials” - Section 2014: “Reinforcement” |

(3)		Shinso Piles (Cast-in-Situ Pile by Manual Excavation) Foundation

The materials for construction of the cast-in-situ shinso piles foundations shall meet with the requirements of Sub-Section 1618: “Shinso Piles (Cast-in-Situ Pile by Manual Excavation)”.

7503	CONSTRUCTION METHOD AND DESIGN REVIEW AND UPDATING REPORT

Before commencement of the works, the Contractor shall prepare and submit to the Engineer, for review and approval, a construction method statement and the detailed report of the Design Review and Updating. These documents shall be attached to the Program which requirements are described in Section 116: “Programme”.

7504	MEASUREMENT

The works for this item shall be measured respectively in accordance with the specified units and method established in the following relevant sections of these Specifications:

Section 400: 	"Fencing"

Section 1300: 	“Bituminous Surface and Base Course”

Section 1618: 	“Shinso Piles (Cast-in-Situ Pile by Manual Excavation)”

Section 1900: 	“Bearings and Expansion Joints”

Section 2000: 	“Concrete for Structures”

Section 2200: 	“Structural Steel Work”;

Sub-Section 2018(1):	“Concrete” (Measurement)

Sub-Section 2018(4):	“Reinforcement” (Measurement)

Sub-Section 2204: 	“Erection”;

Sub-Section 2205: 	“Painting”

Sub-Section 2413:	“Dry Stone Packing and Soling”

Section 3000:	“Sub-Surface Geotechnical Investigation”

Sub-Section 3105: 	"Railing"

Section 3111: 	“Bridge Drainage”

7505	PAYMENT

The works for the Steel Bridge For Flyover as measured above shall be paid under this Section at the respective contract unit rates, for the items shown here below, which shall be the full and the final compensation to the Contractor as per Clause 112 including the cost for Design Review and Updating, provision of all materials, all operations required for construction as designed, as shown in the Drawings and as described in this Item including drawings, tests and all other ancillary works and any incidental work needed to complete the work as per these Specifications and/or directed by the Engineer.

The payment method for “Shinso Piles (Cast-in-Situ Pile by Manual Excavation)” described in Section 1618 shall be applied.

| Item No. | Description | Unit |
| --- | --- | --- |
| 7500 | Steel Bridge for Flyover |  |
| 7500-1 | Design review and update of the flyover bridge as per the specification, the Drawings and instruction by the Engineer | lump sum |
| (1) | Superstructure |  |
| 7500-2 | Structural Steel Works for the Main Girder | ton |
| 7500-3 | Concrete Class A (M35/20, C30/20) for Superstructure | cu.m |
| 7500-4 | Concrete Class B (M30/20, C25/20) for Superstructure | cu.m |
| 7500-5 | Reinforcement for Superstructure | ton |
| 7500-6 | Fencing Type-F | lin.m |
| 7500-7 | Asphalt Concrete Wearing Course for Bridge (50mm Thick) | sq.m |
| 7500-8 | Bitumen Tar Felt | sq.m |
| 7500-9 | Elastomeric Bearing (Abutment) Type 1,500 kN to 3,500 kN | nos. |
| 7500-10 | Elastomeric Bearing (Pier) Type 3,500 kN to 4,000 kN | nos. |
| 7500-11 | Expansion Joint Type-B | lin.m |
| 7500-12 | Bridge Collapse Prevention Device for Steel Girders | set |
| 7500-13 | Casting Catch-Pit | nos. |
| 7500-14 | PVC Drain Pipe (VP) Φ200mm for Bridge Drainage | lin.m |
| 7500-15 | Kerb Type-C | lin.m |
| (2) | Reinforced Concrete Substructure |  |
| 7500-16 | Concrete Class B (M30/20, C25/20) for Substructures | cu.m |
| 7500-17 | Concrete Class E (Lean Concrete) for Substructures | cu.m |
| 7500-18 | Reinforcement for Substructures | ton |
| 7500-19 | Dry Stone Packing and Soling | cu.m |
| (3) | Shinso Piles (Cast-in-Situ Pile by Manual Excavation) Foundation |  |
| 7500-20 | Excavation above Shinso Piles Heads | cu.m |
| 7500-21 | Excavation of Shinso with Steel Lining (D=2.50m ~ 5.00m) | cu.m |
| 7500-22 | Excavation of Shinso with Shotcrete (D=2.50m ~ 5.00m, 100mm thick with steel net) | cu.m |
| 7500-23 | Excavation of Shinso with Shotcrete (D=2.50m ~ 5.00m, 200mm thick with steel H rib) | cu.m |
| 7500-24 | Excavation of Shinso without Protection (D=2.50m ~ 5.00m) | cu.m |
| 7500-25 | Concrete class-B (M30/20, C25/20) for Shinso Piles (D=2.50m ~ 5.00m) | cu.m |
| 7500-26 | Reinforcement for Shinso Piles | ton |
| 7500-27 | Boring (100mm dia.) with SPT (@1m) and corresponding laboratory tests and reporting | lin.m |