BILL OF QUANTITIES 
FOR WATER SUPPLY WORKS

Bill of Quantities for Water Supply Works

A.  Preamble

The Preamble of the Bill of Quantities of Part 1: Bidding Procedures, Section IV: Bidding Forms shall apply, prevail and govern also the Bill of Quantities for Water Supply here below described.

The cost of Items against which the Contractor has failed to enter a rate or price shall be deemed to be covered by other rates and prices entered in the Bill of Quantities for Water Supply Works or the main BOQ of Section IV of the Bidding Documents.

The whole cost of complying with the provisions of the Contract shall be included in the Items provided in the priced Bill of Quantities for Water Supply Works, and where no Items are provided, the cost shall be deemed to be distributed among the rates and prices entered for the related Items of Work.

In case a discrepancy between the total amount stated in the BOQ for Water Supply Works herein detailed, and the total amount stated in the main BOQ of Section IV Item 8000, the lower amount shall be applied and corrections shall not be allowed.

Bill of Quantities for Water Supply Works

| SN | Particular of Items | Quantity | Units | Rate NRs | Amount NRs |
| --- | --- | --- | --- | --- | --- |
| I. CIVIL WORKS |  |  |  |  |  |
| A. Well development |  |  |  |  |  |
| 1 | Mobilization and Demobilization | 1 | job |  |  |
| 2 | Camp and Rig Setting and Preparation of site for work | 1 | job |  |  |
| 3 | Vertical Electrical Sounding work | 1 | job |  |  |
| 4 | Guide Pipe Works (550mm dia) | 10 | m |  |  |
| 5 | Drilling of pilot hole by std bit ranging from 7 5/8" to 9 7/8" dia. | 180 | m |  |  |
| 6 | Electrical Resistivity Logging Test Work | 1 | job |  |  |
| 7 | Complete drilling for total Reaming work (at least two) by std bit above 9 7/8" and below 17 1/2" with drilling rig machine | 180 | m |  |  |
| 8 | Reconditioning of bore hole before lowering of pipe assembly 8-10" dia | 180 | m |  |  |
| 9 | Lowering of pipe assembly of 8-10" dia to a depth of 180m | 180 | m |  |  |
| 10 | Collection of lithological samples (at interval of 1.5m and at the change of strata) | 1 | well |  |  |
| 11 | Supply and place approved size pea gravel (4-8mm) packing in a continuous operation | 34 | m3 |  |  |
| 12 | Cementation for sanitary protection and well support work | 4 | m3 |  |  |
| 13 | Supply and install gravel top up 80mm dia GI pipe (M) and seal bore hole with cement | 5 | m |  |  |
| 14 | Supply and Installation of Pipes and accessories: |  |  |  |  |
|  | 250mm ND mild steel ERW housing pipe of thickness 7mm having male and female ends | 70 | m |  |  |
|  | 200mm ND mild steel ERW housing pipe of thickness 7mm having male and female ends | 80 | m |  |  |
|  | 200mm ND Stainless steel slotted pipe of thickness heavy duty with slots 1.6mmx75mm and percentage opening minimum 50% having male and female ends | 30 | m |  |  |
|  | Supply of 250mmx200mm dia MS reducer | 1 | Nos |  |  |
|  | Supply and Installation of well caps, cross bars, centralizers, bottom plug and other necessary accessories required for complete execution of borehole | 1 | job |  |  |
| 15 | Well development by drilling rig machine for well size of 10" dia (Back washing and inner washing) | 180 | m |  |  |
| 16 | Well development by drilling rig machine for well size of 10" dia (water jetting) | 180 | m |  |  |
| 17 | Well development by compressor machine for well size of 10" dia | 1 | well |  |  |
| 18 | Well development by pump 10" dia | 1 | well |  |  |
| 19 | Time draw down pump test for seep tubewell | 1 | well |  |  |
| 20 | Step draw down pump test for deep tubewell (well test) | 1 | well |  |  |
| 21 | Recovery test for Deep tubewell | 1 | well |  |  |
| 22 | Conducting water quality test and disinfection work | 1 | well |  |  |
| 23 | Preparation of report and submission (3 copies) | 1 | job |  |  |
| B. Platform, RVT, Valve Chamber, Guard and Pump House |  |  |  |  |  |
| 1 | PIPE GI-150 (M) | 1 | m |  |  |
| 2 | PIPE GI-100 (M) | 10 | m |  |  |
| 3 | Layout & setting out including site clearance of grass, shrubs, excavation of loose/soft soil to maximum depth of 150mm including dressing, levelling and disposal the excavated materials within the premises as per drawing, specification and instruction of engineer. | 2,183 | sqm |  |  |
| 4 | Earthwork in excavation of all type of soil in foundation trenches of semi basement pits raft etc depth up to 3.5m including dressing of sides, ramming of bottom, trenches, shoring and disposing the excavated material up to 10m lead as per drawing and specification all complete. | 1,263 | cum |  |  |
| 5 | Supplying & placing of Fe 415/500 reinforcement bar for column beams, slab, etc. placed in position and binding with 14 gauge binding wire including cutting, binding, carrying, lifting, etc. all complete as per drawing, specification and instruction of engineer. | 29 | mt |  |  |
| 6 | Fabrication and installation of shutter with 24 gauge G.I. mosquito net in 38 x 75 mm Salwood frame (based on shutter size 1.092 x 2.058 = 2.25sqm) | 4 | sqm |  |  |
| 7 | Fabrication and installation of Salwood frame for door and windows | 0.15 | cum |  |  |
| 8 | Fabrication and installation of panel shutter in 38 x 75 mm salwood frame (based on shutter size 1.07 x 1.982 = 2.12 sqm) | 1 | sqm |  |  |
| 9 | Providing & Laying dry stone soling in foundation trenches with bond stone blinding in perfect line & level all complete as per drawing & specification. | 59 | cum |  |  |
| 10 | 12.5 mm thick cement sand plastering works in 1:3 ration in walls and floor | 2,118 | sqm |  |  |
| 11 | Random rubble (hammer dressed) stone masonry work with 1:4 cement sand mortar including the cost of scaffolding, curing, cleaning and racking the joints all complete as per drawing, specification & instruction. | 15 | cum |  |  |
| 12 | Supplying mixing, placing, compacting plane/reinforced cement concrete of M10(1:3:6) excluding the cost of reinforcement bars & formwork as per drawing, specification and instruction of engineer all complete. | 51 | cum |  |  |
| 13 | Supplying, mixing, placing, compacting reinforced cement concrete of 1:1.5:3 (M20 grade, 1 Cement :1.5 Sand: 3 Aggregate 20 mm and down well graded) excluding the cost of reinforcement bars & formwork as per drawing, specification and instruction all complete. | 85 | cum |  |  |
| 14 | Supplying, mixing, placing, compacting reinforced cement concrete with waterproof chemical of 1:1:2 (M25 grade, 1 Cement :1 Sand : 2 Aggregate 20 mm and down well graded) excluding the cost of reinforcement bars & formwork as per drawing, specification and instruction all complete. | 150 | cum |  |  |
| 15 | Providing, placing, fixing centering and shuttering for plywood or steel formwork for slabs, walls, columns and beams (with necessary provision of extension of reinforcement for ties) and other RCC works including nails, propping supports and bracings of steel pipes including lead & lift upto 0 to 4 m height & removal and disposal of the same all complete as per drawing specification & approval of engineer. | 1,530 | sqm |  |  |
| 16 | CI Manhole | 45 | No. |  |  |
| 17 | Fabrication of Angle post (MS 50x50x4 2.3m high) | 137 | No. |  |  |
| 18 | Fabrication of MS Grill gate 1.2mx 1.5m | 3 | set |  |  |
| 19 | Barbed Wire Fencing | 429 | kg |  |  |
| 20 | Sand filling with compaction and sprinkling of water | 8 | cum |  |  |
| 21 | Supply and fitting Casement door of aluminum section in naturally anodized color. Section size (100×38×1.30) mm and 5 mm glass. | 9 | sqm |  |  |
| 22 | Supplying and fitting Casement double panel aluminum windows with Ventilation section of (88x78x1.1) mm with sash 38x34x1.1mm and 5mm glass. | 9 | sqm |  |  |
| 23 | Provide and laying of Porcelain glazed tiles overlay in 1:4 cement sand mortar | 12 | sqm |  |  |
| 24 | Providing, mixing and laying neat cement punning on floors in true line & level including curing all completed as per drawing, specification & instruction of engineer. | 77 | sqm |  |  |
| 25 | Dry brick soling in foundation and floors with first class chimney made bricks in true line & level including watering and ramming all complete as per drawing, specification & instruction of engineer. | 49 | sqm |  |  |
| 26 | Provide and lay 25 mm Kota stone flooring on 20mm thick 1:4 cement sand mortar | 22 | sqm |  |  |
| 27 | Providing, mixing and laying 12.5mm thick plaster on wall with 1:4 cement sand mortar in perfect line and level including scaffolding, curing etc. all complete as per drawing, specification & instruction of engineer. | 12 | sqm |  |  |
| 28 | Providing, mixing and laying 20mm thick plaster on wall with 1:4 cement sand mortar in perfect line and level including scaffolding, curing etc. all complete as per drawing, specification & instruction of engineer. | 64 | sqm |  |  |
| 29 | Providing and application of two or more coats of distemper paint over base paint of approved brand and colour on exterior fair face wall surface to give even and uniform shade all complete as per drawing, specification and approval of engineer. | 151 | sqm |  |  |
| 30 | Providing and application of two or more coats of cement paint over base paint of approved brand and colour on exterior fair face wall surface to give even and uniform shade all complete as per drawing, specification and approval of engineer. | 124 | sqm |  |  |
| 31 | 12.5mm dia. GI pipe supply and laying works (with pipe) | 15 | m |  |  |
| 32 | 20mm dia. GI pipe supply and laying works (with Pipe) | 6 | m |  |  |
| 33 | Filling with ordinary soil stacked at site with 200 mm thick layers with well compaction, water sprinkling, etc. all complete in pipe trench as per drawing, specification and instruction of engineer. | 34 | cum |  |  |
| 34 | Providing & laying First class chimney made brick masonry work in foundation and Ground Floor with 1:6 cement sand mortar (1 cement: 6 sand) including lead upto 30 m curing, cleaning & racking out mortar joint and making ducts, recesses where required as per drawing, specification and instruction of engineer. | 98 | cum |  |  |
| 35 | Providing, placing, fixing centering and shuttering for plywood or steel formwork for slabs, walls, columns and beams (with necessary provision of extension of reinforcement for ties) and other RCC works including nails, propping supports and bracings of steel pipes including lead & lift upto 0 to 4 m height & removal and disposal of the same all complete as per drawing specification & approval of engineer. | 150 | sqm |  |  |
| 36 | PVC Pipe 100mm Supply and installation all complete | 3 | Rm |  |  |
| 37 | PVC Pipe 75mm Supply and installation all complete | 18 | Rm |  |  |
| 38 | PVC Pipe 50mm Supply and installation all complete | 5 | Rm |  |  |
| 39 | Stainless Steel Sink Supply and installation all complete | 2 | Nos. |  |  |
| 40 | Fabrication of MS Grill for Windows Supply and installation all complete | 322 | Kg. |  |  |
| 41 | 20mm Gate Valve with fitting Supply and installation all complete | 1 | Nos. |  |  |
| 42 | Chrome Plate (CP) 15mm taps Supply and installation all complete | 2 | Nos, |  |  |
| 43 | Glazed Indian Type WC Set Supply and installation all complete | 2 | Nos. |  |  |
| 44 | Inspection Chamber 450x450x450 with RCC Cover Supply and installation all complete | 4 | Nos. |  |  |
| 45 | Supply, installation and commissioning of Main Distribution Board(MDB) wall mounting concealed type, dust and vermin proof made of 16 SWG mild steel sheet cubical with double cover covering the MCB with the locking arrangement as per drawing and specification. Main Distribution Board 1 no of 20 amp SP MCB Havells/ Siemens of equivalent 1set of 30 A amp bus bars 1 no of earth bus bar 2 nos. of 16 amp SP MCB (4.5 kA) Havells\Siemens or equivalent 4 nos. of 10 amp SP MCB (4.5 kA) Havells\Siemens or equivalent | 1 | No |  |  |
| 46 | Supply and installation of light point wiring from DB to various points with2 nos of 3/22 PVC insulated copper wire through 20 mm internal dia HDPE conduit concealed via junction box as per drawing and specification including the cost of switch and metal box | 17 | No |  |  |
| 47 | Supply and installation of 3 pin (15 A) socket point wiringwith2 nos of7/22 + 1no of 1/18 pvc insulated copper wire sthrough 20 mm internal dia HDPE conduit concealed from SDB to various points as per drawing and specification including the cost of Switch socket and metal boxes | 7 | No |  |  |
| 48 | Supply and installation of 2 pin ( 5 A)socket point wiring with2 nos of3/20 pvc insulated copper wires through 20 mm internal dia HDPE conduit concealed from SDB to various points as per drawing and specification including the cost of Switch socket and metal boxes. | 2 | No |  |  |
| 49 | Supply and installation of 9'' sweep exhaust fan as per drawing and specification including GI sheet boxing. | 2 | No |  |  |
| 50 | Supply and installation of 42'' sweep ceiling fan as per drawing and specification including GI sheet boxing. | 2 | No |  |  |
| 51 | Supply and installation of earth electrode made of 30cm x 30cm x 3mmcopper plate including 8 SWG bare copper wire connection to DB's. | 1 | No |  |  |
| 52 | Chieseling, laying conduits cables the main line distribution from MDB to Floor DB,s with 2 x 7/16 size cable in 20 mm HDPE conduit. | 7 | No |  |  |
| 53 | Made of 25x38mm MS angle and 3x18mm MS Strip with metal polish all complete | 1 | no |  |  |
| 54 | Made of 25x38mm MS angle and 3x18mm MS Strip with metal polish all complete | 3 | no |  |  |
| 55 | Made of sisam wood of 25x38mm section, dunlop at seat and back rest with rexin cover, having hand rest, 3 coats chapra polish all complete | 2 | no |  |  |
| 56 | MS cupboard made of 20 gauge MS Sheet for Clothes Store : Godrej or Equivalent | 1 | no |  |  |
| 57 | Single Bed made of 50x75 salwood section | 4 | no |  |  |
| 58 | Laboratory Equipment as per Attached List (List 1) | 1 | Lot |  |  |
| 59 | Supply and installation of light point wiring from DB to various points with2 nos of 3/22 PVC insulated copper wire through 20 mm internal dia HDPE conduit concealed via junction box as per drawing and specification including the cost of switch and metal box | 2 | pts. |  |  |
| 60 | Supply and installation of 3 pin (15 A) socket point wiringwith2 nos of7/22 + 1no of 1/18 pvc insulated copper wires through 20 mm internal dia HDPE conduit concealed from SDB to various points as per drawing and specification including the cost of Switch socket and metal boxes | 2 | pts |  |  |
| 61 | E/W excavation in hard clay | 10 | cum |  |  |
| 62 | Brick soling on flat with sand | 25 | sqm |  |  |
| 63 | Lab equipment and consumable as per List (List 1) | 1 | job |  |  |
| 64 | Pipe Line Excavation and Refilling (0.6m*0.9m Trench) | 7,711 |  |  |  |
| 65 | HDPE Pipe laying and Jointing | 12,680 |  |  |  |
| 66 | GI Pipe laying and Jointing | 1,600 |  |  |  |
| 67 | Blacktopped dismantle and reinstatement work | 1,000 | sqm |  |  |
| II. PIPE AND FITTINGS SUPPLY INCLUDING INSTALLATION OF FITTINGS AND VALVE |  |  |  |  |  |
| A. Pipes (Supply) |  |  |  |  |  |
|  | PIPE GI-75 (M) | 1,600 | m |  |  |
|  | PIPE HDPE-50/10 | 5,399 | m |  |  |
|  | PIPE HDPE-63/10 | 2,420 | m |  |  |
|  | PIPE HDPE-75/10 | 972 | m |  |  |
|  | PIPE HDPE-90/6 | 199 | m |  |  |
|  | PIPE HDPE-110/6 | 237 | m |  |  |
|  | PIPE HDPE-110/10 | 1,170 | m |  |  |
|  | PIPE HDPE-125/10 | 1,276 | m |  |  |
|  | PIPE HDPE-140/6 | 448 | m |  |  |
|  | PIPE HDPE-140/10 | 246 | m |  |  |
|  | PIPE HDPE-160/6 | 313 | m |  |  |
| B. Fittings and Valves Supply and Installation |  |  |  |  |  |
| 1 | Pipe/Fittings and Valve for tubewell for 100mm riser pipe as per drawing all complete (except the Pipe) | 1 | Set |  |  |
| 2 | Bulk meter (100mm dia) | 3 | No |  |  |
| 3 | Bulk meter (200mm dia) | 1 | No |  |  |
| 4 | Bulk meter (75mm dia) | 2 | No |  |  |
| 5 | DI Double Flange Tail Piece (100mm Dia) | 2 | No |  |  |
| 6 | DI Double Flange Tail Piece (150mm Dia) | 1 | No |  |  |
| 7 | DI Double Flange Tail Piece (75mm Dia) | 7 | No |  |  |
| 8 | DI Double Flanged Bend (100mm dia) | 7 | No |  |  |
| 9 | DI Double Flanged Bend (150mm dia) | 2 | No |  |  |
| 10 | DI Double Flanged Bend (200mm dia) | 1 | No |  |  |
| 11 | DI Flange Tail Piece (100mm Dia) | 8 | No |  |  |
| 12 | DI Flange Tail Piece (150mm Dia) | 2 | No |  |  |
| 13 | DI Flange Tail Piece (200mm Dia) | 5 | No |  |  |
| 14 | DI Flange Tail Piece (300mm Dia) | 2 | No |  |  |
| 15 | DI Flange Tail Piece (75mm Dia) | 12 | No |  |  |
| 16 | DI Flanged Tee (150mm dia) | 1 | No |  |  |
| 17 | DI Flanged Tee (75mm dia) | 2 | No |  |  |
| 18 | DI Mechanical Coupling (100mm Dia) | 6 | No |  |  |
| 19 | DI Mechanical Coupling (150mm Dia) | 4 | No |  |  |
| 20 | DI Mechanical Coupling (200mm Dia) | 4 | No |  |  |
| 21 | DI Mechanical Coupling (75mm Dia) | 13 | No |  |  |
| 22 | DI reducer (200/75mm) | 1 | No |  |  |
| 23 | DI Sluice Valve (100mm Dia) | 4 | No |  |  |
| 24 | DI Sluice Valve (150mm Dia) | 1 | No |  |  |
| 25 | DI Sluice Valve (200mm Dia) | 1 | No |  |  |
| 26 | DI Unequale Tee (200/100mm) | 1 | No |  |  |
| 27 | DI Unequale Tee (200/75mm) | 1 | No |  |  |
| 28 | Gate Valve (75mm) | 6 | No |  |  |
| 29 | DI/HDPE Mechanical Coupling (100mm) | 1 | No |  |  |
| 30 | DI/HDPE Mechanical Coupling (75mm) | 5 | No |  |  |
| 31 | DI Double Flange Bend 90degree (75mm) | 6 | No |  |  |
| 32 | DI Reducer (100/75mm) | 2 | No |  |  |
| 33 | GI Equal Tee (40mm dia) | 5.00 | No |  |  |
| 34 | GI Equal Tee (50mm dia) | 2.00 | No |  |  |
| 35 | GI Equal Tee (75mm dia) | 2.00 | No |  |  |
| 36 | HDPE/GI Flange (110/100mm dia) | 6.00 | No |  |  |
| 37 | HDPE/GI Flange (125/110mm dia) | 6.00 | No |  |  |
| 38 | HDPE/GI Flange (140/125mm dia) | 5.00 | No |  |  |
| 39 | HDPE/GI Flange (50/40mm dia) | 47.00 | No |  |  |
| 40 | HDPE/GI Flange (63/50mm dia) | 3.00 | No |  |  |
| 41 | HDPE/GI Flange (75/63mm dia) | 8.00 | No |  |  |
| 42 | HDPE/GI Flange (90/75mm dia) | 3.00 | No |  |  |
| 43 | 100mm Long GI Nipple (100mm dia) | 18.00 | No |  |  |
| 44 | 100mm Long GI Nipple (125mm dia) | 8.00 | No |  |  |
| 45 | 100mm Long GI Nipple (40mm dia) | 81.00 | No |  |  |
| 46 | 100mm Long GI Nipple (50mm dia) | 6.00 | No |  |  |
| 47 | 100mm Long GI Nipple (65mm dia) | 12.00 | No |  |  |
| 48 | 100mm Long GI Nipple (80mm dia) | 5.00 | No |  |  |
| 49 | HDPE reducer (110mm to down size) | 1.00 | No |  |  |
| 50 | HDPE reducer (110mm to down size) | 1.00 | No |  |  |
| 51 | HDPE reducer (125mm to down size) | 2.00 | No |  |  |
| 52 | HDPE reducer (125mm to down size) | 1.00 | No |  |  |
| 53 | HDPE reducer (140mm to down size) | 3.00 | No |  |  |
| 54 | HDPE reducer (160mm to down size) | 2.00 | No |  |  |
| 55 | HDPE reducer (63mm to down size) | 1.00 | No |  |  |
| 56 | HDPE reducer (75mm to down size) | 3.00 | No |  |  |
| 57 | GI Unequal Tee (100x100x40mm) | 5.00 | No |  |  |
| 58 | GI Unequal Tee (100x100x50mm) | 1.00 | No |  |  |
| 59 | Equal Tee (125mm) | 1.00 | No |  |  |
| 60 | GI Unequal Tee (125x125x75mm) | 1.00 | No |  |  |
| 61 | GI Equal Tee (40mm) | 8.00 | No |  |  |
| 62 | GI Unequal Tee (65x65x40mm) | 2.00 | No |  |  |
| 63 | GI Unequal Tee (65x65x50mm) | 2.00 | No |  |  |
| 64 | GI Unequal Tee (75x75x40mm) | 1.00 | No |  |  |
| 65 | Bronze Valve (100mm) | 6.00 | No |  |  |
| 66 | Bronze Valve (125mm) | 3.00 | No |  |  |
| 67 | Bronze Valve (40mm) | 34.00 | No |  |  |
| 68 | Bronze Valve (50mm) | 3.00 | No |  |  |
| 69 | Bronze Valve (63mm) | 4.00 | No |  |  |
| 70 | Bronze Valve (75mm) | 2.00 | No |  |  |
| III. ELECTROMECHANICAL WORKS |  |  |  |  |  |
| 1 | Submersible pump (including 1 spare pump) complete with riser pipe, control panel, electrical work required for the operation of the system; (Conforming ISO 9001) for 11 lps discharge and 175m total pumping head including 30m for the operation of Pressure Filter | 1 | Job |  |  |
| 2 | Centrifugal pump (including 1 spare pump) complete with control panel, electrical work and accessories required for the operation of the system; (Conforming ISO 9001) for 5 lps discharge 55m total pumping head | 1 | Job |  |  |
| 3 | Centrifugal pump (including 1 spare pump) complete with control panel, electrical work and accessories required for the operation of the system; (Conforming ISO 9001) for 3 lps discharge 125m total pumping head | 1 | Job |  |  |
| 4 | Supply, Installation and commissioning of Pole mounted 100 KVA Transformer, protection system and all accessories (Poles, Bracket etc.) including Line to Control Panels and Distribution Boards with following materials in coordination with the Nepal Electricity Authority with complete required accessories | 1 | Job |  |  |
| 5 | Spare parts, for above pump sets to be detailed by the manufacturer | 3 | sets |  |  |
| 6 | Supply, Installation and Commissioning of 100 KVA, 50Hz. Stand by diesel generator including reserve diesel tank of 1000 liter, MS steel platform for diesel tank erection, fire extinguisher (ABC powder type), all wiring necessary for running the pump | 1 | sets |  |  |
| 7 | Water Treatment System | 1 | PS |  |  |
| IV. Miscellaneous Works |  | 1 | PS |  |  |

|  | LIST 1: (LAB APPARATUS AND CONSUMABLES) | No |
| --- | --- | --- |
| L-1 | Thermometer | 3 |
| L-2 | Reversing thermometer | 1 |
| L-3 | Centrifuge | 1 |
| L-4 | Tintometer | 1 |
| L-5 | Underwater Photometer | 1 |
| L-6 | Nephelometer (Turbidimeter) | 1 |
| L-7 | Colorimeter | 1 |
| L-8 | Reflux flask | 1 |
| L-9 | Condenser (For measure of COD) | 1 |
| L-10 | Conductivitimeter | 1 |
| L-11 | pH meter with probes (Griffin pH meter) | 1 |
| L-12 | DO meter with probles and Electrical Stirrers | 1 |
| L-13 | Karl-Fischer Titrator | 1 |
| L-14 | Jar Test Appratus with Jar beakers (Min 6 nos.) | 1 |
| L-15 | Colony Counter | 1 |
| L-16 | Dry Hot Air Oven or Vaccum Oven | 1 |
| L-17 | Desiccator | 1 |
| L-18 | Chemical Weighing Balance | 1 |
| L-19 | Hot water bath | 1 |
| L-20 | Standard color charts | 3 |
| L-21 | Colored glass discs | 2 |
| L-22 | Bunsen Burner | 2 |
| L-23 | Tripod (Burner) Stand | 2 |
|  | GLASSWARE |  |
| G-1 | Nesslers' tubes (Diff capacities) | LS |
| G-2 | Sample Glass Tubes (Diff. Capacities) | LS |
| G-3 | Beakers (Diff. Capacities) | LS |
| G-4 | Bottles (Diff. Capacities) | LS |
| G-5 | Glass rods (Diff. Capacities) | LS |
| G-6 | Conical Flasks (Diff. Capacities) | LS |
| G-7 | Volumetric Flasks (Diff. Capacities) | LS |
| G-8 | Funnels | 3 |
| G-9 | Pipettes (Diff. Capacities) | LS |
| G-10 | Stands | 3 |
| G-11 | Filter Papers (Watman) | LS |
| G-12 | Membrane filter circle (paper) | LS |
| G-13 | Filter paper holders | 3 |
| G-14 | Evaporating Dish | 3 |
| G-15 | Titrimeter (narro neck calibrated tube well fitted with glass stopper) for Winkler Titration | 2 |
|  | CHEMICALS AND REAGENTS |  |
| C-1 | Distilled Water |  |
| C-2 | Deionized water |  |
| C-3 | Litmus papers |  |
| C-4 | EDTA Solutions |  |
| C-5 | EBT indicators |  |
| C-6 | Starch Indicators |  |
| C-7 | Buffer solutions |  |
| C-8 | Various Chemical reagents |  |