Item for application of STEP

| Item |  | Superiority |  |  |  | Remark |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Economy | Safety | Construction | Quality |  |
| Survey | TSP | X | X | X | X | Ahead |
|  | Tunnel Tomography | X | X | X | X | Ahead |
|  | Tunnel NAVI | X | X | X | X | Ahead |
|  | S-BEAT | X | X | X | X | Ahead |
| Construction | TH-Pipe roof |  | X | X | X | Road Subsidence |
|  | AGF-Tk | X | X | X | X | Excavation for weak ground |
|  | Drill-Jumbo | X | X | X | X | Excavation |
|  | Large Section Excavation Machine |  |  | X | X | Excavation |
| Material | AGSR |  | X |  | X | AGF |
|  | Pipe for AGF |  |  | X | X | AGF |
|  | Tunnel LED | X |  | X | X | Lighting |
|  | Steel Door |  | X |  | X | Safety |
|  | Waterproof Sheet | X |  | X | X | Waterproof |

トンネル前方探査

TSP：TSP(Tunnel Seismic Prediction)トンネル切羽前方探査, Hazama, エフティーエス

Tunnel Tomography： GPSを利用したトモグラフィー探査, 鹿島

Tunnel NAVI：ジャンボの油圧計測などからの前方地質予測, 大林, 古川, 演算工房

S-BEAT：反射法弾性波探査の応用, 清水

幹線道路地盤沈下

TH-Pipe roof：施工精度や曲がり測定・修正機能などにより規定, THパイプルーフ技術協会

軟弱地山掘削（切羽保持・地山変異の防止）

AGF-Tk：端末管事前撤去型（地山の変異低減）、無（小）拡幅型（地山の変異低減・施工性の向上）などにより規定, カテックス

トンネル掘削

Drill-Jumbo：コンピュータ制御、Tunnel NAVI、ドリフタやデュアルダンパーの性能で規定

Large Section Excavation Machine：自由断面掘削機、全断面掘削、自由掘削システム、出力保持などで規定, カヤバシステムマシナリー㈱

AGF

AGSR(シリカレンジ系注入材)：小逸走性薬液、ホモゲル強度などで規定, フジモリ産業

長尺パイプ：端末管撤去タイプ：フジモリ産業

トンネル照明

Tunnel LED：調光タイプ、長寿命、バックアップタイムなどで規定, パナソニック、岩崎電気、星和電気

安全防災

Steel Door：NEXCO仕様、車＆人用の混合タイプ, 三和シャッターなど。

トンネル裏面防水

Waterproof Sheet：不織布と一体になったもの。材質（EVA）、厚み(8mm)などで規定。